3GPP TSG-RAN WG2 Meeting #127bis	R2-2408918

# Hefei, China, 14th – 18th October 2024

Agenda Item:	8.7.4.2

Source:	Ericsson

Title:	DSR enhancements

Document for:	Discussion, Decision

# 1	Introduction

At RAN#105 the XR WID [1] for Rel-19 was updated with the following objective on the scheduling enhancements related to DSR:

-	Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]:

-	Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

In this contribution we continue the discussion on the DSR enhancements.

# 2 Discussion on DSR enhancements

In RAN2#127 it was agreed that DSR will be enhanced to report multiple pairs of remaining time and buffer sizes in Rel19 based on multiple remaining time thresholds.

What remains to be decided is the definition of delay critical data in the DSR, inclusion of non delay critical data, how the multiple pairs of information are selected, the triggering of the new DSR and if any further information should be included. This paper will go into details of these aspects.

## 2.1 Delay critical data in DSR

For this discussion of what is delay critical data it is worth to first take a step back and look at what the purpose is of the DSR. The Rel18 solution provides a mean to more precisely inform the network of the remaining time of the data in the UE buffer to enable the scheduler to take better scheduling decisions. This was shown to increase the XR capacity in simulations during the Rel18 SI and was the basis for the introduction of the DSR feature. Generally to utilize delay information in scheduling well it is beneficial to receive this information early as the scheduling of the data can be better planned, i.e. it is used in the delay scheduling process and is delay critical data. However at what time the scheduler can take advantage of the information is up to scheduler implementation and thus it is also up to network to configure the threshold accordingly. Basically, if the scheduler sees a use of early reporting of the remaining time (i.e. long before the deadline) it will set at high trigger threshold of the DSR. With Rel18 agreement, i.e. what is reported in the DSR is considered “delay critical”, it means that data can even be regarded as delay critical as soon as it arrives to the buffer, if the thresholds are set as such.

With multiple reporting thresholds being added in Rel19 the only difference is now that there are multiple levels of delay critical data, i.e. the data can have different time urgency. But any data that is reported (despite which threshold it belongs to) will still be useful for the delay based scheduling process and thus regarded as delay critical.

All data reported in a DSR is considered delay critical but can have different urgency.

## 2.2 New DSR format

There can be multiple remaining time thresholds configured in the DSR. This is network decision based on the usefulness of the granularity and margin of reporting.

Network configures the values of and number of thresholds used for each LCG.

Potentially not all the configured thresholds for each LCG will in all instances need to be reported, since some thresholds may not contain data. Thus, instead of reporting all thresholds the DSR format can indicate if the next thresholds contain something to be reported, i.e. for each LCG there is reporting pair for the first threshold and it is indicated if the next threshold for that LCG should be reported. The unused bit in Rel18 DSR format can be used for this indication (shown as “n” in the figure below). An example is shown in Figure 1.

Figure 1: Mapping of data from the LCGs, with different remaining times and buffer sizes, to a new DSR format using multiple thresholds and a n-bit indicating if the next threshold is being reported for the LCG.

A bit in the DSR format indicates if a following threshold for the LCG is reported.

Adopt the DSR format shown in Figure 1.

## 2.3 Non-delay critical data

For the scheduler to make a good decision it should have correct information about the buffer size associated with the remaining time. If that is not the case the scheduling decision may be inferior and leading to additional delays. A simple example of this is shown in Figure 2.

Figure 2: Issue with non-delay critical data transparent in the DSR.

It should be noted that it is up to UE implementation how to choose the data to be transmitted, but this selection in turn impacts the scheduling decision in the network. In the example above with a buffer including both delay critical data and non-delay critical data there can be two scenarios based on UE selection:

- If the UE select the delay critical data to transmit first then the network only needs to provide a grant of the delay critical data size to accommodate all the delay critical data.

- If the UE select non-delay critical data to transmit first then the network need to provide a larger grant to accommodate also the delay critical data.

The UE decision should be reflected in the DSR as this provides the means for the network to take the correct scheduling decision. This can simply be done by multiplexing the buffer size of the data that will be transmitted before the delay critical data, as is shown in Figure 3. If the UE decision of transmission is reflected in the DSR then the network can take this into account when performing the scheduling.

The DSR provides the information of the size that needs to be scheduled to accommodate the delay critical data.

Figure 3: UE that select non-delay critical data to be transmitted before the delay critical data multiplex the size of the non-delay critical data into the buffer size of the delay critical data.

## 3.3 Triggering of the DSR

Multiple reporting thresholds allow for an easy way to extend the triggering mechanism by selecting which reporting thresholds trigger a DSR. In the Rel18 solution when data becomes delay critical, i.e. remaining time falls below the threshold, a DSR is triggered to report the delay critical data. With multiple reporting thresholds a simple solution is to make it configurable which of the reporting thresholds that are triggering a DSR, i.e. as soon as the remaining time of any data fall below the threshold a new DSR is triggered. In this way the network can decide when it deems it useful for the DSR to be triggered. An example of such configuration is shown in Figure 4.

Make it network configurable which remaining time threshold that will be used for triggering a DSR.

Figure 4: Example of DSR triggering based on one threshold (Threshold 1 = 26ms) and multiple reporting thresholds (Threshold 2 = 12ms).

## 3.4 Configuration of Rel-19 DSR

The UE needs to know the threshold of when to trigger a DSR and the reporting thresholds on which remaining time and related buffer sizes to be reported per LCG. In theory this could be fixed pre-specified values, but it is more flexible if it can be configured via RRC, as the feature should support multiple different services with different time requirements. It can be configured in the same IE as the Rel-18 DSR is configured, i.e. the MAC-CellGroupConfig IE.

Add per LCG a remaining time triggering threshold and multiple remaining time reporting thresholds per LCG in the IE MAC-CellGroupConfig.

The maximum number of remaining time reporting thresholds needs to be agreed upon. At a minimum at least 4 levels should be possible.

Discuss the maximum number of remaining time reporting thresholds.

# 4	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	All data reported in a DSR is considered delay critical but can have different urgency.

Proposal 2	Network configures the values of and number of thresholds used for each LCG.

Proposal 3	A bit in the DSR format indicates if a following threshold for the LCG is reported.

Proposal 4	Adopt the DSR format shown in Figure 1.

Proposal 5	The DSR provides the information of the size that needs to be scheduled to accommodate the delay critical data.

Proposal 6	Make it network configurable which remaining time threshold that will be used for triggering a DSR.

Proposal 7	Add per LCG a remaining time triggering threshold and multiple remaining time reporting thresholds per LCG in the IE MAC-CellGroupConfig.

Proposal 8	Discuss the maximum number of remaining time reporting thresholds.

# References

RP-241771 Revised WID: XR (eXtended Reality) for NR Phase 3, September 2024