3GPP RAN WG2 Meeting #127bis	R2-2408921

Hefei, China, October 14th – 18th 2024

Agenda Item:	8.1.2.2

Source:	InterDigital

Title:	Remaining details of applicable functionality reporting

Document for:	Discussion, Decision

# Introduction

A RAN2 objective for AI/ML for Air Interface Work Item [1] is to design a general framework for one-sided AI/ML, including the signaling and protocol aspects of Life Cycle Management (LCM) to enable functionality selection, activation, deactivation, switching and fallback. In RAN2#127 [1] the following procedure for reporting applicable functionalities was agreed:

Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - (Between “Step 3” and “Step 4”) UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration). Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. The applicable functionality is initially activated by receiving its configuration when it is provided in Step 5. FFS on initial activation of applicable functionality if configuration of supported functionality is agreed to be provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation.

This contribution discusses the above procedure for applicable functionality determination and reporting, specifically remaining details on the applicable functionality reporting procedure.

# Discussion

## AI/ML functionality deactivation

In RAN2#126 [2], the following was agreed regarding the activation/deactivation of a functionality:

Agreements

1	RAN2 will support functionality activation/deactivation after inference configuration.   FFS initial state of configuration and how activation/deactivation is achieved.   FFS what Deactivation refers to:  examples discussed: 1) fallback to legacy 2) switching, etc.

The general applicability reporting framework agreed in RAN2#127 [1] contains many FFS regarding the activation of an AI/ML functionality, including the initial state of a provided inference configuration (when not provided in Step 5) and the support for L1/L2 (de)activation. Input has been requested from RAN1 via an LS [3], and RAN2 should postpone further discussion on such aspects until a reply LS is received. However, UE behaviour upon functionality deactivation is not a part of this LS and may be further progressed by RAN2.

The two options captured in the above FFS include: 1) falling back to legacy; and 2) switching. It is assumed that “Falling back to legacy” means upon AI/ML functionality deactivation the UE will revert to legacy (i.e., non-AI/ML) operation. “Switching” AI/ML functionalities requires an alternative functionality to be both available and applicable, but what the UE is switching is unclear in the context of functionality-based LCM. For example, if the granularity of functionality is defined on a sub-used case level (e.g., temporal beam prediction vs. spatial beam prediction) then “switching” may refer to switching between sub-functionalities. However, functionality granularity is unclear and being discussed in RAN1 based on the recent LS [3].

Observation 1:	The definition of “switching” is unclear in the context of functionality-based LCM and may require further clarification by RAN1 on the granularity of an AI/ML functionality.

Until a response from RAN1 is received regarding the granularity of AI/ML functionality, and additional clarification is provided on what switching refers to in the context of functionality-based LCM, the default behaviour should therefore be that upon deactivation of an AI/ML functionality, the UE reverts back to legacy (i.e., non-AI/ML based) operation.

Proposal 1:	Upon AI/ML functionality deactivation, UE falls back to legacy (i.e., non-AI/ML) operation as baseline.

## Functionality non-applicability

A high-level framework for applicability reporting was agreed in RAN2#127 [1]. According to agreements, the UE may report applicable functionalities in the following scenarios:

- Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

- As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration).

The network may determine which functionalities are “non-applicable” implicitly (e.g., if a supported functionality is not included in the applicable functionality report) or explicitly (e.g., a UE may explicitly report which functionality(ies) are not applicable).

Having the ability to explicitly indicate an AI/ML functionality as “non-applicable” would be useful, for example, if a previously reported functionality changes from “applicable” to “non-applicable”. In this case the UE may simply report the particular AI/ML functionality which is no longer applicable as “non-applicable”. The alternative (i.e., implicit indication) requires the UE re-send the entire list of applicable functionalities, which may cause significant overhead especially if functionality applicability changes dynamically (e.g., due to changing UE or NW side additional conditions).

Observation 2:	Explicitly reporting an AI/ML functionality as “non-applicable” is useful upon change of applicability (e.g., a previously reported applicable functionality is no longer applicable).

To minimize reporting overhead in the case of applicable functionality change, a UE should therefore be able to explicitly report which functionality(ies) are “non-applicable”.

Proposal 2: 	A UE can explicitly report which AI/ML functionality(ies) are “non-applicable”.

A functionality may be non-applicable for many reasons outside of network control, such as UE speed, lack of model training at the UE etc. However, if a functionality is not applicable due to missing information from the network, this may be easily corrected (e.g., via a UE request/response procedure, or via proactive reporting etc.). As part of the “non-applicability” reporting, the UE should indicate which functionality is non-applicable and the reason for non-applicability (e.g., not trained, missing information, UE/NW side conditions etc.).

Proposal 3: 	A UE indicates the reason an AI/ML functionality is non-applicable.

## Neighbour cell functionality applicability reporting

The currently defined applicability reporting framework is designed to report the applicable functionalities within a serving cell. However, there are advantages if the UE is also able to report the applicable functionalities for neighboring cell(s) as well. For example, this may support network decision on which target cell to hand the UE over to (e.g., the network may wish to prioritize cells in which the UE may continue AI/ML operation) and may also reduce service interruption to AI/ML operation upon mobility.

Observation 3:	Extending applicable functionality reporting to include neighbour/target cell(s) can support continuity of AI/ML operation during mobility.

However, there are several issues which may need resolution before confirming that applicable functionality reporting can be extended to include neighbor/target cells. For example, how the UE receives and validates the NW-side additional conditions for a neighbor/target cell or whether the UAI framework can be used to report the functionality applicability other than the serving cell. Before discussing these issues, it is proposed that RAN2 conclude on the baseline functionality reporting procedure (i.e., until RAN1 response is received).

Proposal 4: 	Postpone applicable functionality reporting for neighbour/target cells until the baseline applicable functionality reporting framework is concluded.

# Conclusion

In this contribution the following observations and proposals are made regarding applicable AI/ML functionality determination and reporting:

Observation 1:	The definition of “switching” is unclear in the context of functionality-based LCM and may require further clarification by RAN1 on the granularity of an AI/ML functionality.

Observation 2:	Explicitly reporting an AI/ML functionality as “non-applicable” is useful upon change of applicability (e.g., a previously reported applicable functionality is no longer applicable).

Observation 3:	Extending applicable functionality reporting to include neighbour/target cell(s) can support continuity of AI/ML operation during mobility.

Proposal 1:	Upon AI/ML functionality deactivation, UE falls back to legacy (i.e., non-AI/ML) operation as baseline.

Proposal 2: 	A UE can explicitly report which AI/ML functionality(ies) are “non-applicable”.

Proposal 3: 	A UE indicates the reason an AI/ML functionality is non-applicable.

Proposal 4: 	Postpone applicable functionality reporting for neighbour/target cells until the baseline applicable functionality reporting framework is concluded.

# References

Draft-RAN2_127_Meeting_Report_v2

R2-2406202 - RAN2#126 chair notes

R2-2407848 LS on applicable functionality reporting for beam management UE-sided model - Intel