3GPP TSG-RAN WG2 Meeting #127bis	R2-2408923

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.1.3

Source:	InterDigital Inc.

Title:	Data Collection for Network-Sided Model Training for the beam management use case

Document for:	Discussion

# 1. Introduction

In RAN2-126bis and RAN2-127, several agreements were made regarding data collection for the training of a network sided model.

RAN2-126 agreements [1]:

Agreements for beam management

1. 	For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature. FFS how to handle case when single RRC message is not sufficient. FFS if there will be any further enhancement needed pending RAN1 agreement.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

3.	Enhance the immediate MDT framework to support periodical reporting. FFS whether and what event-based reporting is supported and FFS on network request reporting.

RAN2-127 agreements [2]:

Agreements

- As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE.  FFS if dynamic activation/deactivation is support.

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size.  This is across all use cases

- When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

- Measurements for data collection purposes and logging based can be controlled based on power state of the UE.  It is up to UE implementation how the UE determines power state.  FFS whether the UE stops autonomously or if it reports to the network.

- FFS whether AS buffer event based reporting is supported.  FFS if we send availability indication or full report if it is supported

- FFS on event-based data collection/logging

- On-demand request from the network is supported.   FFS details on signalling

Agreements

- The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. No standardized RRC segmentation procedure is needed (as for the logged MDT measurements)

- Data collection report will not be transmitted over SRB1.  FFS which SRB is used.

In this contribution, we address some of the open issues and propose a way forward.

# 2. Discussion

### 2.1 Periodic vs Event based logging

Logging L1 measurements periodically without any condition may result in very large logs within a short periodic of time. This is undesirable, as it has been agreed that the UE stops the logging when it reaches its buffer limit. Also, even if the UE can log large amount of measurements, most of the measurements may end up being redundant if they are logged without any conditions. Thus, having conditions/events that can be further checked to perform a logging at a particular logging period is essential.

Observation 1: Periodic logging of L1 measurements without any conditions may result in very large logs that may have redundant information.

This is also in line with the logged MDT framework, where the UE can be configured to do the logging periodically (without any other conditions) or based on events (still periodically but logging performed only if the UE is out of coverage or if camped cell’s signal level is below a threshold, which is referred to as eventL1).   However, it should be noted that the measurements being logged are L1 measurements, and thus the out of coverage or eventL1 will not be the most suitable events. The RRM measurement events (e.g., event Ax, Bx, etc.,) are also not suitable as they are designed for the sake of mobility. Input is required from RAN1 regarding the appropriate events for determining whether to perform logging or not.

Observation 2: The events for logging in logged MDT in IDLE/INACTIVE and events for RRM measurement reporting in CONNECTED are designed for mobility use case and thus not suitable for logging of L1 measurements for the training of a network side model.

Thus, we propose:

Proposal 1: Both periodic logging and event-based logging will be supported. RAN2 will wait for RAN1 progress regarding the events.

### 2.2 Reporting aspects

Periodic and event-based reporting is supported for RRM measurements in CONNECTED state while on demand reporting is supported for logged measurements (e.g., logged MDT, RLF reports, etc.,).

Periodic or event-based reporting are appropriate for the case of RRM, as mobility decisions must be made quickly to prevent high levels of interference (e.g., keep serving the UE with a weak cell while there are better neighbour cells) or prevent radio link failures (e.g., UE leaving the coverage area of the serving cell before getting the handover command). However, for the sake of data collection for the training of a network side model, there is no such latency requirements as the training is to be done offline, and thus on demand reporting, as currently used for logged measurements is sufficient.

Observation 3: There are no strict latency requirements for the collection of data for the offline training of a network-sided model, and thus periodic or event-based reporting are not necessary.

Thus, we propose:

Proposal 2: Only on-demand reporting of the logged data will be specified.

One open issue related to logging is whether the UE sends the logged measurements autonomously (e.g., on stopping the logging due to reaching the buffer or power/battery limitation, etc.,) or sends an indication. Like the reasoning above about periodic or event-based reporting, we also do not see a need for the UE triggering the sending of the logged measurements autonomously. A similar approach to the logged measurement reporting (e.g., for logged MDT, RLF reports, etc.,) is a suitable approach here as well, where the UE indicates the availability of a logged data, and the network may request the UE to send the data. This allows a more flexible approach for the network to request the logged data at the most opportune time (e.g., when there is less traffic/congestion).

Proposal 3: The UE indicates the availability of logged data and sends the logged data only on explicit (on-demand) request from the network.

The current mechanisms for the UE to indicate the availability of logged measurements (e.g., in RRC Reconfiguration/Setup/Resume/Re-establishment complete messages) can be easily reused to indicate the availability of logged data for the training of a network sided model.

Proposal 4: The baseline mechanism for indicating the availability of logged data will be the same as for legacy logged measurements (e.g., indication in RRC complete messages).

Currently, the network sends to the UE a UEInformationRequest message indicating the type of logged measurements that it wants the UE to send, and the UE sends the logged measurements using the UEInformationResponse response message. We believe this mechanism is sufficient also for the case of logged data for network side model training.

RAN2 has made an agreement that SRB1 will not be used for sending the logged measurements but the exact SRB is FFS. SRB2 is currently used to send a UEInformationResponse that contains logged measurements. If RAN2 agrees to use a different SRB than SRB2, then discussion is needed if the UEInformationResponse can be sent via this SRB.

Thus, we propose:

Proposal 5: The baseline for the on-demand reporting of logged data is using UEInformationRequest/Response. FFS if an SRB other than SRB2 can be used to send the UEInformationResponse.

Currently, the UE is allowed to release any logged measurement (e.g., RLF report, logged MDT report, etc.,) after 48 hours. If the UE is not sending the logged data for model training immediately (as proposed in proposals 3 and 4 above) then it is necessary to specify a similar maximum duration of time that the UE is mandated to keep the logged data. The exact time duration can be decided later in the normative phase once we get more idea about the expected size of the logged data (e.g., if the logged data is large then the time duration needs to be small).

In relation to the time duration for keeping the logged data, it is also important to discuss whether the UE keeps the logged data during transition back and forth between CONNECTED and IDLE/INACTIVE. Considering that the inactivity timer for deciding the transition to IDLE/INACTIVE is up to network implementation, and it can be very short (e.g. in few minutes or even seconds), it is reasonable to assume that the duration for keeping the log is expected to be longer than that. Thus, we propose to keep the same approach as for legacy logged measurements such as logged MDT and RLF reports, i.e., UE keeps the logged data across state transitions, at least until the time duration for keeping the logs has expired.

Proposal 6: The UE keeps the logged data that is not sent to the network for at least a specified time duration. The value of the time duration (e.g., 48 hours as in the case of logged MDT or a smaller/larger value) is FFS.

Proposal 7: The UE keeps the logged data that is not sent to the network across RRC state transitions.

# 4. Conclusion

In this contribution, we discussed the open issues regarding logging and reporting of data for the training of a network-sided model for the beam management use case, and the following observations and proposals were made:

Observation 1: Periodic logging of L1 measurements without any conditions may results in very large logs that may have redundant information.

Observation 2: The events for logging in logged MDT in IDLE/INACTIVE and events for RRM measurement reporting in CONNECTED are designed for mobility use case and thus not suitable for logging of L1 measurements for the training of a network side model.

Observation 3: There are no strict latency requirements for the collection of data for the offline training of a network-sided model, and thus periodic or event-based reporting are not necessary.

For the beam management use case with a network-sided model:

Proposal 1: Both periodic logging and event-based logging will be supported. RAN2 will wait for RAN1 progress regarding the events.

Proposal 2: Only on-demand reporting of the logged measurements will be specified.

Proposal 3: The UE indicates the availability of logged data and sends the logged data only on explicit (on-demand) request from the network.

Proposal 4: The baseline mechanism for indicating the availability of logged data will be the same as for legacy logged measurements (e.g., indication in RRC complete messages).

Proposal 5: The baseline for the on-demand reporting of logged data is using UEInformationRequest/Response mechanism. FFS if an SRB other than SRB2 can be used to send the UEInformationResponse.

Proposal 6: The UE keeps the logged data that is not sent to the network for at least a specified time duration. The value of the time duration (e.g., 48 hours as in the case of logged MDT or a smaller/longer value) is FFS.

Proposal 7: The UE keeps the logged data that is not sent to the network across RRC state transitions.

# 5. References

[1]	RAN2-126 Chairman Notes, Fukuoka, Japan, May 2024

[2]	RAN2-127 Chairman Notes, Maastricht, The Netherlands, August 2024