3GPP TSG-RAN WG2 Meeting #127bis	R2-2408928

Hefei, China, 14 – 18 October 2024

Agenda item:	8.6.4

Source:	Nokia

Title:	On conditional LTM

WID/SID:	NR_Mob_Ph4 - Release 19

Document for:	Discussion and Decision

# Introduction

In RAN #105, the Rel-19 WID on NR mobility enhancements Phase 4 was updated [1], where the objectives relevant to conditional LTM (CLTM) are described as:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1]

  - Specify UE evaluated conditions for triggering LTM

  - Aim to support conditional LTM including subsequent LTM

  - Limit specifying the conditional LTM to the scenario where the UE is in non-DC

  - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

As per the WID, discussions on CLTM should begin in this meeting. The primary motivation for CLTM is to introduce robustness to the LTM procedure where the UE may not be able to receive the cell-switch command at right radio condition which may lead to radio link failure. Compared to CHO, the conditional LTM is expected to bring the benefits of robustness along with the interruption time reduction benefits of LTM.

In this contribution, we discuss the baseline procedure for CLTM support and identify the related key issues.

# Discussion

We identify the following key issues to be discussed in RAN2 and describe our preferred way forward for each issue in the sections below.

Issue # 1: Conditional cell switch condition configuration

Issue # 2: Conditional cell switch condition evaluation

Issue # 3: Early synchronization

Issue # 4: Conditional cell switch execution

## Conditional cell switch condition configuration

For C-LTM, the procedures outlined in Rel-18 LTM, which are based on NW triggered cell switch command, would require enhancements to support UE-autonomous cell switch triggering. It is anticipated that the basic principles of CHO will be reused. This implies that the UE will be configured with one or more conditions for conditional execution for one or more candidate cells or beams. When these conditions are met, the UE is expected to autonomously execute a CLTM cell switch to the candidate cell where the condition is satisfied.

To achieve this objective, it is necessary to study both - the types of conditions and how these conditions will be configured to the UE. In LTM, RS/beam-level measurements are used for early DL and UL synchronization procedures, as well as for the selection of a TCI state provided in the cell switch command. Following similar principles, for event-triggered L1 measurement reporting, beam/RS level events have already been agreed upon. It is also expected that at least one of the early DL or UL synchronization procedures will need to be considered mandatory to clearly distinguish CLTM from CHO by the benefit of lower handover interruption time.

Based on these considerations, it would be natural to use the LTM events defined for event based L1 measurement also as conditions for CLTM. For example, conditions similar to event LTM3 (defined for event-triggered L1 meas reporting), where the beam of the candidate cell becomes better than the beam of the serving cell by a certain offset, may be applicable to conditional LTM.

Observation 1: In LTM, RS/beam-level measurements are used for early DL and UL synchronization, for the selection of a TCI state in the cell switch command, and for event-triggered L1 measurement reporting.

Observation 2: It is also expected that at least one of early DL or UL synchronization procedures will be mandatory to differentiate CLTM from CHO.

Proposal 1: LTM events defined for event-based L1 measurement reporting are considered as conditions for C-LTM.

In Rel-18, for FR1, L3 measurements can be used to trigger LTM. This is considered for the cases where UE does not support L1 measurements for some candidate frequencies. L3 measurement condition can also be considered for C-LTM. Here the L3 measurement-events are mapped to LTM configurations instead of CHO configurations. Additionally, NW can configure early L3 measurement events in this case to allow NW to do early sync procedures before the L3 condition for C-LTM

Proposal 2: RAN2 discuss the support for L3 based conditions for C-LTM.

For the configuration of conditions, it is important to understand how it was done in CHO. Conditional mobility is a paradigm introduced in Rel. 16 to enhance mobility robustness. This has been done through enhancement of the RRC measurement framework. For this a short introduction of the RRC measurement framework is needed.

Figure 1: Measurement object, measurement ID and the report configuration and the relation to conditional configuration.

In RRC measurement framework there are three key elements.

- Measurement ID

- Measurement Object

- Reporting configuration.

The measurement ID links the measurement object to reporting configuration.

The measurement object indicates a frequency band to be measured. The measurement object can also include list of cells UE is allowed to measure and the list of cells UE is not allowed to measure. The measurement object is designed in a way that it also allows UE to detect cells that it was not configured to measure with specific SSB locations. UE can scan the band configured in measurement object to detect the SSBs of neighbouring cells.

The reporting configuration indicates how the UE can report the measurements it conducted using the measurement object. The reporting configuration may indicate the reporting type, where it can be periodic, aperiodic or event based. In case of event based, the reporting configuration also indicates the event type, e.g., A3 and the event parameters such as time to trigger and offset.

In Rel. 16, the reporting configuration was enhanced to include the conditional execution condition. The conditional execution condition is indicated as a reporting type. Under this reporting configuration also the execution condition and related parameters are indicated. So instead of triggering a report UE would trigger the execution of a conditional configuration. This is achieved through linking the measurement ID to a conditional configuration ID.

Once a cell that is measured through the measurement object, fulfils the execution condition, UE is expected to decode the conditional configuration that is linked to the measurement. The UE compares if the PCI of the cell that fulfils the execution condition is the same as the cell that fulfils the execution condition. If it is the same, then UE executes handover using the conditional configuration. More details on this can be read in Section 5.3.5.13.4 and 5, TS 38.331. After the execution condition is fulfilled from the UE side, the execution is the same as a non-conditional handover.

CLTM can follow a similar structure in the sense that

- The LTM-CSI-reporting configuration can be enhanced with the conditional execution condition.

  - Following the CHO and Rel. 18 LTM paradigm, this can be determined by the source DU for the initial conditional cell switch and by the candidate DUs for the subsequent conditional cell switches.

- LTM candidate configuration can be linked to a ReportConfigID that is configured with event-based reporting

  - The linking of report configuration with candidate configuration shall be done by source CU.

Observation 3: In Rel. 16 CHO, the reporting configuration was enhanced to include the conditional execution condition, where the conditional execution condition is indicated as a reporting type, and the execution condition and related parameters are indicated.

Proposal 3: LTM Candidate configuration is linked to Report-config-ID that is configured with conditional events.

Proposal 4: The DUs configure the conditional events in LTM reporting configuration.

Proposal 5: The CU can link the reporting configuration that indicates the execution condition to LTM candidate configuration.

## Conditional cell switch condition evaluation

In Rel-18 network energy saving feature, the conditional evaluation is linked to NES mode of the serving cell. In other words, the conditional evaluation for the events will be triggered only when serving cell is in NES mode. Similar triggering can be considered for CLTM, where criteria such as availability of timing advance and early downlink synchronization may be used. This way, LTM can benefit from the increased robustness of conditional cell switch execution without compromising on interruption time during cell switch.

Proposal 6: Network triggering of condition evaluation is supported. FFS options to achieve network control.

If the network wants to allow conditional LTM to be triggered towards the candidate cells for which early TA acquisition and TCI state activation is completed, the event definition need not include candidate resource set information. It can be implicitly formed on completion of the above steps.

Proposal 7: For CLTM, implicit triggering of condition evaluation for a candidate cell based on Early-TA acquisition and Early-TCI state activation is considered.

To allow the network to start the early TA and early TCI state activation at appropriate time prior to CLTM, the network may configure separate LTM events with event parameters for A3 slightly earlier than the events defined as condition for CLTM. Alternatively additional offset for early reporting for the condition can be used to indicate that DU to start the early sync related actions for CLTM.

Proposal 8: C-LTM event with additional offset can be used to trigger L1 measurement report prior to condition evaluation.

## Early synchronization

### RACH-based early TA acquisition

A UE configured with CLTM may perform a cell switch without receiving a cell switch command, when any of the evaluated conditional execution conditions for a candidate cell is met. Therefore, the existing mechanism of RACH-based TA information transfer to the UE in the cell switch command is not applicable to CLTM and a different approach is required, if RACH-less conditional LTM cell switches are to be enabled. The transmission of a random-access response (RAR) to provide TA information to the UE was considered both in Rel-18 and Rel-19 LTM discussions, but was ruled out for non-conditional intra- and inter-CU LTM and left as FFS for conditional LTM:

RAN2#126 agreement: RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM.

Given that, as explained above, the exact timing of a CLTM cell switch is not controlled by the network, to ensure that a TA value is available when the conditional execution condition is met, the estimated TA value must be provided to the UE soon after the random access (RA) preamble transmission via a RAR.

Proposal 9: For a UE configured with CLTM for a candidate cell, support transmission of RAR to the UE.

During Rel-18 LTM discussions, the topic of RAR transmission was extensively studied, with RAR transmission from either the source or the candidate cell being considered. Although RAR from the candidate cell has a shorter delay compared to RAR from source, where the estimated TA value must propagate from the candidate to the source cell via the F1 interface, its main drawback is that, depending on UE capability and cell frequency, it may lead to increased interruption time, since the UE must monitor the PDCCH of the candidate cell and may not be able to receive data from the source while monitoring for RAR. In addition, the RAR-from-source option is fully compatible with the Rel-18 early UL synchronization procedure and minimal changes are required to support the augmentation with a RAR, in contrast to the RAR-from-target option, where more modifications to the behavior of the UE, the source cell and the candidate cell are required.

Proposal 10: For a UE configured with CLTM for a candidate cell, support transmission of RAR from the source cell.

In Rel-18 discussions, multiple options for the container of the RAR were considered and before concluding that no RAR would be supported, the following alternatives were down-selected:

-	Alt1: MAC PDU (like RAR MAC PDU in 4 step RACH)

-	Alt2: MAC CE scheduled by PDCCH with CRC scrambled by C-RNTI

Alt1 has the smallest specification impact, but Alt2 has the advantage that it does not require the UE to monitor PDCCH in type 1 CSS. RAN2 should down-select one of the abovementioned alternatives.

Proposal 11: Down-select one of the following two alternatives for the RAR container and determine the RAR content accordingly:

-    Alt1: MAC PDU (like RAR MAC PDU in 4 step RACH)

-    Alt2: MAC CE scheduled by PDCCH with CRC scrambled by C-RNTI

In Rel. 18 LTM, the entity triggering the early TA acquisition procedure is the source DU, based on measurement reports from the UE. In CLTM, depending on the measurement reporting configuration, it may be useful to also enable triggering of the early TA acquisition procedure by the UE. To this end, an event can be configured to the UE and, when triggered, the UE may either perform RA preamble transmission or request a PDCCH order from the source DU. The latter option is preferred, as it allows for control of the timing of the use of CFRA resources by the network and, consequently, for CFRA preamble sharing among UEs.

Proposal 12: The UE may trigger the RACH-based early TA acquisition procedure by sending a request to the source DU based on an event configured by the network.

After a TA value has been obtained, it must be ensured that at the time of the conditional cell switch, the TA value is valid. The maintenance of the TA can be performed either at the source DU or at the UE. Maintenance at the source DU has the advantage of minimal specification impact. Nevertheless, in some cases, depending on the measurement reporting configuration, e.g., if the UE is not configured to report measurements to the source DU, it may not be possible for the source DU to detect significant change in the UE-candidate cell link which should trigger a refresh of the TA. Moreover, considering that the point of CLTM is for UE to autonomously trigger cell switch, it suffers the drawback that UE would not be aware at switching time whether the TA is valid or not. If the UE triggers cell switch with an expired TA, this would inadvertently lead to failure of the LTM execution procedure.  It would then make sense perform the maintenance of the TA at the UE.

Proposal 13: When CLTM is configured, the maintenance of the TA value should be performed at the UE.

For RACH-less access to the target cell, apart from the TA, the UE must be provided with an UL grant. In Rel. 18 LTM, both CG and DG were supported as they have their own benefits. Providing a CG may incur a large resource reservation overhead, especially when multiple CGs for multiple UEs are provided by the target cell, with the resources being reserved from RRC configuration to cell switch execution. Nevertheless, for non-conditional LTM, thanks to the network triggering of the cell switch and the transmission of cell switch notification, optimizations of the resource management on the network side are possible, as well as monitoring of the CG only after a cell switch has been decided. The lack of network control on cell switch in CLTM calls for alternative solutions to avoid monitoring of the CGs starting from RRC configuration and to enable more flexible resource management on the network side. To this end, as the RAR transmission may constitute the last signaling exchange between the UE and the source cell before a potential conditional cell switch, the RAR transmission and its preceding steps (RA preamble transmission, TA information transfer) may be used for the activation of a configured grant (CG). On the other hand, utilizing a DG mitigates issues related to increased resource reservation which were discussed earlier for CG. Moreover, this makes it easier for the NW and UE to be in sync with respect to UL grant activation and monitoring.

Proposal 14: Both CG and DG are supported. RAN2 to discuss CG and DG activation based on RACH-based early TA acquisition procedure.

### UE-based TA estimation

An alternative way for the UE to obtain the TA required for RACH-less cell switch is UE-based TA estimation. In Rel-18 LTM, the UE may obtain the UE-based TA estimate after configuration with UE-based TA estimation and no further signaling is required. It is up to the network to provide enough time to the UE to obtain the UE-based TA estimate before issuing a cell switch command. In CLTM, a conditional cell switch may be triggered before the UE has acquired a valid TA estimate, leading to a potentially unnecessary RACH-based cell switch with increased interruption time. Therefore, we should consider whether/how to ensure that the UE has a UE-based TA estimate when a conditional cell switch is triggered.

Proposal 15: When UE-based TA estimation and CLTM is configured, consider whether/how to ensure that a UE-based TA estimate is available when a conditional cell switch is triggered.

Furthermore, the issue of the first UL transmission of the UE in a RACH-less CLTM switch is even more pronounced, as the network cannot even exploit the RACH-based early UL synchronization procedure to anticipate the arrival of the UE at the candidate cell and the beam the UE is going to use. Hence, the network may be forced to monitor any provided CG on all beams. To mitigate the issue, the UE may inform the network about the acquisition of a UE-based TA estimate.

Proposal 16: When UE-based TA estimation and CLTM are configured, consider whether/how the UE informs the network about the status of UE-based TA estimation.

### TCI state activation

In non-conditional LTM, TCI state activation and indication decisions are made by the network based on measurement reports provided by the UE, with event-triggered L1 measurement reporting currently being specified in Rel-19 to reduce the excessive resource overhead of periodic reporting. In CLTM, due to the absence of a cell switch command, the network cannot indicate a TCI state to the UE at the time of cell switch. Nevertheless, the network can still use TCI state activation commands and anticipate the arrival of the UE at the target cell using any of the activated TCI states.

Proposal 17: Network-triggered TCI state activation (through a TCI state activation command) shall be supported for CLTM.

In addition, since cell switch triggering is left to the UE based on a network-configured event, it may be reasonable to also enable UE-based TCI state activation to further reduce the need for measurement reporting. This can be accomplished by configuring an autonomous TCI state activation event to the UE. A potential issue of such a configuration is that the network will not be aware of the autonomously activated TCI states. To resolve this issue, the UE may inform the source cell about the autonomously activated TCI states.

Proposal 18: Consider whether/how autonomous TCI state activation is supported for UEs configured with CLTM.

For RACH-less conditional cell switch, the source cell, having knowledge about the network-based and/or autonomously activated TCI states, can inform the candidate cell about the activated TCI states, so that the candidate can activate and monitor the corresponding CGs. If such information is not provided to the candidate cell, it must activate and monitor the CGs immediately after providing them in the RRC configuration.

Proposal 19: For a UE configured with CLTM, the source cell may inform the candidate cells about the activated TCI states.

## Conditional cell switch execution

As discussed above, allowing the UE to trigger a cell switch execution has significant impact on the actions that need to be taken before and after a conditional cell switch. An important consequence is the network’s uncertainty about the timing of the cell switch, which makes the preparation of the network for the UE’s access to the target cell more complicated. A possible way to overcome this challenge and simplify the procedures related to UE’s first access to the target is the transmission of a message from the UE to the source cell, informing the source cell that a conditional cell switch condition has been met. Upon receiving such a notification, the source cell can trigger procedures for UE’s access to the target cell as in Rel. 18 LTM, e.g., send a cell switch notification, or it can even decide whether a cell switch is allowed, in case more network control on the cell switch is desired, and inform the UE about it.

Nevertheless, such a message may never reach the source cell as the UE-source cell link quality may already be too low soon after the conditional cell switch condition is met. In addition, while the UE will be waiting for a response to the message, the link may further deteriorate potentially resulting in radio link failure. Therefore, we should further study if and whether this message should be transmitted and how long the UE should wait for a response.

Proposal 20: Study whether and under which conditions the UE should send a message to the network informing about the triggering of a conditional cell switch condition.

Another situation where it may be useful to delay the execution of the cell switch is when the conditional execution condition is met while the early TA acquisition procedure is in progress, e.g., while the UE is waiting to receive the RAR. In such a case, it may be beneficial to allow for completion of the early TA acquisition so that a RACH-less cell switch can be performed. On the other hand, like the above considerations for the notification message from the UE to the source, while the UE is waiting for the RAR, the UE-source cell link may deteriorate, the RAR may never be received by the UE and the UE may experience a radio link failure.

Proposal 21: If the conditional cell switch execution condition is met while early TA acquisition is in progress, allow for completion of the early TA acquisition procedure before triggering the cell switch. FFS details on when and for how long the cell switch can be delayed.

Furthermore, it is likely that conditional cell switch execution conditions for multiple cells are met. The actions that the UE has performed to prepare a cell switch to a candidate cell prior to the triggering of the condition may be different for each of the triggered candidates, e.g., the UE may have performed DL and UL synchronization with one of the triggered candidates, but only DL synchronization with another. To reduce cell switch interruption time, it is reasonable that the UE prioritizes execution to the former.

Proposal 22: When conditional cell switch execution conditions for multiple candidate cells have been triggered, the UE should trigger the cell switch to a candidate cell for which it has performed cell switch preparation actions resulting in reduced interruption time.

# Conclusions

This document has made the following observations:

Observation 1: In LTM, RS/beam-level measurements are used for early DL and UL synchronization, for the selection of a TCI state in the cell switch command, and for event-triggered L1 measurement reporting.

Observation 2: It is also expected that at least one of early DL or UL synchronization procedures will be mandatory to differentiate CLTM from CHO.

Observation 3: In Rel. 16 CHO, the reporting configuration was enhanced to include the conditional execution condition, where the conditional execution condition is indicated as a reporting type, and the execution condition and related parameters are indicated.

And proposed the following:

Proposal 1: LTM events defined for event-based L1 measurement reporting are considered as conditions for C-LTM.

Proposal 2: RAN2 discuss the support for L3 based conditions for C-LTM.

Proposal 3: LTM Candidate configuration is linked to Report-config-ID that is configured with conditional events.

Proposal 4: The DUs configure the conditional events in LTM reporting configuration.

Proposal 5: The CU can link the reporting configuration that indicates the execution condition to LTM candidate configuration.

Proposal 6: Network triggering of condition evaluation is supported. FFS options to achieve network control.

Proposal 7: For CLTM, implicit triggering of condition evaluation for a candidate cell based on Early-TA acquisition and Early-TCI state activation is considered.

Proposal 8: C-LTM event with additional offset can be used to trigger L1 measurement report prior to condition evaluation.

Proposal 9: For a UE configured with CLTM for a candidate cell, support transmission of RAR to the UE.

Proposal 10: For a UE configured with CLTM for a candidate cell, support transmission of RAR from the source cell.

Proposal 11: Down-select one of the following two alternatives for the RAR container and determine the RAR content accordingly:

-    Alt1: MAC PDU (like RAR MAC PDU in 4 step RACH) including candidate cell ID

-    Alt2: MAC CE scheduled by PDCCH with CRC scrambled by C-RNTI not including cell ID

Further RAR content FFS.

Proposal 12: The UE may trigger the RACH-based early TA acquisition procedure by sending a request to the source DU based on an event configured by the network.

Proposal 13: When CLTM is configured, the maintenance of the TA value should be performed at the UE.

Proposal 14: RAN2 to discuss CG activation based on RACH-based early TA acquisition procedure.

Proposal 15: When UE-based TA estimation and CLTM is configured, consider whether/how to ensure that a UE-based TA estimate is available when a conditional cell switch is triggered.

Proposal 16: When UE-based TA estimation and CLTM are configured, consider whether/how the UE informs the network about the status of UE-based TA estimation.

Proposal 17: Network-triggered TCI state activation (through a TCI state activation command) shall be supported for CLTM.

Proposal 18: Consider whether/how autonomous TCI state activation is supported for UEs configured with CLTM.

Proposal 19: For a UE configured with CLTM, the source cell may inform the candidate cells about the activated TCI states.

Proposal 20: Study whether and under which conditions the UE should send a message to the network informing about the triggering of a conditional cell switch condition.

Proposal 21: If the conditional cell switch execution condition is met while early TA acquisition is in progress, allow for completion of the early TA acquisition procedure before triggering the cell switch. FFS details on when and for how long the cell switch can be delayed.

Proposal 22: When conditional cell switch execution conditions for multiple candidate cells have been triggered, the UE should trigger the cell switch to a candidate cell for which it has performed cell switch preparation actions resulting in reduced interruption time.

# Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Sep 2024