3GPP TSG-RAN WG2 Meeting #127bis	R2-2408929

Hefei, China, 14 – 18 October 2024

Agenda item:	8.3.2

Source:	Nokia

Title:	RRM Measurement Prediction Evaluation Results

WID/SID:	FS_NR_AIML_Mob - Release 19

Document for:	Discussion and Decision

# 1	Introduction

RRM measurement prediction has several sub-use-cases based on the objective and the input and outputs. In RAN#125bis the following cases were agreed:

- Case 1: To predict beam level results, then generate cell level results based on the predicted beam results

- Case 2: To directly predict cell level results based on cell level results.

- Case 3: To directly predict cell level results based on beam level results

The two agreed objectives are:

- Objective 1: Measurement reduction. Achieve the same mobility performance, but with reduced DL radio measurements that the UE needs to take and report, which improves UE energy efficiency and reduces the required signalling.

- Objective 2: Mobility optimization. Predict future UE DL radio measurements and use them to optimize mobility decisions.

Sub-use-case prioritization (RAN#126):

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

The contribution presents our evaluation results for the prioritized sub use-cases together with a preliminary discussion on the system level simulation aspects.

# 2	Discussion

## 2.1	Discussion on Measurement Error Modeling

The measurements taken of the channel always contain a certain amount of error (TS 38.133, Clause 10), which should be taken into account in the measurements used as inputs to the AI/ML models and, in most cases at least, in the ground truth used to train the AI/ML models. The impact of the measurement error should be considered in the evaluation.

Observation 1: The measurement error influences both the inputs to the RRM measurement prediction models and the ground truth used to train them. Consequently, we model the measurement error with a Gaussian distribution with 1.92dB standard deviation, which is applied to measurements used as model input and as ground truth.

Proposal 1: Companies should document the measurement error that is modeled in their simulations.

## 2.2	Intra-Cell Time Domain Prediction Aspects

### 2.2.1 	FR2 to FR2 intra-frequency temporal domain case A

In RRM temporal prediction case A, as illustrated in Figure 2.2-1, we assume no measurement reduction within the observation window. This distinguishes it from RRM temporal prediction case B ([POST127][030][AI mobility]). To address this requirement, time series sequence-to-sequence prediction models are necessary. For evaluation purposes, we present results using various regression models and window sizes for RRM temporal prediction case A, categorized into the following sub-cases.

Figure 2.2-1: RRM temporal domain prediction – case A

Our simulations assumptions are highlighted in Table 2.2.1-1.

Table 2.2.1-1: Simulation parameters for the 7-site scenario based on TR38.843 (FR2)

| Reported simulation assumptions | UE trajectory option | Option 3 |  |
| --- | --- | --- | --- |
|  | UE trajectory boundary processing option | Option 3 |  |
|  | UE speed | 30/60/120 km/h |  |
|  | SoftLOS | Yes |  |
|  | Spatial consistency option | Option B |  |
|  | Number of TX beams | 32 |  |
|  | Number of RX beams | 4 |  |
|  | Measurement reduction rate | N/A |  |
| Data Size (Number of Samples) | Training | 1.04G |  |
|  | Testing | 0.26G |  |
|  |  | Case 1 | Case 2 |
| AI/ML model input/output | Model input | L1 beam RSRP measurements | L3 filtered serving cell RSRP measurements |
|  | Model output | L1 beam RSRP measurements | L3 filtered serving cell RSRP measurements |
| AI/ML model description | Model type | Linear regression, Random forest, Gradient Boosting, CNN | Linear regression, Random forest, Gradient Boosting, CNN, LSTM |
|  | Model complexity in number of parameters | Linear regression, Random forest, Gradient Boosting: N/A CNN: ~300k | Linear regression, Random forest, Gradient Boosting: N/A CNN: 3544 LSTM: 10502 |

Let us first present the results for Case 1, i.e., predicting beam level results, then generating cell level results based on the predicted beam results:

Table 2.2.1-2: Evaluation results for RRM temporal prediction Case A, case 1 – Mean absolute error (MAE) for different window settings and AI/ML methods, UE speed 30km/h

| Observation window & Prediction window | 40ms | 80ms | 100ms | 200ms |
| --- | --- | --- | --- | --- |
| Random forest | 1.2709 | 1.3613 | 1.4147 | 1.6216 |
| Linear regression | 1.2989 | 1.4377 | 1.3866 | 1.6316 |
| CNN | 1.8572 | 2.0349 | 1.4449 | 1.7229 |

Table 2.2.1-3: Evaluation results for RRM temporal prediction Case A, case 1 – Mean absolute error (MAE) for different window settings and AI/ML methods, UE speed 60km/h

| Observation window & Prediction window | 40ms | 80ms | 100ms | 200ms |
| --- | --- | --- | --- | --- |
| Random forest | 1.2087 | 1.3530 | 1.4282 | 1.7538 |
| Linear regression | 1.2209 | 1.3677 | 1.4431 | 1.7672 |
| CNN | 1.2529 | 1.8599 | 1.8172 | 1.9311 |

Table 2.2.1-4: Evaluation results for RRM temporal prediction Case A, case 1 – Mean absolute error (MAE) for different window settings and AI/ML methods, UE speed 120km/h

| Observation window & Prediction window | 40ms | 80ms | 100ms | 200ms |
| --- | --- | --- | --- | --- |
| Random forest | 1.2959 | 1.5517 | 1.6762 | 2.1602 |
| Linear regression | 1.3318 | 1.6 | 1.7286 | 2.2365 |
| CNN | 1.36667 | 2.1786 | 1.7953 | 2.3449 |

We can now proceed to presenting the mean absolute error (MAE) evaluation results for case 2, comparing the prediction performance of various AI/ML models against the ground truth. This case uses cell-level measurements to directly predict cell-level measurements across different speed categories. In the following evaluations, we assume that the observation window and prediction window are equal.

Table 2.2.1-5: Evaluation results for RRM temporal prediction Case A, case 2 – Mean absolute error (MAE) for different window settings and AI/ML methods, UE speed 30km/h

| Observation window & Prediction window | 40ms | 80ms | 100ms | 200ms |
| --- | --- | --- | --- | --- |
| Random forest | 0.6470 | 0.8719 | 0.9423 | 1.1278 |
| Linear regression | 0.8992 | 1.1587 | 1.0454 | 1.3218 |
| Gradient boosting | 0.6077 | 0.8020 | 0.8577 | 1.0181 |
| CNN | 0.8223 | 1.3586 | 1.5047 | 2.0697 |
| LSTM | 0.8607 | 1.3320 | 1.4072 | 1.8266 |

Table 2.2.1-6: Evaluation results for RRM temporal prediction Case A, case 2 – Mean absolute error (MAE) for different window settings and AI/ML methods, UE speed 60km/h

| Observation window & Prediction window | 40ms | 80ms | 100ms | 200ms |
| --- | --- | --- | --- | --- |
| Random forest | 0.8343 | 1.1324 | 1.2442 | 1.5469 |
| Linear regression | 1.043 | 1.3901 | 1.3737 | 1.8779 |
| Gradient boosting | 0.8073 | 1.0544 | 1.1536 | 1.4471 |
| CNN | 1.0489 | 1.8194 | 2.0802 | 3.0026 |
| LSTM | 1.1118 | 1.7443 | 1.8865 | 2.5659 |

Table 2.2.1-7: Evaluation results for RRM temporal prediction Case A, case 2 – Mean absolute error (MAE) for different window settings and AI/ML methods, UE speed 120km/h

| Observation window & Prediction window | 40ms | 80ms | 100ms | 200ms |
| --- | --- | --- | --- | --- |
| Random forest | 1.1216 | 1.5638 | 1.7314 | 2.1903 |
| Linear regression | 1.3000 | 1.8209 | 1.9227 | 2.6911 |
| Gradient boosting | 1.1070 | 1.4759 | 1.6223 | 2.0750 |
| CNN | 1.4244 | 2.5351 | 2.9216 | 4.2788 |
| LSTM | 1.5087 | 2.3970 | 2.5843 | 3.5459 |

These evaluation results lead to the following observations.

Observation 2: A larger observation window and prediction window may result in a higher MAE in temporal domain RRM measurement prediction.

Observation 3:  In addition to window size, the impact of UE speed on MAE performance appears to be linear, meaning that higher UE speeds generally lead to higher MAE in temporal domain RRM measurement prediction.

### 2.2.2 	FR1 to FR1 intra-frequency temporal domain case B

This section explores the various approaches and evaluation results for RRM temporal domain prediction case B. As previously raised in the email discussion [1], there two distinct examples for case B. Example 1, depicted in Figure 2.2.2-1, utilizes a sequence-to-sequence prediction model to leverage past measurement samples and predict future measurement samples across multiple time steps. Unlike Case A, which employs per-sample prediction periodicity without measurement reduction, this scenario aligns the prediction periodicity with the combined length of the prediction window and observation window, enabling measurement reduction (e.g., 50% in Figure 2.2-2).

Figure 2.2.2-1: RRM temporal domain prediction – case B example 1

Example 2, depicted in Figure 2.2.2-2, utilizes a sequence of past measurement samples within the observation window but applies a sample-and-hold method to achieve the desired measurement reduction ratio. In the example depicted in the figure, a 50% reduction is achieved. In this case, the prediction results should always correspond to the subsequent measurement sample. While this example is more complex to implement, we begin our evaluation with it, considering that many companies perceive Example 1 as similar to Case A.

Figure 2.2.2-2: RRM temporal domain prediction – case B example 2

Before delving into the detailed technical discussion and evaluation results, we need to clarify how the L1/L3 filter should be applied for this example. Layer 3 filter applied to the beam consolidated measurements is defined in 3GPP TS 38.331 5.5.3.2 as:

Fn = (1 – a)*Fn-1 + a*Mn where Mn is the latest received measurement result from the physical layer; Fn is the updated filtered measurement result, that is used for evaluation of reporting criteria, for measurement reporting or for U2N Relay (re)selection evaluation; Fn-1 is the old filtered measurement result, where F0 is set to M1 when the first measurement result from the physical layer is received; and for MeasObjectNR, a = 1/2(ki/4), where ki is the filterCoefficient for the corresponding measurement quantity of the i:th QuantityConfigNR in quantityConfigNR-List, and i is indicated by quantityConfigIndex in MeasObjectNR; for other measurements, a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by the quantityConfig; for UTRA-FDD, a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by quantityConfigUTRA-FDD in the QuantityConfig;

In RAN2#127, the following was agreed.

3 In the definition of 3 RRM sub-cases, all cell level measurement result(s) refers to L3 filtered cell level measurement

In intra-frequency temporal domain case B evaluation, the L3 filtering should take into account the impact of the skipped measurement steps in the layer 3 filter implementation. This includes both the predicted or the measured time-steps and the input to the ML model in inference.

Observation 4: In intra-frequency temporal domain case B evaluation, the L3 filtering should take into account the impact of the skipped measurement steps in the layer 3 filter implementation.

This scenario focuses on RRM temporal prediction case B, evaluating the prediction performance of various AI/ML models using mean absolute error (MAE). We compare these model predictions against ground truth data, using cell-level measurements to directly predict cell-level measurements across different speed categories and observation window sizes. The prediction window is configured to output a single future sample, as agreed in [1]. The table of parameters are listed below.

Table 2.2.2-1: Simulation parameters for the 7-site scenario based on TR38.843 (FR1)

| Reported simulation assumptions | UE trajectory option | Option 3 |
| --- | --- | --- |
|  | UE trajectory boundary processing option | Option 3 |
|  | UE speed | 30/60/120 km/h |
|  | SoftLOS | Yes |
|  | Spatial consistency option | Option B |
|  | Number of TX beams | 4 |
|  | Number of RX beams | 1 |
|  | Measurement reduction rate | 50% |
| Data Size (Number of Samples) | Training | 109.92M |
|  | Testing | 27.5M |
| AI/ML model input/output | Model input | Reduced serving cell RSRP measurements (following temporal case B example 2) |
|  | Model output | L3 filtered serving cell RSRP measurements |
| AI/ML model description | Model type | Linear regression, Random forest, Gradient Boosting, CNN, LSTM |
|  | Model complexity in number of parameters | Linear regression, Random forest, Gradient Boosting: N/A CNN: 3544 LSTM: 10502 |

Our evaluation results are then presented in the sequel.

Table 2.2.2-1: Evaluation results for RRM temporal prediction Case B – Mean absolute error (MAE) for different window settings and AI/ML methods, UE speed 30km/h

| Observation window | 240ms | 320ms | 400ms | 800ms |
| --- | --- | --- | --- | --- |
| Random forest | 3.0210 | 3.0940 | 3.0893 | 3.1427 |
| Linear regression | 3.0208 | 3.1538 | 3.1353 | 3.2269 |
| Gradient boosting | 3.0898 | 3.1614 | 3.1518 | 3.1974 |
| CNN | 5.0326 | 5.1456 | 5.0868 | 5.3336 |
| LSTM | 6.6417 | 4.9039 | 4.2063 | 3.6716 |

Table 2.2.2-2: Evaluation results for RRM temporal prediction Case B – Mean absolute error (MAE) for different window settings and AI/ML methods, UE speed 60km/h

| Observation window | 240ms | 320ms | 400ms | 800ms |
| --- | --- | --- | --- | --- |
| Random forest | 3.8884 | 3.9144 | 3.9128 | 4.0037 |
| Linear regression | 3.8978 | 3.9730 | 3.9718 | 4.1206 |
| Gradient boosting | 3.9843 | 4.0132 | 4.0074 | 4.0849 |
| CNN | 5.6801 | 5.8031 | 5.8167 | 5.9056 |
| LSTM | 7.3156 | 5.6241 | 5.0648 | 4.4450 |

Table 2.2.2-3: Evaluation results for RRM temporal prediction Case B – Mean absolute error (MAE) for different window settings and AI/ML methods, UE speed 120km/h

| Observation window | 240ms | 320ms | 400ms | 800ms |
| --- | --- | --- | --- | --- |
| Random forest | 4.9964 | 4.9271 | 4.8860 | 5.0733 |
| Linear regression | 5.2616 | 5.0735 | 5.0211 | 6.1366 |
| Gradient boosting | 5.1366 | 5.0947 | 5.0478 | 5.1800 |
| CNN | 9.8172 | 9.8927 | 10.0028 | 9.6540 |
| LSTM | 14.6510 | 11.5974 | 10.0554 | 7.3447 |

To further evaluate the performance of ML in reducing measurement requirements, we present MAE results for the "sample and hold" baseline method. These results are for a 50% MRRT reduction and are presented for different speed categories.

Table 2.2.2-4: Evaluation results for sample and hold method for Case B – Mean absolute error (MAE) for different UE speed categories and different MRRT

| UE speed | 30km/h | 60km/h | 120km/h |
| --- | --- | --- | --- |
| 50% MRRT | 0.5884 | 0.7750 | 1.0232 |
| 75% MRRT | 1.6559 | 2.1405 | 2.7749 |

Observation 5: The prediction accuracy is dependent on the UE speed. The higher the UE speed, the lower the prediction accuracy.

Observation 6: A longer observation window tends to perform better.

Observation 7: In our evaluation sample and hold is performing slightly better than more complex ML approaches.

## 2.3	Inter-Cell, Inter-Frequency Measurement Prediction

A UE needs measurement gaps to measure frequencies (e.g. inter-frequency, inter-RAT) which cannot be measured while the UE is transmitting or receiving to/from the serving cell. The measurements gaps are configured by the serving cell as part of the measurement configuration that is provided to the UE by means of RRC Reconfiguration message. During the measurement gaps, the UE cannot receive/or send on downlink/uplink shared channels of the serving cell and as such measurement gaps can have a negative impact on the data rate of the UE. AI/ML can be used to reduce the number of costly inter-frequency measurements that a UE needs to perform.

With measurement data from two frequency layers, an ML model can be trained to learn the correlation between the radio measurements in the two layers and can predict layer A measurements based on layer B measurements or can predict if a UE connected to the macro layer B would be able to connect to a micro layer A, as shown in Figure 2.3-2. This may require a cluster approach, where measurement from more than one cell in layer B are used as input to the prediction. A time series of layer B measurements may also be used as input and additional other inputs may be considered. The model may predict the measurement for a selected cell or selected cell(s) in Layer A, or the measurement of the strongest cell in Layer A. In this inter-frequency use case, we start with Case 2, i.e., using cell-level measurements in ML input and output.

Figure 2.3-1: Inter-frequency measurement prediction model

Table 2.3-1: Simulation parameters for the 7-site scenario based on TR38.843

| Reported simulation assumptions | UE trajectory option | Option 3 |
| --- | --- | --- |
|  | UE trajectory boundary processing option | Option 3 |
|  | UE speed | 30/60/120 km/h |
|  | SoftLOS | Yes |
|  | Spatial consistency option | Option B |
|  | Number of TX beams | 4 |
|  | Number of RX beams | 1 |
|  | Measurement reduction rate | 50% |
| Data Size (Number of Samples) | Training | 700K |
|  | Testing | 177K |
| AI/ML model input/output | Model input | RSRP measurements of 2GHz beams/cells |
|  | Model output | RSRP measurements of 4GHz beams/cells |
| AI/ML model description | Model type | Linear regression/gated recurrent unit |
|  | Model complexity in number of parameters | Linear regression: 19 GRU: 753 |

To evaluate the above use case, we set up a simulation scenario as in Table 2.3-1 with two frequency layers (2 GHz and 4 GHz, FR1-FR1) and collected measurements from both frequencies to train a machine learning model. The input is a time series of the RSRP measurements of the strongest three cells in the first frequency (2 GHz) and the output is the RSRP of the co-located cell of the second frequency (4 GHz). This was compared to the baseline method of finding the average RSRP difference between the RSRPs of the two frequencies and adding that difference to the RSRP measurements of the higher frequency (mean shift baseline).

The mean absolute error between the predicted RSRPs and the ground truth is shown in Table 2.3-1, where we summarize the results of cases 1, 2, and 3 (L1 to L1, L3 to L3, L1 to L3) for three speed classes. We can already achieve reasonable prediction error using linear regression, although it’s not that much better than the baseline described above. Using the more complicated recurrent neural network model shows no significant gains over the simpler linear regression model, which highlights the importance of comparing to simpler models in order to determine whether the extra gains are worth the computational effort or not.

Proposal 2: RAN2 to discuss including the mean shift baseline to the evaluation results for inter-frequency measurement prediction.

Table 2.3-2: Evaluation results for inter-frequency measurement prediction

|  | Baseline |  |  | Recurrent Neural Network |  |  | Linear Regression |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | L1-L1 | L3-L3 | L1-L3 | L1-L1 | L3-L3 | L1-L3 | L1-L1 | L3-L3 | L1-L3 |
| 30 km/h | 2.190 | 2.103 | 2.180 | 2.30 | 2.43 | 2.14 | 2.120 | 2.059 | 2.043 |
| 60 km/h | 2.389 | 2.294 | 2.427 | 2.41 | 2.33 | 2.40 | 2.303 | 2.243 | 2.220 |
| 120 km/h | 3.647 | 3.555 | 3.692 | 4.12 | 4.21 | 4.17 | 3.626 | 3.565 | 3.548 |

Observation 8: Higher speed UEs have larger prediction errors.

Observation 9: No significant difference between the methods and the baseline mean shift accuracy.

The simple mean shift baseline approach performs already relatively well due to the co-located deployment in the agreed simulation scenario. Higher gains with ML compared to the baseline could be expected in more complex, non-colocated scenarios, where a simple mean shift would not perform as well.

Observation 10: Higher gains with ML compared to the baseline could be expected in more complex, non-colocated scenarios, where a simple mean shift would not perform as well.

Proposal 3: RAN2 to consider evaluation with a non-colocated scenario.

## 2.4	System-Level Evaluation

In the RRM measurement prediction use case, actions utilizing the predictions have been agreed only for objective 1, measurement reduction. Measurements predicted in the FR2 to FR2 intra-frequency temporal domain case A scenario may utilized in the indirect approaches of the measurement event and failure prediction use cases, but otherwise no specific actions for objective 2 (mobility performance optimization) have been agreed in the RRM measurement prediction use case. Therefore, our assumption is that in the scope of the RRM measurement use case, system-level evaluation is at least considered to evaluate the measurement reduction objective.

Observation 11: Measurements predicted in the intra-frequency temporal domain case A scenario may be utilized in the indirect approaches of the measurement event and failure prediction use cases.

Observation 12: No additional optimization actions for objective 2 (mobility optimization) have been agreed in RRM measurement prediction use case.

Proposal 4: RAN2 to clarify, if in addition to measurement event and failure prediction any further scenarios for objective 2 (mobility optimization) are still to be studied in the context of the RRM measurement prediction use case.

Proposal 5: In the scope of the RRM measurement use case, system-level evaluation is considered to evaluate at least the measurement reduction objective.

The impact measurement reduction may have on the mobility performance cannot be determined just based on the mean absolute error of the predicted measurements, because it depends also on the distribution of the error, the configuration of the mobility procedure, e.g., the measurement events, the network scenario and many other factors. Therefore, system level evaluation is required to study, which measurement reduction rates with which measurement reduction sub-use-cases have no or acceptably small impact on mobility metrics.

Observation 13: For the measurement reduction objective, system level evaluation is required to study, which sub use-cases and measurement reduction rates have an impact on mobility metrics.

The mobility metrics to consider, when evaluating the trade-off between measurement reduction and any mobility performance impact may include at least number of handovers, handover success rate, number of ping-pong or short stay handovers and/or total UE outage due to handovers or failures. While the selection of KPIs should be tailored to each sub-use case and the applied ML approach, we still recommend considering a set of common KPIs.

Proposal 6: RAN2 should to down-select from common mobility related KPIs discussed in TR 36.839 for the metrics that should be evaluated in each sub use-case:

- Priority metrics

  - HO performance: total number of HOs, number of Ping-Pongs or short stay HOs.

  - Failures: total number of HO failure, total number of RLF, handover success rate.

- Additional metrics

  - Total time of outage (handovers and failures), time of stay

  - Measurement performance: measurement accuracy (absolute and relative), measurement prediction error CDF

  - QoS related KPIs such as downlink/uplink throughput, latency.

Proposal 7: RAN2 to study, which measurement reduction rates with which measurement reduction sub-use-cases have no or acceptably small impact on mobility metrics.

## 2.5	FR2 to FR2 intra-frequency spatial domain

One of the sub-use-cases identified in 3GPP TR 38.774 study item on AI/ML for inter-cell mobility in NR is intra-cell measurement prediction, which is an extension of the TR 38.843 beam prediction use case to intra-cell mobility.

Figure 2.4-1: 3GPP TS 38.300 Measurement Model

## 2.4.1 Simulation setup

In RAN2#126, the following was agreed.

10 Methodology of Intra_F_C_S: Intra-frequency intra-cell spatial domain prediction is done by measuring sub set of configured SSB as input to the model to predict L3 cell level measurements for every instance of the same cell. It is only evaluted for FR2 intra-frequency scenario and RRM sub case 1 and 3. Several measurement reduction rates should be aligned among company without defining detail pattern. The detail rate values are FFS.

In the agreed Case 1, an ML model is trained to predict the skipped beam-level measurements (A or A1) and then the cell-level filtered measurement is derived with the TS 38.300 baseline method. In Case 3, either B or C is directly predicted from the subset of beam-level measurements (A or A1). In case the unfiltered cell level measurement B is predicted with ML, the baseline layer 3 filter is applied in a separate step to get the layer 3 cell measurement C.

Table 2.4.1-1 shows the simulation assumptions used in the evaluation of the different cases.

Table 2.4.1-1: Simulation parameters for the 7-site scenario based on TR38.843

| Reported simulation assumptions | UE trajectory option | Option 3 |
| --- | --- | --- |
|  | UE trajectory boundary processing option | Option 3 |
|  | UE speed | 30/60/120 km/h |
|  | SoftLOS | Yes |
|  | Spatial consistency option | Option B |
|  | Number of TX beams | 32 |
|  | Number of RX beams | 4 |
|  | Measurement reduction rate | 50% |
| Data Size (Number of Samples) | Training | 360K |
|  | Testing | 180K |
| AI/ML model input/output | Model input | Reduced set of filtered L1 measurements |
|  | Model output | Filtered L3 measurements |
| AI/ML model description | Model type | Random Forests |
|  | Model complexity in number of parameters | 0.3M |

## 2.4.2 Simulation results

Measurements are reduced by measuring only half of the beams in each beam sweep (i.e. 50% of measurements saved). To predict the L3 measurements from the reduced set of L1 measurements we use the following methods:

- A random forest that takes as an input the reduced set of measurements from the current and previous beam sweep and predicts the unfiltered L3 measurements.

- A non-ML baseline approach that estimates the unfiltered L3 measurements (labelled B in Figure 2.4-1) by taking the RSRP of the best beam among those that have been measured in the current and previous beam sweep (Take Max).

The reduced set of beams is selected such that we alternate between odd-indexed and even-indexed beams every beam sweep in an interleaved manner. The output of those two methods is then filtered to give the predicted filtered L3 measurements (labelled C in Figure 2.4-1). The mean absolute error (MAE) between the predicted and ground truth filtered L3 measurements (RSRPs, in dB) is shown in Table 2.4.2-1.

Clearly, the higher the speed of the UE, the larger the error of the predicted measurements. In addition, a simple approach (take max) where we use the RSRP of the best measured beam in the current and previous beam sweeps as an estimate of the actual best beam RSRP slightly outperforms the random forests approach. This is because interleaving the measurements means every beam is measured once in every two beam sweeps and hence gives an accurate estimate of the L3 measurements.

Table 2.4.2-1: Mean absolute error for three speed classes

|  | Random Forest | Take Max |
| --- | --- | --- |
| 30 km/h | 0.7753 | 0.5410 |
| 60 km/h | 0.7814 | 0.5662 |
| 120 km/h | 0.7985 | 0.6253 |

Observation 14: Taking the max measurement outperforms random forest, when the interleaved approach is used.

So far, the measured subset of beam is fixed and predetermined. However, in our evaluation we have observed that a dynamic selection of the optimal subset of measured beams that adapts to the changing radio conditions can perform better than a fixed subset. To evaluate this observation, we run the same simulation scenario as before, but where the subset of beams to measure dynamically changes based on the previous best beam and its most correlated beams. The predicted layer 3 measurements are derived from the RSRP of the best beam among those that are measured. Table 2.4.2-2 shows the results of this evaluation.

Table 2.4.2-2: Evaluation results (RSRP mean absolute error in dB) for the dynamic selection of the subset of beams to measure for each speed class and measurement saving rate (10%-90%)

|  | 10% | 30% | 50% | 70% | 90% |
| --- | --- | --- | --- | --- | --- |
| 30 km/h | 0.000264 | 0.002404 | 0.012578 | 0.06234 | 1.30469 |
| 60 km/h | 0.000439 | 0.003477 | 0.014861 | 0.07300 | 1.80343 |
| 120 km/h | 0.000587 | 0.005066 | 0.019094 | 0.08862 | 2.22447 |

Observation 15: Iterative selection/optimization of the set of measured beams provides significantly higher accuracy than any other approach without introducing complex models.

Proposal 8: RAN2 to study two sub-use-cases of spatial measurement reduction:

- Fixed selection of the measured beams

- Dynamic selection of the measured beams

# 3	Conclusion

This document has made the following observations:

Observation 1: The measurement error influences both the inputs to the RRM measurement prediction models and the ground truth used to train them. Consequently, we model the measurement error with a Gaussian distribution with 1.92dB standard deviation, which is applied to measurements used as model input and as ground truth.

Observation 2: A larger observation window and prediction window may result in a higher MAE in temporal domain RRM measurement prediction.

Observation 3:  In addition to window size, the impact of UE speed on MAE performance appears to be linear, meaning that higher UE speeds generally lead to higher MAE in temporal domain RRM measurement prediction.

Observation 4: In intra-frequency temporal domain case B evaluation, the L3 filtering should take into account the impact of the skipped measurement steps in the layer 3 filter implementation.

Observation 5: The prediction accuracy is dependent on the UE speed. The higher the UE speed, the lower the prediction accuracy.

Observation 6: A longer observation window tends to perform better.

Observation 7: In our evaluation sample and hold is performing slightly better than more complex ML approaches.

Observation 8: Higher speed UEs have larger prediction errors.

Observation 9: No significant difference between the methods and the baseline mean shift accuracy.

Observation 10: Higher gains with ML compared to the baseline could be expected in more complex, non-colocated scenarios, where a simple mean shift would not perform as well.

Observation 11: Measurements predicted in the intra-frequency temporal domain case A scenario may be utilized in the indirect approaches of the measurement event and failure prediction use cases.

Observation 12: No additional optimization actions for objective 2 (mobility optimization) have been agreed in RRM measurement prediction use case.

Observation 13: For the measurement reduction objective, system level evaluation is required to study, which sub use-cases and measurement reduction rates have an impact on mobility metrics.

Observation 14: Taking the max measurement outperforms random forest, when the interleaved approach is used.

Observation 15: Iterative selection/optimization of the set of measured beams provides significantly higher accuracy than any other approach without introducing complex models.

And proposed the following:

Proposal 1: Companies should document the measurement error that is modeled in their simulations.

Proposal 2: RAN2 to discuss including the mean shift baseline to the evaluation results for inter-frequency measurement prediction.

Proposal 3: RAN2 to consider evaluation with a non-colocated scenario.

Proposal 4: RAN2 to clarify, if in addition to measurement event and failure prediction any further scenarios for objective 2 (mobility optimization) are still to be studied in the context of the RRM measurement prediction use case.

Proposal 5: In the scope of the RRM measurement use case, system-level evaluation is considered to evaluate at least the measurement reduction objective.

Proposal 6: RAN2 should to down-select from common mobility related KPIs discussed in TR 36.839 for the metrics that should be evaluated in each sub use-case:

- Priority metrics

  - HO performance: total number of HOs, number of Ping-Pongs or short stay HOs.

  - Failures: total number of HO failure, total number of RLF, handover success rate.

- Additional metrics

  - Total time of outage (handovers and failures), time of stay

  - Measurement performance: measurement accuracy (absolute and relative), measurement prediction error CDF

  - QoS related KPIs such as downlink/uplink throughput, latency.

Proposal 7: RAN2 to study, which measurement reduction rates with which measurement reduction sub-use-cases have no or acceptably small impact on mobility metrics.

Proposal 8: RAN2 to study two sub-use-cases of spatial measurement reduction:

- Fixed selection of the measured beams

- Dynamic selection of the measured beams

# 4	References

[1] R2-2407849, Summary of [POST127][030][AI mobility] RRM simulation assumptions (OPPO), RAN2#127, Maastricht, Netherlands, August, 2024.