3GPP TSG-RAN WG2 Meeting #127bis	R2-2408930

Hefei, China, 14 – 18 October 2024

Agenda item:	8.3.3

Source:	Nokia

Title:	Discussion on Measurement Event Prediction

WID/SID:	FS_NR_AIML_Mob - Release 19

Document for:	Discussion and Decision

# Introduction

The following agreements were made for the measurement event prediction use case in RAN#125bis

Agreements:

- At least measurement event evaluation based on RRM measurement prediction result will be studied. Direct measurement event prediction is also allowed.

- Clarifications on what is being as input should be provided with results

- Start with A3 as a baseline

The contribution discusses aspects related to A3 measurement event prediction and inter-frequency measurement event prediction.

# 2	Discussion

## 2.1	A3 Measurement Event Prediction

In RAN2#125bis it was agreed to start by studying the use of AI/ML to predict A3 measurement events in time domain.

### 2.1.1   Discussion on the Use Case

Considering the identified objectives for the SI, the starting point for A3 measurement event prediction would be objective 2, i.e., mobility performance optimization. Further study is needed, for instance, on when the predictions are made and what actions are taken based on the predicted measurement events to optimize the mobility performance.

Observation 1: Predicted A3 measurement events need to be studied at least under the second objective of the SI, i.e., mobility performance optimization.

The network should be able to configure the measurement event predictions and how the UE is using them. When a UE predicts an A3 measurement event will occur, according to the configuration it may, for example:

- Report to the network that a measurement event is predicted to occur in a separate measurement event prediction report

- Report measurements based on a predicted measurement event

- Choose an alternative measurement event configuration, e.g., a shorter time-to-trigger

- Report predicted future measurements based on a predicted measurement event

The A3 measurement event predictions may configured by the network to be made by the UE, for example:

- Periodically

- Based on a configured measurement event condition being fulfilled

- Based on an A3 measurement event entry condition being fulfilled

Proposal 1: For the second objective, RAN2 to study at least the following:

- When the A3 measurement event predictions are made

- How the predicted measurement events are used to optimize the mobility performance.

### 2.1.2 Evaluation Aspects

To avoid lengthy discussions in 3GPP, the simulation assumptions and setup already agreed for the RRM measurement prediction and RLF prediction should also be reused for A3 measurement event prediction evaluation as much as possible. Specifically, to re-use the FR2-to-FR2 intra-frequency temporal domain case A scenario, assumption is to re-use the FR2 simulation assumptions for RRM measurement prediction evaluation.

Proposal 2: The A3 measurement event prediction evaluation re-uses the simulation assumptions for RRM measurement prediction and RLF prediction. The FR2-to-FR2 intra-frequency temporal domain case A scenario is taken as a starting point.

Proposal 3: For the A3 measurement event configuration, the following configuration may be taken as a starting point:

| Parameter | Value |
| --- | --- |
| Offset | 2 dB |
| Hysteresis | 1 dB |
| Time-to-Trigger | 160 ms |

In RAN2#127bis the following was agreed.

Agreements - Both direct and indirect are allowed. Companies should indicate what they used and what inputs they are using - Output for indirect: predicted SINR. Based on predicted SINR the time instance the RLF occurs can be determined without further AI/ML models. - Output for direct: probability of RLF within an window - Companies should report the prediction time window they have used in the simulations KPI For indirect - SINR difference, missed RLF detection, false RLF detection, F1 score, time difference of true time RLF and predicted RLF, true RLF prediction. Direct - missed RLF detection, false RLF detection, F1 score. - Whether the RLF will happen based on the following methodology. Above a probability threshold we assume that RLF will happen. This is then compared with true RLF.

Similar to the agreements for RLF prediction use-case, it would be helpful to agree on the baseline methodologies for both direct and indirect measurement event prediction use-cases. In our view, the following approaches can be used.

- For direct A3 event prediction, at least measurements are used as input to the AI/ML model (other inputs are not precluded). The output of the AI/ML model is the predicted occurrence of an A3 event within a window.

- For indirect A3 event prediction, at least measurements are used as input to the AI/ML model. As intermediate step, UE predicts measurements in the future (time domain prediction Case A) and utilizes these to derive at least the A3 event occurrence within a predicted time window (other inputs and outputs are not precluded).

Proposal 4: To align the understanding, RAN2 uses the following methodologies as starting point for discussion:

- For direct A3 event prediction, at least measurements are used as input to the AIML model (other inputs are not precluded). The output of the AI/ML model is the predicted occurrence of an A3 event within a window.

- For indirect A3 event prediction, at least measurements are used as input to the AI/ML model. As intermediate step, UE predicts measurements in the future (time domain prediction Case A), and utilizes these to derive at least the A3 event occurrence within a predicted time window (other inputs and outputs are not precluded).

Since the output of both the direct and indirect approaches are the predicted occurrence of the A3 event, the accuracy of the measurement event prediction ML models may be evaluated with typical ML classification metrics, such as precision, recall or F1-score metrics and confusion matrix.

Proposal 5: For both direct and indirect A3 event prediction, at least F1-score and confusion matrix are considered as evaluation metrics.

The required accuracy metrics and their impact on the network or UE performance depend on how the measurement event predictions are used, which determines, for example, the cost of false positive or false negative predictions. Furthermore, if system-level evaluation of actions taken based on predicted measurement events are studied, they may have further requirements for the simulation assumptions. The actions taken based on predicted A3 measurement events and the evaluation of the impact of these actions to the mobility metrics are FFS.

Observation 2: The required accuracy metrics and their impact on the network or UE performance depend on how the measurement event predictions are used, which determines the cost of false positive or false negative predictions.

## 2.2	Inter-Frequency Measurement Event Prediction

Figure 2.2-1: Frequency domain measurement event prediction

In the RRM measurement prediction use case, one of the studied scenarios is inter-frequency measurement prediction. Inter-frequency measurement prediction is used for study objective 1, i.e., measurement reduction. A potential approach how this may be achieved is frequency-domain measurement event prediction, shown in Figure 2.3-3. Based on measurements in frequency B, a machine learning model is predicting if a measurement event condition in frequency A is fulfilled.

This may, for example, use an indirect approach, where a machine learning model is trained to predict the RSRP of the strongest cell in set A based on measurements of cells or beams in set B, where the sets of cells A and B are of different frequency layers. In a second step, the predicted measurements may be used to determine, if the signal strength of the strongest cell in set A of cells satisfies a specified measurement event condition. In the direct prediction alternative, a machine learning model is trained to predict direct from set B measurements, if the strongest cell of set A satisfies the measurement event condition. The measurement event condition may be as in A4, A5 or B1 measurement events.

Observation 3: Inter-frequency measurement event prediction can utilize the inter-frequency prediction approach in the RRM measurement prediction use case. Alternatively, a direct approach may be studied.

The required inter-frequency measurements can be reduced by configuring the UE to take the inter-frequency measurements, including configuring the required measurement gaps, only when the inter-frequency measurement event condition is predicted to be fulfilled.

Observation 4: Inter-frequency measurement event prediction can be used for study objective 1, i.e., to reduce the number of inter-frequency measurements a UE needs to take.

Costly inter-frequency measurements can be saved by configuring the UE to measure them, including configuring the required measurement gaps, only when the inter-frequency measurement event prediction ML model predicts an inter-frequency measurement event based on intra-frequency measurements, indicating that a connection is likely possible. Since any connection or handover is determined based on actual measurements, the method is robust against AI/ML prediction inaccuracies. The inter-frequency measurement event prediction may reuse the evaluation assumptions of the inter-frequency RRM measurement prediction use case as a starting point.

Observation 5: Inter-frequency measurements can be saved by configuring the UE to measure them, including configuring the required measurement gaps, only when the inter-frequency measurement event prediction ML model predicts an inter-frequency measurement event.

Proposal 6: RAN2 to study inter-frequency measurement reduction based on inter-frequency measurement event prediction.

# 3	Conclusion

This document has made the following observations:

Observation 1: Predicted A3 measurement events need to be studied at least under the second objective of the SI, i.e., mobility performance optimization.

Observation 2: The required accuracy metrics and their impact on the network or UE performance depend on how the measurement event predictions are used, which determines the cost of false positive or false negative predictions.

Observation 3: Inter-frequency measurement event prediction can utilize the inter-frequency prediction approach in the RRM measurement prediction use case. Alternatively, a direct approach may be studied.

Observation 4: Inter-frequency measurement event prediction can be used for study objective 1, i.e., to reduce the number of inter-frequency measurements a UE needs to take.

Observation 5: Inter-frequency measurements can be saved by configuring the UE to measure them, including configuring the required measurement gaps, only when the inter-frequency measurement event prediction ML model predicts an inter-frequency measurement event.

And proposed the following:

Proposal 1: For the second objective, RAN2 to study at least the following:

- When the A3 measurement event predictions are made

- How the predicted measurement events are used to optimize the mobility performance.

Proposal 2: The A3 measurement event prediction evaluation re-uses the simulation assumptions for RRM measurement prediction and RLF prediction. The FR2-to-FR2 intra-frequency temporal domain case A scenario is taken as a starting point.

Proposal 3: For the A3 measurement event configuration, the following configuration may be taken as a starting point:

| Parameter | Value |
| --- | --- |
| Offset | 2 dB |
| Hysteresis | 1 dB |
| Time-to-Trigger | 160 ms |

Proposal 4: To align the understanding, RAN2 uses the following methodologies as starting point for discussion:

- For direct A3 event prediction, at least measurements are used as input to the AIML model (other inputs are not precluded). The output of the AI/ML model is the predicted occurrence of an A3 event within a window.

- For indirect A3 event prediction, at least measurements are used as input to the AI/ML model. As intermediate step, UE predicts measurements in the future (time domain prediction Case A), and utilizes these to derive at least the A3 event occurrence within a predicted time window (other inputs and outputs are not precluded).

Proposal 5: For both direct and indirect A3 event prediction, at least F1-score and confusion matrix are considered as evaluation metrics.

Proposal 6: RAN2 to study inter-frequency measurement reduction based on inter-frequency measurement event prediction.