3GPP TSG-RAN WG2 #127bis	R2-2408939

# Hefei, China, 14-18 October 2024

Agenda Item:	 7.2.1

Source:	Ericsson

Title:	draft LS reply on synchronization source change at the transmitting anchor UE in SL positioning

Document for:	Discussion

# 1	Introduction

RAN4 in LS “R4-2410352” is asking RAN2 whether RAN2/RAN1 has introduced or is working on any solutions to inform an SL UE performing an SL positioning measurement (e.g., SL RSTD, SL Rx-Tx, and SL RTOA) about synchronization reference source change at a UE which is transmitting SL-PRS for the measurement.

In this paper, we discuss and draft a reply LS to RAN4.

RAN1 has replied with below response.

- RAN1 has not introduced a solution to specifically inform the Rx UE about synchronization reference source change of the UE transmitting SL-PRS.

  - RAN1 has introduced a solution for a UE transmitting SL-PRS to inform a UE performing an SL positioning measurement about synchronization reference source.

- RAN1 does not intend to introduce any additional solutions for Rel-18 to inform a measuring UE of changes to synchronization source for a transmitting anchor UE.

- This does not preclude other RAN group to introduce such signalling.

# 2	Discussion

Similar as RAN1, RAN2 can mention that signaling exist whereby a UE can indicate the synch source that it is using. It is then up to UE implementation, if the UE sends the message again to reflect that the synch source is not the same. However, RAN2 does not want to specify any trigger or specific signaling for this case and leaves it to UE implementation.

#### –	SL-RTD-Info

The IE SL-RTD-Info provides time synchronization information of SL Anchor UEs.

-- ASN1START

-- TAG-SL-RTD-INFO-START

SL-RTD-Info ::= SEQUENCE ( SIZE (1.. maxNrOfUEs)) OF RTD-InfoListPerAnchorUE

RTD-InfoListPerAnchorUE ::= SEQUENCE {

applicationLayerID          OCTET STRING,

referenceRTD-Info           ReferenceRTD-Info                  OPTIONAL,

rtd-BetweenAnchorUEs        CHOICE {

subframeOffset              INTEGER (0..1966079),

sl-OffsetDFN                INTEGER (0..1000)

}                                                              OPTIONAL,

rtd-Quality                 SL-TimingQuality                   OPTIONAL,

syncSourceType              ENUMERATED { gnss, gNB-eNB, ue}    OPTIONAL

}

ReferenceRTD-Info ::= SEQUENCE {

syncSourceType        ENUMERATED { gnss, gNB-eNB, ue},

applicationLayerID    OCTET STRING                       OPTIONAL,

nrCell-Identify       SEQUENCE {

nr-PhysCellID             NR-PhysCellID              OPTIONAL,

nr-ARFCN                  ARFCN-ValueNR              OPTIONAL,

nr-CellGlobalID           NCGI                       OPTIONAL

}                                                        OPTIONAL

}

-- TAG-SL-RTD-INFO-STOP

-- ASN1STOP

SL-RTD-Info field descriptions

nrCell-Identify This field provides NR cell identity information. The field is present only if the synchronization source of a SL Anchor UE is gNB/eNB.

referenceRTD-Info This field defines the reference RTD and comprises the following sub-fields: - syncSourceType: This field specifies the synchronization source type. - applicationLayerID: This field provides the application layer ID of the reference UE if the syncSourceType is set to UE.

rtdBetweenAnchorUEs This field specifies the RTD between SL Anchor UEs: - subframeOffset: This field specifies the subframe boundary offset at the TRP antenna location between the reference UE and this neighbour UE in time units where Δfmax=480∙103 Hz and (TS 38.211 [6]). The offset is counted from the beginning of a subframe #0 of the reference UE to the beginning of the closest subsequent subframe of this neighbour UE. Scale factor 1 Tc. - sl-OffsetDFN: This field indicates the timing offset for the UE to determine DFN timing when GNSS is used for timing reference. Value 1 corresponds to 0.001 milliseconds, value 2 corresponds to 0.002 milliseconds, and so on.

rtd-Quality This field specifies the quality of the RTD.

By UE implementation, a UE may be able to inform that the synch source has changed. However, there is no trigger or specific signaling that has been designed to indicate change of synch source.

RAN2 to reply that “RAN2 has neither introduced nor working on any solutions to inform a UE performing an SL positioning measurement (e.g., SL RSTD, SL Rx-Tx, and SL RTOA) about synchronization reference source change at a UE which is transmitting SL-PRS for the measurement. However, it is possible for a UE to indicate the reference synch source that it is using “.

# 3	draft Reply

3GPP TSG-RAN WG2 Meeting #127 	  R2-24xxxxxx

Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Title:	[draft] reply LS on synchronization source change at the transmitting anchor UE in SL positioning

Release:	Rel-18

Work Item:	NR_pos_enh2-Core

Source:	3GPP TSG-RAN WG2

To:	3GPP TSG-RAN WG4, 3GPP TSG-RAN WG1

Contact Person:

Name:			Ritesh Shreevastav

E-mail Address:		ritesh.shreevastav@ericsson.com

Attachments:	None

- Overall Description:

RAN2 would like to thank RAN4 for the LS and below is the answer from RAN2 for the question asked by RAN4:

RAN2 has neither introduced nor working on any solutions to inform a UE performing an SL positioning measurement (e.g., SL RSTD, SL Rx-Tx, and SL RTOA) about synchronization reference source change at a UE which is transmitting SL-PRS for the measurement. However, it is possible for a UE to indicate the reference synch source that it is using.

2. Actions:

To RAN4:

ACTION: 	RAN2 kindly requests RAN4 to consider above answer for their Rel-18 specification work.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN WG2 Meeting #128                    Orlando, USA 		          November 18 – November 22, 2024

TSG-RAN WG2 Meeting #129                    Greece 			February 17 – February 21, 2025

# Conclusion

In the previous sections we made the following observations:

Observation 1	By UE implementation, a UE may be able to inform that the synch source has changed. However, there is no trigger or specific signaling that has been designed to indicate change of synch source.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to reply that “RAN2 has neither introduced nor working on any solutions to inform a UE performing an SL positioning measurement (e.g., SL RSTD, SL Rx-Tx, and SL RTOA) about synchronization reference source change at a UE which is transmitting SL-PRS for the measurement. However, it is possible for a UE to indicate the reference synch source that it is using “.

# References

R4-2410352