3GPP TSG RAN WG2 #127bis						                       R2-2408941

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.1.2.3

Source:	AT&T

Title:	LCM for Positioning use case

Document for:	Discussion/Approval

# Introduction

As part of Rel-18 Study Item on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1], 3GPP has agreed to study the framework for AI/ML for air-interface corresponding to target use cases considering aspects such as performance, complexity, and potential specification aspects. One of the identified use cases was for positioning which included the following representative sub-use cases:

- Direct AI/ML positioning:

  - AI/ML model output: UE location

  - e.g., fingerprinting based on channel observation as the input of AI/ML model

- AI/ML assisted positioning:

  - AI/ML model output: new measurement and/or enhancement of existing measurement

  - e.g., LOS/NLOS identification, timing and/or angle of measurement, likelihood of measurement

In RAN#102 as part of Rel-19 Work Item on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [2], it was agreed to start the work on specification support for the positioning accuracy use case. The work item was updated in RAN#105

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]:

  - Direct AI/ML positioning:

    - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

    - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

    - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

  - AI/ML assisted positioning

    - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

    - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

  - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any

  - Investigate and specify the necessary signalling of necessary measurement enhancements (if any)

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

  - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

In this contribution, we will discuss the specification aspects for LCM of positioning use case.

# LCM for Positioning for UE sided model

In RAN#125bis [3] the following agreements were made on functionalities, supported functionalities, applicable functionalities, proactive and reactive reporting of applicable functionality regarding functionality-based LCM for UE sided models,

Agreements

1.	Which AI/ML-enabled Features/FGs and functionalities are supported should be standardized. The details wait for RAN1’s progress.   “supported” means that the UE is capable of supporting the functionality and doesn’t mean neccesarily that the UE has the model available.  FFS what functionality refers to.

2.	Supported AI/ML-enabled Features/FGs and supported functionalities are included in UE capability.

Agreements for positioning and beam management

- Support proactive reporting of UE-sided applicable functionality, e.g., the UE reports its applicable AI/ML functionalities via UAI message/LPP message.

- Support reactive reporting of UE-sided applicable functionality.  The NW configures AI/ML functionalities via RRC/LPP message.  FFS what the configuration contains. FFS how to report applicable functionality and what is applicable functionality

3	FFS how the two approaches will be specified and whether we can combine them into one procedure.    FFS how to report applicable functionality, what is applicable functionality, how the UE determines which function is applicable or not (if it is needed)

Agreements:

1	For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline.  The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach).

2	“UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19

In RAN2#126[4] the following agreement was made regarding functionality-based LCM for positioning regarding supported functionalities and associated ID.

Agreements

- The LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to indicate supported AI/ML positioning capabilities.  FFS how to handle dynamic capabilities, depending on further RAN1 progress and understanding of the functionality.

- wait for RAN1 for associate ID discussion

- At least for Case 1, existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing the results of the UE sided model inference operation.  FFS further details on signaling enhancements

In RAN2#127[5] the following agreements regarding functionality-based LCM was agreed for Beam management use case.

Agreements on definitions

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling)

- Applicable functionalities refers to functionalities that the UE is ready to apply for inference

- Activated functionalities refers to functionalities already enabled for performing inference

Agreements on procedures - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc - Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. - The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality - We will write an LS to RAN1 to provide our agreements and ask specific questions that RAN2 needs to enable progress.

In the previous meeting there have been several discussions relating to functionality-based LCM. During these discussions there have been various terminologies used with regards to functionality-based LCM such as supported functionalities, applicable functionalities and activated functionalities. In the previous RAN2 meeting these definitions were formally agreed for the Beam Management use case. We believe there should be consistency for the terminology among the different AI/ML use case, therefore, these definitions should also be agreed in the positioning use case. Therefore, we propose the following,

Proposal 1: Supported functionalities refers to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling).

Proposal 2: Applicable functionalities refers to functionalities that the UE is ready to apply for inference.

Proposal 3: Activated functionalities refers to functionalities already enabled for performing inference.

Furthermore, there was discussion in RAN2 regarding the procedure of how to indicate these different functionalities together with NW side additional conditions. We believe that it will be beneficial to have alignment among the different one-sided use cases for the procedure to indicate the different functionalities and the NW side additional conditions. Having a similar procedure for reporting the different functionalities allows easier design and management of different AI/ML use cases. Therefore, we propose that we should use the signalling procedure defined for Beam Management as starting point to design the signalling procedure to report different functionalities for positioning use case.

Proposal 4: Use the following procedure as starting point to design the procedure for functionality-based LCM for positioning.

- Step 1: Network sends the UECapabilityEnquiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

- Step 2: UE sends the UECapabilityInformation message to network, containing supported functionalities at the UE side.

- “Step 3”: Following configurations are provided from NW to UE:

- UE is allowed to do UAI reporting via OtherConfig.

- Network may provide NW-side additional condition.  Decisions on the RRC signalling and whether it is mandatory or optional are FFS.

- The configuration (e.g., inference configuration) of supported functionalities is FFS. Configuration content is FFS.

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. Whether other configuration can be considered by UE (e.g., inference configuration) is FFS.  The mechanism for deciding the applicable functionality if the NW-side additional condition is not provided in step 3 is FFS.

- “Step 4”: UE reports applicable functionality in the following scenarios:

  - Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

  - As response to NW-side additional condition requesting applicable functionality reporting in step 3, Other network configurations (e.g., inference configuration), FFS via UAI or RRCReconfigurationComplete, etc. are FFS.

- Step 5:

  - Network configures UE inference configuration after applicable functionality reporting if inference configuration based on supported functionality is not provided in Step 3 (i.e., inference configuration is provided in Step 5).

  - If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not.

- The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5.  FFS the initial activation state.  FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation.  FFS if multiple applicable functionalities can be activated at the same time.   FFS what is the granularity of functionality

# Conclusion

In this contribution, we discussed a sub-use case on CSI feedback enhancements related to time domain CSI prediction using UE sided model. We made the following proposals.

Proposal 1: Supported functionalities refers to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling).

Proposal 2: Applicable functionalities refers to functionalities that the UE is ready to apply for inference.

Proposal 3: Activated functionalities refers to functionalities already enabled for performing inference.

Proposal 4: Use the following procedure as starting point to design the procedure for functionality-based LCM for positioning.

- Step 1: Network sends the UECapabilityEnquiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

- Step 2: UE sends the UECapabilityInformation message to network, containing supported functionalities at the UE side.

- “Step 3”: Following configurations are provided from NW to UE:

- UE is allowed to do UAI reporting via OtherConfig.

- Network may provide NW-side additional condition.  Decisions on the RRC signalling and whether it is mandatory or optional are FFS.

- The configuration (e.g., inference configuration) of supported functionalities is FFS. Configuration content is FFS.

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. Whether other configuration can be considered by UE (e.g., inference configuration) is FFS.  The mechanism for deciding the applicable functionality if the NW-side additional condition is not provided in step 3 is FFS.

- “Step 4”: UE reports applicable functionality in the following scenarios:

- Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

- As response to NW-side additional condition requesting applicable functionality reporting in step 3, Other network configurations (e.g., inference configuration), FFS via UAI or RRCReconfigurationComplete, etc. are FFS.

- Step 5:

- Network configures UE inference configuration after applicable functionality reporting if inference configuration based on supported functionality is not provided in Step 3 (i.e., inference configuration is provided in Step 5).

- If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not.

- The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5.  FFS the initial activation state.  FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation.  FFS if multiple applicable functionalities can be activated at the same time.   FFS what is the granularity of functionality

# References

[1] RP-213599 Study on AI/ML for Air-Interface, RAN#94e.

[2] RP-234039 New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

[3] RAN2#125bis Chair’s Notes, Changsha, China, 15 - 19 April 2024

[4] RAN2#126 Chair’s Notes, Fukuoka, Japan, 20 - 24 May 2024

[5] RAN1#127 Chair’s Notes, Maastricht, Netherlands, 19 - 23 August 2024