3GPP TSG-RAN WG2 Meeting #127bis	R2-2408942

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.2.2 Functionality aspects

Source: 	Semtech

Title: 	Considerations for energy status report

Document for:	Discussion/Decision

# Introduction

In RAN#102, the A-IoT Rel-19 SID [2] was approved based on the Ambient IoT RAN led study technical report [1]. The SID was updated in RAN #103 [3]. The motivation of Ambient IoT is to support devices that are ultra-low cost and ultra-low power. This tdoc discusses the considerations for functionality aspects.

# Energy Status Report

In the discussions in the RAN2 #128, there was a proposal to study an energy status report [4]:

Agreements

-	RAN2 will study the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages.   FFS details on signaling

-	RAN2 will study the need and use of a simple message “size” reporting to the reader

One of the impacts of the A-IoT devices relying on energy harvesting for device operation is the low reliability of the available energy for operations. A-IoT devices, being low cost devices, would have limited energy storage and limited charging capabilities for data transmissions/receptions with the reader and for processing within the device (e.g., for the “command”). If the reader was to always assume the A-IoT devices have enough energy for the operation when it pages the device, it places a higher burden on the device implementation to satisfy this requirement. Thus, there is a need for the A-IoT device and readers to have procedures to deal with A-IoT devices with insufficient energy.

The proposed energy status report (in RAN2 #128) could be used to report the energy level to the reader or the core network. The A-IoT device would send this report (within Msg1 or an alternative method) based on the energy level thresholds required to transmit/receive the subsequent messages (e.g., Msg 2/3) and the internal processing for the operation. When the reader and/or core network receives a report indicating insufficient energy, the procedure would be delayed for a period of time.

RAN2 to include an energy status report from the A-IoT device to the reader when the device has insufficient energy storage to complete the operation.

# Summary

- RAN2 to include an energy status report from the A-IoT device to the reader when the device has insufficient energy storage to complete the operation.

# References

TR 38.848 Study on Ambient IoT (Internet of Things) in RAN, 2023

RP-234058, “New SID: Study on solutions for Ambient IoT (Internet of Things) in NR”

RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”

RAN2 #128, RAN2 Chair notes, Aug 2024.