3GPP TSG-RAN WG2 Meeting #127bis	R2-2408944

Hefei, China, 14th – 18th of October 2024

Agenda item:	7.7.2

Source:	Nokia, Nokia Shanghai Bell

Title:	FR2-related Release 18 NTN Issues

WID/SID:	NR_NTN_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In this paper we present our view how to resolve the Release 18 NTN issues associated with FR2.

# 2	Discussion

## 2.1	FR2-NTN – feedback from RAN1 LS

RAN1 during their RAN1#118 meeting (August 2024) has completed the work on supporting NR NTN in the FR2 frequency bands. During this discussion RAN1 has realized there are inconsistencies whether FR2-1 or FR2-2 procedures should apply to NTN’s FR2 (i.e. FR2-NTN). It is important as RAN1 specifications define separate behaviour for FR2-1 and FR2-2.

Observation 1: There are unclarities whether FR2-1 or FR2-2 procedures in various specifications should apply to the FR2-NTN.

It was also discovered that a similar issue might be applicable to the existing RAN2 RRC parameters. RAN1 LS [2] lists the following exemplary parameters: subcarrierSpacing, channelBWs-DL, configuredUL-GrantType1-v1650. These might be missing the reference to FR2-2 bands.

Observation 2: RAN1 has discovered certain RRC parameters might not have appropriate indication which FR2-x they belong to (i.e. FR2-1, FR2-2, FR-NTN).

According to [4], corresponding frequency ranges for FR2-1, FR2-2 and FR2-NTN are defined as follows:

- FR2-1: 24250 MHz – 52600 MHz

- FR2-2: 52600 MHz – 71000 MHz

- FR2-NTN: 17300 MHz – 30000 MHz

As can be noticed, FR2-NTN partly overlaps with FR2-1. However, the lower frequency region in FR2-NTN is not comprised within FR2-1.

Observation 3: FR2-NTN partly overlaps with FR2-1. Its lower frequency region is not comprised within FR2-1.

Nevertheless, it can be assumed that most of the parameters and procedures defined for FR2-1 (and not for FR2-2 which covers a completely different part of the spectrum, i.e. frequencies above 52 GHz according to [4]) can be applicable to FR2-NTN. We claim most of the parameters and procedures, as e.g. handover in FR2-NTN is much simplified when compared to FR2-1 (e.g. no beam sweeping). Anyway,  this RAN2 assumption requires a RAN4 confirmation.

Proposal 1: RAN2 assumes the requirements and parameters for FR2-1 are generally applicable to FR2-NTN, unless specified otherwise.

Proposal 2: RAN2 verifies with RAN4 its assumption regarding the applicability of FR2-1 to FR2-NTN.

Regarding RAN2 specifications and how to reflect that FR2-1 applies also to FR2-NTN, we do not think it is a good idea to update each individual parameter (e.g. each field description). Instead, RAN2 should consider a general statement on this matter, obviously if RAN4 confirms our conclusion is valid.

Proposal 3: RAN2 considers a general statement expressing FR2-1 requirements and parameters apply to FR2-NTN. Such information should be also available in appropriate RAN4 specification.

## 2.2	FR2-NTN capability for enhanced RRM requirements for measurements in IDLE/INACTIVE

The need for a separate capability has been discussed based on [5] during RAN2#127 meeting (August 2024). The conclusion was to postpone this to next meeting [3]:

UE capabilities corrections R2-2406451 Remaining Issues on FR2-NTN Support vivo discussion Rel-18 NR_NTN_enh-Core Proposal 1: Introduce a new R18 UE capability without signalling for enhanced FR2-NTN bands RRM requirements. - HW support this - QC thinks this should be discussed and RAN4 and they should request this - Ericsson wonders if we can just update the field with FR2 of this idle mode capability. Vivo thinks this would not be so clear - Samsung thinks this depends on RAN4 - QC prefers to wait Come back in the next meeting to check whether we Introduce a new R18 UE capability without signalling for enhanced FR2-NTN bands RRM requirements

The capability for “Enhanced RRM requirements for measurements in IDLE and INACTIVE modes” has been introduced in Release 17 and is defined as follows [6]:

Enhanced RRM requirements for measurements in IDLE and INACTIVE modes It is optional for UE to support enhanced RRM requirements for measurements for NTN bands (FR1 only and FDD only) in RRC_IDLE/RRC_INACTIVE as specified in TS 38.133 [5]. If UE does not support this feature, other NTN measurement requirements (as specified in TS 38.133 [5], clause 4.2C.2 for RRC_IDLE and clause 5.1C.2 for RRC_INACTIVE) are applied.

As can be seen, this capability is applicable to FR1 and FDD only.

Observation 4: The Rel-17 capability for Enhanced RRM requirements for measurements in IDLE and INACTIVE modes applies to FR1 and FDD only.

With the Release 18 support of FR2-NTN corresponding updates have to be pursued. If the requirements are described in the same section of TS 38.133 and we do not expect there will be UEs supporting  Enhanced RRM requirements for measurements in IDLE and INACTIVE modes in FR1, but not in FR2 (or vice versa) then the existing capability could just be updated with FR2-NTN indication. However, as this is already a legacy Release 17 capability, our preference would be to introduce a separate Release 18 capability, for FR2-NTN.

Proposal 4: Introduce a separate Release 18 capability for Enhanced RRM requirements for measurements in IDLE and INACTIVE modes in FR2-NTN.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: There are unclarities whether FR2-1 or FR2-2 procedures in various specifications should apply to the FR2-NTN.

Observation 2: RAN1 has discovered certain RRC parameters might not have appropriate indication which FR2-x they belong to (i.e. FR2-1, FR2-2, FR-NTN).

Observation 3: FR2-NTN partly overlaps with FR2-1. Its lower frequency region is not comprised within FR2-1.

Proposal 1: RAN2 assumes the requirements and parameters for FR2-1 are generally applicable to FR2-NTN, unless specified otherwise.

Proposal 2: RAN2 verifies with RAN4 its assumption regarding the applicability of FR2-1 to FR2-NTN.

Proposal 3: RAN2 considers a general statement expressing FR2-1 requirements and parameters apply to FR2-NTN. Such information should be also available in appropriate RAN4 specification.

Observation 4: The Rel-17 capability for Enhanced RRM requirements for measurements in IDLE and INACTIVE modes applies to FR1 and FDD only.

Proposal 4: Introduce a separate Release 18 capability for Enhanced RRM requirements for measurements in IDLE and INACTIVE modes in FR2-NTN.

# References

- RP-232669	Revised WID: NR NTN (Non-Terrestrial Networks) enhancements 3GPP TSG RAN Meeting #100 Taipei, Taiwan, June 12-14, 2023

- R1-2407406 LS on FR2-NTN inclusion to specifications 3GPP TSG RAN WG1#118                                               Maastricht, NL, August 19th – 23rd, 2024

- R2-2407573 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #127 Maastricht, Netherlands, Aug 19th – 23rd, 2024

- 3GPP TS 38.101 NR; User Equipment (UE) radio transmission and reception

- R2-2406451 Remaining Issues on FR2-NTN Support 3GPP TSG-RAN WG2 Meeting #127 Maastricht, Netherlands, Aug 19th – 23rd, 2024

- TS 38.306 User Equipment (UE) radio access capabilities V18.3.0 (2024-09)