3GPP TSG-RAN WG2 Meeting #127bis	R2-2408946

Hefei, China, 14 – 18 of October 2024

Agenda item:	8.8.4

Source:	Nokia, Nokia Shanghai Bell

Title:	On MBS Support in Rel-19 NR NTN

WID/SID:	NR_NTN_Ph3 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In December 2023, RAN approved a Rel-19 work item (WI) aimed at defining Phase 3 enhancements to Non-Terrestrial Networks (NTN). There was a minor revision of the WID at RAN#103. Among the objectives captured in the WID [1] the following can be found:

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

In RAN2#125bis meeting, the first agreements on the topic were made and these were as follows [2]:

Agreements: - For MBS broadcast service we don’t restrict the work to any satellite constellation type - We prioritize working on a solution for MBS broadcast but we don’t preclude other broadcast services, namely ETWS - We will cover at least the case where the indicated intended service area covers a portion of a NTN cell - The intended service area can cover the area of more than one NTN cells (or portions thereof) - Can discuss next time whether the broadcast transmission can be limited to the intended service area only (i.e. no transmission happens outside of the intended serive area) - At least the following geographical area formats to model service area can be further considered (the signalling of other information than the geographical information can be considered): - Circles (like for TN coverage) - Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

The topic was subsequently discussed in RAN2#126 with the following decisions made [3]:

Agreements: 1. For MBS broadcast service, both EFC and EMC are supported. 2. RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area. 3. MBS broadcast intended service area is provided via system information 4. For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS 5. When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

Further decisions were taken during RAN2#127 [4]:

1. The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s). 2. RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions 3. For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2) 4.For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

In this paper we present our further view on what is to be addressed for this Rel-19 objective in RAN2.

# 2	Discussion

## 2.1	Support for emergency broadcasting services

For emergency broadcasting services, the following was initially agreed [2]:

RAN2#125bis agreements - We prioritize working on a solution for MBS broadcast but we don’t preclude other broadcast services, namely ETWS

At RAN2#127 [4] it was agreed to send the LS to SA1 [5] and ask this SA WG whether geofencing emergency messages should be supported for ETWS Primary Notification  in an NTN cell [5]:

To SA WG 1: RAN2 kindly request SA1 to take the above information into account and provide feedback whether the capability to geofence emergency messages to an area should be supported for ETWS in a NTN cell, considering the delay requirements for ETWS primary notification.

NTN can support much greater coverage area compared to terrestrial network (TN) due to much larger beam footprint (even with LEO satellite constellation, not to mention GEO). Capability to support multi-beam layout enhances the geographical coverage zone of the tracking area of the satellite. As a result, NTN can provide connectivity to large number of UEs with minimal terrestrial network coverage. Therefore, dissemination of public warning systems like ETWS to larger number of UEs can be an attractive use case for broadcasting over NTN.

The ETWS message contains two parts, the primary and the secondary warning messages, the primary notification message and the secondary notification message. As per TS 22.268, which provides the requirements for the public warning system, the following points about ETWS can be found in the specification:

Duration of the delivery time for PLMN operators is the time from the receipt of the Warning Notification by the PLMN operator, i.e. the edge of the 3GPP network, to the time that the Warning Notification is successfully delivered to the UEs.

Provisioning of delivery of Primary and Secondary Notification may be required:

-	Primary Notification shall be delivered within 4 seconds to the UE in the Notification Area even under congestion situation.

-	Secondary Notification is delivered to the users in the Notification Area even under congestion situation.

Note 1: 	UEs that are out of coverage or switched off are not considered in the requirements.

Note 2: 	Secondary Notification may not always be generated as it depends on the Warning Notification Provider’s Primary Notification shall:

-	convey data which is small enough to be sent quickly on the network.

-	convey small amount of data to indicate the imminent occurrence of Earthquake and Tsunami, etc.

Secondary Notification may:

-	convey a large amount of data in order to deliver text, audio to instruct what to do / where to get help, graphical data such as a map indicating the route from present position to evacuation site, time table of food distribution.

policy.

Note 3: 	Primary Notification may not always be generated (i.e. the warning may start with a Secondary Notification).

Since the primary notification message is the critical part of the ETWS message, broadcast of ETWS over NTN should focus predominantly on the primary notification message of ETWS, if ETWS is to be supported.

Proposal 1:  RAN2 to confirm that for ETWS message dissemination through broadcast over NTN (if supported), initially will focus on primary notification message.

The correct distribution of public warning messages over the affected area is critical to correctly alert impacted UEs. Despite the intention is to alert as many UEs as possible, it may be desirable to alert only those UEs that are within the disaster area. In NTN, cell’s size may be orders of magnitude larger than the actual disaster area, which means that if the alert is cell-wide, many UEs may be warned while being outside of the disaster area. For those UEs outside of the disaster area, apart from disturbing UE’s experience, they are also impacted in terms of UE’s energy savings since devices are required to immediately wake up, acquire system information and notify the user.

Observation 1: In case of ETWS over NTN, UEs outside of the disaster/intended area may be impacted in terms of UE’s energy savings.

Even though we understand that warning messages must be delivered without compromises, we consider that in case of large NTN cells (up to 1000 km in diameter), UEs may benefit from distinguishing whether they are within or outside of the disaster area. For that reason, we propose the following:

Proposal 2: RAN2 to discuss means for geo-fencing broadcast ETWS messages over NTN cells.

So far, the general assumption in RAN2 has been that the gNB decides how to configure the service area for broadcast services and the specific format may be defined similarly as the signalling for TN coverage area in Rel-18 NR NTN. This understanding can apply to MBS but not to,e.g., CMAS, where the specific coordinates are transparent to the gNB and the latter only forwards this information.

Observation 2: In CMAS, area information is transparent to the NG-RAN node.

Based on this, we propose the following:

Proposal 3: RAN2 to decide what is the impact to the gNB in case of supporting geo-fencing for ETWS.

## 2.2	Area contention of the broadcasting services

The 3GPP contribution in [7] contained the following proposal on the potential scenarios to be supported:

R2-2402355 Proposal 1: RAN2 discusses and confirms the intended scenarios to be supported for MBS broadcast service via NR NTN in Rel-19: - S1: The service area is a portion of an NTN cell, and the Content of the service can only be received within the service area (e.g. broadcast via a dedicated beam(s) ). - S2: The service area is a portion of an NTN cell, but the content of the service can be received within the whole cell. - S3: The intended service area consists of a list of NTN cell(s)/tracking area(s).

The proposal was discussed during RAN2#125bis meeting resulting with the following outcome [2]:

RAN2#125bis agreements: - We will cover at least the case where the indicated intended service area covers a portion of a NTN cell - The intended service area can cover the area of more than one NTN cells (or portions thereof) - Can discuss next time whether the broadcast transmission can be limited to the intended service area only (i.e. no transmission happens outside of the intended serive area)

During RAN2#126 it was additionally agreed that:

2. RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area.

Thus, scenario 1 from [7] has been excluded.

Observation 3: The decision taken at RAN2#126 excludes scenario 1 (The service area is a portion of an NTN cell, and the Content of the service can only be received within the service area) from further considerations in Rel-19.

At RAN2#127 it was agreed that [4]:

For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2) For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

As can be noticed, no firm requirements have been defined for the UE regarding MRBs handling when the UE is outside of intended service area.

Observation 4: UE’s behaviour concerning MRBs handling when the UE is outside of intended service area is unspecified (i.e. the UE may or may not establish or release the MRBs).

In our view at least the action when the UE can establish the MRBs should be clear. I.e. the UE should be allowed to establish the MRBs based on its location versus MBS service area, not just whenever it can receive the MBS. Thus, we suggest the following:

Proposal 4: Make the RAN2#127 agreement UE-binding and capture that the NTN-capable UE is not allowed to/shall not establish MRBs if it can receive broadcast service, but its location is not within the MBS service area.

At RAN2#127 it was also decided the UE may release the MRBs when it leaves the intended area. That is acceptable, but we shall not let the UE keep the MRBs for excessively long time. Thus, we suggest the following:

Proposal 5: UE may release MRBs when leaving the intended area. However, releasing MRBs is eventually subject to additional conditions, such as the time or distance that has elapsed since leaving the MBS area, as the UE cannot keep the MRBs eternally.

It was debatable how the UE determines it has left the service area. We acknowledge that it requires new means when compared to MBS in terrestrial networks. Nevertheless, it is expected the NTN-capable UE will monitor its geo-position, so it should be able to realize it has left or entered the MBS service area.

Proposal 6: NTN-capable UE realizes it is within or outside of MBS service area based on the GNSS location it is using for other NTN purposes. I.e. there is no need to update UE’s GNSS location specifically for this purpose.

## 2.3	Supporting MBS in Earth-moving deployments

During RAN2#126 the following has been agreed [3]:

1. For MBS broadcast service, both EFC and EMC are supported.

This is in line with the objective captured in the WID [1], wherein the aim is to provide broadcast services despite the satellite constellation and the NTN cell deployment. However, this raises a new problem for MBS signalling when satellites in NTN system are in continuous motion. For example in EMC the footprint will be continuously moving, and a particular satellite will be able to provide MBS just temporarily in a certain area.

Observation 5: MBS service will be provided by certain satellite only during limited period (i.e. when satellite’s footprint overlaps with the MBS area).

Thus, cell identifiers associated with the MBS service area will change over time.

Observation 6: MBS service area provided by NTN will be covered by cells whose identifiers change over time.

On the other hand, current MBS framework was designed considering earth-fixed cells without moving gNBs. For example, a broadcast session that is initiated from an application function is forwarded to the gNBs in the MBS service area. More specifically, AMF picks the gNBs that reside in the service area of the MBS session and sends Broadcast Setup Request only to those gNBs.

Observation 7: In legacy MBS design AMF selects gNBs that are in the service area corresponding to MBS session.

However, currently there is no mechanism for moving cells, e.g., a gNB coming into an MBS service area cannot dynamically receive/release Broadcast Session. If RAN2 agrees there is a need to support such scenario, we shall also let RAN3 know and ask whether the existing mechanisms (e.g. mapped cell identifier) is sufficient.

Proposal 7: As RAN2 decided to support MBS via EMC in NTN, RAN2 sends the LS to RAN3 asking this WG to define corresponding NW signalling, if mapped cell ID is insufficient.

## 2.4	SIB or MCCH for signalling intended broadcast area

At RAN2#126 (May 2024) the following has been agreed [3]:

For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS

At RAN2#127 (August 2024) the topic was discussed via offline thread [6] and the following was agreed [4]:

2. RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions

According to the MBS description in [7], the MBS configuration information is carried on MCCH logical channel. It comprises the information on the MBS broadcast sessions that are provided, related scheduling information and it can also contain the list of neighbour cells where the same broadcast MBS services are provided.

Observation 8: MCCH provides the list of MBS broadcast sessions, scheduling information, and the list of neighbours with the same MBS services.

In order to receive MCCH, the UE needs to acquire appropriate information from SIB1 and SIB20 [7].

Observation 9: The information from SIB1 and SIB20 is essential for the UE to be able to received MCCH.

One of the Rel-19 NTN questions that remain is where this service area related information is to be provided for NTN cells supporting MBS. For example, if the intended service area information is provided in SIB (e.g. SIB20 or SIB21), then we need a separate list of provided broadcast sessions in the cell, although the list already exists in the MCCH. Another approach could be providing the intended service area for each broadcast sessions in the MCCH in the same order.

At RAN2#127 it has been agreed that the UE that is not within the intended service area may not reacquire the MCCH.

Observation 10: UE that is not within the intended service area may not reacquire the MCCH.

This behaviour seems to be reasonable, and it shall be up to the UE implementation how to realize whether it is within or outside of the intended service area. This RAN2#127 conclusion also does not restrict the solution to be necessarily SIB20-based or MCCH-based. MCCH-based solution allows to send the notification via PDCCH addressed with MCCH RNTI, so just MBS-interested UEs are receiving this notification. The indication can explicitly tell if new service is to be provided or if it has changed.

Observation 11: MCCH-based solution allows to send the notification via PDCCH addressed with MCCH RNTI and indicate with a single bit if new service will be provided.

The solution relying on SIB20 has at least the following drawbacks:

- changes to service area impacts all UEs in the cell

- there should be a mapping between the service area provided in SIB20 and the MBS session info available in MCCH

Thus, in our view MCCH-based solution is clearly a better approach, so once again we propose the following:

Proposal 8: RAN2 follows the legacy approach and provides MBS related information (such as list of MBS sessions, intended broadcast service area) in MCCH (within MBSBroadcastConfiguration).

## 2.5	Format of the service area

The following agreements were made during the RAN2#125bis meeting that are relevant for the format of the intended service area [2]:

RAN2#125bis agreements: - We will cover at least the case where the indicated intended service area covers a portion of a NTN cell - The intended service area can cover the area of more than one NTN cells (or portions thereof) - At least the following geographical area formats to model service area can be further considered (the signalling of other information than the geographical information can be considered): - Circles (like for TN coverage) - Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

The topic was concluded during RAN2#127 [4] with the following decision:

1. The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

For the set of referenceLocation and radius, RAN2 can reuse the solution defined in Rel-18 NTN. In order to signal the terrestrial network (TN) coverage area, a new system information block (SIB25) conveys the following information elements (IEs) [7]:

CoverageAreaInfo-r18 ::= SEQUENCE { tn-AreaId-r18 TN-AreaId-r18, tn-ReferenceLocation-r18 ReferenceLocation-r17, tn-DistanceRadius-r18 INTEGER(0..65536) }

TN coverage area is described using a reference location and the radius. Together they constitute a circular area with a centre defined by the referenceLocation IE. A list of such referenceLocation and radius pairs should be defined also for MBS intended service area.

Proposal 9: RAN2 introduces a list of ReferenceLocation-r17 and corresponding radius to signal the intended broadcast area.

Regarding polygons, signalling impact can be heavier than for the reference location and radius. Here for a single polygon multiple vertices need to be signalled, possible each with a full ReferenceLocation IE. RAN2 shall discuss if this approach can be optimized somehow and what is the number of vertices that shall be given per each polygon.

Proposal 10: RAN2 to investigate how many vertices should be signalled for a single polygon. In addition it shall be discussed if each vertex of the polygon needs to be represented with a ReferenceLocation  IE.

# 3	Conclusion

This document has made the following observations:

Observation 1: In case of ETWS over NTN, UEs outside of the disaster/intended area may be impacted in terms of UE’s energy savings.

Observation 2: In CMAS, area information is transparent to the NG-RAN node.

Observation 3: The decision taken at RAN2#126 excludes scenario 1 (The service area is a portion of an NTN cell, and the Content of the service can only be received within the service area) from further considerations in Rel-19.

Observation 4: UE’s behaviour concerning MRBs handling when the UE is outside of intended service area is unspecified (i.e. the UE may or may not establish or release the MRBs).

Observation 5: MBS service will be provided by certain satellite only during limited period (i.e. when satellite’s footprint overlaps with the MBS area).

Observation 6: MBS service area provided by NTN will be covered by cells whose identifiers change over time.

Observation 7: In legacy MBS design AMF selects gNBs that are in the service area corresponding to MBS session.

Observation 8: MCCH provides the list of MBS broadcast sessions, scheduling information, and the list of neighbours with the same MBS services.

Observation 9: The information from SIB1 and SIB20 is essential for the UE to be able to received MCCH.

Observation 10: UE that is not within the intended service area may not reacquire the MCCH.

Observation 11: MCCH-based solution allows to send the notification via PDCCH addressed with MCCH RNTI and indicate with a single bit if new service will be provided.

And proposed the following:

Proposal 1:  RAN2 to confirm that for ETWS message dissemination through broadcast over NTN (if supported), initially will focus on primary notification message.

Proposal 2: RAN2 to discuss means for geo-fencing broadcast ETWS messages over NTN cells.

Proposal 3: RAN2 to decide what is the impact to the gNB in case of supporting geo-fencing for ETWS.

Proposal 4: Make the RAN2#127 agreement UE-binding and capture that the NTN-capable UE is not allowed to/shall not establish MRBs if it can receive broadcast service, but its location is not within the MBS service area.

Proposal 5: UE may release MRBs when leaving the intended area. However, releasing MRBs is eventually subject to additional conditions, such as the time or distance that has elapsed since leaving the MBS area, as the UE cannot keep the MRBs eternally.

Proposal 6: NTN-capable UE realizes it is within or outside of MBS service area based on the GNSS location it is using for other NTN purposes. I.e. there is no need to update UE’s GNSS location specifically for this purpose.

Proposal 7: As RAN2 decided to support MBS via EMC in NTN, RAN2 sends the LS to RAN3 asking this WG to define corresponding NW signalling, if mapped cell ID is insufficient.

Proposal 8: RAN2 follows the legacy approach and provides MBS related information (such as list of MBS sessions, intended broadcast service area) in MCCH (within MBSBroadcastConfiguration).

Proposal 9: RAN2 introduces a list of ReferenceLocation-r17 and corresponding radius to signal the intended broadcast area.

Proposal 10: RAN2 to investigate how many vertices should be signalled for a single polygon. In addition it shall be discussed if each vertex of the polygon needs to be represented with a ReferenceLocation  IE.

# References

- RP-240775 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3 3GPP TSG RAN Meeting #103 Maastricht, The Netherlands, March 18-22, 2024

- R2-2403733 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #125bis Changsha, China, Apr 15th – 19th, 2024

- R2-2405703 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #126 Fukuoka, Japan, May 20th – 24th, 2024

- R2-2407573 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #127 Maastricht, The Netherlands, Aug 19th – 23rd, 2024

- R2-2407823 LS on requirements for ETWS primary notification 3GPP TSG-RAN WG2 Meeting #127 Maastricht, The Netherlands, Aug 19th – 23rd, 2024

- R2-2407619 [Offline 307] [R19 NR NTN] service area information (Apple) 3GPP TSG-RAN WG2 Meeting #127 Maastricht, The Netherlands, Aug 19th – 23rd, 2024

- 3GPP TS 38.331 NR RRC, v18.3.0, September 2024.

- R2-2402355, Discussion on support of broadcast service via NR NTN, CATT, China Broadnet, 3GPP TSG RAN2#125bis meeting, Rel-19, April 2024.