3GPP TSG-RAN WG2 Meeting #127bis	R2-2408948

Hefei, China, 14 – 18 October 2024	revision of R2-2406851

Agenda item:	8.6.3

Source:	Nokia

Title:	On L1 Measurement Reporting Enhancements for Rel-19 LTM

WID/SID:	NR_Mob_Ph4 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The phase 4 mobility enhancements work item description captures the measurement enhancements in the [1] as follows:

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1]

  - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM

  - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1]

    - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work

  - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

Following agreements were made in RAN2#126 [3]:

Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

Following agreements were made in RAN2#127 [3]:

Agreements on L1 MR event evaluation

- Event LTM1 is not defined.

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

- R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

6. MAC layer handles the event evaluation and measurement report triggering.

Agreements on L1 measurement reporting

- Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

In this paper we present our further view on the Release 19 measurement framework for LTM.

# 2	Discussion

## 2.1 Event triggered Reporting for LTM

### 2.1.1 Current Beam Measurements for Event Evaluation

#### 2.1.1.1 Activated TCI state for PDCCH as current beam for event evaluation

In RAN2#127 the following has been agreed:

6. Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

Additionally, the following RAN1 decisions have been taken at RAN1#118:

Agreement
For the identification of the serving cell RS for event evaluation, 
At least the following options are further studied in RAN1, where different options could apply to different LTM event
Option. 1: Derived from QCL (type-D) RS(s) of the indicated joint/DL TCI state for the serving cell
Option. 2: Derived from QCL RS(s) or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell
QCL RS or SSB is configured by the network
Option. 3: Measurement RS(s) is/are explicitly configured
Option. 4: Derived from QCL RSs of activated TCI states with the best quality, or SSB which is QCLed with the QCL RSs of activated TCI states with the best quality.
Option 6: Derived from QCL RSs of activated TCI states, or SSB which is QCLed with the QCL RSs of activated TCI states 
The RSs of the candidate cell(s) for event evaluation are explicitly configure
Note: Companies are encouraged to take into account the RAN2 agreement (i.e current beam rather than best beam) for their further study.

Supporting LTM does not necessarily imply using only the Rel-17 unified TCI state framework in the serving cell. Also, it is not accurate to claim that a candidate cell prepared for Rel-18 LTM must support the Rel-17 unified TCI state framework for beam management. In RAN1 117, a CR [5] was endorsed to support the application of LTM TCI states after a cell switch, even when the target cell is configured with Rel-15 TCI states. Thus, Rel-18 LTM does not impose any constraint on serving or candidate cells to exclusively use the Rel-17 unified TCI state framework for beam management purposes.

Therefore, the serving cell for which the event triggered LTM reporting is configured may use R15 TCI framework but current agreements do not support (nor exclude) the R15 TCI State framework to be used by the serving cell for which the event triggered reporting is configured.

In case of the serving cell supports Rel-15 TCI framework, the TCI state for the measurement resource should be derived based on the activated TCI state for PDCCH. There seems to not be an immediate technical issue or disadvantage to support the derivation of current beam RS for LTM event evaluation based on activated TCI State.

Observation 1: the indicated TCI State refers to the use of unified TCI State framework for the serving cell.

Observation 2: the serving cell may operate using R15/16 TCI state framework i.e. non-unified TCI state. Thus, to support event triggered LTM reporting different network deployments, the activated TCI state for PDCCH should be used for deriving the current beam RS.

Proposal 1: From RAN2 perspective, for LTM 3/4/5 event evaluation the current beam RS for serving cell measurements can be derived based on activated TCI state for PDCCH (R15/16).

### 2.1.2 On using the same RS type for Event Evaluation

In RAN2, a working assumption has been made to use the same RS type for both serving and candidate beams for events LTM3 and LTM5. As discussed, the RSs of a candidate cell could be SSBs and/or CSI-RSs. For the serving cell, the measurement RS would be derived from the serving beam, which could also be an SSB or a CSI-RS. It is important to note that the serving beam could change dynamically based on serving cell measurements. Therefore, in scenarios where the serving beam and candidate beam are associated with different RS types (e.g., the serving cell beam is QCL’ed with an SSB, and the candidate RS is a CSI-RS), it is unclear how event evaluations for LTM3 and LTM5 should be conducted and what RS information should be reported when the event criteria are met.

Observation 3: When the serving beam and candidate beam are associated with different RS types it is unclear how event evaluations for LTM3 and LTM5 should be conducted and what RS information should be reported.

Specifically, in cases where the RS derived for the serving beam is an SSB, and the RS configured for the candidate beam is a CSI-RS, the SSB QCL’ed with the candidate CSI-RS should be used for event evaluation. However, it is unclear whether the SSB ID (e.g., SSBRI) or the CSI-RS ID (e.g., CRI) should be reported when the event criterion is met. Therefore, it needs to be discussed how to ensure that the same type of RS is used for both the serving and candidate cells for events LTM3 and LTM5, and the impact on corresponding reporting.

Proposal 2: RAN2 should study whether and how the same type of RS can be used for both the serving and candidate cells for events LTM3 and LTM5, and the implications for reporting.

### 2.1.3		Event Evaluation at MAC Layer

The RAN plenary (RP-242356) has revised the Mobility enhancement WI objectives and no further attention was needed to manage potential overlaps between the MIMO WI and Mobility WI.

  - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1]

To prevent too frequent event triggered reporting, a mechanism to perform at least time domain filtering would be needed. It has been agreed that for LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific offset can be applied.

For the LTM event evaluation, the TTT concept, similar to L3 reporting, could be adopted. In the TTT approach, the UE is configured to observe whether the evaluated criteria is fulfilled for the duration of the TTT (e.g. LTM3 event). In a certain case the TTT duration could be configured to be ‘0’ which means that if the event criteria is fulfilled, the event and the associated reporting is triggered immediately. As the MAC layer is used for event evaluation, the TTT related procedure (start /stop/expiry) is performed at MAC layer where the event evaluation procedure is defined.

We consider few alternatives for the implementation of such TTT based event evaluation at MAC layer.

Alternative 1:

In this alternative, the L1 is providing the measurement results to MAC layer (e.g. on RS1… RSk) that includes the candidate RS measurements and serving cell RS measurements, depending on the LTM event type. MAC layer determines based on the configured event criteria when the event criteria is fulfilled and starts the TTT timer. MAC layer also observes that whether the criteria is fulfilled for the duration of the TTT. Once the TTT expires, the event triggered report is generated, reporting the RS or RSs fulfilling the criteria. Modeling of the TTT timer handling may options such as whether the TTT timer start/stop/expiry is modelled explicitly or only the reporting procedure is modelled as result of the evaluation.

Figure 1. Event evaluation by MAC layer processing the L1 measurements.

Alternative 2:

On second alternative, In this alternative, the L1 responsible for observinf the event criteria based on the candidate and serving RS measurements. If the event criteria is fulfilled an indication for the RS is provided to the MAC layer (e.g. on RS-1… RS-k). MAC layer manages TTT timer (start/stop/expire) based on the L1 indications. E.g the TTT timer is started based on L1 indication, and once the TTT expires determines the event triggered report is criteria is fulfilled and starts the TTT timer. Once the TTT expires, the event triggered report is generated at MAC.

Figure 2. Event evaluation by MAC layer based on L1 indication.

Observation 4: L1 would be responsible for either the indications to MAC layer or for providing the measurement results.

Proposal 3: The event evaluation is performed at MAC layer by having a beam specific time to trigger (TTT) timer.

Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

Based on the agreement. the RS associated with indicated TCI state is used for event evaluation. Since the event triggered procedure and BM management are sperate procedure and as result of L1 beam management reporting the RS corresponding to the indicated beam may change it should be discussed how it affects the LTM event evaluation. As an example the TTT timer may be running for candidate RS based on the event criterion and the serving cell reference (current beam) may change and it should cause the evaluation to be restarted.

Proposal 4: If the serving cell reference (indicated beam) for event evaluation changes, the evaluation for the candidate RS is restarted.

### 2.1.4 Report on Leave

The measurement framework for NR has numerous components which were defined over several releases, starting from Rel-15. One such configurable aspect is reportOnLeave. It is a flag indicating if the UE should report also when one of the cells that previously met a reporting criteria has now left the cellsTriggeredList. A similar mechanism could be defined for LTM L1 measurements events where the report on leave could report on beam level that the reporting criteria is not fulfilled anymore.

Proposal 5: Support report on leave for LTM L1 measurement reporting framework.

If the aforementioned proposal is agreeable, RAN2 needs to discuss how report on leave in MAC layer should look like. Among the questions to be answered one can list the following:

- Report on leave is RS-specific or candidate cell specific?

- Does report on leave use the same MAC CE format as the report when entry condition is met?

- Is the report on leave type of event indicated explicitly or implicitly?

- Is the leaving condition an equivalent of the entry condition or can it be configured separately and in a different manner?

Proposal 6: RAN2 considers the aforementioned list of open issues when designing the report on leave for L1 measurement reporting.

### 2.1.5 Periodic Transmission of the MAC CE Report

Similar to L3 event triggered reporting on RRC, the periodicity i.e. one-time report/periodic report should be configurable per event triggered reporting configuration.

Proposal 7: Support periodic reporting of the LTM event triggered report, with configurable periodicity and/or number of repetitions.

### 2.1.6 MAC CE Reporting Container

#### 2.1.6.1 Candidate Cell Identifier

The R18 CSI-resource config may include reference signals associated with one or more candidate cells. As the result of event evaluation, multiple RS indices of the same or different candidate cells may fulfil the reporting criteria, the MAC CE format should potentially have a way to indicate RSs of multiple candidate cells for which the event is reported.

Proposal 8: The LTM Report MAC CE includes indication of a candidate cell identifier that points to the configured LTM candidate identifier(s).

#### 2.1.6.2 Number of Reported Cells

The LTM reporting events may be triggered for multiple candidate cells, the report format should support reporting RS for multiple candidate cells in a single MAC CE.

Observation 5: As the LTM reporting events may be triggered for multiple candidate cells, the report format should support reporting for multiple candidate cells in a single MAC CE.

Proposal 9: The LTM Report MAC CE supports reporting of candidate RSs for multiple candidate cells in a single MAC CE.

#### 2.1.6.3 Event/Reporting Identifier

Since the event triggered reporting supports multiple event types, at least one field in MAC CE should indicate the event for which the reporting is triggered and the reporting configuration. For example, a field in the MAC CE should point to the reporting configuration associated with the event and resource configuration for which the reporting is performed.

Proposal 10: the LTM Report MAC CE includes an identifier that identifies at least the reporting event type (e.g. LTM3) and the associated reporting configuration (and resource configuration) for which the reporting is performed.

#### 2.1.6.4 RS index

Candidate cell beam(s) for event evaluation:

For the candidate cell measurements, the measurement resources (e.g. SSBs) listed in the LTM-CSI-Resource config can be used to determine whether certain beam of a candidate cell fulfills the reporting criteria. the LTM CSI resource config may comprise of measurement resources of one more candidate cells.

Figure . CSI resources and reporting configuration for event LTM3/5

Additionally, if the CSI resource config includes the RSs of one candidate cell (or the RSs of a subset of candidate cells) the network could configure multiple LTM CSI resource configs with respective report configurations.

Same resource configuration approach could be used also for the LTM4 event (Figure ). Also the same approach could be used for the LTM2 event, but without candidate cell resources (for reporting) associated with the event.

Figure . CSI resources and reporting configuration for event LTM4.

Rel-18 L1 reporting format for LTM includes RS index value that is determined based on the configuration order of the RS in the resource configuration. The bit length of the RS index field in R18 is LI reporting format is determined by the number of configured RS in the resource set.

Observation 6: In L1 UCI reporting format the number of configured RS in the resource set is used to determine the bit width of the Resource identifier field.

The bit width for resource indicator field may thus vary from (0 bits) to 9 bits for LTM: For each resource set up to 64 indices can be included and RS for up to 8 candidate cells can be configured. This results in up to 512 RS indices which means that up to 9 bits per RS index may be needed.

Supporting such range and flexibility is not feasible for MAC CE format which typically has fixed size fields. The value for RS index bit field should be based on the maximum number of RS index values (64) supported per candidate cell:

Proposal 11: The LTM Report MAC CE includes RS index field wherein the RS index field size is fixed 6 bits (to reflect up to 64 RS indices).

If lower number of resource indicators (thus fewer bits) is needed for reporting for a candidate cell, the RS index field can only encode resource indicators on required bit positions while maintaining the size allocated per RS index.

To differentiate reported RSs of different candidate cells, the RS index field encodes the resource indicator field that points to the reference signals, in the LTM CSI-ResourceConfig per PCI. The resource indicator ID space (of 64) is reused for each candidate cell in the CSI-resource config.

Proposal 12: The LTM Report MAC CE includes RS index field that encodes resource indicator pointing to the reference signals of the candidate cell in the LTM-CSI-ResourceConfig associated with the reporting configuration.

Proposal 13: The resource indicator ID space is reused for each candidate cell in the LTM-CSI-configuration associated with reported event.

#### 2.1.6.5 Number Of Reported RS

The LTM Report MAC CE supports reporting of variable number of candidate RS per candidate cells up to the configured number of RSs.

Proposal 14: The LTM Report MAC CE supports reporting of variable number of candidate RS per candidate cell(s), up to the configured number of RSs.

#### 2.1.6.6 RSRP reporting

Providing the RSRP value associated with reported RS identifier gives the network essential information on the quality of the RS that triggered the event.

Proposal 15: The LTM Report MAC CE includes RSRP value associated with each reported RS identifier.

#### 2.1.6.7 Differential RSRP reporting

For the Rel-18 LTM L1 report, the RSRP is reported in differential format. In differential RSRP reporting format the first listed RSRP value is in absolute format (7 bit) and other listed values are in differential format (4 bit) relative to the absolute RSRP value. The differential RSRP reporting could bring overhead savings. Whether to use absolute RSRP values in each report or rely on differential RSRP values could be also specific to certain events.

Proposal 16: RAN2 to study the RSRP value reporting using differential RSRP mechanism in MAC CE.

#### 2.1.6.8 Serving Cell Information

It is important for the NW to know the serving’s beam RSRP, especially when LTM events using the RSRP of the serving beam are applied. At the same time, it can be safely assumed the network know current RS index associated with the serving cell.  Thus, it can be omitted from the report.

Observation 7: the serving cell RS index/identifier should be implicitly known by the network thus it could be omitted from the MAC report

Observation 8: Although events such as LTM 3/5 make already the comparison between current beam RS and candidate RS i.e. so that reported RS quality  are relative to the current beam quality it is beneficial to include the serving cell quality to provide more information to network.

Proposal 17: Include serving cell RSRP measurement for events for which the current beam RS is used for event evaluation i.e. LTM 2/3/5

In order to consider to inter-working with the beam management procedure, it might be beneficial UE to include RS associated with indicated (and activated TCI states) in the event based measurement report. Network can use this to delay LTM cell switch if another serving cell beam is strong e.g. based on beam management procedure.

Proposal 18: UE might include RS associated with indicated (and activated TCI states) in the event based measurement reporting.

#### 2.1.6.9 Truncated Format for MAC CE

When multiplexing the LTM Report MAC CE on UL grant, the UL grant may have limited size i.e. not all the reported information can be included. Thus, the truncation of the MAC CE should be supported. The details can be discussed later once the work on the MAC CE format matures.

Proposal 19: Support truncation of LTM report MAC CE. Details FFS (once the work on MAC CE format matures).

### 2.1.7 Scheduling Request to indicate a reporting event

In some use cases it may be beneficial to configure SR ID for indicating that reporting event has been triggered but the reporting procedure should be designed so that the dedicated SR configuration is not mandatory or required for the event triggered reporting procedure. If the SR ID is associated with LTM event triggered reporting, it should be possible to share the same SR ID with SR for other purposes. If the SR is not configured, the MAC CE reporting the event is provisioned by triggering CBRA.

Proposal 20: An SR ID can be associated with event triggered reporting. The SR can be used to indicate a reporting event/request for UL resource for provision of the MAC CE

Proposal 21: The SR ID for event triggered reporting can be shared with SR for other purposes.

Proposal 22: The SR for indicating event triggered reporting is optionally configured.

Proposal 23: If no SR for indicating event triggered reporting is configured, the UE can use normal SR, if configured.

Proposal 24: If SR is not configured, CBRA procedure (for BSR) is triggered for providing the MAC CE reporting the measurements.

### 2.1.8. Transmitting the MAC CE for LTM Reporting

RAN2 agreed MAC CE is used as a reporting container. The LTM report MAC CE should be able to be provisioned on any UL grant.

Proposal 25: The LTM Report MAC CE can be multiplexed in any available UL grant.

Proposal 26: If the SR for indicating reporting event is configured and UE has available UL grant, the UE is not required to transmit the SR and it can multiplex the MAC CE on any available UL grant.

# 3	Conclusion

This paper discussed RAN2 aspects of LTM L1 measurement framework which shall be enhanced as a part of Release 19. The following observations have been made:

Observation 1: the indicated TCI State refers to the use of unified TCI State framework for the serving cell.

Observation 2: the serving cell may operate using R15/16 TCI state framework i.e. non-unified TCI state. Thus, to support event triggered LTM reporting different network deployments, the activated TCI state for PDCCH should be used for deriving the current beam RS.

Observation 3: When the serving beam and candidate beam are associated with different RS types it is unclear how event evaluations for LTM3 and LTM5 should be conducted and what RS information should be reported.

Observation 4: L1 would be responsible for either the indications to MAC layer or for providing the measurement results.

Observation 5: As the LTM reporting events may be triggered for multiple candidate cells, the report format should support reporting for multiple candidate cells in a single MAC CE.

Observation 6: In L1 UCI reporting format the number of configured RS in the resource set is used to determine the bit width of the Resource identifier field.

Observation 7: the serving cell RS index/identifier should be implicitly known by the network thus it could be omitted from the MAC report

Observation 8: Although events such as LTM 3/5 make already the comparison between current beam RS and candidate RS i.e. so that reported RS quality  are relative to the current beam quality it is beneficial to include the serving cell quality to provide more information to network.

And the following has been proposed:

Proposal 1: From RAN2 perspective, for LTM 3/4/5 event evaluation the current beam RS for serving cell measurements can be derived based on activated TCI state for PDCCH (R15/16).

Proposal 2: RAN2 should study whether and how the same type of RS can be used for both the serving and candidate cells for events LTM3 and LTM5, and the implications for reporting.

Proposal 3: The event evaluation is performed at MAC layer by having a beam specific time to trigger (TTT) timer.

Proposal 4: If the serving cell reference (indicated beam) for event evaluation changes, the evaluation for the candidate RS is restarted.

Proposal 5: Support report on leave for LTM L1 measurement reporting framework.

Proposal 6: RAN2 considers the aforementioned list of open issues when designing the report on leave for L1 measurement reporting.

Proposal 7: Support periodic reporting of the LTM event triggered report, with configurable periodicity and/or number of repetitions.

Proposal 8: The LTM Report MAC CE includes indication of a candidate cell identifier that points to the configured LTM candidate identifier(s).

Proposal 9: The LTM Report MAC CE supports reporting of candidate RSs for multiple candidate cells in a single MAC CE.

Proposal 10: the LTM Report MAC CE includes an identifier that identifies at least the reporting event type (e.g. LTM3) and the associated reporting configuration (and resource configuration) for which the reporting is performed.

Proposal 11: The LTM Report MAC CE includes RS index field wherein the RS index field size is fixed 6 bits (to reflect up to 64 RS indices).

Proposal 12: The LTM Report MAC CE includes RS index field that encodes resource indicator pointing to the reference signals of the candidate cell in the LTM-CSI-ResourceConfig associated with the reporting configuration.

Proposal 13: The resource indicator ID space is reused for each candidate cell in the LTM-CSI-configuration associated with reported event.

Proposal 14: The LTM Report MAC CE supports reporting of variable number of candidate RS per candidate cell(s), up to the configured number of RSs.

Proposal 15: The LTM Report MAC CE includes RSRP value associated with each reported RS identifier.

Proposal 16: RAN2 to study the RSRP value reporting using differential RSRP mechanism in MAC CE.

Proposal 17: Include serving cell RSRP measurement for events for which the current beam RS is used for event evaluation i.e. LTM 2/3/5

Proposal 18: UE might include RS associated with indicated (and activated TCI states) in the event based measurement reporting.

Proposal 19: Support truncation of LTM report MAC CE. Details FFS (once the work on MAC CE format matures).

Proposal 20: An SR ID can be associated with event triggered reporting. The SR can be used to indicate a reporting event/request for UL resource for provision of the MAC CE

Proposal 21: The SR ID for event triggered reporting can be shared with SR for other purposes.

Proposal 22: The SR for indicating event triggered reporting is optionally configured.

Proposal 23: If no SR for indicating event triggered reporting is configured, the UE can use normal SR, if configured.

Proposal 24: If SR is not configured, CBRA procedure (for BSR) is triggered for providing the MAC CE reporting the measurements.

Proposal 25: The LTM Report MAC CE can be multiplexed in any available UL grant.

Proposal 26: If the SR for indicating reporting event is configured and UE has available UL grant, the UE is not required to transmit the SR and it can multiplex the MAC CE on any available UL grant.

# References

[1]	RP-240299	Revised WID: NR mobility enhancements Phase 4

[2] R2-2403731 	Report from session on V2X/SL, R19 NES and MOB 3GPP TSG-RAN WG2 Meeting #125bis Changsha, China, April 15 – 19, 2024

[3] R2-2405701	Report from session on V2X/SL, R19 NES and MOB 3GPP TSG-RAN WG2 Meeting #126	 Fukuoka, Japan, May 20 – 24, 2024

[4] 3GPP TS 38.331 NR RRC NR; Radio Resource Control (RRC); Protocol specification, v18.3.0 (September 2024)

[5] R1-2405716	Correction on QCL assumption after LTM cell switch command 3GPP TSG RAN WG1 #117	Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024