3GPP TSG-RAN WG2 Meeting #127bis	R2- 2408949

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.4.2

Source:	InterDigital, Inc.

Title:	Discussion on LP-WUS operation in RRC_IDLE/INACTIVE modes

Document for:	Discussion

# 1. Introduction

In RAN2#127 [1], following workding assumption and agreement were made:

Agreement LP-WUS configuration (Whether dedicated configration is needed or not) Baseline: The network does not need to be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS. LP-WUS entry/exit condition Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ. The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

In this contribution, we discuss LP-WUS operation in RRC_IDLE/INACTIVE mode.

# 2. Discussion

Dedicated configuration for LP-WUS

For LP-WUS monitoring, an LP-WUR UE needs to receive LP-WUS configuration (e.g., LP-SS configuration, entry/exit condtions). RAN2 agreed to receive LP-WUS configuration via system information, however, a necessity of dedicated configuration is still FFS. From our view, dedicated LP-WUS configuration from a gNB is not valid since an idle/inactive LP-WUR UE is allowed to camp on another cell via cell (re-)selection procedure [3].

Proposal 1: Do not support dedicated LP-WUS configuration for idle/iacitve LP-WUR UEs.

Dedicated (de-)activation signaling for monitoring LP-WUS

When the UE is monitoring LP-WUS, the UE needs an additional latency to wake up MR and monitor PO from LP-WUS monitoring. The additional latency can be problematic for some use cases which are delay sensitive, e.g., the additional latency can be longer than the required delay. To avoid the additional latency, UE dedicated (de-)activation signaling needs to be supported. For example, when a UE is expected to receive DL data with delay sensitive traffic (e.g., XR) frequently, then the network can deactivate the LP-WUS monitoring (i.e., not to start LP-WUS monitoring) via the dedicated deactivation signaling so that the UE can receive the delay sensitive data  without additional wake-up delay from LP-WUS monitoring. This (de-)activation signlaing can be configured with the RRC release message, similar to one of the legacy configuration using for idle/inactive state (e.g., suspendConfig, cellReselectionPriorities) via the RRC release message.

Proposal 2: Support dedicated LP-WUS monitoring (de-)activation per UE via RRC release message.

Subgrouping for LP-WUS

In RAN1#118, the following agreements and summary were made on subgrouping [2].

Agreement For RRC idle/inactive state, support the following option for at least indicating subgroup information using LP-WUS: - Option 2: A LP-WUS indicates a codepoint value corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs - UE monitoring one or more MOs (up to X MOs) for the same beam within an LO is supported - Value of X is larger than 1. FFS on additional details of X. Agreement For the maximum number of subgroups per PO (X), down-select from the following options in RAN1#118bis: - Option 1: X = 8 - Option 2: X = 16 - Option 3: X = 32 - Option 4: X = 64 - Option 5: X = 128 - Option 6: X = 256 For decision in RAN1#118bis, companies are encouraged to provide the maximum number of information bits per LP-WUS (Z), the number of OFDM symbols occupied by LP-WUS per MO, the number of MOs for their preferred option.

Following the legacy PEI, subgrouping is an efficient power saving operation to reduce false alarms and monitor paging occasions in IDLE/INACTIVE modes. Similar to the legacy subgrouping method, subgroup based wake-up indication and corresponding LP-WUS information are supported by LP-WUS. The contents of the indication is up to RAN1 since the LP-WUS signal design and payload have been discussed by RAN1.

Proposal 3: For PEI based subgrouping, detailed LP-WUS information and information delivery method (e.g., bitmap, codepoint, maximum number of subgroups per PO) are up to RAN1 decision.

Confirmation for entry/exit condtions

In RAN1#118, the following agreement and the conclusion were made for idle/inactive UEs [2].

Agreement Send an LS to RAN2 and RAN4 to convey the following In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. Request RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs. Final LS in R1-2407559. Conclusion RAN1 will not initiate work on entry/exit conditions based on RRM measurement and RRM measurement offloading/relaxation conditions unless triggered by RAN2 and RAN4

In the last RAN2 meeting [1], RAN2 made an working assumption for entry/exit conditions based on LR-based thresholds with potential revision if RAN1/RAN4 reached different conclusions. As RAN1 will not initiate work on entry/exit conditions unless triggered by RAN2 according to the conclusion, RAN2 should be a working group which needs to discuss whether the working assumption is confirmed or not.

According to the discussions so far, there are many scenarios which require LR measurement envolvement. For example, as captured in RAN1 agreement, LR may not support LP-WUS reception on all the bands supported by MR. In this case, as MR measurement can be quite different from LR measurement, solely based on MR-based threshold may result in frequent ping pong issues between starting LP-WUS monitoring and waking up the MR. To avoid the issues, utilization of LR-based thresholds should be confirmed in RAN2.

Proposal 4: RAN2 confirms the current working assumption for entry/exit conditions based on the RAN1 conclusion. (WA: If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds).

Entry condition with MR and LR measurements

Since both measurements (e.g., serving cell) via MR and LR can be considered to determine satisfying the entry condtion [1], RAN2 considers further whether/how to use both metrics via MR and LR (e.g., RSRP and (LP-)RSRP) for the determination when both MR-based and LR-based thresholds are configured. If RAN2 decides to consider always two metrics, additional power consumption with LR measurement and complexity of UE behavior (i.e., when to measure LP-RSRP via LR) cannot be avoided.

As a solution, RAN2 considers some conditions when to use metric(s) via MR only or both MR and LR for determining whether satysifying the entry condtion. For example, RAN2 can consider to use metric via MR only when the measurement configuration/parameters for MR and LR is similar (e.g., similar bandwidth, carrier frequency, band) that assumes results of MR measurement is similar to the results of LR measurement (e.g.,  LR-based threshold assumed to be satisfied upon MR-based threshold is satisfied by MR measurement). In another example, LR-based thresholds can be considered only when LR channel needs to be evaluated. For example, if MR measurement is good enough then the UE can start LP-WUS monitoring without utilizing LR-based thresholds.

Proposal 5: For satisfying LP-WUS entry condition, support conditions whether to use MR measurement only or both MR measurement (e.g., RSRP) and LR measurement (e.g., LP-RSRP). Detail conditions are FFS.

Exit condition with LR measurement

RAN2 agreed that entry/exit condition for LP-WUS monitoring is configured by SIB. To support entry/exit conditions for LP-WUS monitoring, the UE is configured with conditions. For example, a threshold of LP-RSRP/LP-RSRQ value of received LP-SS can be configured. The satisfied condition (e.g., above the threshold) of received LP-SS can be regarded as successful reception and the UE can keep monitoring LP-WUS. In contrast, if the condition is not satisfied by detecting number of reception failures (e.g., below the threshold), the UE stops LP-WUS monitoring and starts to monitor the legacy PO.

Proposal 6: For satisfying LP-WUS exit condition, support number of LP-WUS reception failures.

Measurement both NR-SS and LP-SS

While measuring of entry/exit conditions, a UE being capable of OFDM-based LP-WUR can measure both NR-SS (existing SSB) and LP-SS (i.e., if supported) simultaneously. The measured values can be used for the decision of entry/exit condition. Since the qualities of NR-SS and LP-SS are different metrics (e.g., RSRP and LP-RSRP, respectively), RAN2 should discuss how to use the different metrics for the decisions. For example, using the higher quality or combined quality.

Proposal 7: For OFDM-based WUS, support measuring both NR-SS and LP-SS quality measurement for entry/exit conditions of LP-WUS monitoring. How to use two different metrics (e.g., NR-SS and LP-SS) for decision is further discussed.

Information for LP-WUS configuration

According to the evaluation results from the TR [4], it is obvious that offloading RRM measurement from MR to LP-WUS is essential to avoid frequent wake up of MR from ultra deep sleep and achieve power saving gain from LP-WUS. Considering limited capacity of LP-WUR and reduced power consumption, providing information on neighboring cells (e.g., whether to provide LP-WUS related configuration) should be considered for LP-WUR based RRM measurement. Also, this information may be beneficial that the UE can (re-)select cell with supporting LP-WUS configuration while performing the cell (re-)selection procedure.

Proposal 8: Information on neighboring cells (e.g., LP-WUS capable) should be supported.

# 3. Conclusion

In this contribution, the following proposals were made:

Proposal 1: Do not support dedicated LP-WUS configuration for idle/iacitve LP-WUR UEs.

Proposal 2: Support dedicated LP-WUS monitoring (de-)activation per UE via RRC release message.

Proposal 3: For PEI based subgrouping, detailed LP-WUS information and information delivery method (e.g., bitmap, codepoint, maximum number of subgroups per PO) are up to RAN1 decision.

Proposal 4: RAN2 confirms the current working assumption for entry/exit conditions based on the RAN1 conclusion. (WA: If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds).

Proposal 5: For satisfying LP-WUS entry condition, support conditions whether to use MR measurement only or both MR measurement (e.g., RSRP) and LR measurement (e.g., LP-RSRP). Detail conditions are FFS.

Proposal 6: For satisfying LP-WUS exit condition, support number of LP-WUS reception failures.

Proposal 7: For OFDM-based WUS, support measuring both NR-SS and LP-SS quality measurement for entry/exit conditions of LP-WUS monitoring. How to use two different metrics (e.g., NR-SS and LP-SS) for decision is further discussed.

Proposal 8: Information on neighboring cells (e.g., LP-WUS capable) should be supported.

# 4. References

[1] Chairman’s notes, 3GPP TSG RAN WG2 #127.

[2] Chairman’s notes, 3GPP TSG RAN WG1 #118.

[3] 3GPP 38.304, NR User Equipment (UE) procedures in idle mode and RRC inactive state.

[4] TR 38.869 V2.0.0 (2023-12), “Study on low-power wake up signal and receiver for NR (Release 18)”.