3GPP RAN WG2 Meeting #127	R2-2407188

Maastricht, Netherlands, August 19th – 23rd 2024

Agenda Item:	8.8.4

Source:	InterDigital

Title:	Support for broadcast service in non-terrestrial networks

Document for:	Discussion, Decision

# Introduction

An objective for Rel-19 NR non-terrestrial networks (NTN) is to provide enhanced signaling support for multicast/broadcast services (MBS). MBS provides an important add-value for NR NTN system, leveraging the large coverage of the NTN compared to TN. Terrestrial MBS features are equally available for NR NTN in the 5G specifications, but for some cases the intended service area is expected to be smaller than the coverage of a Uu cell, requiring enhancements to notify the service area of a Broadcast service.

Detailed objectives from the WID [1] are as follows:

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

This contribution discusses necessary enhancements to support broadcast service in non-terrestrial networks, specifically the impact of NTN characteristics on broadcast services such as cell size and cell movement.

# Discussion

## Broadcast service area description

Agreements discuss how the geographical service area interacts with cell coverage [2]:

- ”We will cover at least the case where the indicated intended service area covers a portion of a NTN cell”

- ”The intended service area can cover the area of more than one NTN cells (or portions thereof)”

Knowing which cell(s) serve the intended service area is important for content reception and service continuity. In legacy MBS the service area was defined by a list of cell IDs/tracking areas so there was no issue, however, it is unclear how the new geographic description of service area will be incorperated into the existing procedure. For example, if the geographic information and cell ID list are both available for a service area, the UE can reintepret the cell ID list as meaning the cells serving the intended service area instead of defining it. If the intention was to have the geographical description replace entirely the cell ID list then additional information may be needed.

Observation 1:	How the geographic description of service area interacts with the legacy MBS cell ID/TAI list needs to be clarified (e.g., whether the UE reinterprets the cell ID list or it is replaced entirely).

As a first step, RAN2 should clarify that at least which cells are serving the geographic service area are known to the UE. Whether this is via the existing procedure or some other means can be discussed further, possible pending progress on how the geographic area is signalled to the UE.

Proposal 1:	Which cell(s) serve a geographic service area is indicated to the UE. FFS whether the legacy cell ID list is reused, or new signalling is introduced.

In RAN2#126 [3] it was agreed that both (quasi)earth fixed and earth moving cells are supported for MBS broadcast service. Supporting NGSO satellites requires consideration that the cells move continuously relative to earth. For example, a cell within the boundaries of a country at time T1 may cross into a region where the MBS session is not applicable at time T2. This means that the cells which make up an MBS session area will change over time due to satellite movement, requiring methods to dynamically update the MBS session area.

Observation 2:	Satellite movement in NSGO deployments could cause the cell(s) serving a service area to constantly change.

Figure 2: Due to satellite movement, the cells within a session area will change over time.

To avoid possible service continuity issues, the UE should be aware of how long each cell is serving the intended service area. Whether the existing mechanisms (e.g., t-Service) are sufficient can be further studied, as well as where this information is provided (e.g., within the MBS cell ID list or separately via SIB19).

Proposal 2:	In NGSO deployments, the duration a cell serves a geographic service area is indicated to the UE. FFS whether t-Service is reused, or new signalling is introduced.

The Rel-17/18, UE can trigger various actions (e.g., measurements) prior to cell coverage expiry to help the UE find a suitable cell before coverage disappears. RAN2 may also study whether similar enhancements are needed in NGSO deployments to ensure service continuity during a broadcast service. It is noted that the service continuity discussion may also require coordination with RAN3 (e.g., for network signaling aspects).

Proposal 3:	Study whether existing procedures are sufficient to ensure broadcast service continuity in NGSO deployments.

## Service area information provisioning

In RAN2#127, the method of service area information provisioning was discussed in offline [307] [4] and then the following agreement was made [5].

2. RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions.

The RAN2 confirmation implies that UE has already known the intended service area for a specific broadcast service prior to MCCH acquisition and so the broadcast service and the associated intended service area need to be signalled to the UE in advance. Thus SIB20 should signal the service area information for each potential TMGI so that UE knows the intended service area for a specific service and determines when UE acquires MCCH.

Proposal 4:	SIB20 is used to provide the service area information.

# Conclusion

In this contribution the following observations and proposals are made regarding support for MBS in NTN

Observation 1:	How the geographic description of service area interacts with the legacy MBS cell ID/TAI list needs to be clarified (e.g., whether the UE reinterprets the cell ID list or it is replaced entirely).

Observation 2:	Satellite movement in NSGO deployments could cause the cell(s) serving a service area to constantly change.

Observation 3:	Restricting broadcast transmission to an intended service area is highly limiting to implementation, complex, and suboptimal for DL coverage.

Observation 4:	Describing a service area geographically means that even though a serving cell ID belongs to a service area, the UE may not be located in the right area of the cell to access the service.

Observation 5:	Defining service area(s) within a cell may require updated UE location information to be acquired and/or verified to determine which service area(s) are applicable for the UE.

Proposal 1:	Which cell(s) serve a geographic service area is indicated to the UE. FFS whether the legacy cell ID list is reused, or new signalling is introduced.

Proposal 2:	In NGSO deployments, the duration a cell serves a geographic service area is indicated to the UE. FFS whether t-Service is reused, or new signalling is introduced.

Proposal 3:	Study whether existing procedures are sufficient to ensure broadcast service continuity in NGSO deployments.

Proposal 4:	SIB20 is used to provide the service area information.

# References

RP-241789 "Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3", Thales, CATT

R2-2404102 RAN2#125bis chair notes

R2-2406202 RAN2#126 chair notes

R2-2407619 [Offline 307] [R19 NR NTN] service area information (Apple)

RAN2#127 chair notes