3GPP TSG-RAN WG2 Meeting #127bis	R2-2408959

Hefei, China, Oct 14th – 18th, 2024

# Agenda item:	8.6.2

# Source:	ETRI

# Title:	Further Discussion to Support the inter -CU LTM

# Document for:	Discussion, Decision

- Introduction

In RAN2#127 meeting, RAN2 reached the following agreements to support inter-CU LTM [1]:

Agreements on inter-CU LTM Reference configuration: - Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported. CSI resource and report configuration: - The Rel-18 signaling structure for LTM CSI resource and report configuration is reused for inter-CU LTM, i.e. a common CSI resource configuration and cell-specific CSI report configuration. - The source CU is responsible to generate the common CSI resource configuration. Early DL sync: - For inter-CU LTM, the R18 candidate TCI State activation/deactivation design (including MAC CE and related UE handling) is reused. Inter-CU LTM switch: - For inter-CU LTM cell switch, it’s the source DU that triggers the MAC CE and informs the source CU about the target LTM cell. RLC and PDCP re-establishment in inter-CU LTM: - If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. As baseline introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update, however dependent on SA3 response, we can revisit it. Handling of candidate configuration after inter-CU LTM cell switch: - As in Rel-18 LTM, the UE keeps its LTM candidate cell configurations after at least a inter-CU LTM cell switch procedure where the UE is not configured with DC, unless these are explicitly released by the network. DRB/PDU session mismatch in subsequent inter-CU LTM: - RAN2 understand NW implementation can handle the concern raised in P6, R2-2407421. LTM and L3 HO: - L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM): - When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network. - The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations. - For the execution order between CHO and LTM, Rel-18 principle is applied. Inter-CU SCG LTM: - Inter-CU SCG LTM preparation can be initiated by source SN. - The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message. - For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN. - RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works. - Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19. Inter-CU MCG LTM: - SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration). - Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied). - For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

In this paper, we discuss some issues to be discussed further to support conditional intra-CU LTM.

- Discussion

Update of candidate LTM cell configuration update

When discussing stage-2 signaling procedure for inter-CU LTM, the focus was mainly on the preparation of inter-CU LTM configuration, and the sharing or update of the LTM configuration was not discussed in detail. Unlike Rel-18 intra-CU LTM, signaling for updating candidate LTM cell configurations between network nodes needs to be defined in Rel-19 inter-CU LTM. Some papers about the need for sharing or updating the LTM configuration has been proposed and some of them was discussed and agreed. The operation where a source gNB-CU informs candidate gNB-CU(s) of the common CSI resource configuration can also be considered an example of LTM configuration update. A change in PDU sessions can be another use case for the LTM configuration update. A UE can keep the LTM configuration relatively long time due to the support of subsequent LTM and may add a new PDU session while the LTM configuration is still active. In such cases, it is necessary to verify whether the existing candidate cells can support the new PDU session, and based on the results, an update of the existing LTM configuration may be required.

While the detailed update procedure for the LTM configuration should be handled by RAN3, it is necessary to discuss, from the RAN2 perspective, when and how the LTM configuration should be updated. The results of this discussion can be informed to RAN3 for consideration when designing the signaling process. The followings can be discussed regarding the update of LTM configuration.

- Message for LTM configuration update: The HO REQUEST/ACKNOWLEDGE messages can be reused for updating the candidate LTM configuration, or new messages can be defined specifically for this procedure.

- Sharing of the updated LTM configuration among candidate cells: The timing for sharing the prepared LTM configuration among candidate cells has not yet been determined. One option is for the source gNB-CU to inform the target gNB-CU of the LTM configuration when a cell switch occurs. Alternatively, the source gNB-CU could share the LTM configuration with candidate cells when notifying them of the common CSI resource configuration. Similarly, the updated LTM configuration could either be shared with the target cell at the time of the next cell switch or distributed to all candidate cells immediately after the LTM configuration is updated.

Proposal 1: The source gNB can trigger the update of the candidate LTM cell configurations while the candidate LTM cell configuration is active.

Proposal 2: RAN2 is kindly asked to discuss when and how the updated LTM configuration is informed to candidate gNB-CU(s).

Security Key Update

RAN2 has identified four potential solutions for security key updates during Inter-CU LTM and shared these with SA3 for further analysis of their security impacts. However, the impact of these solutions was not discussed within RAN2. While we need to wait for SA3's conclusions, it would be beneficial to analyze the impact of each solution from RAN2's perspective. Based on these discussions, we may be able to narrow down the candidate solutions before receiving feedback from SA3 or provide additional input to SA3.

For instance, the first option, where an NCC value or index is included in the cell switch command MAC CE, may have an impact on the cell switch decision procedure. Currently, the consensus is that the gNB-DU triggers the CSC MAC CE and then notifies the gNB-CU of the cell switch. However, the gNB-DU does not know whether the target cell is an intra-CU or inter-CU cell, nor does it have any security key information. If the first option is adopted, the gNB-DU would need to inform the gNB-CU of the cell switch before triggering the CSC MAC CE and only transmit the CSC MAC CE after receiving a response from the gNB-CU. Given the additional delay introduced by this cell switch decision procedure, the gNB-DU would either need extra information on NCC value (or index) for each LTM candidate inter-cell, or it would need to make the cell switch decision earlier.

Proposal 3: RAN2 to discuss the impact of each four options from RAN2 perspective and, based on the discussion, narrow down the candidate solutions or provide additional input to SA3.

- Conclusions

In this paper, we discuss further issues on inter -CU LTM related issue. Following are the proposals made in this paper:

Proposal 1: The source gNB can trigger the update of the candidate LTM cell configurations while the candidate LTM cell configuration is active.

Proposal 2: RAN2 is kindly asked to discuss when and how the updated LTM configuration is informed to candidate gNB-CU(s).

Proposal 3: RAN2 to discuss the impact of each four options from RAN2 perspective and, based on the discussion, narrow down the candidate solutions or provide additional input to SA3.

- References

- Chair notes RAN2#127;