3GPP TSG-RAN WG2 #127bis 	R2-2408961

Hefei, China, 14th – 18th October 2024

Agenda Item:	8.2.3

Source:	Ericsson

Title:	Discussion on DL messages for Ambient IoT UEs

Document for:	Discussion/Decision

# 1	Introduction

RAN2 has started work on the support of Ambient IoT devices WI (RP-234058). The SID [1] has the following objectives led by RAN2:

- RAN2-led: - Study and decide which functions are needed for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission, and study those functions. For example: - Paging - Random access - Data transmission, including necessary radio resource control aspects, respecting the limitation in the General Scope - Interactions with upper layers For functionalities not listed above, they are studied only if found essential.

In RAN2#127, the following agreements related to this were made:

1 Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices.

1 At least the following information are considered useful to be visible to the reader from CN: - The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful. - Targeted for one or more than one devices. - Approximate number of target devices (if available). FFS on mandatory/optional. 2 RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIOT interface as upper layer data.

In this contribution we continue to discuss downlink messaging for A-IoT devices and address the open issues from the previous meetings.

# 2	Discussion

## 2.1 Reader awareness

In RAN2#127, it was agreed that approximate number of targeted devices (if available) may be provided to the reader, so that the reader would be able to configure the parameters required for random access accordingly. Although, RAN2 agreed that it would be beneficial for the reader to know the A-IoT service type, the discussion on whether more information on command type, e.g. read/write/disable, would be beneficial has not been concluded. It may be beneficial for the reader to know whether the DL command requires UL response from the device(s) so that the reader can determine whether to allocate resources for the UL response.

The reader can determine whether there is a need to allocate resources for UL transmission, if it knows whether the DL command requires an UL response.

Moreover, it may also be beneficial for the reader to know whether the command is a write/disable command as the response after such command (if considered required) would be a message for acknowledgement. Hence the size of such acknowledgment message would be certain, i.e., known to the reader. The reader can then allocate resources in the UL accordingly.

The size of a response message to a write/disable command (if required) can be known to the reader.

The command type, e.g. read/write/disable, can be useful for the reader to know from UL scheduling standpoint.

Therefore, we propose the following:

The command type, e.g. read/write/disable, is considered useful for the reader to know.

Regarding the discussion on whether the information above is provided to the reader mandatorily or optional; we think that the information on service type, command type and whether service is intended for a single device or multiple devices shall always be provided to the reader mandatorily. However, for approximate number of targeted devices we think that it should be optional as it may not be always possible for the network to have such information.

It is should be mandatory that the reader is informed about the service type, command type and whether the DL message is addressed to single or multiple devices.

It should be optional that the reader is informed about the approximate number of targeted devices.

## 2.2 Multiple/subsequent A-IoT paging

In RAN2#127, it was agreed to support a mechanism for the reader to trigger multiple/subsequent A-IoT paging messages that are associated with the same request from the CN. To indicate that a paging message corresponds to the same or different request from the CN, when compared with the previous paging message, and ID can be provided in the paging message. This ID would then be associated with the request from the CN.

In the A-IoT paging message, reader provides an ID that is associated with the CN request.

When an A-IoT device receives a paging message and observes that the ID provided in the paging message is different from the one provided in the previous paging message, the A-IoT device performs the requested procedure regardless of whether it has completed the same procedure in the previous paging round. Otherwise, i.e., if the ID provided in the paging message is same as the one provided in the previous paging message, the device only needs to perform the requested procedure if it has not completed successfully in the previous paging round. If the A-IoT device has completed the procedure successfully in the previous paging round, it can ignore the paging message in this paging round. Note that successful completion may be either due to receiving an acknowledgment explicitly or implicitly after transmitting Msg3. This is a separate discussion and up to RAN2 to decide.

If paging message indicates an ID different that the one indicated in the previous paging round, i.e., paging messages are associated with separate CN requests, the device performs the requested procedure.

If paging message indicates the same ID with the one indicated in the previous paging round, i.e., paging messages are associated with same CN request, the device performs the requested procedure only if it has not completed the procedure successfully in one of the earlier paging rounds associated with the same CN request. Otherwise, the device can ignore the paging message.

How is the ID generated?

The ID can be generated either by the reader or the CN:

- When the reader receives a request from the CN, it generates the ID associated with the CN request and provides it in the paging message triggered by the CN request. In the subsequent paging message(s) triggered by the reader, same ID is provided in the paging message.

- The ID is generated by the CN and provided to the reader along with the CN request.

If the CN generates the ID, CN can coordinate so that the same ID is provided to multiple readers, if needed, along with the CN request, e.g., inventory. This would make it possible to avoid receiving redundant responses from an A-IoT device in case it happens to be covered by multiple readers. Otherwise, i.e., if the reader generates the ID, there may be a need for coordination between readers. We suggest capturing both options with pros and cons in the TR and inform SA2 and RAN3 that it may be beneficial if the ID is generated by the CN.

Capture the following options on how ID is generated and the pros and cons in the TR: a) the reader generates the ID based on the request from the CN, b) CN generates the ID and provides it to the reader along with the request.

What happens when the A-IoT device runs out of energy?

In case an A-IoT device will not be able to monitor the DL continuously, e.g., due to running out of energy, after responding to a paging request successfully, the device should perform the requested procedure received in a paging message after waking up, e.g., after harvesting sufficient energy, regardless of whether the ID provided in that paging message is different. This is since it would not be possible for the device to know “for certain” if there has been a wrap around for the ID in an asynchronous system. Note that in practice introducing a rather large value ranges for ID can help to a certain extent, but it may not be so beneficial in overall when impact on message size, device capability etc. are considered.

When an A-IoT device is not able to monitor the DL continuously, e.g., due to running out of energy, it performs the requested procedure received in a paging message after waking up, regardless of whether the ID provided in the paging message is different.

## 2.3 Multiple reader scenario for A-IoT devices

Multiple readers may be deployed to serve an area, and their coverage can overlap. For example, in [1], a reader can transmit paging messages to A-IoT device(s) whose responses are received by another reader.

One may also need to consider the case where an inventory procedure started by one reader is interleaved by another inventory procedure started by another reader assuming that there is no coordination between readers in time and/or frequency domains. For example, an A-IoT device which is in the coverage of two readers as shown in Figure 1, may receive paging messages from both readers assuming that there is no coordination between the readers in time and/or frequency domains. In the absence of a (reader) identifier, it will be difficult for the device to realize which reader it has responded, or it should respond to.

Figure 1 A-IoT device in the overlapping coverage.

Based on the following agreement: “A-IoT paging message indicate information from which the device can determine resources to be used for response”, the information within a paging message indicates the resources to be used when transmitting the response. For example, for slotted-Aloha access, where devices determine the slot for the uplink message transmission, an A-IoT device under coverage of multiple readers may get confused if subsequent paging messages are received. It can be beneficial to introduce a reader identifier for an A-IoT device to determine when/how to transmit. Such identifier can be provided in the paging message.

Figure 2 A-IoT device receiving paging message from multiple readers in overlapping coverage area.

Besides, in some use cases (e.g., for proximity determination), the CN may need the device to response to all A-IoT paging messages sent from multiple readers even if the A-IoT paging messages are associated with the same CN request. The CN could instruct the reader to include the reader ID in the A-IoT paging messages in case it only needs the device to respond once to the A-IoT paging messages associated with the same CN request and sent from different readers, otherwise the CN could instruct the reader to not include the reader ID in the A-IoT paging messages if it needs the device to response to the A-IoT paging messages sent from different readers even if the A-IoT paging messages are associated with the same CN request, in this case the device will always respond.

The coverage from multiple readers may overlap, e.g., where a transmitting reader can be different than the receiving reader, or an inventory procedure started by one reader can be interleaved by another inventory procedure started by another reader.

In some use cases (e.g., for proximity determination), the CN may need the device to respond to the A-IoT paging messages sent from multiple readers even if the A-IoT paging messages are associated with the same CN request.

Study if it would be beneficial to introduce a reader ID that can be provided in the paging message.

## 2.4 Resource information in A-IoT paging message

RAN2 has agreed that A-IoT paging message shall indicate resource information for UL response transmission:

AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion.

Furthermore, RAN1#116bis made the following related agreements regarding D2R multiple access:

Agreement (from agenda item 9.4.2.1) Study time-domain multiple access of D2R transmissions. Further details, including pros/cons, are FFS. Agreement (from agenda item 9.4.2.1) Study frequency-domain multiple access of D2R transmissions, at least by utilizing a small frequency-shift in baseband. Further details, including pros/cons, are FFS. Agreement (from agenda item 9.4.2.1) Whether code-domain multiple access is feasible and necessary for D2R transmissions for all devices is FFS.

Multiple access triggered by the reader can be contention free or contention based and in both cases the reader needs to provide the resources time/frequency in the A-IoT paging message. Based on RAN1 agreements, the D2R response triggered by the reader can be in time and frequency domain. The reader can allocate a specific time-frequency resource (for contention free case) or a set of time-frequency resources that the devices can choose from (for contention-based case). The UL resources provided in the A-IoT paging message may depend on reader’s awareness of the number of devices in its vicinity i.e., if the reader is provided a rough number of targeted devices.

The FFS in the related agreement from the last RAN2 meeting intends to address how to provide those resources and what kind of resources can be provided.

In an A-IoT paging message, it should be possible to allocate dedicated time and/or frequency resources for a single or multiple devices.

# 3	Conclusion

In this contribution we discussed downlink/paging messages for A-IoT devices. Based on the discussion in the previous sections we made the following observations:

Observation 1	The reader can determine whether there is a need to allocate resources for UL transmission, if it knows whether the DL command requires an UL response.

Observation 2	The size of a response message to a write/disable command (if required) can be known to the reader.

Observation 3	The command type, e.g. read/write/disable, can be useful for the reader to know from UL scheduling standpoint.

Observation 4	The coverage from multiple readers may overlap, e.g., where a transmitting reader can be different than the receiving reader, or an inventory procedure started by one reader can be interleaved by another inventory procedure started by another reader.

Observation 5	In some use cases (e.g., for proximity determination), the CN may need the device to respond to the A-IoT paging messages sent from multiple readers even if the A-IoT paging messages are associated with the same CN request.

Based on the discussion in the previous section, we propose the following:

Proposal 1	The command type, e.g. read/write/disable, is considered useful for the reader to know.

Proposal 2	It is should be mandatory that the reader is informed about the service type, command type and whether the DL message is addressed to single or multiple devices.

Proposal 3	It should be optional that the reader is informed about the approximate number of targeted devices.

Proposal 4	In the A-IoT paging message, reader provides an ID that is associated with the CN request.

Proposal 5	If paging message indicates an ID different that the one indicated in the previous paging round, i.e., paging messages are associated with separate CN requests, the device performs the requested procedure.

Proposal 6	If paging message indicates the same ID with the one indicated in the previous paging round, i.e., paging messages are associated with same CN request, the device performs the requested procedure only if it has not completed the procedure successfully in one of the earlier paging rounds associated with the same CN request. Otherwise, the device can ignore the paging message.

Proposal 7	Capture the following options on how ID is generated and the pros and cons in the TR: a) the reader generates the ID based on the request from the CN, b) CN generates the ID and provides it to the reader along with the request.

Proposal 8	When an A-IoT device is not able to monitor the DL continuously, e.g., due to running out of energy, it performs the requested procedure received in a paging message after waking up, regardless of whether the ID provided in the paging message is different.

Proposal 9	Study if it would be beneficial to introduce a reader ID that can be provided in the paging message.

Proposal 10	In an A-IoT paging message, it should be possible to allocate dedicated time and/or frequency resources for a single or multiple devices.

# References

TR 38.848, “Study on Ambient IoT (Internet of Things) in RAN (Release 18)” V18.0.0 (2023-09)

RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, CMCC, Huawei, T-Mobile USA, Maastricht, Netherlands, March 18-21, 2024.