3GPP TSG-RAN WG2 #127-bis                                             	    R2-2408964

Hefei, China, October 14th –18th, 2024

Agenda item:	   8.6.4

Source: 		Qualcomm Incorporated

Title: 	Conditional intra-gNB LTM

Document for:	Discussion

# 1	Introduction

The WID for mobility enhancements includes the following objective [1]:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

This paper discusses aspects related to conditional intra-gNB LTM.

# 2	Unified LTM/C-LTM framework

## 2.1	C-LTM candidate cells are subset of LTM candidate cells

In L3 mobility, a UE may receive an L3 HO command towards a candidate cell for which the UE has previously received a stored CHO configuration. Similarly, the network should be allowed to trigger LTM towards a target cell that is a C-LTM candidate cell. This implies that C-LTM candidate cells are a subset of LTM candidate cells.

Observation 1: In L3 mobility, the network may trigger L3 HO towards a CHO candidate cell.

Proposal 1a: C-LTM candidate cells are a subset of LTM candidate cells. Network-triggered LTM towards a C-LTM candidate cell is always supported.

The UE determines which subset of the LTM candidate cells are also C-LTM candidate cells based on configuration. Specifically, if the LTM candidate cell configuration also includes configuration for C-LTM condition evaluation, then the LTM candidate cell is also a C-LTM candidate cell.

Proposal 1b: A UE identifies an LTM candidate cell as a C-LTM candidate cell if the LTM candidate configuration includes a configuration for C-LTM condition evaluation.

## 2.2	Subsequent C-LTM is baseline

The WID states that RAN2 should aim to support subsequent C-LTM. Since avoiding RRC configuration between cell switches is a key advantage of LTM, subsequent C-LTM should be the baseline.

Proposal 2a: Subsequent C-LTM is the baseline: UE does not release LTM/C-LTM configuration following a C-LTM cell switch.

Once a cell switch is triggered, it makes no difference whether that triggering is a network trigger or based on UE evaluation of conditions. For instance:

- In both cases, the UE applies the stored RRC configuration and sends the RRC complete message.

- In both cases, the target cell notifies the gNB-CU of the UE’s successful access

- In both cases, the gNB-CU switches the UP datapath towards the target cell.

This implies the UE should be able to perform any sequence of network-triggered LTM/C-LTM cell switch executions in any order.

Proposal 2b: RAN2 to support a unified framework for LTM and C-LTM, where the UE may perform any sequence of LTM/C-LTM cell switch executions among the candidate cells.

# 3	UE evaluated conditions

In L3 conditional mobility, events are reused from the event-triggered L3 measurement reporting framework. A similar approach can be followed for C-LTM, which has the advantage of simplifying specification and implementation efforts.

Proposal 3: RAN2 to reuse the LTM events for measurement enhancements as triggering events for C-LTM, including events LTM2/3/4/5. FFS additional events.

# 4	RACH-less C-LTM

A key advantage of LTM is that it supports RACH-less cell switch. This benefit should be retained for C-LTM.

Proposal 4: RAN2 to support RACH-less C-LTM. Details are FFS.

# Conclusion

This contribution discussed aspects related to conditional intra-gNB LTM. The following observations and proposals have been made:

C-LTM candidate cells are subset of LTM candidate cells

Observation 1: In L3 mobility, the network may trigger L3 HO towards a CHO candidate cell.

Proposal 1a: C-LTM candidate cells are a subset of LTM candidate cells. Network-triggered LTM towards a C-LTM candidate cell is always supported.

Proposal 1b: A UE identifies an LTM candidate cell as a C-LTM candidate cell if the LTM candidate configuration includes a configuration for C-LTM condition evaluation.

Subsequent C-LTM is baseline

Proposal 2a: Subsequent C-LTM is the baseline: UE does not release LTM/C-LTM configuration following a C-LTM cell switch.

Proposal 2b: RAN2 to support a unified framework for LTM and C-LTM, where the UE may perform any sequence of LTM/C-LTM cell switch executions among the candidate cells.

UE evaluated conditions

Proposal 3: RAN2 to reuse the LTM events for measurement enhancements as triggering events for C-LTM, including events LTM2/3/4/5. FFS additional events.

RACH-less C-LTM

Proposal 4: RAN2 to support RACH-less C-LTM. Details are FFS.

# References

- RP-242356; Revised Work Item: NR mobility enhancements Phase 4; 3GPP TSG RAN meeting #105; Melbourne, Australia, Sep 9-12, 2024