3GPP TSG-RAN WG2 Meeting #127bis	R2-2408971

Hefei, China, Oct 14th – 18th, 2024		Revision of R2-2407537

Agenda item:	8.9.2

Source: 	Sequans Communications

Title: 	Support of Store & Forward

Document for:	Discussion and Decision

# Introduction

During previous meeting, RAN2 made the following agreements:

Agreements:

1.	RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options)

2.	RAN2 will consider both single satellite pass and multiple satellite pass scenarios

3.	RAN2 will consider both MO and MT data within scope

4.	UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature)

5.	RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected.

7.	RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

In this contribution, we address some aspects related to support of Store & Forward from AS point of view.

# Discussion

## S&F mode usage

From SA2 architecture, the NW should inform about whether S&F Satellite operation is applied. In our understanding, this corresponds to the fact that the satellite would operate in “S&F mode” rather than in “normal mode”. We assume this is supposed to match with the availability of a feeder link, i.e. the satellite operates in “S&F mode” when no feeder link is present while it operates in normal mode when the feeder link is available. However, there is no need to make such linkage; a satellite could e.g. always operate in S&F mode independently of feeder link presence (possibly because it can’t transmit all the data stored). That is to say, we believe it makes more sense to consider an indication that the satellite operates in S&F mode, rather than an indication that the feeder link is available.

Observation 1: It is more generic to use a “S&F mode” indication, rather than a “feeder link available” indication

It is also important to understand the rationale of this indication. This is not detailed in SA2 conclusions. In our understanding, this indication could be used e.g.:

- From AS perspective:

  - To bar UEs not supporting S&F mode

- From NAS perspective:

  - For NW selection:  As S&F mode is a degraded form or service, this might be considered while performing PLMN selection

- From application perspective: for application transport profile selection

  - E.g. TCP should be avoided in S&F mode, instead UDP/SMS etc should be used

We discuss barring of UEs not supporting S&F mode later. To allow possible usage by NAS/application, we think the indication should be forwarded to upper layers.

Proposal 1: The “S&F mode” indication is forwarded to upper layers for possible usage by NAS/Application

The NAS behavior will be different if using split MME architecture or Full CN. Hence, the S&F mode could further indicate the supported architecture. This would be useful if e.g. separate UE NAS capabilities are defined for both architectures (otherwise this could be directly indicated by NAS signaling). In our view, this is more up to SA2 discussions, and they will notify RAN2 if this is required.

Similarly, if e.g. multiple PLMNs/GW would need to be supported, the indication would need to be per PLMN. But this would be up to SA2 to decide.

## S&F mode switching

A straightforward approach for the “S&F mode” indication would be to broadcast one bit. However, this has some drawbacks.

In general, whenever the feeder link is available, it would make sense for the eNB to indicate “normal mode” (i.e. to not indicate “S&F mode”), given the service provided would then be “Real Time” and much more efficient. In normal mode, the eNB could offer services to UEs not supporting S&F.

We assume this is also SA2 understanding and request, as they indicate: “When feeder link is not available and the network supports S&F operation, the network shall be able to inform UE(s) whether S&F Satellite operation is applied”. However this will introduce some additional complexity, hence we propose to confirm this understanding.

Proposal 2: Support switching between “S&F mode” and “normal mode”

Depending of feeder link availability, that would lead to switch continuously between “S&F mode” and “normal mode”. Some UEs might see always “S&F mode”, some “normal mode”, and some might always see the switch as the satellite crosses the sky.

As a result, if S&F operation mode switching is supported, we believe the switching should not trigger SI change notification. This would be both costly for NW resources, and for UE power consumption.

Observation 2: Toggling a “S&F mode” bit would yield signaling overhead/UE power consumption

Instead of broadcasting a “S&F mode” bit, an alternative would be to indicate the S&F mode time range (e.g. start/stop times of as UTC time). This avoids SI change overhead. This can also be used for neighbor cells.

Proposal 3: Consider broadcasting “S&F mode” time range instead of a “S&F mode” bit, to avoid SI change notification (e.g. absolute start/duration)

## Handling of UEs not supporting S&F

An eNB operating in S&F mode provide a degraded service compared to normal mode. In general, it seems difficult for a UE not supporting S&F to access.

However, there are some scenarios/use cases where a S&F system might accommodate even UEs not supporting S&F. For instance, it seems SMS service could be supported by Full CN architecture. Similarly, if an operator deploys a specific delay tolerant service based on UDP, the full CN architecture could support that.

Observation 3: In some scenarios, UEs not supporting S&F could still be accommodated by S&F eNB

Hence, we believe it should be up to NW deployment to allow or disallow access of UEs not supporting S&F.

Proposal 4: It should be up to NW deployment to allow/disallow access of UEs not supporting S&F

UEs not supporting S&F are twofold:

- Legacy UEs

- R19 UEs not supporting S&F

Regarding legacy UEs, as reminded during previous meetings, they can be barred (if desired) by legacy cellBarred and cellBarredNTN. To allow R19 S&F capable UEs to connect, the legacy indication should be bypassed for R19 S&F UE, conditional to the presence of a new R19 barring indication (e.g., “cellBarredSF”).

Proposal 5: Legacy barring indication is skipped by R19 S&F UE upon presence of a new R19 barring indication, which is then used instead

Regarding R19 UEs not supporting S&F, in scenarios where it is desired to bar them, they could be barred either with the legacy indication (as legacy UEs would be barred too), or by using the “S&F mode” indication (i.e., whenever the eNB operates in “S&F mode”, such UEs would be barred).

Using the new dynamic S&F mode indication for barring the non-S&F R19 UEs has some advantages. This is not possible for legacy UEs, for which only legacy barring bit can be used. But for non-S&F R19 UEs, it may be much more efficient to use this indication, as the NW may not want to constantly toggle legacy barring bit on/off in case of S&F mode switching.

Proposal 6: Reuse “S&F mode” indication to bar R19 UEs not supporting S&F operation

Similarly as for legacy UEs, there may be some scenarios where such UEs could still access a S&F system. To enable above flexibility, the barring should be under NW control, e.g. there should be an indication on whether UEs not supporting S&F are barred or not during “S&F mode” operation (e.g., “SFbarringActivated”).

Proposal 7: Add an indication to control access of R19 UEs not supporting S&F operation (i.e. to activate barring upon S&F mode indication or not)

To enable this, R19 UEs not supporting S&F operation should also bypass the legacy barring indication.

Proposal 8: Legacy barring indication is also skipped by R19 non-S&F UE upon presence of the new R19 barring indication, which is then used instead

When the NW does not activate barring of non S&F R19 UEs, it is also expected to allow legacy UEs (the use case being e.g. only SMS service on full CN is deployed).

The following table summarizes the different scenarios:

|  | Scenario: in S&F mode, only allows S&F capable UEs | Scenario: in S&F mode, all UEs allowed |
| --- | --- | --- |
| Legacy UEs | In S&F mode: Disallow (legacy barring bit set) In normal mode: Allow (legacy barring bit unset) (up to NW to toggle if desired) | Allow (legacy barring bit not set) |
| R19 UEs not supporting S&F | In S&F mode: Disallow In normal mode: Allow (legacy barring bit bypassed as cellBarredSF is present, SFbarringActivated set) | In S&F mode: Allow In normal mode: Allow (legacy barring bit not set, SFbarringActivated unset) |
| R19 UEs supporting S&F | In S&F mode: Allow In normal mode: Allow (legacy barring bit bypassed as cellBarredSF is present) | In S&F mode: Allow In normal mode: Allow |

## Satellite ID

In SA2 solution, NAS can optionally provide list of Satellite IDs (for re-attempt of Attach/TAU procedure, for signalling/data, for S&F monitoring). RAN2 already broadcasts a Satellite ID, which is used for indexing the satellite assistance information for serving/neighbour cells.

#### –	SatelliteId

The IE SatelliteId is used to identify the satellite assistance information of the serving or neighbour satellites.

SatelliteId information element

-- ASN1START

SatelliteId-r18 ::= INTEGER (0..255)

-- ASN1STOP

It seems straightforward that NAS reuses the existing Satellite ID, i.e. there is no need to introduce new Satellite IDs for S&F. It also enables to reuse AS features such as discontinuous coverage which indicates when a given satellite will provide coverage.

Proposal 9: NAS Satellite IDs correspond to existing broadcasted SatelliteIds (no need for new parameter)

# Conclusion

In this contribution, we make the following observations and proposals:

Observation 1: It is more generic to use a “S&F mode” indication, rather than a “feeder link available” indication

Proposal 1: The “S&F mode” indication is forwarded to upper layers

Proposal 2: Support switching between “S&F mode” and “normal mode”

Observation 2: Toggling a “S&F mode” bit would yield signaling overhead/UE power consumption

Proposal 3: Consider broadcasting “S&F mode” time range instead of a “S&F mode” bit, to avoid SI change notification (e.g. absolute start/duration)

Observation 3: In some scenarios, UEs not supporting S&F could still be accommodated by S&F eNB

Proposal 4: It should be up to NW deployment to allow/disallow access of UEs not supporting S&F

Proposal 5: Legacy barring indication is skipped by R19 S&F UE upon presence of a new R19 barring indication, which is then used instead

Proposal 6: Reuse “S&F mode” indication to bar R19 UEs not supporting S&F operation

Proposal 7: Add an indication to control access of R19 UEs not supporting S&F operation (i.e. to activate barring upon S&F mode indication or not)

Proposal 8: Legacy barring indication is also skipped by R19 non-S&F UE upon presence of the new R19 barring indication, which is then used instead

Proposal 9: NAS Satellite IDs correspond to existing broadcasted SatelliteIds (no need for new parameter)

# References

- 3GPP TR 23.700-29 V19.0.0,	 “Study on integration of satellite components in the 5G architecture Phase 3”