3GPP TSG-RAN WG2 Meeting #127bis	R2-2408975

Hefei, China, 14 – 18 October 2024

Agenda item:	7.25.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on early implementation of R18 measurement gap enhancements

WID/SID:	NR_MG_enh2-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

RAN4 sent an LS to RAN2 in R2-2407930 (R4-2413936), requesting RAN2 to consider making the Rel-18 measurement gap enhancements associated with the features interRAT-NeedForInterruptionNR-r18 and nr-NeedForInterruptionReport-r18 early implementable starting from Rel-17.

These features were already added in Rel-18 based on the TS 36.331 CR 4929 (interRAT-NeedForInterruptionNR-r18) and TS 38.331 CRs 4063 and 4510 (nr-NeedForInterruptionReport-r18); however, at that time, it was not known that the features would need to be early implementable, so the normal steps taken to support early implementation in the specifications had not been taken:

-	Original CRs were not added to the “List of CRs Containing Early Implementable Features and Corrections” in Annex G of 36.331 and Annex C of 38.331.

-	Cover sheets of the original CRs did not have the magic sentence (“Implementation of this CR by a Release 17 UE will not cause compatibility issues”).

Although it would be possible to still add the relevant CRs to the list of early implementable features in the annexes of 36.331 and 38.331, it would be impossible to retroactively add the magic sentence to those original CRs, so two approaches were considered to support the requested early implementation of the features:

- Approach 1: “Re-add” the features in new CRs to ensure the magic sentence is captured

- Approach 2: List the features in Annex G of 36.331 and Annex C of 38.331, but skip the magic sentence

This paper presents these approaches based on an (informal) email discussion held between companies shortly before the RAN2#127bis meeting.

# 2	Discussion

## 2.1	Approach 1

Approach 1 followed the example of a similar request RAN4 had made earlier in the year requesting RAN2 to make the Rel-18 2Tx diversity feature early implementable (LS: R4-2403857).

At that time, to address the limitation that the original CRs were lacking the magic sentence, RAN2 agreed to new category B CRs (R2-2402031, R2-2402032), essentially re-adding the feature from scratch. To do so, the changes took the following steps:

- Delete the feature from 38.306 and 38.331

- Add the feature back to 38.306 and 38.331 immediately below the deletion (i.e. in the same CRs)

- Add the magic sentence in the cover sheets of the CRs

- Add the feature to Annex C (early implementable features) of 38.331

Approach 1 would have followed the same steps as above. However, in the case of nr-NeedForInterruptionReport-r18 and interRAT-NeedForInterruptionNR-r18, the NR and LTE specs are both impacted, which would have required changes to 36.306 and 36.331 on top of 38.306 and 38.331.

In the case of the 2Tx diversity feature, there was no associated procedures so the changes were considered quite simple. However, in the case of nr-NeedForInterruptionReport-r18 there is quite a lot of associated procedural and configuration-related text, so deleting and re-adding the additional text makes the change quite cumbersome. Despite the additional complexity, some companies thought Approach 1 could be advantageous by making it explicitly clear what procedures/configurations were associated with the nr-NeedForInterruptionReport-r18 feature, since the capability was originally added in a different CR from the procedural text and associated RRC configurations (ASN.1).

On the other hand, some companies questioned whether Approach 1 could be done in a backward compatible way at this point after specification freeze since it involved deleting and re-adding the ASN.1. (This question was not concluded, but it again shows the added complexity of this approach.)

Observation 1: Approach 1, despite capturing the magic sentence and being explicit about all aspects for early implementation, requires quite complicated CRs and the backward compatibility was not confirmed.

## 2.2	Approach 2

Approach 2 only requires updating the tables in Annex G and Annex C of 36.331 and 38.331, respectively. This approach avoids all of the complexities of Approach 1.

The main drawback of Approach 2 is that it skips having the magic sentence in the (original) CRs in which the features were added. However, this drawback can be mitigated by clarifying, e.g. in the cover sheets of the CRs in which the features are added to the annexes of the RRC specifications, that there are no interoperability concerns with early implementation of the features in Rel-17.

One other concern raised with this approach was related to the nr-NeedForInterruptionReport-r18 feature being implemented across multiple CRs (e.g. one CR for the capability and another CR for the procedures/configuration), where it was thought that there could be some ambiguity regarding what exactly a Rel-17 UE would need to implement to support this feature. To avoid this ambiguity it was decided that this could be clarified by listing both relevant CRs in the Annex C of 38.331 and clarifying that both CRs would need to be supported simultaneously.

With the above clarifications, there seemed to be good support for Approach 2 among companies, and CRs have been prepared in R2-2408976 (36.331) and R2-2408977 (38.331) to support the RAN4 request accordingly.

Observation 2: Approach 2 is straightforward and seems sufficiently clear for a Rel-17 UE to implement the features early.

Proposal 1: Agree to CRs in R2-2408976 and R2-2408977, to support Rel-17 early implementation of interRAT-NeedForInterruptionNR-r18 and nr-NeedForInterruptionReport-r18, respectively.

# 3	Conclusion

This document has made the following observations:

Observation 1: Approach 1, despite capturing the magic sentence and being explicit about all aspects for early implementation, requires quite complicated CRs and the backward compatibility was not confirmed.

Observation 2: Approach 2 is straightforward and seems sufficiently clear for a Rel-17 UE to implement the features early.

And proposed the following:

Proposal 1: Agree to CRs in R2-2408976 and R2-2408977, to support Rel-17 early implementation of interRAT-NeedForInterruptionNR-r18 and nr-NeedForInterruptionReport-r18, respectively.