3GPP TSG-RAN WG2 #127bis       			      		  R2-2408979

Hefei, China – October 14 - 18, 2024		             Revision of R2-2407191

Agenda item:	8.10.2

Source: 	Kyocera

Title: 	SON/MDT reports for LTM

Document for:	Discussion

# Introduction

For Rel-19 the WID included further enhancement of data collection for SON/MDT for LTM, CHO with candidate SCGs, subsequent CPAC [1].  In this contribution, information that should be needed in the LTM RLF report to assist the network with LTM HO optimization is discussed.

# Discussion

The MRO enhancement for Rel-19 objective is described as follows, which includes the enhancement for LTM.

The objective of this work item is to specify data collection enhancement in NR standalone and MR-DC for SON/MDT purpose. The specific objectives of this work item are

- MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]

- Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3]

- Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

During RAN2#127 the following agreements regarding LTM were reached:

Only the field description associated to the timeConnFailure IE needs to be updated accordingly. Otherwise, we don’t expect any further specification impact for timeConnFailure and reconnectCellId is foreseen, TBC.

introduce a new field in RLF report to indicate the LTM recovery cell id.

RAN2 include the specific access type in the RLF report, i.e. whether it is RA-based or RA-less cell switch. FFS details, e.g. if explicit or implicitly signalled.

We do not include in the RLF report, the time since the last reception of PDCCH order towards the target cell till reception of cell switch command.

RA-based access will not be a new triggering condition for the SHR report.

HO interruption time will not be a new triggering condition for the SHR report

Reuse the existing approach of using timeUntilReconnection in RLF-report also for LTM failure scenarios.

We will not define and log timeSinceLTM-Reconfig (like timeSinceCHO-Reconfig) within RLF-report and SHR in LTM failure and near failure cases.

We will not log interruption time in SON reports (e.g. SHR) for LTM.

We aim to log some info to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not, TBD if explicit/implicit.

Log L3 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure. RAN2 assumes this is already possible with existing spec.

2.1.	Enhancements for LTM

For Rel-18, the procedure for LTM is depicted in the following Figure 1.

Figure 1	 Signalling procedure for LTM (Figure 9.2.3.5.2-1 in [2])

With reference to Figure 1, one of the key features in LTM is the availability of a TA value prior to the cell switch to the target cell.  Currently, there are two ways for the UE to acquire the early TA values associated with each candidate cell.  The serving cell may send the TA value in the LTM cell switch command MAC CE when triggering LTM cell switch or the UE performs TA measurement for the candidate cells after being configured by RRC. Considering the possibility of LTM failure due to inaccurate or invalid TA value(s), RAN2 should consider whether the TA value(s) of all candidate cells or at least the TA associated with the target cell should be included in the RLF report.

During the discussions in RAN2#127, it wasn’t clear yet if the information associated with early TA value(s) need to be included in the RLF report.  Another related aspect is whether the UE should include how it accesses the target cell after it received the Cell Switch Command, i.e., whether it’s RACH-less, CBRA or CFRA.  It has been pointed out at the meeting that if the UE attempted RACH-less then the NW could know that the UE must have obtained early TA value from the Cell Switch Command or from UE-based TA measurement.  However, considering the report is associated with RLF or LTM failure, the following conditions should also be considered:

Necessity of including early TA value in the RLF report:

In one example, the early TA value may be obtained by UE-based TA measurement and after receiving the Cell Switch Command the UE’s RACH-less attempt fails.  Under this situation it would be useful for the NW to know what TA value was used and how the UE obtained the TA value during the failed access attempt.

In another example, the UE may have available both the UE-based TA measurement as well as the TA value provided by the NW via the Cell Switch Command, while the UE will always use the TA value provided by the NW; however, it should be considered whether the UE could also store in the SHR and RLF the UE-based TA measurement, since it could allow the NW to better understand whether it can rely on the UE to perform TA measurements by itself.

Based on the above cases, it should be useful for the UE to provide both the TA value and the type of early TA obtained by the UE.  Furthermore, it may be useful for the UE to not only include the early TA value associated with the target cell, but also the TA values associated with UE-based TA measurements of the configured LTM candidate cells.

- Proposal 1	To support LTM optimization, RAN2 should include the early TA value(s) associated with the target cell and the type of early TA in the RLF-report upon LTM failure.

Necessity of including the type of access attempt value in the RLF report:

Assuming Proposal 1 is agreeable, it should also consider whether it would be useful for the UE to include the access type in the RLF report. The straightforward reason for the UE to also include the access type in the RLF report is just to ensure that the UE actually chose the correct type of access attempt in case the access failed, e.g., whether the UE really used RACH-less when the UE has available early TA value upon receiving the Cell Switch Command.

In another example, the UE may have available both the early TA value provided within the Cell Switch Command and also an early TA value from the UE-based TA measurement.  However, due to poor radio condition. the RACH-less access attempt may fail.  Upon Subsequent LTM, the UE may still fail its access attempt, but using CBRA (i.e., no early TA value is available).  The latest RLF report with CBRA included, would not indicate whether early TA value is available or was used in the initial failed access attempt, since the new RLF report would have overwritten the early RLF report. This may require some enhancements to the RLF report to not discard data from the previous RLF report.  However, this is another case when both access attempt information and TA value information can assist with the NW to better understand the failure condition.

- Proposal 2	RAN2 should include in the RLF report the type of access attempt used in the LTM execution (i.e., RACH-less, CBRA or CFRA).

With regards to the early TA measurements, it would also be helpful to the NW to know whether the early TA measurement (esp. for the UE-based TA measurement) is completed before the UE receives the Cell Switch Command or the elapsed time between the completion of the early TA measurement and Cell Switch Command reception.  In contrast to the PDCCH ordered early TA, the NW cannot track the timing of when the UE obtained its own TA value based on UE-based TA measurement.  Depending on whether the UE has completed the early TA measurement prior to receiving the Cell Switch Command, the UE may end up with various options for access attempt.  By providing the elapsed time between the completion of the early TA measurement and Cell Switch Command reception, it will provide the NW with the potential reason why the early TA value may have caused the access failure.

- Proposal 3	RAN2 should include in the RLF report the elapsed time between the early TA measurement completion and the reception of the Cell Switch Command.

In our view, the accuracy of the TA measurements (esp. for the case of UE-based TA measurement) is confined within the cyclic prefix used by each cell.  The cyclic prefix used in calculating the UE-based TA value.  CP is typically determined based on different frequency bands and deployment scenarios (e.g., macrocells would require larger CP to account for the larger delay spread).

- Observation 	The cyclic prefix used by each cell will impact the accuracy of the early TA values.

Although, the NW may also have available this information, it may be more convenient to have the UE provide this information together with the early TA values, regardless of whether they are provided to the UE or UE-based TA measurements.  In some cases, the network may dynamically change the numerology or CP length based on channel conditions or other factors.  If the network decides to change the CP length (e.g., due to mobility or interference), it informs the UE through updated SIB2. It indicates whether the cell uses normal cyclic prefix (NCP) or extended cyclic prefix (ECP).

- Proposal 4	RAN2 should discuss whether it will be helpful to include the cyclic prefix (e.g., NCP vs ECP) used for calculating the early TA values and the associated numerology.

# Conclusion

In this contribution, we addressed some potential enhancements to the RLF Report in support of Rel-18 LTM.  RAN2 is kindly asked to take into account the observation and proposals below:

- Proposal 1	To support LTM optimization, RAN2 should include the early TA value(s) associated with the target cell and the type of early TA in the RLF-report upon LTM failure.

- Proposal 2	RAN2 should include in the RLF report the type of access attempt used in the LTM execution (i.e., RACH-less, CBRA or CFRA).

- Proposal 3	RAN2 should include in the RLF report the elapsed time between the early TA measurement completion and the reception of the Cell Switch Command.

- Observation 	The cyclic prefix used by each cell will impact the accuracy of the early TA values.

- Proposal 4	RAN2 should discuss whether it will be helpful to include the cyclic prefix (e.g., NCP vs ECP) used for calculating the early TA values and the associated numerology.

# References

- RP-234038, “New WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4”, CMCC

- TS38.300, “NR and NG-RAN Overall Description; Stage 2”,