3GPP TSG-RAN WG2 Meeting #127bis			     R2-2408983

Hefei, China, October 14 – 18, 2024

Agenda Item:	8.7.6 (NR_XR_Ph3-Core)

Source: 	LG Electronics Inc.

Title:         	Discussion on rate control signaling for XR

Document for: 	Discussion and Decision

Introduction

This contribution discusses the following objective which is newly included in the WID.

- Specify uplink congestion signalling [RAN2]: - Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

Discussion

The purpose of this objective is to make the XR application adjust its data rate and performance to be adapted to network congestion. As a similar feature, RBR (Recommended bit rate) MAC CE is already defined in NR to control and adjust bit rate and someone may consider to reuse this RBR MAC CE for XR application. However, the main target of RBR MAC CE is for VoIP traffic and this has quite different QoS requirement and traffic characteristic compared to XR application. Therefore, just copy/paste of RBR MAC CE would not be sufficient for XR rate control and RAN2 should check first whether each field in the RBR MAC CE is needed or not.

According to the current MAC specification, there are four fields in the RBR MAC CE, i.e., LCID, direction, a bit rate, and a bit rate multiplier. The LCID is associated with one DRB and this means that the bit rate is controlled by per DRB in the current RBR MAC CE. However, one IP flow is mapped to one QoS flow and one DRB can be mapped with multiple QoS flows which has similar/same QoS requirement. Given that this objective is to control a specific XR application, if per DRB granularity is used, the unintended QoS flow in the same DRB can be unnecessarily impacted by per DRB control. In addition, the above objective is agreed to introduce RAN-level congestion signalling which is complementary to existing upper layer solutions (e.g., L4S). Since the L4S uses ECN (explicit congestion notification) field in IP header and one IP flow is mapped to one QoS flow, the desirable way to cooperate with L4S information is to define per QoS flow granularity for this new RAN-level congestion signalling.

Observation 1. If per DRB granularity is introduced like RBR MAC CE, the unintended QoS flow in the same DRB can be unnecessarily impacted by per DRB control.

For direction, it is targeted for uplink rate control and this new congestion indication would be transmitted only from the gNB to the UE for controlling uplink data rate of QoS flows in a specific XR application of the UE. If downlink data rate of QoS flows in a specific XR application is changed, the network can change downlink data transmission based on the changed downlink data rate of QoS flows in a specific XR application without any information from the UE.

Observation 2. Uplink direction is implicitly indicated to the UE without explicit direction information in the MAC CE and no downlink direction information is needed to the UE.

A bit rate is essential information for XR uplink rate control and should be included in the new uplink congestion signalling. However, the issue is that the bit rate table for the current RBR MAC CE may not be reused since XR traffic characteristic is different from the VoIP and the current table would be inappropriate to support much higher uplink data rate for a XR service. Considering a new buffer size table to have finer granularity introduced in Rel-18 XR, we think that a new bit rate table for XR uplink rate control should be introduced for finer granular uplink rate control of a XR application.

For the bit rate multiplier field in the RBR MAC CE, it is a useful tool to support higher data rate and finer granularity for XR uplink rate control. Thus, this concept/field should be included in the new uplink congestion signalling, but the value can be different from the bit rate multiplier field in the current RBR MAC CE.

Observation 3. A bit rate table and a bit rate multiplier defined in RBR MAC CE is essential information, but the current value would not be sufficient for XR uplink rate control.

Proposal 1. The XR rate control signalling is defined per QoS flow and a new MAC CE is defined to contain a QoS flow ID, a bit rate, and a bit rate multiplier.

On top of the new MAC CE for XR uplink rate control per QoS flow, it would be good to consider how to determine uplink congestion per QoS flow. As per the MAC specification, the UE should check buffer status and report buffer status information per LCG to the network, then the network schedules an uplink grant based on the received BSR. In this condition, the network can recognize the amount of buffered data and if the buffered data is growing, the network can determine whether there is uplink congestion in the UE or not. However, there is no buffer status information per QoS flow in the current BSR MAC CE and the network may not correctly recognize which QoS flow has a congestion problem in the UE. We think that even though the new MAC CE for XR uplink rate control per QoS flow is introduced, if there is no clear way to determine uplink congestion per QoS flow, it is doubtful how the network can correctly use the new MAC CE for XR uplink rate control per QoS flow. Thus, RAN2 should consider a way to report the amount of buffered data per QoS flow in the BSR.

Observation 4. No buffer status information per QoS flow is included in the current BSR MAC CE and it is hard to determine whether there is uplink congestion per QoS flow in the UE.

Proposal 2. The UE reports the amount of buffered data per QoS flow.

Conclusion

Based on the above discussions, we present the following observations and proposals:

Observation 1. If per DRB granularity is introduced like RBR MAC CE, the unintended QoS flow in the same DRB can be unnecessarily impacted by per DRB control.

Observation 2. Uplink direction is implicitly indicated to the UE without explicit direction information in the MAC CE and no downlink direction information is needed to the UE.

Observation 3. A bit rate table and a bit rate multiplier defined in RBR MAC CE is essential information, but the current value would not be sufficient for XR uplink rate control.

Observation 4. No buffer status information per QoS flow is included in the current BSR MAC CE and it is hard to determine whether there is uplink congestion per QoS flow in the UE.

Proposal 1. The XR rate control signalling is defined per QoS flow and a new MAC CE is defined to contain a QoS flow ID, a bit rate, and a bit rate multiplier.

Proposal 2. The UE reports the amount of buffered data per QoS flow.