3GPP TSG-RAN WG2 Meeting #127bis	R2-2408984

Hefei, China, Oct. 14 – 18, 2024

Agenda item:	8.7.6

Source:	Samsung

Title:	Discussion on UL rate control for Rel-19 XR

Document for:	Discussion & Decision

Introduction

In the Rel-19 XR WID [1], it is specified that

Specify uplink congestion signalling [RAN2]: - Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

In this paper, we provide our view on the UL congestion/rate control signalling per QoS flow/DRB.

Discussion

First, we review the current Recommended Bit Rate mechanism, to decide whether new mechanism are required to achieve the goal specified in the WID.

Legacy Bit Rate Recommendation Procedure

The legacy bit rate recommendation procedure includes two parts:

Part 1: UE may send bit rate recommendation query message to gNB, to request gNB to indicate the recommended bit rate for a specific LCH and direction (either UL or DL), and

Part 2: gNB may transmit bit rate recommendation message to UE, to indicate the recommended bit rate for the UE for a specific LCH and direction (either UL or DL).

Target scenario of the WID is pertaining to gNB-initiated scenario (since UL congestion is supposed to be detected by gNB). Note that, although the legacy bit rate recommendation procedure was intended for a scenario that UE initiates the procedure by sending bit rate recommendation query message if requested by upper layers, it is still allowed for gNB to proactively send the bit rate recommendation message without query from UE, i.e., Part 1 is not a pre-requisite for Part 2. It means that a solution for the target UL congestion/rate control signalling can be developed based on the Part 2 of the legacy procedure, e.g., when UL congestion is detected by gNB, it can send bit recommendation message to UE to adapt the UL source rate.

Observation 1: Targeted scenario of the UL congestion/rate control signalling specified in WID requires NW-initiated solution, which is compatible to the legacy bit rate recommendation procedure.

Proposal 1: The targeted UL congestion/rate control signalling is developed within the framework of the legacy bit rate recommendation procedure.

Then, we should discuss whether the legacy bit rate recommendation procedure is eligible to support per QoS flow/DRB UL congestion/rate control as specified in the WID.

Per DRB Signaling

In the legacy bit rate recommendation procedure, gNB can indicate recommended bit rate to UE for a specific LCH. Without loss of generality, LCH is typically linked to a certain DRB/SRB (except some specific cases of split bearer and PDCP duplication), such that per DRB signalling is mostly well-supported by the legacy procedure. There is little demand to develop additional solutions to support DRB-specific signaling.

Observation 2: UL congestion/rate control signaling per DRB is supported by legacy bit rate recommendation procedure.

Per QoS Flow Signaling

First, if we consider the possibility that multiple QoS flows are allowed to be mapped to a single DRB, to support per QoS flow signalling for the targeted UL congestion/rate control, we need a new MAC CE with QoS flow-level bit rate recommendation.

While, since how to map the QoS flow to DRB is within the scope of gNB implementation, gNB can deliberately configure one-to-one mapping between QoS flow and DRB, for the QoS flow(s), whose UL source rate might be adapted when UL congestion happens. But gNB may face too much restriction in serving multiple QoS flows of different types of traffi.

Observation 3: The targeted UL rate control per QoS flow can be achieved by gNB implementation with one to one mapping between Qos flow and DRB, while it enforces gNB to sacrifice the flexibility and the extensibility in radio resource management.

So, it is more desirable to consider introducing a new MAC CE featured with the QoS flow level bit rate control signalling.

Proposal 2: A new MAC CE featured with QoS flow level bit rate recommendation is introduced for UL bit rate control in Rel-19 XR.

Conclusion

Based on the discussion above, we have the following observations and proposals:

Observation 1: Targeted scenario of the UL congestion/rate control signalling specified in WID requires NW-initiated solution, which is compatible to the legacy bit rate recommendation procedure.

Proposal 1: The targeted UL congestion/rate control signalling is developed within the framework of the legacy bit rate recommendation procedure.

Observation 2: UL congestion/rate control signalling per DRB is supported by legacy bit rate recommendation procedure.

Observation 3: The targeted UL rate control per QoS flow can be achieved by gNB implementation with one to one mapping between Qos flow and DRB, while it enforces gNB to sacrifice the flexibility and the extensibility in radio resource management.

Proposal 2: A new MAC CE featured with QoS flow level bit rate recommendation is introduced for UL bit rate control in Rel-19 XR.

References

[1] RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3