3GPP TSG-RAN WG2 Meeting #127bis	R2-2408985

Hefei, China, Oct. 14 – 18, 2024

Agenda item:	8.7.4.2

Source:	Samsung

Title:	DSR enhancements for Rel-19 XR

Document for:	Discussion & Decision

Introduction

In the last RAN2#127 meeting, RAN2 made the following agreements for DSR enhancements:

Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

In this paper, we further discuss the essential aspects on the DSR enhancements.

Discussion

DSR Report

In RAN#127 meeting, we agree that the multiple remaining time thresholds is for DSR reporting. The next issue would be what information UE should report based on the multiple remaining time thresholds.

As with the remaining time reported in legacy DSR, for the multiple remaining time fields per LCG of the enhanced DSR, we can consider the option that each of them is supposed to be used to report the shortest remaining value of the running PDCP discardTimers for each remaining time interval, between each two consecutive remaining time thresholds, as well as that between 0 and the smallest threshold.

Similarly, along with reporting the multiple remaining time values for multiple remaining time intervals per LCG, multiple buffer size values are reported, where each indicates the volume of the corresponding delay-critical data, whose remaining time falling within the corresponding remaining time interval, respectively.

For example, as illustrated in Fig. 1, if NW configures two remaining time thresholds (i.e., Threshold 1 < Threshold 2) for an LCG, among the buffered PDCP SDUs, UE can report the shortest remaining time below Threshold 1 (t1) as one remaining time value, and report the shortest remaining time within Threshold 1 and Threshold 2 (t2) as another remaining time value. Correspondingly, the two buffer size values would be the volume of the delay-critical data with remaining time below Threshold 1 (b1), and that with remaining time within Threshold 1 and Threshold 2 (b2+b3), respectively.

Fig. 1. Multi-pair remaining time and buffer size reporting for DSR enhancement.

Moreover, since there could be more than one, varying number of such information pairs (remaining time and buffer size) for each LCG due to the absence of delay-critical data for some remaining time interval, we can consider introducing an extension (E) field for the MAC CE format to dynamically indicate whether another information pair follows for the same LCG. Note that we do not need to care about how to individually indicate which remaining time interval the presented information pair is associated to, since the reported remaining time implicitly implies the interval it belongs to.

For example, a resultant MAC CE format for the enhanced DSR is provided as follows, with the BT and LCG bitmap fields taking the same roles as those in the legacy DSR MAC CE. Note that the subscripts i and j in Remaining Timei,j/Buffer Sizei,j/BTi,j indicate the corresponding field is jth field of that type of ith LCG, respectively.

Fig. 2. Proposed enhanced DSR MAC CE format.

Proposal 1. For DSR enhancement, for LCG(s) configured with multiple remaining time thresholds, UE reports the following information for each remaining time interval between each two consecutive remaining time thresholds as well as that between 0 and the smallest remaining time threshold, for each of the LCG(s):

- the shortest remaining time value among those falling within the corresponding remaining time interval. FFS other conditions/restrictions.

- total amount of UL data whose remaining time falling within the corresponding remaining time interval. FFS other conditions/restrictions.

Proposal 2. An extension (E) field is introduced for enhanced DSR MAC CE, to indicate whether another pair of buffer size and remaining time fields follows for the same LCG.

DSR Triggering

When there are multiple remaining time thresholds, configured for an LCG, we need to specify, based on which, the DSR should be triggered. Since the intention of introducing the multiple remaining time thresholds is to report the holistic delay status across different remaining time ranges, it is desirable to consider the smallest threshold as the triggering threshold, which would otherwise, will trigger DSR at the point that only the information associated to the part of the remaining time intervals (associated to larger thresholds) is always reported.

Proposal 3. For an LCG configured with multiple remaining time thresholds, the smallest threshold among the multiple thresholds is considered for enhanced DSR triggering for the LCH(s) of the LCG.

Conclusion

Based on the discussion above, we have the following observations and proposals:

Proposal 1. For DSR enhancement, for LCG(s) configured with multiple remaining time thresholds, UE reports the following information for each remaining time interval between each two consecutive remaining time thresholds as well as that between 0 and the smallest remaining time threshold, for each of the LCG(s):

- the shortest remaining time value among those falling within the corresponding remaining time interval. FFS other conditions/restrictions.

- total amount of UL data whose remaining time falling within the corresponding remaining time interval. FFS other conditions/restrictions.

Proposal 2. An extension (E) field is introduced for enhanced DSR MAC CE, to indicate whether another pair of buffer size and remaining time fields follows for the same LCG.

Proposal 3. For an LCG configured with multiple remaining time thresholds, the smallest threshold among the multiple thresholds is considered for enhanced DSR triggering for the LCH(s) of the LCG.