3GPP TSG-RAN WG2 Meeting #127bis	R2-2408988

Hefei, China, 14th Oct – 18th Oct, 2024

Agenda Item		:	8.8.4

Source		:	LG Electronics Inc.

Title		:	Discussion on support of broadcast service in NTN

Document for		:	Discussion and Decision

1. Introduction

In the past meetings, the following statements were agreed.

RAN2#126 Agreements: - RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area. - For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS - When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details) RAN2#127 Agreement: - RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions - For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2) - For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

In this contribution, we provide our views on the expected UE behavior outside of the intended service area and how to provide the intended service area to UE.

2. Discussion

In the past meeting, RAN2 discussed to select which message is used to provide the intended service area. The possible options are: SIB20/SIB21/MBSBroadcastConfiguration. For UE supporting broadcast service in NTN feature, the intended service area is not related to any other information included in SIB20 and SIB21. UE is still possible to acquire MCCH information wihtin the cell coverage. The intended service area is not needed to acquire MCCH information. SIB20 contains the information required to acquire the MCCH/MTCH configuration for MBS broadcast. Furthermore, the intended service area is not associated with the frequency but broadcast service. SIB21 contains the mapping between the current and/or neighbouring carrier frequencies and MBS FSAI. Therefore, it is not suitable to include the intended service area into SIB20 or SIB21.

Observation 1	 The intended service area is not needed for acquiring MCCH/MTCH information. Furthermore, it is associated with broadcast service session, not frequency.

Two options other than MBSBroadcastConfiguration affect all NTN UEs camping on the serving cell although some UEs that does not support broadcast service in NTN feature. Let us assume that the intended service area is provided via system information other than MBSBraodcastConfiguration. The UEs not supporting broadcast service in NTN feature will suffer from redundant SIB1 acquisition when the information of the intended service area is changed.

Observation 2	The UEs not supporting broadcast service in NTN feature will suffer from redundant system information acquisition due to the change of the information of the intended service area if the intended service area is provided via SIB20 or SIB21.

MBSBroadcastConfiguration provides the information associated with the broadcast session. We expect that there is smallest impact if the intended service area is provided via MBSBroadcastConfiguration rather than two options.

Proposal 1	The intended service area is provided via MBSBroadcastConfiguration message.

Now we discuss how to implement the intended service area within MBSBroadcastConfiguration message. The mutliple broadcast services can be associated to a sinlge service area. It is efficient to contain ID of the intended service area into the broadcast session information to prevent duplicate signalling. The intended service area is provided by the list. The broadcast MBS session information refers associated intended service area as bit string.

Proposal 2	The intended service area is provided with the list. Adopt Text Proposal 1 as a baseline.

Proposal 3	MBS-SessionInfo refers to the associated service area in the list by bitmap. Adopt Text Proposal 2 as a baseline.

According to the agreement of the last meeting, the format of the intended service area can be provided as a form of circle shape or polygon. As the format has two choices, RAN2 can reuse to define the signalling of the intended service area as the existing “Warning Area Coordinates” in TS 38.413, similarly to CMAS in SIB8. “Warning Area Coordinates” can represent both circle shape and polygon shape.

Proposal 4	The signalling of the intended service area can be defined as Warning Area Coordinates in CMAS that can be represented as circle or geographical shape. Adopt Text Proposal 3 as a baseline to introduce the format of the intended service area.

In RAN2#127, RAN2 agreed that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. And RAN2 also agreed that UE does not have any established broadcast MRB outside the intended service area according to the RAN2#127 agreements. In fact, the UE supporting MBS may not acquire MCCH information upon receiving MCCH information change notification of on-going MBS broadcast service when the UE does not have any established broadcast MRB. UE is already able to skip the MCCH acquisition outside of the intended servcie area when the MCCH change notification of on-going MBS broadcast service.

Observation 3	According to the current specification, UE may not acquire MCCH information upon MCCH change notification of on-going broadcast service when it does not have any established broadcast MRB.

Proposal 5	When UE outside the intended service area of interest, the UE already can skip acquiring MCCH information upon receiving MCCH change notification due to the change of on-going MBS broadcast service in current sepc. No spec impact.

It could be argued that the UE outside the intended service area of interest should also be able to skip the acquisition of MCCH information upon receiving MCCH change notification due to the start of a new MBS broadcast service. In our understanding, the UE may be able to such behaviour if the intended service area is not changed at all. However, if the intended service area is changed, the UE cannot be sure whether it outside the intended service area of interest or not. Furthermore, even if UE performs such behaviour, the UE has to acquire MCCH information anyway upon it enters the intended service area of interest.

Observation 4	UE acquires MCCH information due to the start of a new MBS broadcast service only when it does not receive the MBS broadcast service of interest yet.

Observation 5	The gain of skipping MCCH information due to the start of a new MBS broadcast service is not clear.

Proposal 6	Even if UE outside the intended service area of interest, the UE interested in receiving MBS broadcast service shall acquire MCCH information for the change due to the start of a new MBS broadcast service.

3. Conclusion

In this contribution, following statements are proposed:

Observation 1	 The intended service area is not needed for acquiring MCCH/MTCH information. Furthermore, it is associated with broadcast service session, not frequency.

Observation 2	The UEs not supporting broadcast service in NTN feature will suffer from redundant system information acquisition due to the change of the information of the intended service area if the intended service area is provided via SIB20 or SIB21.

Proposal 1	The intended service area is provided via MBSBroadcastConfiguration message.

Proposal 2	The intended service area is provided with the list. Adopt Text Proposal 1 as a baseline.

Proposal 3	MBS-SessionInfo refers to the associated service area in the list by bitmap. Adopt Text Proposal 2 as a baseline.

Proposal 4	The signalling of the intended service area can be defined as Warning Area Coordinates in CMAS that can be represented as circle or geographical shape. Adopt Text Proposal 3 as a baseline to introduce the format of the intended service area.

Observation 3	According to the current specification, UE may not acquire MCCH information upon MCCH change notification of on-going broadcast service when it does not have any established broadcast MRB.

Proposal 5	When UE outside the intended service area of interest, the UE already can skip acquiring MCCH information upon receiving MCCH change notification due to the change of on-going MBS broadcast service in current sepc. No spec impact.

Observation 4	UE acquires MCCH information due to the start of a new MBS broadcast service only when it does not receive the MBS broadcast service of interest yet.

Observation 5	The gain of skipping MCCH information due to the start of a new MBS broadcast service is not clear.

Proposal 6	Even if UE outside the intended service area of interest, the UE interested in receiving MBS broadcast service shall acquire MCCH information for the change due to the start of a new MBS broadcast service.

4. References

[1]

5. Text proposal 1

–	MBSBroadcastConfiguration

The MBSBroadcastConfiguration message contains the control information applicable for MBS broadcast services transmitted via broadcast MRB.

Signalling radio bearer: N/A

RLC-SAP: UM

Logical channel: MCCH

Direction: Network to UE

MBSBroadcastConfiguration message

-- ASN1START

-- TAG-MBSBROADCASTCONFIGURATION-START

MBSBroadcastConfiguration-r17 ::= SEQUENCE {

criticalExtensions                CHOICE {

mbsBroadcastConfiguration-r17     MBSBroadcastConfiguration-r17-IEs,

criticalExtensionsFuture          SEQUENCE {}

}

}

MBSBroadcastConfiguration-r17-IEs ::= SEQUENCE {

mbs-SessionInfoList-r17               MBS-SessionInfoList-r17                                              OPTIONAL,   -- Need R

mbs-NeighbourCellList-r17             MBS-NeighbourCellList-r17                                            OPTIONAL,   -- Need S

drx-ConfigPTM-List-r17                SEQUENCE (SIZE (1..maxNrofDRX-ConfigPTM-r17)) OF DRX-ConfigPTM-r17   OPTIONAL,   -- Need R

pdsch-ConfigMTCH-r17                  PDSCH-ConfigBroadcast-r17                                            OPTIONAL,   -- Need S

mtch-SSB-MappingWindowList-r17        MTCH-SSB-MappingWindowList-r17                                       OPTIONAL,   -- Need R

lateNonCriticalExtension              OCTET STRING                                                         OPTIONAL,

nonCriticalExtension                  MBSBroadcastConfiguration-r19-IEs                                    OPTIONAL

}

MBSBraodcastConfiguration-r19-IEs ::= SEQUENCE {

mbs-SessionInfoList-r19               MBS-SessionInfoList-r19                                              OPTIONAL,   -- Need R

mbs-ServiceAreaList-r19               MBS-ServiceAreaList-r19                                              OPTIONAL,   -- Need R

nonCriticalExtension                  SEQUENCE {}                                                          OPTIONAL

}

-- TAG-MBSBROADCASTCONFIGURATION-STOP

-- ASN1STOP

MBSBroadcastConfiguration field descriptions

pdsch-ConfigMTCH Provides parameters for acquiring the PDSCH for MTCH. When this field is absent, the UE shall use parameters in pdsch-ConfigMCCH to acquire the PDSCH for MTCH.

mbs-SessionInfoList Provides the configuration of each MBS session provided by MBS broadcast in the current cell.

mbs-NeighbourCellList List of neighbour cells providing one or more MBS broadcast services via broadcast MRB that are provided by the current cell. This field is used by the UE together with mtch-NeighbourCell field signalled for each MBS session in the corresponding MBS-SessionInfo. When an empty mbs-NeighbourCellList list is signalled, the UE shall assume that MBS broadcast services signalled in mbs-SessionInfoList in the MBSBroadcastConfiguration message are not provided in any neighbour cell. When a non-empty mbs-NeighbourCellList is signalled, the current serving cell does not provide information about MBS broadcast services of a neighbour cell that is not included in mbs-NeighbourCellList, i.e., the UE cannot determine the presence or absence of an MBS service of a neighbour cell that is absent. When the field mbs-NeighbourCellList is absent, the current serving cell does not provide information about MBS broadcast services in the neighbouring cells, i.e. the UE cannot determine the presence or absence of an MBS service in neighbouring cells based on the absence of this field.

mbs-ServiceAreaList List of the areas where ongoing MBS session is intended to be provided. This information is only provided when the serving cell is NTN cell.

6. Text proposal 2

–	MBS-SessionInfoList

The IE MBS-SessionInfoList provides the list of ongoing MBS broadcast sessions transmitted via broadcast MRB and, for each MBS broadcast session, the associated G-RNTI and scheduling information.

MBS-SessionInfoList information element

-- ASN1START

-- TAG-MBS-SESSIONINFOLIST-START

MBS-SessionInfoList-r17 ::=      SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionInfo-r17

MBS-SessionInfo-r17 ::=          SEQUENCE {

mbs-SessionId-r17                TMGI-r17,

g-RNTI-r17                       RNTI-Value,

mrb-ListBroadcast-r17            MRB-ListBroadcast-r17,

mtch-SchedulingInfo-r17          DRX-ConfigPTM-Index-r17                      OPTIONAL, -- Need S

mtch-NeighbourCell-r17           BIT STRING (SIZE(maxNeighCellMBS-r17))       OPTIONAL, -- Need S

pdsch-ConfigIndex-r17            PDSCH-ConfigIndex-r17                        OPTIONAL, -- Need S

mtch-SSB-MappingWindowIndex-r17  MTCH-SSB-MappingWindowIndex-r17              OPTIONAL  -- Cond MTCH-Mapping

}

DRX-ConfigPTM-Index-r17 ::=          INTEGER (0..maxNrofDRX-ConfigPTM-1-r17)

PDSCH-ConfigIndex-r17  ::=           INTEGER (0..maxNrofPDSCH-ConfigPTM-1-r17)

MTCH-SSB-MappingWindowIndex-r17  ::= INTEGER (0..maxNrofMTCH-SSB-MappingWindow-1-r17)

MRB-ListBroadcast-r17 ::=            SEQUENCE (SIZE (1..maxNrofMRB-Broadcast-r17)) OF MRB-InfoBroadcast-r17

MRB-InfoBroadcast-r17 ::=            SEQUENCE {

pdcp-Config-r17                      MRB-PDCP-ConfigBroadcast-r17,

rlc-Config-r17                       MRB-RLC-ConfigBroadcast-r17,

...

}

MRB-PDCP-ConfigBroadcast-r17 ::=     SEQUENCE {

pdcp-SN-SizeDL-r17                   ENUMERATED {len12bits}                   OPTIONAL, -- Need S

headerCompression-r17                CHOICE {

notUsed                              NULL,

rohc                                 SEQUENCE {

maxCID-r17                           INTEGER (1..16)               DEFAULT 15,

profiles-r17                         SEQUENCE {

profile0x0000-r17                    BOOLEAN,

profile0x0001-r17                    BOOLEAN,

profile0x0002-r17                    BOOLEAN

}

}

},

t-Reordering-r17                     ENUMERATED {ms1, ms10, ms40, ms160, ms500, ms1000, ms1250, ms2750}    OPTIONAL -- Need S

}

MRB-RLC-ConfigBroadcast-r17 ::=      SEQUENCE {

logicalChannelIdentity-r17           LogicalChannelIdentity,

sn-FieldLength-r17                   ENUMERATED {size6}                       OPTIONAL, -- Need S

t-Reassembly-r17                     T-Reassembly                             OPTIONAL  -- Need S

}

MBS-SessionInfoList-v1900 ::=      SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionInfo-v1900

MBS-SessionInfo-v1900 ::=          SEQUENCE {

mbs-ServiceAreaID-r19                  BIT STRING (SIZE(maxServAreaMBS-r19))       OPTIONAL, -- Need S

}

-- TAG-MBS-SESSIONINFOLIST-STOP

-- ASN1STOP

MBS-SessionInfoList field descriptions

g-RNTI G-RNTI used to scramble the scheduling and transmission of MTCH.

headerCompression If rohc is configured, the UE shall apply the configured ROHC profile(s) in downlink.

mbs-SessionId Indicates an identifier of the MBS session provided by the MTCH.

mrb-listBroadcast A list of broadcast MRBs to which the associated broadcast MBS session is mapped to.

mtch-neighbourCell Indicates neighbour cells which provide this service on MTCH. The first bit is set to 1 if the service is provided on MTCH in the first cell in mbs-NeighbourCellList, otherwise it is set to 0. The second bit is set to 1 if the service is provided on MTCH in the second cell in mbs-NeighbourCellList, and so on. If the service is not available in any neighbouring cell and mbs-NeighbourCellList is signalled, the network sets all bits in this field to 0. The field is absent when mbs-NeighbourCellList is absent or an empty mbs-NeighbourCellList is signalled. If this field is absent when mbs-NeighbourCellList is absent or a non-empty mbs-NeighbourCellList is signalled, the related service may or may not be available in any neighbouring cell, i.e. the UE cannot determine the presence or absence of an MBS service in neighbouring cells based on the absence of this field. If this field is absent and an empty mbs-NeighbourCellList is signalled, then the UE shall assume that MBS broadcast services signalled in mbs-SessionInfoList in the MBSBroadcastConfiguration message are not provided in any neighbour cell.

mtch-schedulingInfo Indicates the index of DRX configuration entry in drx-ConfigPTM-List that is used for scheduling the MTCH. The value 0 corresponds to the first entry in drx-ConfigPTM-List, the value 1 corresponds to the second entry in drx-ConfigPTM-List and so on. In case mtch-schedulingInfo is absent for a G-RNTI (i.e. no PTM DRX), the UE shall monitor for PDCCH scrambled with G-RNTI in any slot according to the search space configured for MTCH [see TS 38.213 [13], clause 10.1].

mtch-SSB-MappingWindowIndex Indicates the index of MTCH-SSB-MappingWindowCycleOffset configuration entry in MTCH-SSB-MappingWindowList. The value 0 corresponds to the first entry in MTCH-SSB-MappingWindowList, the value 1 corresponds to the second entry in MTCH-SSB-MappingWindowList and so on. This field is set to the same value for all MBS sessions mapped to the same G-RNTI.

pdcp-SN-SizeDL Indicates that PDCP sequence number size of 12 bits is used, as specified in TS 38.323 [5]. When the field is absent the UE applies the value as specified in 9.1.1.7.

pdschConfigIndex Indicates the index of PDSCH configuration entry in pdschConfigList for MTCH. Value 0 corresponds to the first entry in pdschConfigList, the value 1 corresponds to the second entry in pdschConfigList and so on. When the field is absent the UE applies the first entry in pdschConfigList for MTCH.

sn-FieldLength Indicates that the RLC SN field size of 6 bits is used, see TS 38.322 [4]. When the field is absent the UE applies the value as specified in 9.1.1.7.

t-Reassembly Timer for reassembly in TS 38.322 [4], in milliseconds. Value ms0 means 0 ms, value ms5 means 5 ms and so on. When the field is absent the UE applies the value in specified in 9.1.1.7.

t-Reordering Value in ms of t-Reordering specified in TS 38.323 [5]. Value ms1 corresponds to 1 ms, value ms10 corresponds to 10 ms, and so on. When the field is absent the UE applies the value as specified in 9.1.1.7.

mbs-ServiceAreaID Indicates area which is intended by the network to provide this service. The first bit is set to 1 if the service is provided on the first area in mbs-ServiceAreaList, otherwise it is set to 0. The second bit is set to 1 if the service is provided on the first area in mbs-ServiceAreaList, and so on. When the network sets all bits in this field to 0, the related service may not be available in serving cell or any neighbouring cell. The field is absent when mbs-ServiceAreaList is absent or an empty mbs-ServiceAreaList is signalled. If this field is absent when mbs-ServiceAreaList is absent, the related service may be available in serving cell or any neighbouring cell, i.e. the UE considers the related service may be available in serving cell and neighbouring cells indicated by mtch-neighbourCell. If this field is absent and a non-empty mbs-ServiceAreaList is signalled, then the UE shall assume that MBS broadcast services signalled in mbs-SessionInfoList in the MBSBroadcastConfiguration message may not be available in serving cell and any neighboring cell.

| Conditional Presence | Explanation |
| --- | --- |
| MTCH-Mapping | The field is mandatory present if the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 is more than 1, and searchspaceMTCH is not set to zero (including the case where searchSpaceMTCH is absent and searchSpaceMCCH is not set to zero). Otherwise, it is absent, Need R. |

7. Text proposal 3

–	MBS-ServiceAreaList

The IE MBS-ServiceAreaList indicates a list of areas where ongoing MBS session is intended to be provided via broadcast MRB in the NTN cell.

MBS-ServiceAreaList information element

-- ASN1START

-- TAG-MBS-SERVICEAREALIST-START

MBS-ServiceAreaList-r19 ::=     SEQUENCE (SIZE (0..maxServAreaMBS-r19)) OF MBS-ServiceArea-r19

MBS-ServiceArea-r19 ::=         SEQUENCE {

warningAreaCoordinatesSegment   OCTET STRING                                    OPTIONAL,   -- Need R

}

-- TAG-MBS-SERVICEAREALIST-STOP

-- ASN1STOP

MBS-ServiceAreaList field descriptions

warningAreaCoordinatesSegment If present, carries a segment, with one or more octets, of the geographical area where the MBS session is valid as defined in xxx. The first octet of the first warningAreaCoordinatesSegment is equivalent to the first octet of Warning Area Coordinates IE defined in and encoded according to TS 23.041 [29] and so on.