3GPP TSG-RAN WG2#127-bis		      					R2-2409001

Hefei, China, October 14 – 18, 2024

Agenda item:	8.6.4

Source: 	Kyocera

Title: 	Discussion on Conditional Intra-CU LTM

Document for:	Discussion

Introduction

RAN#105 approved the revised WID for NR mobility enhancements Phase 4 [1]. The objectives for conditional Intra-CU LTM support are stated in the revised WID as follows [1].

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

In this contribution, the potential issues for supporting Conditional Intra-CU LTM are discussed, based on the existing 3 phases for LTM, i.e., LTM preparation phase, Early sync phase and LTM cell switch execution phase.

Discussion

LTM preparation phase

In the LTM preparation phase of Rel-18 Intra-CU LTM, the network configures the UE with LTM Config within RRC Reconfiguration [4].

For the conditional LTM preparation phase, it’s straightforward that the conditional LTM candidate cell(s) is pre-configured by RRC Reconfiguration as well. However, it’s unclear whether the existing LTM Config can be reused for the Conditional LTM preparation. The following options would be considered:

- Option 1a: Extend the existing LTM Config for Conditional LTM.

- Option 1b: Introduce a new Conditional LTM Config.

With regard to Option 1a, it’s easy to extend by adding new IE(s) for Conditional LTM to the existing LTM Config. On the other hand, with Option 1b, introduces a new Conditional LTM Config, which can be separately from the existing LTM Config.

RAN2 should discuss whether to extend the existing LTM Config (Option 1a) or introduce a new conditional LTM Config (Option 1b).

Early sync phase

The Early sync phase is one of the key features of LTM to reduce latency [2]. In Rel-18, the UE-based TA measurement and the PDCCH-ordered early RACH were introduced for Early TA acquisition. These can be reused for Conditional Intra-CU LTM, in general. Therefore, it should be confirmed that the Rel-18 early sync phase is applied in the Conditional intra-CU LTM.

RAN2 should confirm the Rel-18 Early TA acquisition for RACH-less LTM, including the UE-based TA measurement and the PDCCH-ordered early RACH, is supported for Conditional Intra-CU LTM.

On the other hand, it’s already clear that the Cell Switch Command MAC CE is not used in Conditional LTM. So, for the PDCCH-ordered early RACH, there is no way for the UE to receive the TA value, even if the early RACH was initiated. Therefore, introducing the RAR-based early TA acquisition for Conditional LTM, the UE can get the TA value from the LTM candidate cell.

RAN2 should discuss whether the RAR-based early TA acquisition is introduced for Conditional Intra-CU LTM.

The PDCCH-ordered early RACH has another potential issue. In Rel-18 Intra-CU LTM, the UE performs the early RACH when it receives the PDCCH order from the serving gNB, which assumes the PDCCH order is always reachable to the UE. Since CHO is meant to improve HO performance esp. in the case of poor radio conditions[4], RAN2 should further consider whether the handling of PDCCH order from Rel-18 should also be reconsidered in Rel-19, since it’s possible that PDCCH order is sent when the UE’s radio condition worsens. In this case, the UE may not send PRACH, so it’s not able to acquire the TA value from the LTM candidate cell(s). It results in the RACH-based LTM which increases latency. Therefore, for more robust early RACH, the following options for enhancements with early RACH should be considered:

- Option 2a: Upon the configured condition is satisfied, i.e., Conditional RA transmission.

- Option 2b: Upon the conditional LTM configuration is configured from the network immediately.

Regarding Option 2a, if the configured condition is satisfied then the UE performs the early RACH, i.e., Conditional/Event-triggered RACH, so that the UE can acquire the TA value without receiving the PDCCH order. Where the term “condition” means the measurements of one of the configured target cells meeting the threshold condition. The benefit of this option is that the early RACH can be performed just before the conditional LTM execution, assuming a proper configuration, so TA value is always expected to be valid. However, the LTM candidate gNB(s) need to reserve the dedicated RACH resources until the UE sends PRACH, in case of CFRA.

Regarding Option 2b, the UE always initiates the early RACH when it receives RRC Reconfiguration with the conditional LTM configuration. The benefit is that the LTM candidate gNB(s) can minimize the duration for reserving the dedicated RACH resources since the UE performs the early RACH immediately upon reception of the conditional LTM configuration. However, if the UE’s location is too far from the LTM candidate gNB(s) when it’s configured with the conditional LTM configuration, it’s possible that the PRACH is unreachable, which results in random access failure. In addition, the UE needs to send PRACHs to every LTM candidate cells whenever it’s configured, regardless of whether the UE really access to the LTM candidate cells. It causes unnecessary UE power consumption and/or potential interference.

Given the observations above, both options have advantages and disadvantages. In our view, Conditional LTM would anyway need to support a means for the UE to execute early RACH on its own.

RAN2 should discuss which option(s) is supported to trigger the early RACH, the condition is satisfied (Option2a) or the conditional LTM configuration is configured (Option2b).

LTM cell switch execution phase

In Rel-18 Intra-CU LTM, it was totally up to gNB implementation when to transmit the LTM cell switch command, based on L1 measurement report (or L3 measurement report).

For the Conditional Intra-CU LTM, the UE should execute the LTM cell switch based on the radio condition, which is expected to be evaluated by the existing measurement events, like CondEvent for CHO [3]. Before tackling the details of events, it should be first discussed whether to reuse L3 events, L1 events or both, for the Conditional LTM evaluation. The pros and cons would be analysed as follows:

- Option 3a: L3 events.

  - Benefit: It has less specification impact if the existing L3 events for CHO (CondEvent) is reused.  L3 measurement results are stable by L3 filter, and it’s reliable enough for Conditional LTM evaluation because it has been used in the L3 handover decision and the CHO evaluation for a long time.

  - Drawback: L3 measurement cannot keep up with the rapid change of radio environment, e.g., caused by the UE in high-mobility state and/or FR2.

- Option 3b: L1 events.

  - Benefit: The L1 events being specified in Rel-19 can be reused. L1 measurement results can keep up with the rapid change of radio environment, and it fits with the LTM concept. The current LTM decision by the gNB is also based on L1 measurement reports.

  - Drawback: L1 measurement results are highly variable, so it’s difficult to specify a standardized L1 filter.

- Option 3c: Combination of L3 and L1 events.

  - Benefit: There may be potential to take the advantages from L3 and L1 events.

  - Drawback: Combinations need to be defined and may be complicated.

RAN2 should discuss which event may apply to the conditional intra-CU LTM, e.g., L3 events (Option 3a), L1 events (Option 3b) or combination of L3 and L1 events (Option 3c).

Conclusion

In this contribution, the issues for supporting conditional intra-CU LTM are discussed. RAN2 is kindly asked to take into account the proposals below:

Proposal 1	RAN2 should discuss whether to extend the existing LTM Config (Option 1a) or introduce a new conditional LTM Config (Option 1b).

Proposal 2	RAN2 should confirm the Rel-18 Early TA acquisition for RACH-less LTM, including the UE-based TA measurement and the PDCCH-ordered early RACH, is supported for Conditional Intra-CU LTM.

Proposal 3	RAN2 should discuss whether the RAR-based early TA acquisition is introduced for Conditional Intra-CU LTM.

Proposal 4	RAN2 should discuss which option(s) is supported to trigger the early RACH, the condition is satisfied (Option2a) or the conditional LTM configuration is configured (Option2b).

Proposal 5	RAN2 should discuss which event may apply to the conditional intra-CU LTM, e.g., L3 events (Option 3a), L1 events (Option 3b) or combination of L3 and L1 events (Option 3c).

References

- RP-242356 “Revised Work Item: NR mobility enhancements Phase 4”, Apple Inc, China Telecom, RAN#105

- TS38.300, “NR and NG-RAN Overall Description; Stage 2”, 3GPP

- TS38.331, “Radio Resource Control (RRC) protocol specification”, 3GPP

- RP-201274, “Summary of WI on NR mobility enhancements”, Intel Corporation (rapporteur), RAN#88e