3GPP TSG-RAN WG2 Meeting #127bis	R2-2409002

Hefei, China, Oct 14th – 18th, 2024

Agenda item	:		8.8.4

Source	: 	Sharp

Title	: 	UE behaviour for MBS related procedures

Document for	:	Discussion and Decision

Introduction

In the last RAN2 #127 meeting, RAN2 reached some agreements/decisions as follows [1];

Agreements: - The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s). - RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions - For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2) - For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

In this paper, we’d like to discuss UE behaviours for MBS related procedures.

Discussion

DL UE Behaviour inside the Intended Service Area

In the last meeting, RAN2 had an offline discussion for service area information. In this discussion, it was discussed which provides the service area information, SIB20 or MBSBroadcastConfiguration in MCCH, but there was no conclusion. If service area information is included in SIB20, UE can consider whether UE needs to (re)acquire MCCH or not simply. However, if it is inluded in MBSBroadcastConfiguration, it is difficult for UE to consider whether UE needs to (re)acquire MCCH or not. In this paper, we'd like to analyse the UE behaviour for MCCH acquisition in a case the service area information is included in MBSBroadcastConfiguration.

When the MBS session information is updated, the association information between the updated MBS session and the intended service area should also be updated. In addition, when the intended service area information is updated, the information that links the session information to the intended service area should also be updated. In other words, any changes of the MCCH information may occur when either the session information or the intended service area information is updated, e.g. in the following cases:

- Intended service area information is updated, for example, when intended service areas are out of cell range especially in the case of Earth Moving Cells, or when some areas are no longer available due to the changed circumstances.

- The case of updating session information, for example, when an MBS service provider updates the area of the service.

Observation 1. If intended service area information is included in the message carried by MCCH, since session information is associated with intended service area, MCCH modification is occurred when any changes for either intended service area or MBS session information are occurred.

In the last meeting, it was agreed that the UE may not need to (re)acquire up-to-date MCCH when the UE is not in any intended service area of its interested broadcast services.

Iin the current specification, if the NW sends an 'MCCH modification indication' to the UE when there are any changes for MCCH, the UE initiates the MCCH acquisition procedure when UE receives the indication, regardless of whether it is inside or outside the intended service area. This is described in TS38.331[2] and TS38.212[3], and 2-bit bitmap information in DCI format 4_0 is used to indicate the change of the MCCH.

The 2-bit bitmap indicates the following information.

(bit1) the start of new MBS service(s)

(bit2) modification of MCCH information other than the change caused by start of new MBS service(s).

Based on the current mechanism, UE which is not in any intended service area of its interested broadcast services can consider whether MCCH acquisition is needed or not based on this second bit (bit2).

Observation 2. In the current specification, the UE initiates the MCCH acquisition procedure based on the MCCH modification indication regardless of whether UE is in any intended service areas or not.

Observation 3. The MCCH modification indication can be used to avoid the unnecessary MCCH (re)acquisition operations.

However, the second bit (bit2) indicates other causes than the change related to intended service area. Therefore, UE which is not in any intended service area of its interested broadcast services needs to acquire MCCH even if the change is not related to intended service area. So, we think RAN2 should discuss whether MCCH modification indication needs to be extended or not to differentiate the change related to intended service area from other causes to avoid unnecessary MCCH re-acquisition operations outside the intended service area.

Proposal 1. RAN2 should discuss whether MCCH modification indication needs to be extended or not to differentiate the change related to intended service area from other causes to avoid unnecessary MCCH re-acquisition operations outside the intended service area.

UL UE Behaviours considering the Intended Service Area

In the last meeting, RAN2 agreed UE behaviours for MRB establishment/release.

- For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2)

- For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

There are some other remaining MBS related procedures (e.g. MII (MBS interest indication), cell reselection priorities handling). If these procedures are performed by UEs which are not intended to receive the service, there are negative impacts for NW operation, especially if UL resources are used by the procedure. For example, if UE is allowed to send MII from outside of the intended service area, UL resources are wasted by UEs which are not intended to receive the service. To prevent such negative impacts, the specification should also limit UE behaviours for these MBS related procedures. Therefore, if the MBS service is associated to the intended service area(s), MII should not be sent by UE located outside of the intended service area.

Observation 4. If UE is allowed to send MBSInterestIndication message from outside of the intended service area, UL resources are wasted by UEs which are not intended to receive the MBS service.

Proposal 3. MBSInterestIndication message should not be sent by UE located outside of the intended service area.

Conclusion

In this paper, we made the following observations and proposals;

Observation 1. If intended service area information is included in the message carried by MCCH, since session information is associated with intended service area, MCCH modification is occurred when any changes for either intended service area or MBS session information are occurred.

Observation 2. In the current specification, the UE initiates the MCCH acquisition procedure based on the MCCH modification indication regardless of whether UE is in any intended service areas or not.

Observation 3. The MCCH modification indication can be used to avoid the unnecessary MCCH (re)acquisition operations.

Proposal 1. RAN2 should discuss whether MCCH modification indication needs to be extended or not to differentiate the change related to intended service area from other causes to avoid unnecessary MCCH re-acquisition operations outside the intended service area.

Observation 4. If UE is allowed to send MBSInterestIndication message from outside of the intended service area, UL resources are wasted by UEs which are not intended to receive the MBS service.

Proposal 3. MBSInterestIndication message should not be sent by UE located outside of the intended service area.

Reference

- R2-2407902, Report of 3GPP TSG RAN WG2 meeting #127, Maastricht, Netherland

- 3GPP TS 38.331 V18.2.0 (2024-06)

- 3GPP TS 38.212 V18.3.0 (2024-06)