3GPP TSG-RAN WG2 Meeting #127bis	R2-2409003

Hefei, China, 14th – 18th Oct, 2024

Agenda item	:		8.8.4

Source	: 	Sharp

Title	: 	Discussion on MBS service area information

Document for	:	Discussion and Decision

Introduction

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]

- Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2]

- Specify the necessary signaling between CN and NG-RAN. [RAN3]

According to the R19 NR NTN WID[1] scope, NTN MBS broadcast service was proposed by RAN2.

And in RAN2#127 meeting, RAN2 made following agreements about Intended Service Area.

Agreements (excerpted ):

- The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

- RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions

In this paper, we discuss where/how to contain information about the intended service area.

Discussion

In the previous RAN2 discussion, it was agreed that the intended broadcast service are is defined by a geographical area represented by a (set of ) referenceLocation and radius (i.e. circle(s)) or by a (set of ) polygon(s). However, it is unclear which of the following interpretation is the intension of agreement.

- Each geographical area can consist of either a (set of ) circle(s) only or by a (set of ) polygon(s) only.

  - i.e. some geographical areas consist of a (set of ) circle(s) only, and some other geographical areas consist of a (set of ) polygon(s) only.

- All geographical areas consist of either a (set of ) circle(s) only or by a (set of ) polygon(s) only.

If interpretation A) is the intension, this contains an unnecessary restriction that the shape of a geographical area is selected exclusively, and this restriction has no technical meaning. So, we think it is better to confirm that a geographical area represented by a (set of) referenceLocation and radius, and/or polygon.

Proposal 1: RAN2 confirm that a geographical area is represented by a (set of) referenceLocation and radius, and/or polygon.

Considering the representation of a geographical area, there should be a situation that the certain areas is excluded from the service area (e.g. countries, parcels, exclusion zones). In this case, circles, represented by reference points and radii, or polygons, represented by a list of coordinates, may be placed in large numbers to represent a single area and may be too complex, as shown in Figure 1.

Fig1, Expression of intended service area with circles and polygons, or with Exclude Area

Combining these two representations could address this concern. However, the amount of data required to represent the area shape could be increased. MCCH carrying the intended broadcast service area information is transmitted frequently, so information size should be reduced as possible.

Observation1: Complex combinations of circles and polygons are required to express the shapes which just exclude particular areas from the intended service area.

Observation 2: Since MCCH carrying the intended broadcast service area information is transmitted frequently, information size should be reduced as possible.

To reduce the information size for intended service area, and to realize more efficient representation, in addition to defining intended service area where MBS services are available, we think it would be possible to define excluded areas, and the intended broadcast service area should be defined by combining this excluded area information with the coverage indicated by the intended service area as agreed in previous discussions.

Observation 3: The complex shape could be expressed more efficiently by introducing the concept of Exclude Area, i.e. areas to be excluded from the intended service area.

Proposal 3: RAN2 should consider introducing the concept of Exclude Area.

Conclusion

In this paper, we made following observations and proposals;

Proposal 1: RAN2 confirm that a geographical area is represented by a (set of) referenceLocation and radius, and/or polygon.

Observation 1: Complex combinations of circles and polygons are required to express the shapes which just exclude particular areas from the intended service area.

Observation 2: Since MCCH carrying the intended broadcast service area information is transmitted frequently, information size should be reduced as possible.

Observation 3: The complex shape could be expressed more efficiently by introducing the concept of Exclude Area, i.e. areas to be excluded from the intended service area.

Proposal 3: RAN2 should consider introducing the concept of Exclude Area.

Reference

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3

- Draft_RAN2_127_Meeting_Report_v2 #127, Maastricht, Netherland