3GPP TSG-RAN-WG2 Meeting #127bis	R2- 2409004

Hefei, China, Oct 14th – 18th, 2024

Agenda item	:		8.8.2

Source	: 	Sharp

Title	: 	Discussion for DL coverage enhancement

Document for	:	Discussion and Decision

Introduction

In the RAN2 #127 meeting, RAN2 made some agreements as follows [1].

Agreements:

1.	From RAN2 point of view, if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration (there might still be impacts on DTX aspects)

2.	From RAN2 point of view, If the SSB periodicity is larger than 160ms, for example ssb-PeriodicityServingCell, measurement gap periodicity, SMTC configuration, ssb-Periodicity-r17 for NonCellDefiningSSB-r17 may need to be extended. And the field description of nAndPagingFrameOffset may need to be enhanced to consider the SSB periodicity higher than 160ms.

3.	RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity

4.	If there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g. because of extreme SSB periodicity) the existing NTN bar bit can be used. FFS about the behaviour for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set.

In this paper, we discuss the impacts for beam-hopping / larger SSB periodicity and DTX configuration.

Discussion

It was agreed that RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity. SMTC is a parameter used to measure neighbour cells and can specify the window of the SSB to be measured per cell.

SSB periodicity can be supported up to 160ms in the current specification. So, if SSB periodicity needs to be extended, the extension is necessary for several parameters, including SMTC.

These parameters are configured per cell level. In the previous RAN2 discussion, it could be considered that one cell was composed by some beams and each beam was associated to SSB index.

In this case, if the beam hopping is equally performed for all beams, the transmission periods of the SSBs corresponding to all SSB indexes will also be equal, but if the beam hopping is unequally performed due to population bias in the coverage area or other reasons, the transmission periods of some SSBs would be different from the periods of other SSBs.

However, based on the incoming LS from RAN1 as below and WID, enhancements to the existing SSB patterns are not within the scope.

Question1 : Can RAN1 provide the information on their progress on whether the existing SSB pattern for an NR cell (e.g. SSB position in burst, SSB index number, etc.) is changed in Rel-19 NR NTN, and whether the SSB periodicity is extended compared with existing TN values?. Answer to Q1: According to the updated WID (RP-241667), SSB enhancement other than SSB periodicity extension is not considered. Thereby, RAN1 understanding is that enhancements to the existing SSB patterns, e.g. SSB position in burst, SSB index number, are not within the scope. The extension of the SSB periodicity is still under RAN1’s discussion.

Therefore, NW should configure each satellite beam to be equally distributed in a single cell. (i.e. beams which have a different hopping periodicity would be a different cell.)

Proposal.1: RAN2 assumes each cell has a single SSB periodicity for all beams associated to SSB index.

In addition, if the SSB transmission periodicity is changed based on the cell state ("off", "common messages only" and "active traffic”), the signalling overhead becomes large to indicate the cell state change and/or changed SSB transmission periodicity dynamically. Therefore, we think the SSB transmission periodicity should be configured statically.

Proposal.2: RAN2 assumes SSB periodicity is not changed dynamically.

On the other hand, a DTX mechanism is considered for power sharing purpose and the current DTX/DRX parameters are configured per cell level.

Even though NW can reconfigure the cell DTX parameters to align with the ON/OFF schedule of concerned beam each time UE moves to a different beam area, the signalling overhead becomes large.

Observation 1: R18 Cell DTX mechanism can be adopted for NTN beam power sharing, but current parameters are per cell level.

Therefore, it should be considered to discuss the enhancement how to adopt the DTX/DRX feature with reducing the signalling overhead.

There are at least two types of options to adopt NES DTX for NTN beam power sharing, and it should be considered whether a single cell should have one DTX configuration or a single cell can have multiple active DTX configuration.

Option 1: one NTN payload serves small cells formed by one or a few beams and each cell is associated to a DTX configuration like R18 NES operation.

Option 2: one NTN payload serves some cells formed by one or multiple beams and each beam is associated to a DTX configuration.

If option 1 is used, there are few changes from R18 NES specification for RRC Connected mode and there are less works in RAN2.

However, if option 1 is used, Handover should be occurred frequently compared with option 2. Therefore, it is needed to check the impacts for this.

In the option 1 case, HO should be intra-CU HO and there are no security related issues which would be factors of latency and/or overhead of HO. For mobility delay, if RACH-less HO can be used for this HO, latency would be reduced, but L2 reset should be triggered and this might be a cause of latency and overhead. LTM can be a solution to avoid L2 reset, but if we consider based on the normal HO, option 2 is preferable.

Proposal 3: RAN2 assumes each beam can be associated to an independent DTX configuration for DL coverage enhancement.

Conclusion

In this paper, we made following observations and proposals;

Proposal.1: RAN2 assumes each cell has a single SSB periodicity for all beams associated to SSB index.

Proposal.2: RAN2 assumes SSB periodicity is not changed dynamically.

Observation 1: R18 Cell DTX mechanism can be adopted for NTN beam power sharing, but current parameters are per cell level.

Proposal 3: RAN2 assumes each beam can be associated to an independent DTX configuration for DL coverage enhancement.

Reference

- R2-2407902 Report of 3GPP TSG RAN WG2 meeting #127, Maastricht, Netherlands

- R2-2407919 (R1-2407538) Reply LS on DL coverage enhancements