3GPP TSG RAN2 Meeting #127bis                                                              R2-2409008

14th Oct. – 18th Oct. 2024, Hefei, China

Agenda item:	8.11.2

Source:	Samsung

Title:	Random access in SBFD

Document for:	Discussion & Decision

Introduction

The Rel-19 WID Evolution of Duplex operation introduces NR specification support for gNB-side SBFD within a TDD carrier, enhancements to inter-gNB and inter-UE CLI handling, and BS RF and RRM requirements [1].

In the Rel-19 WID, it is assumed that SBFD is deployed at the gNB side and that the TDD UE operates in half-duplex. Subband full-duplex (SBFD) operation on a time-domain resource is limited to FDM in non-overlapping DL and UL subbands. SBFD enhancements for SBFD-aware UEs assume SBFD operation for NR single carrier in FR1 or FR2-1. From the perspective of the SBFD-aware UE, SBFD operation is assumed within a single configured DL and UL BWP pair with aligned center frequencies. For the semi-static indication of frequency/time locations of the SBFD subbands to SBFD-aware UEs on a legacy DL or F symbol/slot, it is assumed that one UL subband is configured for SBFD operation in an SBFD symbol.

The Rel-19 WID objectives include introduction of specification support for random access in SBFD symbols at least for RRC_CONNECTED mode and a study phase for RRC_IDLE/INACTIVE modes [1].

- Specify SBFD operation to support random access in SBFD symbols by UEs in RRC CONNECTED mode [RAN1, RAN2]

- Study and specify, if justified, SBFD operation to UE in RRC_IDLE/INACTIVE mode for random access [RAN1, RAN2]

  - RAN#104 to check whether to proceed normative work

During the previous meeting #127, RAN2 agreed as follows:

Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS. RAN2 to strive for a common SBFD CBRA framework independent of RRC state. FFS whether/how early indication is used during a SBFD RA procedure. RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed.

During the previous meeting #118, RAN1 agreed as follows:

Agreement Extend the previous agreements for UEs in RRC_CONNECTED mode to UEs in RRC_IDLE/INACTIVE mode. Agreement For RAN1 discussion purpose, ‘additional-ROs’ is defined as the following: - For RACH configuration Option 1, additional-ROs include the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, and the ROs across SBFD symbols configured as downlink and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon. - For RACH configuration Option 2, additional-ROs are the ROs configured by the additional RACH configuration. Agreement Regarding RO validation for RACH configuration Option 1 with Alt 1-1, an RO across SBFD symbols configured as downlink and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon is treated the same as an RO in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon. - The RO includes at least one DL symbol configured by tdd-UL-DL-ConfigurationCommon Agreement For RACH configuration Option 1 with Alt 1-1, for determination of lowest RO of additional-ROs in SBFD symbols, select one from the following alternatives. - Alt 1: The parameter msg1-FrequencyStart in rach-ConfigCommon is applied with no reinterpretation. - Alt 2-1: The parameter msg1-FrequencyStart in rach-ConfigCommon is reinterpreted as the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs. - Alt 2-2: Use a fixed value (e.g., 0) for the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs. - Alt 2-3: The frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs is equal to mod (msg1-FrequencyStart, bandwidth of UL usable PRBs). - Alt 2-4: The frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRB is msg1-FrequencyStart*scaling factor, where the scaling factor=UL usable PRB sizeUL BWP size. - Note: Other alternatives are not precluded Agreement (confirmed in Wednesday session) For RACH configuration Option 1 with Alt 1-1, for the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, support the following conditions on top of what was agreed in RAN1#116bis for RO validation. - Condition#1: A valid RO starts at least Ngap symbols after a last downlink non-SBFD symbol. - Condition#2: A valid RO starts at least Ngap symbols after the SSB. - Note: The Ngap here is the same as the Ngap in the current specification. Agreement For RACH configuration Option 1 with Alt 1-1, the legacy SSB-RO mapping rule is reused for additional-ROs. - FFS the association period and association pattern period. Agreement For SBFD aware UEs, at least support the following: - PRACH transmission with preamble repetitions within additional-ROs (not across additional-Ros and legacy ROs) Note: SBFD aware UEs support PRACH transmission with preamble repetitions only within legacy-ROs (not across additional-ROs and legacy ROs) as in legacy releases. Agreement (amended in Thursday session as shown in red) For RACH configuration Option 2, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration, - For FR2, use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211) - FFS whether to introduce new parameter(s) to determine the slot number for ROs in SBFD symbols. - Working Assumption: For FR1, use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211) - FFS whether to introduce new parameter(s) to determine the subframe number for ROs in SBFD symbols. Agreement For SBFD-aware UEs and RACH configuration Option 1 with Alt 1-1, consider the following options for initial PRACH transmission attempt in one random access procedure: - Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations. - Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs. Agreement For SBFD-aware UEs and RACH configuration Option 2, consider the following options for initial PRACH transmission attempt in one random access procedure: - Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations - Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs. Agreement For SBFD aware UEs and RACH configuration Option 1 with Alt 1-1, consider the following options for PRACH transmission re-attempt in one random access procedure: - Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure Agreement For SBFD aware UEs and RACH configuration Option 2, consider the following options for PRACH transmission re-attempt in one random access procedure: - Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure Agreement For RACH configuration Option 2, support Alt 2-3 (the additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs). Working Assumption For RACH configuration Option 2, use legacy SSB-RO mapping rule for the additional-ROs configured by the additional RACH configuration, separate from the SSB-RO mapping for the legacy-ROs configured by the legacy RACH configuration. - FFS: Whether/How to handle the case where the legacy ROs overlap with additional ROs

In this contribution, we would like to discuss the detailed random access in SBFD symbols in RRC Connected and Idle/Inactive mode.

Discussion

To increase the SBFD symbol utilization, random access in SBFD symbols are proposed and studied. Random access in SBFD symbols increases the number of available UL transmission opportunities in the TDD UL-DL frame configuration period for the PRACH/preamble transmissions and reduces the random access latency.

According to RAN1 discussion so far, there could be a RACH Occasion (RO) within a SBFD symbol. And such RO may start from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots can be valid based on network configuration. Figure 1 illustrates an example of RO across two SBFD slots.

Utilizing such RO in SBFD symbols, RAN1 and RAN2 require to design/ extend the existing random access procedure such as RO configuration, Msg.1/preamble transmission, and UE power ramping behaviour.

Figure 1 An example of RO across SBFD slots

RACH configurations

In RAN1#116, two possible RACH configuration options for SBFD-aware UEs were identified. Note that as by RAN1#116bis agreement, Option 1 does not allow to configure separate prach-ConfigurationIndex.

- Option 1: Use one single RACH configuration with possible enhancement

- Option 2: Use two separate RACH configurations (one legacy and one additional RACH configuration)

To ensure clarity in RAN2 discussions and progress, we suggest confirming the RAN1 Agreement as follows:

Proposal 1: RAN2 to confirm that the below two RACH configuration options are considered for SBFD based random access:

- Option 1: Use one single RACH configuration, and only based on the existing parameters of the single RACH configuration

- Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration

The primary motivation behind RACH Configuration Option 1 with Alt. 1-1 is to minimize signalling overhead. By employing a single RACH configuration, UEs with SBFD capabilities can differentiate whether the RO coincides with separately configured SBFD slots/symbols or within a legacy UL slots. Consequently, these SBFD-aware UEs can utilize the RO on SBFD. On the other hand, UE without SBFD capabilities (legacy UE) may only be able to distinguish ROs on legacy UL slots as the effective ROs.

In order to implement the RACH configuration Option 2 with two distinct RACH configurations, it is logical for RAN2 to define an additional RRC IE specifically for the SBFD based RACH configuration, and signal this configuration through system information. Proposal 3 suggests that for RACH Configuration Option 2 (which involves using two separate RACH configurations, one being a traditional RACH configuration and another being an additional RACH configuration), RAN2 should specify an extra RRC IE dedicated to the novel SBFD based RACH configuration.  Also, considering different aspects of SBFD resources and legacy UL resources, the SBFD RACH configuration may require to have different parameters such as power control related parameters.

Proposal 2: For RACH configuration Option 2, RAN2 needs to specify a new RRC IE for the new SBFD based RACH configuration with a new set of parameters.

And we believe either Option 1 or Option 2 would utilize SIB1 for the container.

Proposal 3: RAN2 to confirm that the RACH configuration for SBFD is transmitted via SIB1, for either option 1 or 2. FFS dedicated RRC signalling detail.

Prioritization of SBFD RA

Considering a User Equipment (UE) supporting the SBFD feature, it could make use of both legacy Random Access Opportunities (RO) and SBFD RO for random access. In situations where there are two ROs configured, the UE must determine which RO to utilize for its random access.

To ensure that the SBFD-aware UE benefits from this additional SBFD RO, we should first establish whether such a UE should be capable of utilizing the legacy RO. Proposal 4 suggests to confirm that the SBFD- aware UE can indeed utilize the legacy RO for random access.

Proposal 4: RAN2 to confirm that the SBFD-aware UE can utilize the legacy RO for random access.

Moreover, when considering the relationship between the SBFD RO and the legacy RO, we might explore the possibility of introducing rules, conditions, or prioritization mechanisms among the ROs. Given these considerations, we recommend that RAN2 needs to deliberate over various options regarding the initial Physical Random Access Channel (PRACH) transmission attempt within a single random access procedure. These options include selecting the closest RO for random access regardless of its type (legacy or SBFD), choosing between legacy-ROs and SBFD-ROs based on specific conditions or prioritizations, or leaving the decision entirely up to the UE implementation.

Proposal 5: For SBFD-aware UEs, RAN2 need to discuss the following options for initial PRACH transmission attempt in one random access procedure:

- 1) Select the nearest RO for random access, irrespective of its type (legacy or SBFD)

- 2) Select one type between legacy-ROs and SBFD-ROs based on certain specified/ configured conditions/ prioritizations.

- 3) It’s up to UE implementation

Early Indication

Another consideration is discussing and deciding on the necessity of an 'Early Indication' mechanism for Release 19 SBFD. Similar to previous releases of NR that supported an early indication mechanism for REDCAP, we believe that a Layer 1-based and/or Layer 3-based early indication mechanism could be considered for Release 19 SBFD.

The reason behind considering an early indication mechanism is the same as for previous NR features; namely, the inability to promptly exchange or retrieve UE radio access capabilities until later during the initial access procedure once the UE identity has been verified and NAS and AS authentication have successfully completed.

The example of NR standalone mode operation for FR1 TDD 30kHz is considered in below Figure 2. Any UE in RRC_IDLE must connect to the network and first perform the NAS initial access procedure (“with stored context”) before DL/UL data scheduling.

Figure 2 NAS initial access procedure (“with stored context”) in NR SA mode

It should be considered that about 8-10 DL/UL RRC/NAS messages must be exchanged between the network and the UE during NAS initial access procedure. If an early indication type of mechanism becomes available for SBFD-aware UEs, then any PUSCH containing RRC and NAS messages during the transition from RRC_IDLE to RRC_CONNECTED mode can already be transmitted usint the SBFD UL subband following RACH Message 5 (RRC Setup Complete + NAS Registration Request) upon entering RRC_CONNECTED mode. Hence, an early indication could reduce the initial access delay significantly.

This is because SBFD UL subbands can only be configured for the UE by the RRC re-configuration msg which itself can occur only after the UE has been identified and authenticated by gNB and AMF.

As a candidate for early indication (EI) of SBFD-aware UEs, we consider L1-based EI which could rely on RACH/preamble partitioning and L3-based EI that could rely on Msg.3 (or later) transmissions.

Proposal6. RAN2 supports an Early Indication (EI) mechanism for the Rel-19 SBFD-aware UEs

Note that for SBFD-aware UEs supporting random access in SBFD symbols, implicit Early Indication is already possible for RACH configuration option 2 when an SBFD-aware UE uses the additional ROs.

Furthermore, when RACH configuration option 1 is configured or indicated through SIB1, the choice of ROs used for random access attempts by SBFD-aware UEs depends on the RO selection and validation rules. If an SBFD-aware UE utilizes a legacy RO, Early Indication may require a Layer 3 (L3)-based approach.

It is also crucial to consider that feature independence must be achievable for deploying SBFD on the network side. Operators may opt to deploy SBFD solely for operations involving UEs in RRC_CONNECTED mode, meaning they may choose not to leverage the random access in SBFD resources feature. Consequently, Early Indication upon transitioning to RRC_CONNECTED mode would still necessitate reliance on an L3-based solution.

Proposal 7: RAN2 considers the L3-based EI for Rel-19 SBFD-aware UEs as baseline

Type-2 Random Access

In the Type-2 random access procedure, RACH Msg.1 and Msg.3 (PUSCH) are transmitted together as MsgA. The RACH Msg.2 (RAR) and Msg.4 (for contention resolution) are transmitted together as MsgB. Type-2 is applicable for UEs in RRC_CONNECTED and in RRC_IDLE/INACTIVE modes. Both CBRA and CFRA are possible for Type-2 random access. For MsgA transmission, the gNB configures the MCS and PUSCH allocation. For MsgB reception, a new RNTI is defined. The Type-2 procedure reduces signaling overhead and decreases RA procedure delay.

In Rel-16 NR, the random access causes for Type-1 are considered as valid for Type-2 random access. No new/additional random access cause is introduced for Type-2.

When Type-2 random access is available and configured in the serving cell, the UE selects the Type-1 or Type-2 procedure at the initiation of the random access procedure based on the network configuration.

Most expected design and specification impact to support Type-1 random access procedure in SBFD symbols occur with respect to the Msg.1/preamble transmission such as RO configuration/validation, SSB-RO mapping rules, and UE power ramping behavior. Support of the Type-2 procedure for MsgA transmission shares these common design aspects. For (Type-2) PUSCH in MsgA in SBFD symbols, network configuration can be used similar to existing Rel-16 NR principles.

Therefore, we do not see significantly increased specification effort to also support Type-2 random access in SBFD symbols in addition to the Type-1 procedure.

Proposal 8: For SBFD-aware UEs, support the Type-2 random access procedure in SBFD symbols.

Conclusion

RAN2 is requested to discuss and agree to the following proposals:

Proposal 1: RAN2 to confirm that the below two RACH configuration options are considered for SBFD based random access:

- Option 1: Use one single RACH configuration, and only based on the existing parameters of the single RACH configuration

- Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration

Proposal 2: For RACH configuration Option 2, RAN2 needs to specify a new RRC IE for the new SBFD based RACH configuration with a new set of parameters.

Proposal 3: RAN2 to confirm that the RACH configuration for SBFD is transmitted via SIB1, for either option 1 or 2. FFS dedicated RRC signalling detail.

Proposal 4: RAN2 to confirm that the SBFD-aware UE can utilize the legacy RO for random access.

Proposal 5: For SBFD-aware UEs, RAN2 need to discuss the following options for initial PRACH transmission attempt in one random access procedure:

- 1) Select the nearest RO for random access, irrespective of its type (legacy or SBFD)

- 2) Select one type between legacy-ROs and SBFD-ROs based on certain specified/ configured conditions/ prioritizations.

- 3) It’s up to UE implementation

Proposal 6: RAN2 supports an Early Indication (EI) mechanism for the Rel-19 SBFD-aware UEs

Proposal 7: RAN2 considers the L3-based EI for Rel-19 SBFD-aware UEs as baseline

Proposal 8: For SBFD-aware UEs, support the Type-2 random access procedure in SBFD symbols.

- References

[1] RP-240789 “New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”; RAN#103