3GPP TSG-RAN WG2 Meeting #127bis	R2-2409013

Hefei, China, Oct. 14th – 18th, 2024

Agenda item:		8.2.2

Source:		Samsung

Title:			Discussion on segmentation functionalities for Ambient IoT

Document for:	Discussion & Decision

# 1 Introduction

The agreements that have been made related to A-IoT transport block size and upper layer packet size in the last RAN2 meeting is as follows.

Agreements

-	Send an LS to RAN1 regarding TBS size.  Ask SA2 on the max and typical upper layer packet size.

Based on the agreements, in this contribution, we provide our general views on A-IoT segmentation functionalities.

# 2 Discussion

In NR RLC AM, each RLC SDU is assigned its own sequence number (SN) and, if segmentation is needed, all the segments of an RLC SDU share the same SN with respective segment offset. An RLC AM receiver reassembles these RLC SDU segments into the orignal RLC SDU using the SN and the segment offset. Once an SN gap among the received SDUs is detected, the RLC AM receiver starts reassembly timer based on the assumption that the missing SDU(s) may still get retransmitted via HARQ protocol at lower layer. When the reassembly timer expires, the RLC AM receiver could send a RLC status report comprising the SN(s) of the missing SDU(s) to the RLC AM transmitter. Accordingly, the ARQ function of the RLC AM transmitter performs retransmission(s) based on the received status report. However, RAN2 agreed not to support HARQ and RLC AM in RAN2#125bis meeting.

Agreements

4. No HARQ and RLC AM

Since the intent of allocating SN and segment offset to an RLC SDU segment is to facilitate its re-transmission with possibility of being received out-of-order (due to HARQ reordering) when being reassembled, in A-IoT, if ARQ and HARQ are not to be supported, there seems to be no real need to consider SN and segment offset in A-IoT for segmentation (if is supported).

Proposal 1: Do not support sequence number and segment offset for A-IoT segmentation functionalities.

Although sequence number and segment offset functionalities are not considered, it is necessary for the A-IoT receiver to know whether the data is segmented or not; otherwise the subsequent data transmission for the same SDU may not be occurred.

Proposal 2: Introduce segment type (e.g. not last segment or last segment) for A-IoT segmentation functionalities.

One may consider tagging each segment with segment number (e.g., ith segment of an SDU) as well as the total number of segments, via respective field with fixed length. However, given that the length of the field confines the maximum number of segments for an SDU, it may not be scalable and also necessitate additional complexity. Considering ultra-low complexity for A-IoT devices, it is not essential to introduce any segment number and the number of segments.

Proposal 3: Do not support segment number and the number of segments for A-IoT segmentation functionalities.

If the segmented data is received successfully, some kind of explicit feedback (e.g. ACK) needs to be introduced to trigger next segmented data transmission; otherwise it is not guaranteed that segments are sent in order.

Proposal 4: Introduce explicit feedback (e.g. ACK) to trigger next segment transmission for A-IoT segmentation functionalities.

In RAN2#126 meeting, RAN2 agreed the following agreement on retransmission/repetition.

Agreements

4	AS-layer (above PHY layer) RLC-like retransmission/repetition is not supported. This doesn't preclude the reader and device sending the payload again as new transmission from MAC perspective. FFS how we handle segmentation case (if needed).

It is possible that a certain segment transmission is failed. In this case one may consider that failure feedback (e.g. NACK) is also needed for the transmitter to send it again as new transmission from MAC perspective. We think that it is not a must considering that PHY layer repetition is supported.

Proposal 5: RAN2 to discuss whether retransmission of the segment (i.e., sending it as new transmission from MAC perspective) is supported or not for A-IoT segmentation functionalities.

If segment number and the number of segments are not supported for A-IoT segmentation functionalities, AS layer would not need to buffer all the data. For example, all buffered data could be stored in upper layer(s) (i.e. in non-volatile memory) and AS layer just transmits the un-transmitted segment(s).

Proposal 6: Do not support AS layer buffering for A-IoT segmentation functionalities, i.e., all buffered segment(s) are stored in upper layer(s).

Since the reply LS from RAN1 and SA2 has not been received yet, it is a bit premature to conclude whether A-IoT segmentation functionalities are required in both D2R and R2D transmissions or either D2R or R2D transmissions. We think that there will be no difference about the feasible solution itself in both directions so it is suggested to consider both D2R and R2D segmentation at least during SI phase.

Proposal 7: Consider both D2R and R2D segmentation for A-IoT functionalities during SI phase.

# 3	Conclusion

Based on above, RAN2 is requested to discuss and agree on the following proposals:

Proposal 1: Do not support sequence number for A-IoT segmentation functionalities.

Proposal 2: Introduce segment type (e.g. not last segment or last segment) for A-IoT segmentation functionalities.

Proposal 3: Do not support segment number and the number of segments for A-IoT segmentation functionalities.

Proposal 4: Introduce explicit feedback (e.g. ACK) to trigger next segment transmission for A-IoT segmentation functionalities.

Proposal 5: RAN2 to discuss whether retransmission of the segment (i.e., sending it as new transmission from MAC perspective) is supported or not for A-IoT segmentation functionalities.

Proposal 6: Do not support AS layer buffering for A-IoT segmentation functionalities, i.e., all buffered segment(s) are stored in upper layer(s).

Proposal 7: Consider both D2R and R2D segmentation for A-IoT functionalities during SI phase.

# 4	Reference