3GPP TSG-RAN WG2 Meeting #127bis	R2-2409018

Hefei, China, 14 – 18 October 2024

Agenda item:	8.7.4.2

Source:	Nokia, Nokia Shanghai Bell

Title:	DSR Enhancements

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The following agreements were made in RAN2 #127 on DSR enhancements:

DSR enhancements – multiple thresholds and grouping of data - Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. - For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR FFS whether PDU set importance needs to be included - FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3)

In this contribution we discuss some of the FFSs related to DSR enhancements.

# 2	Discussion

In WG meeting #127, RAN2 agreed that the network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. The following FFSs were identified:

- whether there is any impact on delay critical data definition due to multiple reporting thresholds

- whether to include non-delay critical data ahead of delay critical data in the buffer size calculation

Moreover, the following FFSs from WG meeting #126 are still open:

- whether DSR triggering is impacted, i.e. how the NW configures DSR triggering and reporting thresholds

- whether PDU set importance needs to be included

On the third FFS, we think that a DSR carrying information on multiple pairs of remaining time and buffer size for a LCG should not have any impact on the DSR triggering. The purpose of reporting multiple pairs per LCG is to provide more granular information to assist the gNB in radio resource management. Therefore, we think that the threshold for DSR trigger (i.e., remainingTimeThreshold) and the remaining time threshold(s) for reporting multiple pairs of buffer size and remaining time per LCG should be separately configured. Of course, one of the reporting remaining time threshold(s) could have the same value as the remaining time threshold for DSR trigger. But we see no strong need to limit the DSR trigger to be the highest or lowest reporting threshold, as it could be beneficial to report the data that will soon become delay critical when a DRS is triggered by a lower threshold. The reporting remaining time threshold(s) could even be configured relatively to the triggering threshold if needed. It is more flexible to allow triggering threshold and reporting threshold(s) to be separately configured.

Proposal 1: remaining time threshold(s) for DSR triggering and reporting in case of multiple pairs of remaining time and buffer size are separately configured.

On the first FFS, since in the previous FFS we mention that the presence of multiple remaining time thresholds does not impact the DSR triggering, the same will happen with the delay critical data definition. Essentially, all data, whose remaining time is below the configured remainingTimeThreshold for DSR triggering will be considered as delay-critical data.

If any of the threshold configured for DSR reporting is higher than the triggering threshold, those non-delay critical data will be reported as well in the DSR. When they become delay critical after the reporting, there is no need to trigger another DSR based on current procedure since it has already been reported, which could reduce frequent DSR trigger as well.

Proposal 2: the presence of multiple reporting thresholds would not impact the delay critical data definition, which is used for DSR triggering, but non-delay critical data can also be reported in the DSR with multiple pairs if it is configured with any threshold higher than the triggering threshold.

With the possibility of reporting data with longer remaining time than the triggering threshold, DSR could serve the purpose of BSR as well when all the data are reported. It could make more sense than legacy to allow disabling BSR when all the data are already reported in DSR. It is up to NW to configure the thresholds and whether to configure disabling BSR. However, it is currently not possible to disable BSR but enable DSR.

Proposal 3: allow to configure certain LCG as to be reported in DSR only without BSR.

On the second FFS, it is clear that with the current DSR the NW is unaware when non-delay critical data is ahead of delay critical data in the queue. Therefore, since the amount of non-delay critical data has not been reported in the DSR (and the gNB is not able to extract it from the BSR), the gNB schedules resources based on what reported in the buffer size field of the DSR, and the allocated resources may not be sufficient for the transmission of delay-critical data. To illustrate this problem, let’s consider an example where data belonging to different PDU sets with different importance levels arrive in the UE buffer interlaced. Separately signalling the buffer size and remaining time of high importance and low importance data in the DSR may not be sufficient for the gNB to determine how many resources are needed for a UE to be able to transmit delay-critical data. Then, the only way to avoid this issue is by including non-delay critical data ahead of delay critical data in the buffer size calculation to be reported in the DSR. This issue is schematically illustrated in Figure 1. Note that, even if illustrated for delay-critical and low-importance data, the issue may also consist for high-importance delay-critical and non-delay critical data.

Figure 1: DSR with low-importance and high importance data

Proposal 4: include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR. FFS how to extend this to DSR with multiple reporting thresholds.

It should also be discussed how the new DSR works together with legacy DSR when there are LCGs configured with single and multiple reporting thresholds. Similarly to BSR, the simplest way would be to report the new DSR whenever there is any LCG that requires to report multiple pairs. Otherwise, legacy DSR is reported.

Proposal 5: when there is any LCG that needs to report multiple pairs, the new DSR format is used, otherwise legacy DSR is used.

# 3	Conclusion

DSR enhancements for XR are discussed in this contribution with the following proposals made:

Proposal 1: remaining time threshold(s) for DSR triggering and reporting in case of multiple pairs of remaining time and buffer size are separately configured.

Proposal 2: the presence of multiple reporting thresholds would not impact the delay critical data definition, which is used for DSR triggering, but non-delay critical data can also be reported in the DSR with multiple pairs if it is configured with any threshold higher than the triggering threshold.

Proposal 3: allow to configure certain LCG as to be reported in DSR only without BSR.

Proposal 4: include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR. FFS how to extend this to DSR with multiple reporting thresholds.

Proposal 5: when there is any LCG that needs to report multiple pairs, the new DSR format is used, otherwise legacy DSR is used.