3GPP TSG-RAN WG2 Meeting #127bis	R2-2409019

Hefei, China, Oct.14~18, 2024

Agenda item:		8.2.3

Source:		Samsung

Title:			Discussion on A-IoT paging

Document for:	Discussion & Decision

# 1 Introduction

The agreements that have been made related to A-IoT paging in the last RAN2 meeting is as follows.

- Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices

- wait for further RAN1 progress on indication of the start of access occasion.

Based on the agreements, in this contribution, we discuss further on A-IoT paging.

# 2 Discussion

## Issue 1: Paging message transfer

As agreed in the last meeting, the AIoT paging message, associated with the same request from CN, is allowed to be transmitted for multiple times. The intention is to ensure that the AIoT devices can recevie the AIoT paging message. Normally, when receiving one AIoT paging message, the paged device can response it at certain occasion. If the device confirms its successful response, it needn’t provide another response w.r.t. the AIoT paging message associated with the same request from CN. Thus, the device should be able to determine whether it needs to response a received Paging message. The potential solution is that the AIoT paging message can include one indication information, which is used to associate with the request from the CN. Such indication information can be used by the device to avoid the duplicated response to the AIoT paging message. For such indication information, companies proposed different formats, e.g., transaction ID, session ID. In our understanding, this is stage-3 details, which relies on the details content of AIoT paging message. It can be discussed during normative stage.

Proposal 1-1: The AIoT paging message can include the indication information, which is used by the device to avoid the duplicated response w.r.t. the same request from CN. The details on such indication information is left to normative work.

Among the AIoT paging messages associated with the same CN request, the configuration infomration in each message may be changed. Fig. 1 shows one example on the resource configuration change for different AIoT paging messages. Specifically, the CN request is to inventory 100 devices. At the beginning, the AIoT reader may need assign a long period (i.e., period 1) to collect responses since large number of devices cause more collisions. After Period 1, 80 devices have been inventoried. Thus, the AIoT devices can assign a short period (i.e., period 2) for inventory considering the small number of remaining devices.

Fig. 1 Configuration information change among AIoT paging messages

Proposal 1-2: RAN2 is kindly asked to discuss whether the AIoT Paging messges associated with the same request from CN can contain different configuration information (e.g., resource configuration) or not.

With multiple transmission of AIoT paging message, the Reader should be able to determine when to stop its transmission. There are various reasons to stop the transmisison of AIoT paging message, e.g., the required number of devices are successfully inventoried, the time period used for inventory expires, CN requests to stop the inventory, etc. Thus, when determing to stop the transmission, the AIoT reader can send an indication to devices.

Proposal 1-3: RAN2 is kindly asked to discuss how to stop the transmission of AIoT paging message.

## Issue 2: information contained in the AIoT paging message

For the content of the AIoT paging messages, the following agreements were reached:

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion.

- ID information to identify the paged device(s)

This ID information is used to determine which devices are paged. It depends on the information received from the CN, which may have the following options:

- Opt1: ID of a specific device: this ID can be permanent ID from the CN

- Opt2: ID identifying a group of devices: this ID can be a group ID, or a mask to determine the a group of devices with the same information determined by such mask;

- Opt3: No ID information, which indicates that all devices needs to be paged.

- Opt4: ID list of several devices

The ID information from Opt1~3 can be simply included in the AIoT paging message. For Opt4, one possible way is that each AIoT paging message contains ID of one device, and the contention free RA can be triggered. Another way is that AIoT paging message contains multiple device IDs, which may result in large size of paging message. Considering the multiple transmission of paging message, the latter way may need more resource, and the paging message may have high probability of failure transmission. Thus, if CN provides a ID list to AIoT reader, the AIoT paging message can page each device one by one. In this sense, the AIoT paging message can include, ID of a specific device, ID identifying a group of devices, or no ID information represening to page all devices.

Proposal 2-1: In AIoT paging message, the paged device(s) can be indicated via the following options:

  - ID of a specific device

  - ID identifying a group of devices

  - No ID information to indicate that all devices are paged.

The above options for the ID information comes from the CN. The issue is whether those ID information is visible to AIoT reader or not. If it is invisible to reader, the ID information can be included in the AIoT paging message as a “container”; if it is visible to reader, the CN should provide those ID information to AIoT reader explicitly.

Proposal 2-2: RAN2 is kindly asked to discuss whether the ID information for the paged device(s) is visible to AIoT reader or not.

- Procedure type indication

According to the latest TP, the general procedure is given as below:

The potential procedure type depends on the service type and the random access type, which can be listed as the following table.

| Service type | Random access type | Steps |
| --- | --- | --- |
| Inventory only | Contention-free | Step A, Step B (D2R data transmission only) |
|  | Contention-based | Step A, Step B(Msg1 with/without data Msg2 D2R data transmission) |
| Inventory + Command | Contention-free | Step A, Step B (D2R data transmission only), Step C1, Step C2 |
|  | Contention-based | Step A, Step B(Msg1 with/without data Msg2 D2R data transmission), Step C1, Step C2 |
| Command only | Contention-free | Baseline: Step A, Step B (D2R data transmission only), Step C1, Step C2 |
|  | Contention-based | Baseline: Step A, Step B(Msg1 with/without data Msg2 D2R data transmission), Step C1, Step C2 |
|  | No RA procedure | TBD: Step A’(incl. command), Step C2 |
|  | With RA procedure | TBD: Step A’(incl. command), Step B (Msg1 with/without data Msg2), Step C2 |

According to the above table, the procedure types can be summarized as the following classes:

- Class 1 (contention-free RA with one data transmission): D2R data transmission only

- Class 2 (contention-based RA with one data transmission): contention based RA + D2R data transmission

- Class 3 (contention-free RA with multiple data transmission): D2R data transmission + the following-up D2R/R2D data transmision

- Class 4 (contention-based RA with multiple data transmission): Contention based RA + D2R data transmission + the following-up D2R/R2D data transmission

To differentiate the above procedure classes, one dimension is the RA type, i.e., contention-based or contention-free. Based on the RA type, the device can be aware whether the first D2R transmission is D2R data transmission or AIoT Msg 1. Another dimension is whether there is multiple data transmission or not. This depends on the service type, inventory+command or command only may introduce multiple data transmission, while the inventory only procedure only has one data transmission. Thus, the second dimension can be differentiated via the service type.

Proposal 2-3: in order to differentiate different procedure types, the AIoT paging message can include the RA type and the service type.

- Resource allocation information

We have agreed to support the contention-free procedure and contention-based RA procedures:

- Contention-free RA procedure

The AIoT device can perform D2R data transmission during dedicated time domain/access occasion. The AIoT paging message can indicate such dedicated time domain/access occasion via, e.g., the sequence number of time domain/access occasion (as Fig. 1 (a)), the number of time domain/access occasion since the AIoT paging message is received by AIoT device (as Fig. 1 (b)), can be indicated via AIoT paging message.

Fig. 2 Indication of dedicated access occasion

- Contention-based RA procedure

The slotted-ALOHA-like random access can be considered as a baseline, where an A-IoT device will randomly select a time domain resource for very first D2R message transmission. The A-IoT reader can provide information assisting the determination of the time resource/random access occasion via AIoT paging message. One possible information is a parameter Q, which indicates 2Q time resource/random access occassions in total so that the A-IoT device can randomly select one resource/occasion among them for D2R transmission.

Proposal 2-4: Depending on the RA type, the AIoT paging message can include the following resource allocation information:

  - Contention-free RA: information indicating a dedicated time resource/random access occasion

  - Contention-based RA: a parameter Q indicating 2Q time resource/random access occassions in total

In addition, the frequency-domain random access resource information may be included if the FDM is applied for resource allocation. However, this depends on RAN1 progress.

Proposal 2-5: RAN2 is kindly asked to discuss whether the frequency-domain resource information can be included in the AIoT paging message to support the FDM based resource allocation.

# 3 Conclusion

Based on the discussion above, we have the following observations and proposals:

Paging message transfer

Proposal 1-1: The AIoT paging message can include the indication information, which is used by the device to avoid the duplicated response w.r.t. the same request from CN. The details on such indication information is left to normative work.

Proposal 1-2: RAN2 is kindly asked to discuss whether the AIoT Paging messges associated with the same request from CN can contain different configuration information (e.g., resource configuration) or not.

Proposal 1-3: RAN2 is kindly asked to discuss how to stop the transmission of AIoT paging message.

Paging message content

Proposal 2-1: In AIoT paging message, the paged device(s) can be indicated via the following options:

  - ID of a specific device

  - ID identifying a group of devices

  - No ID information to indicate that all devices are paged.

Proposal 2-2: RAN2 is kindly asked to discuss whether the ID information for the paged device(s) is visible to AIoT reader or not.

Proposal 2-3: in order to differentiate different procedure types, the AIoT paging message can include the RA type and the service type.

Proposal 2-4: Depending on the RA type, the AIoT paging message can include the following resource allocation information:

  - Contention-free RA: information indicating a dedicated time resource/random access occasion

  - Contention-based RA: a parameter Q indicating 2Q time resource/random access occassions in total

Proposal 2-5: RAN2 is kindly asked to discuss whether the frequency-domain resource information can be included in the AIoT paging message to support the FDM based resource allocation.