3GPP TSG-RAN2 Meeting #127bis	R2-2409021

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.2.2

Source:	Qualcomm Incorporated

Title:	Views on Functionality Aspects of Ambient IoT

WID/SID:	FS_Ambient_IoT_solutions – Release 19

Document for:	Discussion and Decision

# Introduction

Rel-19 Ambient IoT SID [1] describes the objectives including the RAN2 protocol and function studies.

In RAN2 #125bis meeting, the following agreements were made for CP and UP functions separately.

Agreements

- RRC connection management is not supported.  FFS how the resource configuration is provided to the device (if needed based on RAN1 progress)

- RRM L3 measurement reporting is not supported by Ambient IoT devices.

- RAN2 assumes, AIoT devices are not required to support ASN.1 encoding/decoding.

- Periodical System information and MIB are not supported by AIoT devices. This doesn’t preclude any RAN1 defined broadcast signals.

- RAN2 assumes that RRC layer is not necessary between the reader and the device.   RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC

Agreement

- SDAP is not supported for UP protocol stack.

- PDCP layer is not needed.  FFS how to handle AS security (if needed pending SA3 dicsussion) and any other really needed functionalities.

- RLC layer is not needed.   FFS how to handle segmentation (if needed and depending on RAN1 design and upper layer packet size).  RAN2 considers segmentation and reassembly would add complexity, however further discussions are needed.

- No HARQ and RLC AM

- FFS about the level of visibility required by the reader and what information is necessary for AS layer operations.

- RAN2 assumes that no per-packet QoS and no per-QoS flow is supported at AS level (for both UL/DL).  FFS how to handle the general QoS requirements from SA2

RAN2#126 agreed the following regarding the function aspects:

Agreements on functionality

1	Multiple “AIoT logical channels” for upper layer data are not supported.   FFS if AIoT logical channel concept is used depending on final modeling issue.

2	legacy NR BSR/SR is not needed for A-IoT communication.

3	FFS whether further indication of device message size/status is needed

4	AS-layer (above PHY layer) RLC-like sretransmission/repetition is not supported.  This doesn’t preclude the reader and device sending the payload again as new transmission from MAC perspective.   FFS how we handle segmentation case (if needed)

Further, RAN2 #127 made the following agreements related to function aspects.

Agreements

-	RAN2 will study the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages.   FFS details on signaling

-	RAN2 will study the need and use of a simple message “size” reporting to the reader

Agreements

- At least the following information are considered useful to be visible to the reader from CN

-	The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful

-	targeted for one or more than one devices;

-	approximate number of target devices (if available).

FFS on mandatory/optional

2	RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIOT interface as upper layer data.

In this paper, we discuss the functionality aspects of Ambient IoT (AIoT). We will discuss some remaining control plane functionality aspects, followed by user plane functionality aspects and then provide a summary.

# Discussion

The traffic types of DO-DTT and DT with the focus on indoor inventory and command should be considered. The service requirements on inventory procedure and command procedure should be supported from RAN perspective.

In inventory procedure, the AIoT application in core network would like to discover the AIoT devices who are nearby the Readers and sends the inventory request to Readers. The inventory request message may include filter criteria, and other upper layer information. The Reader that receives the inventory request performs the inventory procedure. The Reader forwards the inventory request to all AIoT devices. The inventory command from Reader could be the group signaling which are sent to all AIoT devices who have harvested enough energy and are able to receive.

In RAN2 #125bis meeting, it has been agreed that the AIoT access procedure is triggered by the Reader and Reader may provide the information that the AIoT devices need to respond for the AIoT access trigger. The AIoT devices who are triggered by the inventory request will perform the AIoT access towards the Reader in response to the received inventory request.

Figure 1 Ambient IoT inventory

In command procedure, the AIoT application issues a Command Request including command container, a list of AIoT device identifiers that the command is destined or filter criteria that identify the AIoT devices that are supposed to act upon the command. Therefore, the trigger message sent from Reader may include the device information or filter criteria for triggering AIoT devices to perform AIoT access. The trigger message may also include the command container if AIoT application provides commands to Reader in the command procedure.

## Control plane functionalities

RAN2#125bis discussed about the targeted lower complexity and cost of AIoT devices and agreed that traditional Control Plane functions like RRC connection management, RRM L3 measurement reporting, ASN.1 encoding/decoding, legacy periodical SIB/MIB, legacy paging message, legacy RRC layer, legacy DRX, tracking area update from the device etc. will not be supported by AIoT.

However, RAN2 still has to conclude on some of the remaining CP aspects which will be helpful to resolve the following aspect captured in RAN2#125bis: RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC.

### AIoT Device Registration, Subscription and CN Awareness

Due to the target on very low complexity, cost as well as power consumption, and given that potentially a very large number of AIoT devices may be present in a given vicinity but perform only bursty data transmissions always triggered by the Reader, one of the fundamental architectural differences between AIoT and existing 3GPP technologies may be how the devices are exposed to the Core Network functions.

In our understanding, SA2 would need to discuss and conclude on this. From RAN2 point of view, we think requiring such AIoT devices to support legacy UE-registration-like procedures would be very ambitious not only because of the complexity of the procedure and protocol, but also due to the number of communication steps required. The AIoT devices will not have enough energy source or energy storage to perform multiple steps just for the purpose of registration. Additionally, having a ‘subscription’ similar to legacy UE subscriptions per AIoT device may be a high burden for the core network to manage.

The AIoT device may harvest energy, accumulate it up to the required minimum amount of energy for communication, perform the communication steps and may fully deplete its energy. If the AIoT device runs out of energy after some communication steps, it may not be able to store any configurations, i.e. it will likely ‘forget’ everything. In other words, it is plausible that every communication session starts as if the device has just been powered up from the power off state. Then, it would not make sense to have the AIoT device perform the legacy registration procedures every time it needs to communicate.

It is plausible that for AIoT devices, each communication session may start as if the device has just been powered up from the power off state.

Legacy UE-registration-like procedure is not supported by AIoT. In other words, a specific tag is not required to maintain registration with the CN.

### NAS functions

NAS control protocol in NR performs the functions listed in TS 23.501 such as registration, authentication, mobility management, security control etc. Supporting all these legacy NAS functionalities by the AIoT devices may be infeasible especially given their complexity, cost, processing power and energy/power constraints.

In our view, the functionalities like security and authentication for AIoT communication should be provided by AIoT Application function using a separate command protocol. We understand such details are being discussed in SA2 and SA3.

Legacy NAS or NAS-like functions are not supported by AIoT devices. Consequently, NAS-based data transfer (CP-CIoT-like) is not supported.

RAN2 assumes new AIoT upper layer protocol (e.g. Command Protocol) between AIoT device and AIoT Controller/AIoT Application Function will be introduced by SA2/3.

## User plane functionalities

According to the AIoT UP agreements made in the previous RAN2 meetings, the legacy NR SDAP/PDCP/RLC protocol layers are not supported for AIoT UP protocol. Some functionalities in these protocols may need further discussion that may need other WG’s further inputs as well. The similar function could be implemented in other UP protocol layer or a new L2 layer.

For the AIoT MAC, not all legacy NR MAC functionalities are needed for AIoT.

### Logical and transport channels

RAN2#126 agreed the following.

Multiple “AIoT logical channels” for upper layer data are not supported.   FFS if AIoT logical channel concept is used depending on final modeling issue.

There is still FFS on the AIoT logical channel concept. The NR logical channels are used to carry the control signaling and UP data with different QoS requirements. Similar to NR UP data, the AIoT data can be carried in MAC PDU. In AIoT MAC, the upper layer control signaling and AIoT data can be mapped into one single logical channel and all MAC PDUs belonging to this one logical channel instruct the transport blocks to be delivered to the physical layer. Alternatively, the upper layer control signaling and AIoT data can be mapped to two separate logical channels. AIoT MAC can multiplex the MAC PDU from these two logical channels into one transport block.

The concept of logical channels as in NR MAC is supported for AIoT.

Regarding the transport channels, since RAN1 has already agreed to support PRDCH and PDRCH, from AIoT MAC perspective, at least two transport channels mapping to physical channels, i.e., PRDCH and PDRCH should be supported. Similar to NR DL-SCH and UL-SCH, AIoT should support transport channels RD-SCH and DR-SCH.

At least two transport channels, i.e., RD-SCH and DR-SCH are supported for AIoT.

### Segmentation and reassembly

In RAN2 #125bis, there were some discussion on whether the legacy NR RLC-like segmentation and reassembly are supported for AIoT. In RAN2 #127, the segmentation issue was again discussed, and an LS was sent to RAN1 and SA2 for checking the expected maximum and minimum TBs in both D2R and R2D directions (to RAN1) and maximum data block size delivered from upper layer to AIoT AS layer (to SA2).

Whether to support segmentation/assembly is not only related to the AIoT data packet size or the TB size but is also related to the capability of the AIoT backscatter link. Whether an AIoT data packet needs to be segmented or not is also dependent on how much energy is left in AIoT device and which resources are available in the backscatter link.

For AIoT device, the data rate in the backscatter link is limited due to the limited capability of AIoT device. Consequently, the TB size granted in the backscatter link may not be able to carry the whole AIoT data packet in all scenarios. In some cases, the AIoT application function may use the Read Command to ‘read X bytes from the AIoT memory’. However, it assumes the AIoT device should always support to read and send the ‘X bytes’ as the minimum TB in one transmission if there is no AS segmentation support. The problem is that Reader cannot guarantee always allocating such minimum TB size supporting ‘X bytes’ regardless of the communication range between Reader and device, the energy level and the capability of AIoT device. It makes little sense to require a AIoT device to keep transmission duration nearly one second with a low data rate to support the minimum TB size for reading and sending ‘X bytes’ from AIoT memory. Therefore, we believe the AS segmentation/reassembly should be supported for AIoT.

Segmentation/reassembly is supported for AIoT.

One may argue that segmentation and reassembly might add complexity, however, we believe such added complexity can be minimized by careful studies, and here we provide a simple solution as an example to show how it works.

The simple procedure would be similar to the modeling of ‘Request-Grant’ between AIoT device and Reader. The ‘Request-Grant’ modeling could be also like the stop-and-wait protocol with window size one.

It should also be noted that ARQ/HARQ are already excluded from the WID with the understanding that AIoT is not targeted to be a lossless communication. Since the window size is one, there should be no reordering issue. However, some segments may be lost and reassembly may fail. If Reader cannot reassemble the segment, it just informs the application the inventory/command has failed. Then the reliability and error can be handled by application layer which is aligned with the general principle of AIoT design.

According to the WID (ARQ/HARQ are excluded), AIoT is not targeted to be a lossless communication.

Basically, the AIoT device can piggyback an indication of remaining message size to Reader and the Reader provides the TB based on the indication from the device. It should be noted that the indication of remaining message size is only for memory size indication, and it does not mean to require AIoT device to store any segments/data in the AS buffer. The AIoT device transmits the amount of AIoT data based on the received TB size from the Reader and can request next TB if there is still remaining packet payload to be sent.

RAN2 has agreed that ‘RAN2 will study the need and use of a simple message “size” reporting to the reader.’

The AIoT device segments the AIoT upper layer data based on the scheduled TB size from the Reader.

To simplify further, the indication can be just one bit request (i.e., inform reader ‘more data’ or ‘the last data’ is there instead of indicating the actual size) and let the Reader handle the TB size estimation until AIoT device no more continues to indicate there is more data. This would be understood by Reader as last segment has been sent.

The AIoT device indicates the remaining message/segment size to Reader, or alternatively, indicates one bit memory flag (i.e., more data or the last data) to Reader.

# Overall protocol stack

Based on the above discussion on the needed functionalities, in our view the overall AS protocol stack may look something like below.

Figure 3 AIoT protocol stack

As discussed above, the NR legacy UP protocols, i.e., SDAP/PDCP/RLC are not needed. NAS protocol would be replaced by simplified command protocol terminating at the AIoT Application Function. Note that the exact protocol layers in the Reader facing the AF side is not shown since that is unchanged and depends on whether Reader is a separate entity or a part of BS or UE based on topology which could be discussed in RAN3.

In our view, a new AIoT layer 2 protocol is a simple and clear design for AIoT device and Reader. The new AIoT L2 protocol sits between AIoT MAC and the application layer/command protocol.

A new simplified AIoT L2 protocol will be introduced.

Simplified AIoT protocol stack is unified for AIoT CP and UP (i.e. no differentiation between CP and UP protocol stacks).

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Observation 1.	It is plausible that for AIoT devices, each communication session may start as if the device has just been powered up from the power off state.

Observation 2.	According to the WID (ARQ/HARQ are excluded), AIoT is not targeted to be a lossless communication.

Observation 3.	RAN2 has agreed that ‘RAN2 will study the need and use of a simple message “size” reporting to the reader.’

Proposal 1:	Legacy UE-registration-like procedure is not supported by AIoT. In other words, a specific tag is not required to maintain registration with the CN.

Proposal 2:	Legacy NAS or NAS-like functions are not supported by AIoT devices. Consequently, NAS-based data transfer (CP-CIoT-like) is not supported.

Proposal 3:	RAN2 assumes new AIoT upper layer protocol (e.g. Command Protocol) between AIoT device and AIoT Controller/AIoT Application Function will be introduced by SA2/3.

Proposal 4:	The concept of logical channels as in NR MAC is supported for AIoT.

Proposal 5:	At least two transport channels, i.e., RD-SCH and DR-SCH are supported for AIoT.

Proposal 6:	Segmentation/reassembly is supported for AIoT.

Proposal 7:	The AIoT device segments the AIoT upper layer data based on the scheduled TB size from the Reader.

Proposal 8:	The AIoT device indicates the remaining message/segment size to Reader, or alternatively, indicates one bit memory flag (i.e., more data or the last data) to Reader.

Proposal 9:	A new simplified AIoT L2 protocol will be introduced.

Proposal 10:	Simplified AIoT protocol stack is unified for AIoT CP and UP (i.e. no differentiation between CP and UP protocol stacks).

# References

- RP-240826, Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA