3GPP TSG-RAN WG2 Meeting #127bis	R2-2409023

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.12.2

Source:	Samsung

Title:	Discussion on RAN2 impacts

WID/SID:	NR_MIMO_Ph5

Document for:	Discussion and Decision

# Introduction

Rel-19 MIMO WID RP-242394 includes the following detailed objectives.

RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources Acronyms: - sTRP: single TRP (transmit-receive point) - mTRP: multiple TRP (transmit-receive point) - CJT: coherent joint transmission - DU: distributed unit - CRI: CSI-RS resource indicator RAN2: Specify higher layer support of the enhancements listed above. RAN4: Specify necessary core requirements for the enhancements listed above.

The objectives focus on the following aspects.

- UE initiated/event-triggered beam reporting,

- increased number of ports (up to 128) for CSI measurement and reporting,

- inter-TRP calibration for CJT,

- 3Tx for UL,

- asymmetric DL sTRP/UL mTRP.

In this paper, we share our views on RAN2 impacts for each objective based on the RAN1 agreements so far and the RAN1 LS on RRC parameter and MAC CE (R1-2407285).

# Discussion

## 2.1 UE initiated/event-triggered beam reporting

Based on RAN1 progress so far (see agreements in the Appendix), we summarize the current agreed procedure of UE initiated beam reporting from the several aspects. There are still many FFSs which may lead to additional procedure and details.

- Measurement object: intra/inter-cell SSB, and periodic CSI-RS for BM

- Trigger event

  - Event-2: L1-RSRP of at least one new beam becomes a threshold (RRC configured) better than the current beam

    - Current beam: QCL RS of indicated TCI state, or SSB QCLed to the indicated TCI state

    - New beam: explicitly configured in RS resource set associated to the report configuration

    - Report content: CRI/SSBRI and L1-RSRP (differential value) for N ≥ 1 beam(s) (and current beam if enabled by RRC) in the report instance, with at least one beam meeting the event

    - optional: Filter by a time window and counter M

  - (Working assumption) Event-1: Quality of the current beam is worse than a certain threshold.

  - (Working assumption) Event-7: Quality (e.g., L1-RSRP) of at least one new beam becomes a threshold (RRC configured) better than the M-th best beam among the activated TCI states

- Report procedure

- Mode A (report in UCI using DG)

- Step 1: Transmit report notification in PUCCH (FFS dedicated SR or new UCI in dedicated periodic PUCCH)

- Step 2: Receive DCI scheduling DG for UCI

- Step 3: Transmit report in UCI using DG PUSCH

  - Mode B (report in UCI using type-1 CG)

    - Step 1: Transmit report notification in PUCCH (FFS dedicated SR or new UCI in dedicated periodic PUCCH)

    - Step 2: Transmit report in UCI using type-1 CG PUSCH, after at least X symbols

From RAN2 perspective, the RRC and MAC impacts are analysed as follows.

On the RRC aspect, we can follow RRC parameters list in RAN1 LS R1-2407285 and later updates, including the configuration of

- Enable mode A/B,

- event-2 (threshold, time window, counter M) and report content (N, report current beam or not),

- new beam to be measured, etc.

In addition, we expect more information from RAN1 on PUCCH/PUSCH resource configuration for the report notification and actual report. We can wait for more RAN1 progress on resource configuration.

One may notice the similarity and the different purposes of MIMO UE initiated/event-triggered beam reporting and LTM event-triggered beam reporting, which are under parallel discussion in two WIs. Based on RAN1 LS, the relevant parameters for UE initiated/event-triggered beam reporting are provided in CSI-ReportConfig. For LTM event-triggered beam reporting, although event configuration (e.g., thresholds, etc.) can be same, it is intuitive to include LTM event configuration in LTM-CSI-ReportConfig.

CSI-MeasConfig ::=                  SEQUENCE {

nzp-CSI-RS-ResourceToAddModList     SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource   OPTIONAL, -- Need N

nzp-CSI-RS-ResourceToReleaseList    SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-ResourceId OPTIONAL, -- Need N

nzp-CSI-RS-ResourceSetToAddModList  SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet

OPTIONAL, -- Need N

nzp-CSI-RS-ResourceSetToReleaseList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL, -- Need N

csi-IM-ResourceToAddModList         SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource           OPTIONAL, -- Need N

csi-IM-ResourceToReleaseList        SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-ResourceId         OPTIONAL, -- Need N

csi-IM-ResourceSetToAddModList      SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSet     OPTIONAL, -- Need N

csi-IM-ResourceSetToReleaseList     SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSetId   OPTIONAL, -- Need N

csi-SSB-ResourceSetToAddModList     SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSet   OPTIONAL, -- Need N

csi-SSB-ResourceSetToReleaseList    SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSetId OPTIONAL, -- Need N

csi-ResourceConfigToAddModList      SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig

OPTIONAL, -- Need N

csi-ResourceConfigToReleaseList     SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfigId

OPTIONAL, -- Need N

csi-ReportConfigToAddModList        SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfig  OPTIONAL, -- Need N

csi-ReportConfigToReleaseList       SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfigId

OPTIONAL, -- Need N

...omitting...

ltm-CSI-ReportConfigToAddModList-r18   SEQUENCE (SIZE (1..maxNrofLTM-CSI-ReportConfigurations-r18)) OF LTM-CSI-ReportConfig-r18

OPTIONAL, -- Need N

ltm-CSI-ReportConfigToReleaseList-r18  SEQUENCE (SIZE (1..maxNrofLTM-CSI-ReportConfigurations-r18)) OF LTM-CSI-ReportConfigId-r18

OPTIONAL  -- Need N

]]

}

For now in the early phase of the design, we think it would be easier to design the RRC signalling separately for the two features, and further optimization can be considered when both features are mature.

Proposal 1: For the configuration of UE initiated beam reporting, follow RRC parameter list from RAN1 and discuss signalling details for the provided parameters, and wait for more information from RAN1 on PUCCH/PUSCH resource configuration for the report notification and actual report.

On the MAC aspect, there is no direct impact from the LS. We can wait for more RAN1 progress. At the same time, RAN2 can identify potential impacts based on the current agreed procedure.

Proposal 2: For the MAC aspect of UE initiated beam reporting, wait for more RAN1 progress.

## 2.2 CSI enhancements

Objective 2 (increased number of ports (up to 128) for CSI), 3 (inter-TRP calibration for CJT), and 4 (3Tx for UL) can be grouped as CSI enhancements, for which RAN2 impacts focus on RRC configuration as informed in RAN1 LS R1-2407285. We do not see any MAC impacts so far on these objectives.

In particular, RAN1 has sent a LS R2-2404114 informing the signalling requirements for codebook parameter CBSR (codebook subset restriction).

Proposal 3: For CSI enhancements, follow RRC parameter list from RAN1, and discuss signalling details for the provided parameters.

## 2.3 Asymmetric DL sTRP/UL mTRP

For this objective, we consider the scenario of dense UL TRPs. UE receives DL from a macro TRP and transmits UL to multiple micro TRPs, so-called “asymmetric DL sTRP/UL mTRP” scenario. There is a pathloss offset between the DL pathloss RS from the DL TRP and the actual pathloss for UL TRP.  Due to this pathloss offset, power control for UL transmission (including PUCCH, PUSCH, SRS, PDCCH-order PRACH as RAN1 agreed so far) to the UL-only TRP are impacted (see RAN1 agreements in the Appendix).

Based on the LS R1-2407285 and the following RAN1 agreements,

Agreement RAN1#117 For the association between PL offset and joint/UL TCI state, support the following - Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s). Agreement RAN1#118 For the asymmetric DL sTRP/UL mTRP scenarios, the value range of PL offset includes at least [-10, 60] dB - FFS: Extending the range to lower than -10dB - Step size is 4dB

- a PL offset value is configured per joint/UL TCI state, MAC CE is used to update the PL offset value,

- the value range of PL offset includes at least [-10, 60] dB, step size is 4dB, FFS: Extending the range to lower than -10dB.

Proposal 4: For the configuration of asymmetric DL sTRP/UL mTRP, follow RRC parameter list from RAN1 and discuss signalling details for the provided parameters.

RAN2 can start to work on the PL offset update MAC CE design. The MAC CE should at least include serving cell indication, BWP ID, PL offset update, TCI state indication. Various MAC CE designs can be considered. Here we list some details to be discussed.

- Whether the PL offset update MAC CE is applied to a single serving cell or multiple serving cells?

For simplicity, we propose the MAC CE is applied to a single serving cell with a Serving Cell ID field.

- How to indicate the PL offset value

  - Option 1: indicate absolute PL offset, with a step size of 4dB, and wait for RAN1 for the value range.

  - Option 2: indicate delta PL offset with respect to the RRC configured value, with a step size of 4dB and wait for RAN1 for the value range.

- How many joint/UL TCI states are mapped to a PL offset?

- Option 1: one TCI state (either joint or UL) is indicated per PL offset.

- Option 2: multiple TCI states (either joint or UL) are indicated per PL offset,  considering multiple TCI states can have the same updated PL offset value.

- How many TCI states and the corresponding PL offsets are included in the MAC CE?

The PL offset update MAC CE is applied to TCI states for PUCCH, PUSCH, SRS, and PDCCH-order PRACH. For PUCCH and PUSCH, up to 8 TCI states can be first activated by MAC CE and DCI indicates one to be applied. For SRS, the DCI-indicated TCI state is applied if followUnifiedTCI-StateSRS is configured, otherwise for periodic SRS resource TCI state is configured by RRC, and for SP/AP SRS resource TCI state is indicated by MAC CE. For PDCCH-order PRACH, RAN1 is discussing how to indicate TCI state. The MAC CE should be applicable to all these cases.

- Since both joint TCI states configured in DL BWP and UL TCI states configured in UL BWP are applicable, DL BWP ID should be indicated for joint TCI states and UL BWP ID should be indicated for UL TCI states. Whether joint TCI state(s) or UL TCI state(s) is indicated can be determined based on RRC configuration unifiedTCI-StateType: if unifiedTCI-StateType is configured with joint for the serving cell, DL BWP ID and joint TCI states are included in the MAC CE; if unifiedTCI-StateType is configured with separate for the serving cell, UL BWP ID and UL TCI states are included in the MAC CE.

Proposal 5: Introduce a new MAC CE for PL offset update. The MAC CE includes at least serving cell indication, DL/UL BWP ID, PL offset update information, joint/UL TCI state ID.

Proposal 6: For the PL offset update MAC CE, discuss the following aspects for the MAC CE design.

- Whether the PL offset update MAC CE is applied to a single serving cell or multiple serving cells?

- How to indicate the PL offset value

  - Option 1: indicate an absolute PL offset, with a step size of 4dB, and wait for RAN1 for the value range.

  - Option 2: indicate a delta PL offset with respect to the RRC configured value, with a step size of 4dB, and wait for RAN1 for the value range.

- How many joint/UL TCI states are mapped to a PL offset?

  - Option 1: one TCI state (either joint or UL) is indicated per PL offset.

  - Option 2: multiple TCI states (either joint or UL) are indicated per PL offset, considering multiple TCI states can have the same updated PL offset value.

- The TCI states in the MAC CE are at least applicable to PUCCH, PUSCH, SRS, and PDCCH-order PRACH.

- If unifiedTCI-StateType is configured with joint for the serving cell, DL BWP ID and joint TCI states are included in the MAC CE; if unifiedTCI-StateType is configured with separate for the serving cell, UL BWP ID and UL TCI states are included in the MAC CE.

For PHR, RAN1 has agreed that the PL offset is applied to Type 1 PHR calculation for actual/reference PUSCH, FFS whether/how to consider PL offset for Type 3 PHR calculation. From RAN2 perspective, one issue to be discussed is whether PHR trigger is impacted. In TS 38.321 clause 5.4.6, one of the trigger events of PHR is as follows, where pathloss change for any PL RS is considered to determine the PHR trigger.

- phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB for at least one RS used as pathloss reference for one activated Serving Cell of any MAC entity of which the active DL BWP is not dormant BWP since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission; NOTE 1: The path loss variation for one cell assessed above is between the pathloss measured at present time on the current pathloss reference and the pathloss measured at the transmission time of the last transmission of PHR on the pathloss reference in use at that time, irrespective of whether the pathloss reference has changed in between. The current pathloss reference for this purpose does not include any pathloss reference configured using pathlossReferenceRS-Pos in TS 38.331 [5]

The pathloss value of PL RS is determined/calculated in PHY layer, which is transparent to MAC. In our view, the PL offset should be applied when the pathloss is calculated in PHY layer. The pathloss that is used to determine PHR trigger should already take into account the PL offset when it is calculated in PHY layer. Thus, there should be no change to the PHR trigger condition in MAC layer.

Proposal 7: The pathloss that is used to determine PHR trigger already takes into account the PL offset when calculated in PHY layer. Confirm the PL offset has no impact on the existing PHR trigger condition in MAC (i.e., the condition that pathloss changes more than a threshold).

In the latest WID, 2TA for sDCI mTRP operation for asymmetric DL sTRP/UL mTRP is added as a new objective. In Rel-18, the applicable scenario for 2TA operation is mDCI mTRP, with no consideration for sDCI mTRP. From RAN2 perspective, in the current 2TA operation, each TCI state, instead of each coresetPoolIndex, is associated to the TA of a TAG. To extend 2TA operation to sDCI mTRP, we think the current 2TA operation can be applied with minor changes. The RA procedure with 2TA and behaviour on TAT/TAG management are applicable to sDCI mTRP with 2TA. The restriction of mDCI (i.e., coresetPoolIndex) in relevant RRC fields (e.g., tag2) can be removed. We do not foresee much impact on 2TA operation in MAC procedure. We can wait for more RAN1 progress on this objective and further check the RAN2 impacts.

Proposal 8: For 2TA extension to sDCI mTRP, wait for more RAN1 progress.

Regarding how to apply the PL offset, for PUSCH/PUCCH/SRS transmission RAN1 has made the following agreements (see Appendix).

- Support the association between PL offset and joint/UL TCI state for PUSCH/PUCCH/SRS transmission

- Reuse the legacy uplink power control formulation by replacing legacy PL with UL PL which is derived from the DL PL RS and the PL offset.

For PRACH transmission, RAN1 has made the following agreements (see Appendix).

- Support applying PL offset on PDCCH-order PRACH towards a UL TRP in FR1

- The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission (FFS how to indicate TCI state)

We notice that RAN1 has only considered PDCCH-order PRACH. It is not clear whether it refers to both PDCCH-order CBRA and PDCCH-order CFRA or not. It is also not clear whether PL offset is needed for PRACH in UE initiated RA (e.g., for BFR, SR, etc.) or not. If we consider UE initiated RA is always performed towards to a normal TRP with both DL and UL operation, the PL offset is not needed. We propose to discuss these issues and confirm with RAN1 if necessary.

Proposal 9: Discuss whether PL offset is applied to PRACH in both PDCCH-order CBRA and PDCCH-order CFRA, whether PL offset is needed for PRACH in UE initiated RA.

Moreover, in the current Random Access procedure UE measures RSRP of DL pathloss RS and compares it with a RSRP threshold to determine RA resource selection (e.g., for NUL/SUL, 4-step/2-step, Msg1/3 repetition). Normally, UE measures RSRP of DL PL RS and transmits PRACH to the same TRP. However, in asymmetric DL/UL scenario, UE can measure RSRP from a DL TRP and transmits PRACH to a UL-only TRP. Since the measured RSRP of a DL TRP and the actual RSRP of UL-only TRP to be used for RSRP comparison can be different due to the PL offset, the pathloss offset between the two TRPs may have an impact on the RSRP comparison for RA resource selection. We propose to discuss whether the PL offset has an impact on the RSRP comparison for RA resource selection.

Proposal 10: Discuss whether the PL offset has an impact on the RSRP comparison for RA resource selection.

# Conclusion

Based on the above analysis, we have the following proposals for RRC and MAC impacts.

RRC:

Proposal 1: For the configuration of UE initiated beam reporting, follow RRC parameter list from RAN1 and discuss signalling details for the provided parameters, and wait for more information from RAN1 on PUCCH/PUSCH resource configuration for the report notification and actual report.

Proposal 3: For CSI enhancements, follow RRC parameter list from RAN1, and discuss signalling details for the provided parameters.

Proposal 4: For the configuration of asymmetric DL sTRP/UL mTRP, follow RRC parameter list from RAN1 and discuss signalling details for the provided parameters.

MAC:

Proposal 2: For the MAC aspect of UE initiated beam reporting, wait for more RAN1 progress.

Proposal 5: Introduce a new MAC CE for PL offset update. The MAC CE includes at least serving cell indication, DL/UL BWP ID, PL offset update information, joint/UL TCI state ID.

Proposal 6: For the PL offset update MAC CE, discuss the following aspects for the MAC CE design.

- Whether the PL offset update MAC CE is applied to a single serving cell or multiple serving cells?

- How to indicate the PL offset value

  - Option 1: indicate an absolute PL offset, with a step size of 4dB, and wait for RAN1 for the value range.

  - Option 2: indicate a delta PL offset with respect to the RRC configured value, with a step size of 4dB, and wait for RAN1 for the value range.

- How many joint/UL TCI states are mapped to a PL offset?

  - Option 1: one TCI state (either joint or UL) is indicated per PL offset.

  - Option 2: multiple TCI states (either joint or UL) are indicated per PL offset,  considering multiple TCI states can have the same updated PL offset value.

- The TCI states in the MAC CE are at least applicable to PUCCH, PUSCH, SRS, and PDCCH-order PRACH.

- If unifiedTCI-StateType is configured with joint for the serving cell, DL BWP ID and joint TCI states are included in the MAC CE; if unifiedTCI-StateType is configured with separate for the serving cell, UL BWP ID and UL TCI states are included in the MAC CE.

Proposal 7: The pathloss that is used to determine PHR trigger already takes into account the PL offset when calculated in PHY layer. Confirm the PL offset has no impact on the existing PHR trigger condition in MAC (i.e., the condition that pathloss changes more than a threshold).

Proposal 8: For 2TA extension to sDCI mTRP, wait for more RAN1 progress

Proposal 9: Discuss whether PL offset is applied to PRACH in both PDCCH-order CBRA and PDCCH-order CFRA, whether PL offset is needed for PRACH in UE initiated RA.

Proposal 10: Discuss whether the PL offset has an impact on the RSRP comparison for RA resource selection.

# References

[1] RP-242394 WID revision: NR MIMO Phase 5

[2] R1-2407285 LS to RAN2 on RRC and MAC CE impacts for Rel-19 NR MIMO Ph5

# Appendix

## 5.1 RAN1 agreements on UE initiated beam reporting

- RAN1#116

R1-2401681	Moderator Summary #2 on UE-initiated/event-driven beam management	Moderator (ZTE)

Agreement

On UE-initiated/event-driven beam report, at least of following aspects should be included:

- Trigger-event detection for beam reporting by UE

  - UE monitors RS to assess if a beam-reporting trigger condition has been met

  - FFS: Trigger condition for declaring beam-reporting event

- Beam-report transmission by UE

  - Signaling contents in the beam report

  - Down-selection one or more options (strive for one) between the following options as signaling medium/container for beam report transmission

    - MAC-CE

    - UCI

    - Others are not precluded.

On UE-initiated/event-driven beam report, the following aspects may be included:

- UE requesting UL resource(s) for the beam report

- UE notifying transmission of beam report

- gNB preconfigured resources

Other procedure(s) as required

R1-2401787	Moderator Summary #3 on UE-initiated/event-driven beam management	Moderator (ZTE)

Agreement

On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, RAN1 further study at least the following aspects: quality metrics, event-definition and threshold.

- Further study trigger events, including the following example as a starting point

  - Event-1: Quality of the current beam is worse than a certain threshold.

  - Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

  - Event-3: Quality of a new beam is better than a certain threshold.

  - Event-4: Quality of the current beam is worse than a threshold 1, and quality of at least one new beam is better than a threshold 2.

  - Others are not precluded.

- Note: Companies are encouraged to provide details on procedure (e.g. how it is used) related to their preferred event

Agreement

On UE-initiated/event-driven beam reporting, at least support L1-RSRP as a measurement quantity on SSB for intra-cell and inter-cell, and periodic CSI-RS for beam management

- Notes: measurement results may be contained in the beam report and/or used as quality metric(s) to initiate/trigger the reporting.

- FFS: Semi-persistent CSI-RS and aperiodic CSI-RS.

- FFS: Whether/how to support L1-SINR measurement, assuming legacy RS or RS combination (e.g., CMR only, CMR+ZP/NZP-IMR) for Rel-16 SINR is reused.

- FFS: Whether/how to specify filtering operation for L1-RSRP.

Agreement

On UE-initiated/event-driven beam reporting, regarding signaling content(s), at least support DL RS resource indicator and L1-RSRP

- FFS: Study and decide whether additional contents can be supported.

- FFS: L1-RSRP format, e.g., absolute and/or differential value.

- Note: Above does not imply to preclude discussion on L1-RSRP filtering.

- The actual reported content depends on the triggering event

  - Support of one or multiple events will be discussed separately

- RAN1#116bis

R1-2402713	Moderator Summary #2 on UE-initiated/event-driven beam management	Moderator (ZTE)

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported:

- Mode A (dynamically scheduling UCI by gNB):

  - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report

    - FFS: Request format, e.g., SR or a new UCI type.

  - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report.

  - Step 3: Beam report is transmitted in second UL channel.

    - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both

  - This mode is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature).

  - No new DCI format is introduced.

- Mode B (UCI in pre-configured resource(s) for second UL channel):

  - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report

    - FFS: Notification format, e.g., SR or a new UCI type.

  - Step 2: UE transmits the beam report in the second UL channel.

    - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both

  - The notification in Step1 is in a separate reporting instance from the beam report in Step 2.

FFS: Whether UE receives acknowledge information with response to each step for all modes

For above procedures, cross-CC beam reporting is supported for both modes.

- FFS: Details.

R1-2402714	Moderator Summary #3 on UE-initiated/event-driven beam management	Moderator (ZTE)

Agreement

On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, at least support Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

- At least L1-RSRP is supported as quality metrics used for Event-2

  - FFS: How the L1-RSRP is used to determine the triggering event (e.g. timer, counter, filter coefficient)

  - FFS: Whether the network controls how the L1-RSRP is used to determine the triggering event

- Regarding RS measurement for the new beam for Event-2, down-select one or more of the following:

  - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured by RRC (e.g., reusing legacy configuration of RS measurement or in TCI-State) or MAC-CE

  - Option-3b (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s).

  - Option-3c (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of configured TCI state(s).

- Note-1: ‘New/current beam’ is for discussion purpose.

- Note-2: Other trigger events/quality metrics (e.g., L1-SINR) are not precluded.

- Note-3: For above implicit manner(s), if there are two QCL RSs in a TCI state, the measurement RS is derived from RS w.r.t. QCL-TypeD, if applicable.

Agreement

On UE-initiated/event-driven beam reporting, regarding Event-2, the threshold value is RRC configured

R1-2403690	Summary #4 on UE-initiated/event-driven beam management	Moderator (ZTE)

Agreement

On UE-initiated/event-driven beam reporting, regarding Event-2, ‘current beam’ is a beam corresponding to the indicated TCI state.

- Regarding RS measurement for the current beam for Event-2, Option-2a is supported:

  - Option-2a (implicit manner): The RS for current beam is implicitly derived from a QCL RS of indicated TCI state.

    - FFS: The RS for current beam can be either the QCL RS in the indicated TCI state or the SSB which is QCLed with the QCL RS in the indicated TCI state.

  - FFS: Option-2c (explicit manner): The RS for current beam is explicitly configured by RRC or MAC-CE.

    - Note: SSB or CSI-RS can be configured

Agreement

On UE-initiated/event-driven beam reporting, further study the following trigger events:

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-3: Quality of a new beam is better than a certain threshold.

- Event-4: Quality of the current beam is worse than a threshold 1, and quality of at least one new beam is better than a threshold 2.

- Event-5: Absolute value of the difference between the quality of the current beam and the quality of at least one new beam is lower than a threshold.

- Event-6: When the current beam is not in the best K>1 beams (out of configured beams for measurement and reporting).

- Event-7a: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the worst quality.

- Event-7b: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the best quality.

- Event-8: Quality of M>1 new beams, such as L1-RSRP, become a threshold value better than the current beam.

- Event-9: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the configured reference RS (can be SSB or CSI-RS).

Agreement

On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, the following options are provided for down-selection (other options are not precluded) in RAN1#117

- Option-1 (variable size): N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax}

  - The N beam(s) should satisfy the condition of Event-2

  - Nmax is configured by gNB

  - FFS: Whether the indication of payload size should be provided additionally.

- Option-1a (variable size): N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax}

  - At least one of N reported beam(s) should satisfy the condition of Event-2

  - Nmax is configured by gNB

  - FFS: Whether the indication of payload size should be provided additionally.

  - FFS: Details on how value of N is determined by the UE

- Option-1b: N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax}

  - The N beam(s) should satisfy the condition of Event-2

  - Nmax is configured by gNB

  - Payload size does not vary as a function of N

  - FFS: Zero-padding can be provided if N is less than Nmax.

- Option-2: Only N=1 beam is reported in the report instance

  - The reported beam should satisfy the condition of Event-2

- Option-3: N ≥ 1 beam(s) are reported in the report instance,

  - At least one of N reported beam(s) should satisfy the condition of Event-2

  - N is configured by gNB

- Other options are not precluded.

- FFS: Whether the measurement results for current beam is always reported or can be enabled by RRC.

- FFS: When current beam is reported, whether the current beam is counted in the N reported beams.

- The selected option shall satisfy Event-2.

- RAN1#117

R1-2404713	Moderator Summary #1 on UE-initiated/event-driven beam management	Moderator (ZTE)

Agreement

On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, at least Option-3 is supported

- Option-3: N ≥ 1 beam(s) are reported in the report instance,

  - At least one of N reported beam(s) should satisfy the condition of Event-2

  - N is configured by gNB

    - FFS: candidate value of ‘N’.

  - FFS: RRC can enable or disable whether current beam is always reported in addition to the N beams

- FFS: Option-1/1a/1b/2.

- Above applies at least for the single CC case

R1-2404714	Moderator Summary #2 on UE-initiated/event-driven beam management	Moderator (ZTE)

Working Assumption

On beam report transmission procedure for UE-initiated/event-driven beam reporting

- For mode-A, at least support one-bit indication in the first PUCCH channel to request a resource for a second UL channel to carry beam report.

  - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling.

- For mode-B, at least support one-bit indication in the first PUCCH channel to notify a second UL channel to carry beam report.

  - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling.

- FFS: Whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved.

- FFS: details on the dedicated RRC signaling

- Above applies at least for the single CC case.

Agreement

Regarding RS measurement for the current beam for Event 2, for Option-2a, support the both schemes as follows.

- Scheme-1: RS for current beam is the QCL RS in the indicated TCI state

  - FFS: Whether/How to handle the case if only one TRS is configured in the indicated TCI state.

- Scheme-2: the RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state.

- Enabling one of either Scheme-1 or Scheme-2 is selected by NW.

  - FFS: The above selection is via an explicit RRC parameter or an implicit manner, e.g., if the RS(s) for new beam are CSI-RS, Scheme-1 is enabled; otherwise, Scheme-2 is enabled.

  - (Working Assumption) Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam.

- The above QCL RS is the RS w.r.t. QCL-TypeD, if there are two QCL RSs in the indicated TCI state.

Agreement

Regarding RS measurement for the new beam for Event 2, at least Option-3a is supported

- Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured

- FFS: Option-3b/3c

  - Option-3b: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s).

  - Option-3c: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of TCI state(s) in a configured subset of the legacy RRC-configured TCI state list

R1-2404715	Moderator Summary #3 on UE-initiated/event-driven beam management	Moderator (ZTE)

Agreement

On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, for a report instance where N ≥ 1 beam(s) are reported, the following is supported.

- RRC can enable or disable whether current beam is always reported

    - When enabled by RRC, the current beam + N beams from the measurement RSs for new beam(s) are reported

      - Note: The reported current beam is NOT counted in the N reported beams.

    - When disabled by RRC, N beams are reported.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-A, the DCI format in Step-2 comprises UL-grant DCI format, and the second channel in Step-3 is at least PUSCH.

- The UL-grant DCI format at least comprises DCI format 0_1/0_2.

  - FFS: DCI format 0_3

- FFS: How to trigger the UEI beam report by the UL-grant DCI format

- FFS: the DCI format in Step-2 comprises DL-grant DCI format, and the second channel in Step-3 is PUCCH.

    - 1-bit field in the DL-grant DCI format is introduced to indicate the transmission of the UEI beam report

      - The PUCCH resource for HARQ-ACK transmission can be reused to carry both the HARQ-ACK and UEI beam report.

    - The DL-grant DCI format at least comprises DCI format 1_1/1_2.

      - FFS: DCI format 1_3

R1-2405638	Moderator Summary #4 on UE-initiated/event-driven beam management	Moderator (ZTE)

Agreement

Regarding the triggering event determination for Event 2:

- If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

  - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam

Above feature is subject to UE capability.

- Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs

FFS: Whether the above is captured in RAN1 or RAN2 specification.

Agreement

On UE-initiated/event-driven beam reporting, regarding trigger events, the following Event-1 and 7a/7b, are provided for down-selection or combination in RAN1#118 (possible outcome is that no new event is supported)

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-7a: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the worst quality.

- Event-7b: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the best quality.

Agreement

Regarding explicit RS configuration for new beam measurement for Event 2, down-select the following options in the RAN1#118:

- Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration;

  - FFS: The RS in the RS resource set can be updated by MAC-CE.

- Option-2: A list of RS(s) for new beam measurement can be configured by RRC, and a subset can be activated for new beam measurement by MAC-CE.

  - FFS: If a list size is small, MAC-CE activation is not needed

- Option-3: A list of RS resource (s) for new beam measurement can be configured by RRC, and a subset of RS resource(s) in the list can be provided for new beam measurement by indicated TCI state.

- Others are not precluded.

- FFS: Each RS for new beam measurement should be associated with a configured joint/DL TCI state which can be used as the indicated TCI state

Agreement

Regarding RS measurement for the current beam for Event 2, for Option-2a, besides for scheme-1 and scheme-2, further study the following for handling the case that only one TRS is configured in the indicated TCI state.

- Option-1: Introducing additional scheme: the RS for current beam can be a CSI-RS for beam management derived from the QCL RS in the indicated TCI state;

- Option-2: Further support TRS as measurement RS of current beam for determining L1-RSRP

- Option-3: Introducing additional scheme: The RS for current beam is explicitly configured by RRC or MAC-CE (Option-2C in RAN1 116b agreement).

- Option-4: No further enhancement (i.e., in such case, Scheme-2 is used)

- Others are not precluded.

- RAN1#118

R1-2406044	Moderator Summary #1 on UE-initiated/event-driven beam management	Moderator (ZTE)

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, for regarding Mode-B, the pre-configured resource(s) for the second channel in Step-2 is at least type 1 CG-PUSCH.

- FFS: PUCCH as the second channel

- FFS: Whether the PUSCH can be with UL data

Agreement

Regarding explicit RS configuration for new beam measurement for Event 2, at least Option-1 is supported

  - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration

  - If legacy UE capability signaling cannot be reused, introduce a UE capability signaling of indicating the maximum number of the configured RS(s) in the RS resource set.

  - FFS: The RS in the RS resource set can be updated by MAC-CE

Agreement

On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, the following differential L1-RSRP report format is supported.

CRI or SSBRI #1

CRI or SSBRI #2

…

CRI or SSBRI #N

L1-RSRP #1

Differential L1-RSRP #2

…

Differential L1-RSRP #N

Differential L1-RSRP for current beam, if report mode that current beam is always reported is enabled by RRC

Note: Other contents are not precluded

.

    - Differential L1-RSRP #2~#N/current beam is determined based on the difference between measured L1-RSRP corresponding to the CRI/SSBRI #2~#N/current beam and the measured L1-RSRP corresponding to CRI/SSBRI #1

      - L1-RSRP#1 is the largest measured RSRP among reported ones, and an absolute L1-RSRP.

      - FFS: range and step size of differential L1-RSRP

    - FFS: Whether/how to report additional indication of which CRI/SSBRI(s) satisfy the condition of Event-2.

    - FFS: Additional report content(s) (e.g., reporting configuration ID, indication for synchronization state, event ID, or cell ID).

R1-2406045	Moderator Summary #2 on UE-initiated/event-driven beam management	Moderator (ZTE)

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-A, down-select one of the following

- Alt-1 (dedicated SR for mode-A): Introduce RRC parameter, e.g., reportResourceRequest-UEIBR, corresponding to the one-bit indication in the first PUCCH channel

  - The RRC parameter is associated with the dedicated SchedulingRequestId.

    - Note: The detailed signaling is up to RAN2.

- Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeA-UEIBR, for the periodic PUCCH resource configuration.

  - Note: The RRC parameter is NOT associated with SchedulingRequestId.

  - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR.

  - The dedicated RRC parameter at least comprises the following:

    - periodicityAndOffset

    - PUCCH-ResourceID

- Above applies at least for the single CC case.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A, select one of the following options

- Option-1: Introduce a new 1-bit field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report

  - FFS: DCI format 0_3

- Option-2: Reuse CSI request field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report

  - FFS: DCI format 0_3

- FFS: Whether/how to handle the case that multiple CSI report configuration(s) for the UE-initiated/event-driven beam report are associated with the same first PUCCH resource and/or the same scheduled PUSCH

Conclusion

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-A, there is no RAN1 consensus on additionally supporting that the DCI format in Step-2 comprises DL-grant DCI format, and the second channel in Step-3 is PUCCH.

R1-2406046	Moderator Summary #3 on UE-initiated/event-driven beam management	Moderator (ZTE)

Agreement

On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, the candidate value of ‘N’ at least comprises {1, 2, 3, 4}

- FFS: additional candidate value(s) of {5, 6, 7, 8}

- FFS: If ‘N’ is not RRC configured, only one L1-RSRP and CRI/SSBRI are reported by default.

Agreement

Regarding RS measurement for the current beam for Event 2, for Option-2a, besides for scheme-1 and scheme-2, further down-select one of the following for handling the case that only one TRS is configured in the indicated TCI state in RAN1#118bis

- Option-1: Introducing additional scheme: the RS for current beam can be a CSI-RS for beam management derived from the QCL RS in the indicated TCI state;

- Option-2: Further support TRS as measurement RS of current beam for determining L1-RSRP

- Option-3: Introducing additional scheme: The RS for current beam is explicitly configured by RRC or MAC-CE (Option-2C in RAN1 116b agreement).

- Option-4: No further enhancement

Note: If there is no consensus in RAN1 on one of Options 1/2/3, Option 4 will be taken.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH carry the beam report, for the second UL channel in Mode-B, at least one or both of the following should be supported:

- Option-1: The same Type-1 CG PUSCH can carry UL-SCH and the beam report.

- Option-2: The Type-1 CG PUSCH is a dedicated type-1 CG PUSCH for carrying the beam report

  - Note: This PUSCH can NOT carry UL-SCH. This PUSCH can NOT carry any other UCI.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-B, UEI beam report is carried on a first available transmission occasion of the second UL channel X symbols after sending the last symbol of report notification on the first PUCCH channel.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding resource mapping/configuration between first and second channel in Mode-B, for a given CSI report configuration, the following is provided for down-selection.

- Option-1 (one-to-one): Only one first PUCCH resource and only one pre-configured resource for second UL channel can be associated with the CSI report configuration for UE-initiated/event-driven beam reporting.

  - Option-1A: Same periodicity between first PUCCH resource and pre-configured resource for second UL channel.

  - Option-1B: No restriction in terms of periodicity.

- Option-2 (one-to-M): Only one first PUCCH resource and one or more pre-configured resource(s) for second UL channel can be associated with CSI report configuration for UE-initiated/event-driven beam reporting.

R1-2407470	Moderator Summary #4 on UE-initiated/event-driven beam management	Moderator (ZTE)

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, for at least Mode-B, the beam report should be carried in the second UL channel in the CC where the corresponding CSI report configuration is configured.

- Above applies to both cross-CC and same-CC beam report.

- Note: Above is applied to the case that second UL channel is PUSCH.

- FFS: Whether the first and second channels can be from the same/different CC.

Working Assumption

On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported.

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality.

  - M is RRC configured with subjective to UE capability signalling

    - UE may only indicate a single candidate value or not support Event-7.

- The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise

- The additionally supported events will be lower priority compared to event 2.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-A and/or Mode-B, further study the following for first PUCCH transmission

- UCI multiplexing/dropping/prioritization rule

- Conditions for the transmission of the first PUCCH.

- Whether the PUCCH resource in the first PUCCH channel can be associated with multiple CSI report configurations for UE-initiated/event-driven beam reporting from one or multiple CC(s).

- Whether/how to re-transmit the first PUCCH channel.

Whether/how to apply prohibit-timer or maximum number of (re)transmission(s) for first PUCCH channel.

## 5.2 RAN1 agreements on asymmetric DL sTRP/UL mTRP (not exhaustive)

Agreement RAN1#116

For the asymmetric DL sTRP/UL mTRP deployment scenarios, support to associate a UL TCI state with a PL offset:

- When a UL TCI state associated with a PL offset is applied for the PUSCH/PUCCH/SRS transmission, the UE shall calculate the Tx power of the PUSCH/PUCCH/SRS based on the DL PL RS and PL offset associated with this UL TCI state.

  - Reuse the legacy uplink power control formulation by replacing legacy PL with UL PL which is derived from the DL PL RS and the PL offset.

  - FFS: The UE can update UL PL in a way that new UL PL = current UL PL + an update delta indicated by the NW.

Agreement RAN1#116

For the asymmetric DL sTRP/UL mTRP deployment scenarios, separate DL/UL TCI state mode of Rel-17/18 unified TCI framework can be configured for both FR1 and FR2.

- Joint TCI state mode can be configured at least for FR1

Agreement RAN1#116bis

For FR1, a joint TCI state can be associated with a PL offset.

- When a joint TCI state associated with a PL offset is applied for the PUSCH/PUCCH/SRS transmission, the UE shall calculate the Tx power of the PUSCH/PUCCH/SRS based on the DL PL RS and PL offset associated with this joint TCI state.

  - Reuse the legacy uplink power control formulation by replacing legacy PL with a PL which is derived from the DL PL RS and the PL offset.

  - FFS: The UE can update UL PL in a way that new UL PL = current UL PL + an update delta indicated by the NW.

Agreement RAN1#116bis

Support applying PL offset on PDCCH-order PRACH towards a UL TRP in FR1.

- Note: The DL reference timing determination for PDCCH-order PRACH transmission to an UL TRP is still based on the DL RS defined in current RAN4 specification

- Above is subject to a separate UE capability signaling

Agreement RAN1#118

For indicating a PL offset for PDCCH-order PRACH transmission at least for FR1, support Alt3:

- Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission

  - FFS the detailed design of DCI format: e.g., how to indicate one of the indicated joint/UL TCI states or whether to apply the PL offset in the indicated TCI state or not.

Agreement RAN1#117

For the association between PL offset and joint/UL TCI state, support the following

- Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s).

Agreement RAN1#118

For the asymmetric DL sTRP/UL mTRP scenarios, the value range of PL offset includes at least [-10, 60] dB

- FFS: Extending the range to lower than -10dB

- Step size is 4dB