3GPP TSG-RAN2 Meeting #127bis	R2-2409028

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.2.4

Source:	Qualcomm Incorporated

Title:	Views on Random Access Aspects of Ambient IoT

WID/SID:	FS_Ambient_IoT_solutions – Release 19

Document for:	Discussion and Decision

# Introduction

Rel-19 Ambient IoT SID [1] describes the RAN2 objectives and Ambient IoT random access should be studied.

In RAN2 #125bis meeting, the following agreements were made regarding the Ambient IoT random access.

- RAN2 confirms slotted-ALOHA is the baseline for Ambient IoT random access

- We will study the support for access triggering for a single device, group of devices, or all devices.    RAN2 to discuss the contention-based and contention-free access procedures and detailed solutions.

- Random Access is triggered by the reader

- Reader provides the information that the device needs to respond to the random access trigger.  FFS what those parameters are

- Study the solution and benefits of both 2-step like random access procedure and 4-step like random access procedure.  FFS the details on each procedure and how we call it.

- Handling of contention resolution failure and access failure at the device will be studied in RAN2, including failure detection and re-access.  FFS details

- For the very first access message from the device to reader in random access an ID is included.  RAN2 to discuss whether a temporary identifier is included, or the permanent device ID is included (considering other WGs input as well).

RAN2 #126 made the following agreements related to Ambient IoT random access.

Agreements on “4 step” RA

1	A-IoT Msg1: the device sends an ID to the reader.  ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).  FFS on ID size.  This doesn’t preclude any other RAN1 agreed information

2	A-IoT Msg2: the reader echos the ID received in Msg1.   Further information may be included in mgs2 based on RAN1 agreements

3	A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request)

4  The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose.

5  “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed.

RAN2 will not use “Msg4” term for further discussion of the random access.

Agreements on 2 step CB RA

- A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed.  This doesn’t preclude any other RAN1 agreed information

- A-IoT Msg2: the reader may echo some information from Msg1.  FFS what some information is.  “Msg2” usage/presence can be further discussed

Further, RAN2 #127 made the following agreements related to Ambient IoT random access.

=> wait for further RAN1 progress on indication of the start of access occasion. => RAN2 assumes that Msg3 transmission resource can explicitly be indicated in Msg2 (based on RAN1 agreements/design). Wait for RAN1 for further details.

Agreements

-	For 3-step CBRA Support fixed random ID size is 16 bit.   The ID is randomly generated.

-	Failure/success indication of D2R will be studied.   FFS if it would be implicit or explicit and for which use case it is needed.  FFS whether it is applied only to some cases.

Agreements

-	for 2step CBRA, RAN2 design will support msg2.  Whether it is needed it is up to the reader.  FFS when it is needed.  For 2-step CBRA (when mgs2 is needed), the random ID (fixed 16bits) is also included in A-IoT Msg1, and is echoed in A-IoT Msg2.   FFS if there will be devices support only 2-step RA and any other optimizations will be needed for such devices.

-	In contention-free access, the A-IoT device directly sends the upper layer data (e.g. device ID) in its very first D2R message after being triggered (i.e. skip contention resolution Msg1/2).   FFS if a short AS ID is also included in the message and what type of ID for scheduling purposes.

-	FFS if reader assigns the AS ID for scheduling purposes

In this paper, we discuss the random-access aspects of Ambient IoT (AIoT) and provide our view.

# Discussion

The Reader may directly indicate the dedicated AIoT access resource to AIoT device for random access. It is beneficial for the command use case that Reader can identify one particular AIoT device to act upon the command.

Proposal 1: Reader can provide a dedicated resource (contention-free) to the AIoT device in the trigger message (e.g., if the Reader has already inventoried and knows the device identifier).

When AIoT trigger message indicates the AIoT device to respond, it is straightforward and efficient to provide the corresponding access resource in the AIoT trigger message. Reader can estimate rough number of devices and provide the appropriate AIoT access resources. The same principle can also be applied to the subsequent AIoT trigger message.

Proposal 2: The AIoT access occasion resources could be provided in the AIoT trigger message and the subsequent AIoT trigger message.

The contention-based AIoT access occasion could be provided by Reader to those AIoT devices who can receive the trigger message. The AIoT devices may select AIoT access occasion among the resources provided by the Reader. The resource selection in the time domain of the AIoT access occasions should be supported. Other schemes of the resource selection of the AIoT occasions can be further studied by RAN1/RAN2.

Proposal 3: The AIoT devices selects the AIoT access occasion among the resources provided by Reader. The resource selection in the time domain of the AIoT access occasion is supported. Other schemes of the resource selection of the AIoT occasions can be further studied by RAN1/RAN2.

In the inventory or command procedure, the trigger message may optionally include the filter criteria sent by the upper layer to identify the AIoT devices that are supposed to respond to the inventory request or command message. The Reader forwards the filter criteria in the inventory signaling to the AIoT devices.

For AIoT access, the AIoT device should check the filter criteria provided by Reader before performing the AIoT access. Only if the criteria are satisfied, the AIoT devices are allowed to perform the AIoT access to the Reader. What information are included in the filter criteria can be up to SA2 for further study.

Proposal 4: If the Reader provides filter criteria in the initial trigger message, only those AIoT devices that meet the filter criteria can perform AIoT access.

In the past RAN2 meetings, the 3-step or 2-step AIoT random access were discussed. Since AIoT access is triggered by Reader, Reader will be responsible for providing the access resource considering different services, i.e., inventory request or command message. Reader may have preference on which type of AIoT access to be triggered in response to the service request.

For example, if Reader would like to issue the ‘Write’ command to a small number of AIoT devices, a 2-step AIoT access can be selected for AIoT devices to feed back a simple confirmation after the command execution. In another example, Reader may indicate AIoT device to perform 3-step AIoT access if a large number of AIoT devices will perform AIoT access by using the contention-based resource. Therefore, Reader should be able to indicate the type of AIoT access to AIoT devices in the trigger message.

Proposal 5: Reader indicates the type of AIoT access (e.g., 2-step or 3-step) to AIoT devices in the initial trigger message.

If Reader does not indicate the type of AIoT access to AIoT devices, AIoT devices may evaluate criteria to determine the type of AIoT access.

Proposal 6: If the type of AIoT access is not indicated by Reader in the initial trigger message, AIoT device may evaluate criteria to determine the type of AIoT access. FFS the details of criteria.

RAN2#126 discussed whether the ‘Msg4’ is required for 3-step AIoT random access. RAN2 agreed to not use the term ‘Msg4’ for 3-step AIoT random access. The so-called ‘Msg4’ could be a failure indication for AIoT Msg3 or the acknowledgement like the Msg4 of NR RACH procedure. Besides, if there are subsequent data transmissions, the subsequent R2D message could be an implicit ACK for the previous D2R message.

RAN2 #127 discussed the AIoT Msg2 usage in 2-step AIoT random access. When the AIoT Msg2 is needed is FFS. In our view, it depends on the use cases. If the AIoT function would like to only collect the AIoT data (i.e., ‘read’ command) from the device regardless of which AIoT device sends the data, no success/failure indication is needed. If the use case is to count the AIoT devices (i.e., ‘inventory’ command) under the Reader coverage, the AIoT Msg2 is also not required if Reader is able to detect the D2R message sent from AIoT device. In that sense, the ‘Msg4’ in 3-step CBRA and AIoT Msg2 in 2-step CBRA could have the similar use case and they can be optional in some cases.

Proposal 7: Similar to ‘Msg4’ in 3-step CBRA, the AIoT Msg2 in 2-step CBRA can be optional in some cases.

While some scenarios would need the subsequent R2D message (e.g. for acknowledging the reception of the AIoT device ID and/or AIoT upper layer data sent in the previous D2R message or for triggering the next round of AIoT re-access or AIoT data communication), in other scenarios there would be no such subsequent R2D message (e.g. Reader requesting information from temperature sensors and no intention to ACK/NACK successful reception).

It is beneficial for AIoT device to have correct expectation on next step after transmitting AIoT device ID and/or AIoT upper layer data. Since Reader should determine the next step of AIoT device and the information carried in the subsequent R2D message may be different, the earlier R2D message from Reader can indicate the presence and information to be carried in the subsequent R2D message.

Proposal 8: An R2D message (e.g., AIoT trigger message) can indicate the presence and information carried in the subsequent R2D message (e.g., ‘Msg4’ in 3-step CBRA).

In some cases, the AIoT access and/or AIoT data reporting procedure may fail. If AIoT Msg2 fails to echo the random ID sent from AIoT Msg1, or the AIoT device does not receive AIoT Msg2 at all, the contention resolution is considered unsuccessful. In such cases, the AIoT device won’t be able to transmit the AIoT device ID and/or AIoT upper layer data to the Reader. In legacy NR (4-step) RACH, UE performs re-attempt by sending the AIoT Msg1 again after a backoff time if UE fails to receive the RAR within a given time window.

There are also other cases that the device ID and/or AIoT upper layer data sent in AIoT Msg3 is not received successfully at the Reader even if the contention resolution is successful. In such cases also, AIoT device should be able to perform AIoT re-access. Therefore, RAN2 should study if any other opportunities are provided to AIoT devices for re-access or to handle the different failure cases.

Proposal 9: If the AIoT device’s contention resolution is unsuccessful or the AIoT data transmission is failed, the AIoT device should be able to perform AIoT re-access. FFS details of AIoT re-access.

Due to the nature of backscatter communication of the AIoT device and poor energy storage, the AIoT device with lower capability may have to first harvest energy before performing AIoT re-access. Thus, one option is that AIoT device perform re-access only upon reception of next trigger message from Reader. The AIoT device can have opportunity to charge more energy before the AIoT re-access.

For AIoT device with higher capability, it may have large energy storage, and such device may not need to wait till the next trigger message for energy harvesting and may be able to autonomously perform re-access e.g., even before the next trigger from the Reader. Thus, RAN2 is suggested to study the AIoT devices to perform AIoT re-access with different options.

Proposal 10: RAN2 to study the following options for AIoT devices to perform AIoT re-access.

Option 1: AIoT devices perform re-access only upon reception of next trigger message from Reader.

Option 2: AIoT devices autonomously perform re-access without waiting for next trigger message from Reader.

When performing re-access, AIoT device may select the access occasion to transmit the AIoT Msg1 again. The access occasions for AIoT re-access can be configured by the next trigger message and the access occasions may be different from the resource configured by the initial trigger message.

If AIoT device is able to autonomously perform re-access, it also depends on whether the device has been configured with multiple access occasions for re-access by the initial trigger message. It is also possible that Reader may directly indicate the access occasions for re-access.

Proposal 11: During AIoT re-access, AIoT device can transmit the AIoT Msg1 again in a newly selected access occasion or in an indicated access occasion.

RAN2 #127 discussed whether a short AS ID is needed and what type of ID is for subsequent scheduling purposes. In RAN2 discussion, the usage of the short AS ID may be mainly for the subsequent data transmission in the AIoT contention-free access. Meanwhile, there were also discussions in RAN1 related to the ‘ID associated with devices’ in D2R scheduling and R2D reception. Essentially, whether the AS ID is really needed is dependent on the RAN1 designs on R2D or D2R reception/transmission, e.g., the ID associated with the specific device for D2R scheduling or the ID for the device to receive the R2D message. Thus, RAN2 should first check with RAN1 whether the short AS ID is needed for subsequent D2R scheduling and R2D reception.

Proposal 12: RAN2 should check with RAN1 whether the AS scheduling ID is needed for subsequent D2R scheduling and R2D reception.

If RAN1 can confirm the usage of the short AS ID, RAN2 can further study the candidate solutions for the short AS ID. Considering the low capability of AIoT device, it is not desirable to require device to maintain any additional ID assigned from Reader because it might require additional device memory to store the ID and the ID information may be lost if device is out of energy. Since the subsequent data transmission may not be expected be too many, it makes little sense to require AIoT device to maintain many IDs. In that sense, any additional ID assigned by the Reader is not preferred.

For AIoT contention-based RA, the random ID generated in AIoT Msg1 can be reused as the AS ID for the subsequent data scheduling.

Proposal 13: For AIoT contention-based access, if random ID is generated in AIoT Msg1 and contention resolution is successful in AIoT Msg2, the random ID can be reused as the AS ID for subsequent data scheduling.

For AIoT contention-free RA, there are several options listed in the email discussion ([POST127][033][AIoT] Random Access). But we believe the simplest solution is that the AS ID could be derived from the part of the full AIoT device ID which should be sent to Reader in response to the trigger message. For example, after Reader receives the full AIoT device ID from the AIoT device, Reader can echo back partial upper layer AIoT device ID and use the partial AIoT device ID as AS ID.

Proposal 14: For AIoT contention-free access, the AS ID for subsequent scheduling is derived from partial upper layer AIoT device ID.

Since AIoT device is expected to be ultra-low complexity device with ultra-low power consumption for the very-low end IoT applications, some very low-end AIoT devices, e.g., device type 1, may requires more rounds of energy harvesting to accumulate enough energy for inventory response.

In some cases, the AIoT devices may miss the inventory/command due to inability of accurate sync signal tracking. In other cases, the congestion may be heavy in the massive IoT access procedure. In that sense, the multiple rounds of AIoT access should be supported. It will be helpful if Reader could periodically transmit inventory command to discover the nearby AIoT devices.

Proposal 15: RAN2 to study the multiple rounds of AIoT access procedure, e.g., the periodical transmission of inventory, by Reader.

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Proposal 1: Reader can provide a dedicated resource (contention-free) to the AIoT device in the trigger message (e.g., if the Reader has already inventoried and knows the device identifier).

Proposal 2: The AIoT access occasion resources could be provided in the AIoT trigger message and the subsequent AIoT trigger message.

Proposal 3: The AIoT devices selects the AIoT access occasion among the resources provided by Reader. The resource selection in the time domain of the AIoT access occasion is supported. Other schemes of the resource selection of the AIoT occasions can be further studied by RAN1/RAN2.

Proposal 4: If the Reader provides filter criteria in the initial trigger message, only those AIoT devices that meet the filter criteria can perform AIoT access.

Proposal 5: Reader indicates the type of AIoT access (e.g., 2-step or 3-step) to AIoT devices in the initial trigger message.

Proposal 6: If the type of AIoT access is not indicated by Reader in the initial trigger message, AIoT device may evaluate criteria to determine the type of AIoT access. FFS the details of criteria.

Proposal 7: Similar to ‘Msg4’ in 3-step CBRA, the AIoT Msg2 in 2-step CBRA can be optional in some cases.

Proposal 8: An R2D message (e.g., AIoT trigger message) can indicate the presence and information carried in the subsequent R2D message (e.g., ‘Msg4’ in 3-step CBRA).

Proposal 9: If the AIoT device’s contention resolution is unsuccessful or the AIoT data transmission is failed, the AIoT device should be able to perform AIoT re-access. FFS details of AIoT re-access.

Proposal 10: RAN2 to study the following options for AIoT devices to perform AIoT re-access.

Option 1: AIoT devices perform re-access only upon reception of next trigger message from Reader.

Option 2: AIoT devices autonomously perform re-access without waiting for next trigger message from Reader.

Proposal 11: During AIoT re-access, AIoT device can transmit the AIoT Msg1 again in a newly selected access occasion or in an indicated access occasion.

Proposal 12: RAN2 should check with RAN1 whether the AS scheduling ID is needed for subsequent D2R scheduling and R2D reception.

Proposal 13: For AIoT contention-based access, if random ID is generated in AIoT Msg1 and contention resolution is successful in AIoT Msg2, the random ID can be reused as the AS ID for subsequent data scheduling.

Proposal 14: For AIoT contention-free access, the AS ID for subsequent scheduling is derived from partial upper layer AIoT device ID.

Proposal 15: RAN2 to study the multiple rounds of AIoT access procedure, e.g., the periodical transmission of inventory, by Reader.

# References

- RP-240826, Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA