3GPP TSG-RAN WG2 Meeting #127bis	R2-2409039

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.1.2.2

Source:	LG Electronics Inc.

Title:	NW side Data Collection

Document for:	Discussion and Decision

1. Introduction

During the last meeting, RAN2 reached the following agreements.

RAN2 #127 - As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is support. - UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size. This is across all use cases - When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging. - Measurements for data collection purposes and logging based can be controlled based on power state of the UE. It is up to UE implementation how the UE determines power state. FFS whether the UE stops autonomously or if it reports to the network . - FFS whether AS buffer event based reporting is supported. FFS if we send availability indication or full report if it is supported - FFS on event based data collection/logging - On-demand request from the network is supported. FFS details on signalling - The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. No standardized RRC segmentation procedure is needed (as for the logged MDT measurements) - Data collection report will not be transmitted over SRB1. FFS which SRB is used.

This contribution aims to specify the configuration and procedure involved in data collection for L1 measurement results.

2. Discussion

Logging architecture

RAN2 has agreed to introduce logging mechanism for data collection. When logging is necessary, the UE transfers data from the L1 (Physical layer) to the L3 (RRC layer), and it reports the logged data via L3 reporting. However, if logging is not required, data collection and reporting through RRC may lead to unnecessary internal operation within the UE to exchange data between layers.

For beam management use case, the data contents, such as L1-RSRP and/or beam index, may not differ depending on the purpose of data collection (e.g., inference, monitoring, or training). Therefore, whether the data is reported via L1 or L3 can be determined by whether the UE logs the data, not by the specific purpose of collection. Consequently, it is more efficient to consider L3 reporting (e.g., MDT reporting) only for logging-based reporting, i.e., when logging is not necessary, data is transmitted via L1 reporting (e.g., CSI reporting)

<Figure 1. Example of Logging architecture>

Proposal 1: To consider L3 reporting (e.g., MDT reporting) only for logging-based reporting, i.e., when logging is not necessary, training data is transmitted via L1 reporting (e.g., CSI reporting)

Configuration

In order to enable logging within the RRC, a dedicated CSI configuration for RRC logging can be newly established. This involves configuring the resource configuration for periodic, aperiodic, and semi-persistent CSI-RS, separate from the logging and reporting settings for each resource/resource set. However, CSI reporting may still be required for inference and monitoring operations where logging is unnecessary for the same resource/resource set, in which case the conventional CSI reporting via the L1 layer will continue to be used. Establishing a separate resource configuration solely for logging, identical to the one used for CSI reporting, would result in unnecessary duplication. Therefore, it is preferable that resource configuration for logging refer to the resource configuration of conventional CSI reporting settings.

Observation 1. CSI reporting may still be required for inference and monitoring operations where logging is unnecessary, in which case the conventional CSI reporting via the L1 layer will continue to be used. Establishing a separate resource configuration solely for logging, identical to the one used for CSI reporting, would result in unnecessary duplication.

Proposal 2. To consider that resource configuration for logging refer to the resource configuration of conventional CSI reporting settings

To configure L3 reporting based on L1 measurements, the configuration can be affected from two parts. One part is for L1 measurement-related configuration (i.e., conventional CSI reporting setting), while the other part is for L3 reporting-related configuration (i.e., conventional Measurement reporting setting).

<Figure 2. Configuration structure>

The L1 measurement-related configuration can be further divided into three sub-parts: Triggering State, Report and Resource Configuration, and Physical Layer Configuration.

- Triggering State: Contain trigger states for dynamically selecting one or more aperiodic and semi-persistent reporting configuration and/or triggering one or more aperiodic CSI-RS resource sets

- Report and Resource Configuration: Contain reporting configuration for a periodic/semi-persistent/aperiodic report and resource configuration including a group of one or more NZP-CSI-RS-ResourceSet, CSI-IM-ResourceSet and/or CSI-SSB-ResourceSet

- Physical Layer configuration: Contain references to NZP CSI-RS resources, CSI-IM resources, and SSB resources in a CSI-RS resource set

The L3 reporting-related configuration can be further divided into three sub-parts: Measurement Object, Logging and Reporting configuration, and Measurement ID configuration.

- Measurement Object (MO) : Regarding logging, measurement object configuration can include reference signal configuration for obtaining L1 measurement results. The Reference Signal configuration for logging can be specified in the referenceSignalConfig IE, or in another IE at the same/different hierarchical level within the message structure, but it could be discussed in stage 3 discussion.

- Report configuration:  Logging configuration and Reporting configuration can be included in report configuration.

  - Logging configuration can include logging interval to indicate the periodicity for logging.

  - Reporting configuration can include reporting interval, reporting amount, and reporting

- Measurement Identification

  - Each measurement Id is linked with measurement object and report configuration.

Resource configuration for logging in MO can be linked to one of configurations in CSI reporting settings. There may five alternatives as below:

- Alternative 1. Link to reference signal, such as certain CSI-RS(s)/SSB(s) in L1 measurement-related configuration as shown in (1) of Figure 2.

- Alternative 2. Link to resource set ID(s) associated with nzp-CSI-RS/csi-IM/csi-SSB resource(s) in L1 measurement-related configuration as shown in (2) of Figure 2.

- Alternative 3. Link to resource config ID(s) associated with at least one resource set ID in L1 measurement-related configuration as shown in (3) of Figure 2.

- Alternative 4. Link to report config ID(s) associated with resource config ID(s) in L1 measurement-related configuration as shown in (4) of Figure 2.

- Alternative 5. Link to entry index value of triggering state list associated with report config ID in L1 measurement-related configuration as shown in (5) of Figure 2.

From the perspective that the logging object is ultimately the reporting object, Alternative 4 makes sense to indicate logging object. Alternative 4 can also reuse the legacy DCI/MAC CE for CSI reporting to trigger logging. On the other hand, for Alternative 1-3, a separate DCI/MAC CE/RRC signalling is needed to indicate when to start/stop logging, especially for aperiodic/semi-persistent CSI-RS.

Proposal 3. If P2 is agreeable, the resource configuration of MO links to the report configuration ID of the CSI reporting settings

There may be a relationship between the logging configuration and the reporting configuration, because the appropriate reporting interval can be determined based on the logging interval. For example, the logging interval may be shorter than the reporting interval, or the reporting interval may be a multiple of the logging interval. Therefore, considering the relationship between the two, it is appropriate to configure the logging configuration to be included in the reporting configuration.

Proposal 4. To confirm logging configuration is configured in report configuration

Performing Measurement

Periodic CSI logging

In legacy CSI reporting, periodic CSI-RS does not need an activation command for measuring resource set(s). Periodic CSI reporting can also be triggered upon the UE receives CSI configuration. With the same mechanism, measurement/logging for periodic CSI reporting can be triggered upon the UE receives the logging configuration.

Proposal 5. To confirm that periodic CSI measurement/logging can be triggered upon logging configuration associated with corresponding resource/resource set is configured in measurement configuration.

Aperiodic/semi-persistent CSI logging

According to TS 38.214, aperiodic/semi-persistent CSI reporting can be triggered by DCI/MAC CE for periodic/aperiodic/semi-persistent CSI-RS. As mentioned in Proposal 3, the logging object is ultimately a reporting object. A consequently simple way to measure/log reporting objects is to reuse the DCI/MAC CE for CSI reporting to trigger logging of that resource/resource set. When a UE receives a CSI reporting DCI/MAC CE, it can decide whether to log or report the L1 measurement results depending on its logging needs.

| CSI-RS Configuration | Periodic CSI Reporting | Semi-Persistent CSI Reporting | Aperiodic CSI Reporting |
| --- | --- | --- | --- |
| Periodic CSI-RS | No dynamic triggering/activation | For reporting on PUCCH, the UE receives an activation command, as described in clause 6.1.3.16 of [10, TS 38.321]; for reporting on PUSCH, the UE receives triggering on DCI | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |
| Semi-Persistent CSI-RS | Not Supported | For reporting on PUCCH, the UE receives an activation command, as described in clause 6.1.3.16 of [10, TS 38.321]; for reporting on PUSCH, the UE receives triggering on DCI | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |
| Aperiodic CSI-RS | Not Supported | Not Supported | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |

<TS 38.214. Table 5.2.1.4-1: Triggering/Activation of CSI Reporting for the possible CSI-RS Configurations. >

Proposal 6. To confirm that aperiodic/semi-persistent CSI measurement/logging can be triggered upon receiving CSI reporting DCI/MAC CE as legacy CSI reporting mechanism.

Logging Mechanism

Logging Contents

Conventionally, logged measurement information may include time information, location information, serving cell/PLMN information, etc. For Beam management use cases, the logging data includes L1 measurement results such as L1-RSRP and/or beam index. Additionally, a measurement ID may also be included to indicate which logging configuration is associated with. After RAN2 decides what information to include, it is necessary to consult with RAN1 to confirm if there is any additional information required for logging.

Proposal 7. In addition to L1 measurement results, to discuss what should be included in the logging data, such as time information, location information, serving cell/PLMN information, measurement ID, etc.

Periodic CSI logging

The UE can log data at each logging interval. Logging interval can start when measurement configuration is configured/activated, when previous logging interval is expired, and/or when next reporting interval begins. If the logging interval is not configured, the UE can log data when the UE measures the beam(s).

<logging with periodic reporting w/ logging interval>

Proposal 8. For periodic CSI logging, logging interval is configured/applied. FFS on when logging interval starts, FFS on UE operation when logging interval is not configured

Semi-persistent CSI logging

For semi-persistent reporting, the UE may log data at each logging interval since it receives DCI/MAC CE for semi-persistent reporting. Logging interval can start when the UE receive DCI/MAC CE for semi-persistent reporting, or when previous logging interval is expired. If the logging interval is not configured, the UE can log data when the UE measures the beam(s).

<logging with semi-persistent reporting w/ logging interval>

Proposal 9. For semi-persistent CSI logging, logging interval is configured/applied. FFS on when logging interval starts, FFS on UE operation when logging interval is not configured

For periodic/semi-persistent CSI logging, when the UE logs data based on logging interval, there may two alternatives regarding layer for applying logging interval:

- Alternative 1. L1 layer transmits L1 measurement results to L3 layer based on logging interval

- Alternative 2. L1 layer transmits L1 measurement results regardless of logging mechanism. L3 layer log L1 measurement results related information based on logging interval

To prevent unnecessary internal operation within the UE to exchange data between layers, Alternative 1 may be desirable.

Proposal 10. For periodic/semi-persistent logging, L1 layer transmits L1 measurement results to L3 layer based on logging interval

Aperiodic CSI logging

For aperiodic CSI logging, the logging interval is not suitable for aperiodic CSI logging since the UE can log data when it receives DCI/MAC CE. Therefore, the logging interval is not configured/applied to aperiodic CSI logging.

<logging with aperiodic reporting>

Proposal 11. For aperiodic CSI logging, logging interval is not configured/applied

Reporting Mechanism

For Periodic logging

As with the legacy reporting mechanism, the UE can report logged data according to report interval and/or report amount. When report interval is expired, the UE can report logged data. The report interval can start when measurement configuration is configured/activated, or when previous reporting interval is expired (if the number of reporting does not exceed report amount). If report amount is configured, the UE can report logged data the number of times set in report amount.

<reporting with report interval/amount>

Proposal 12. For periodic CSI logging, reporting interval and reporting amount are configured/applied. The UE reports logged data at report intervals as many times as set in the reporting amount.

For Aperiodic/semi-persistent logging

For aperiodic/semi-persistent CSI logging, logging is controlled by DCI/MAC CE. Since the UE and the network cannot predict when a DCI/MAC CE for logging will occur, reporting according to the reporting interval is not suitable. Accordingly, triggering reporting based on the amount of logged data can be considered for efficient reporting. For example, when the number of logged data exceed a certain amount, the UE can report to the network.

Proposal 13. For aperiodic/semi-persistent logging, reporting interval and reporting amount are not configured/applied.

Proposal 14. For aperiodic/semi-persistent logging, to consider reporting based on number of logged data

3. Conclusion

Proposal 1: To consider L3 reporting (e.g., MDT reporting) only for logging-based reporting, i.e., when logging is not necessary, training data is transmitted via L1 reporting (e.g., CSI reporting)

Observation 1. CSI reporting may still be required for inference and monitoring operations where logging is unnecessary, in which case the conventional CSI reporting via the L1 layer will continue to be used. Establishing a separate resource configuration solely for logging, identical to the one used for CSI reporting, would result in unnecessary duplication.

Proposal 2. To consider that resource configuration for logging refer to the resource configuration of conventional CSI reporting settings

Proposal 3. If P2 is agreeable, the resource configuration of MO links to the report configuration ID of the CSI reporting settings

Proposal 4. To confirm logging configuration is configured in report configuration

Proposal 5. To confirm that periodic CSI measurement/logging can be triggered upon logging configuration associated with corresponding resource/resource set is configured in measurement configuration.

Proposal 6. To confirm that aperiodic/semi-persistent CSI measurement/logging can be triggered upon receiving CSI reporting DCI/MAC CE as legacy CSI reporting mechanism.

Proposal 7. In addition to L1 measurement results, to discuss what should be included in the logging data, such as time information, location information, serving cell/PLMN information, measurement ID, etc.

Proposal 8. For periodic CSI logging, logging interval is configured/applied. FFS on when logging interval starts, FFS on UE operation when logging interval is not configured

Proposal 9. For semi-persistent CSI logging, logging interval is configured/applied. FFS on when logging interval starts, FFS on UE operation when logging interval is not configured

Proposal 10. For periodic/semi-persistent logging, L1 layer transmits L1 measurement results to L3 layer based on logging interval

Proposal 11. For aperiodic CSI logging, logging interval is not configured/applied

Proposal 12. For periodic CSI logging, reporting interval and reporting amount are configured/applied. The UE reports logged data at report intervals as many times as set in the reporting amount.

Proposal 13. For aperiodic/semi-persistent logging, reporting interval and reporting amount are not configured/applied.

Proposal 14. For aperiodic/semi-persistent logging, to consider reporting based on number of logged data