3GPP TSG-RAN WG2 Meeting #127bis	R2-2409040

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.1.5

Source:	LG Electronics Inc.

Title:	Model Transfer/Delivery

Document for:	Discussion and Decision

1. Introduction

In Rel-18 AIML study, different options were introduced for model delivery/transfer to UE, training location, and model delivery/transfer format combinations for UE-side models and UE-part of two-sided models:

Table 4.3-1: Model delivery/transfer cases

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| y | model delivery (if needed) over-the-top. | Outside 3GPP Network | UE-side / NW-side / neutral site |
| z1 | model transfer in proprietary format. | 3GPP Network | UE-side / neutral site |
| z2 | model transfer in proprietary format. | 3GPP Network | NW-side |
| z3 | model transfer in open format. | 3GPP Network | UE-side / neutral site |
| z4 | model transfer in open format of a known model structure at UE, i.e., an exact model structure as has been previously identified between NW and UE and for which the UE has explicitly indicated its support. | 3GPP Network | NW-side |
| z5 | model transfer in open format of an unknown model structure at UE, i.e., any other model structure not covered in z4, including any model structure that is only partially known. | 3GPP Network | NW-side |
| Note: The definition of various Cases is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization. |  |  |  |

To support AI/ML model transfer/delivery, the following solutions are also considered in RAN2:

-	Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling.

-	Solution 2a: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling.

-	Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling.

-	Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data.

-	Solution 2b: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via User Plane (UP) data.

-	Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data.

-	Solution 4a: OTT server can transfer/deliver AI/ML model(s) to UE (e.g., transparent to 3GPP).

-	Solution 4b: OAM can transfer/deliver AI/ML model(s) to UE.

During Rel-19 discussion, RAN1 deprioritized case z2, z3, and z5, and has focused on case z4.

RAN1 #116 From RAN1 perspective, the model transfer/delivery Case z5 is deprioritized for Rel-19. RAN1 #116bis Conclusion From RAN1 perspective, the model transfer/delivery Case z2 is deprioritized at least for UE-sided model in Rel-19 due to the following reasons: - Risk of proprietary design disclosure - Burden of offline cross-vendor collaboration Conclusion From RAN1 perspective, the model transfer/delivery Case z3 is deprioritized for Rel-19 due to the following reasons (compared to Case y): - No much benefit compared to Case y - Risk of proprietary design disclosure - Large burden of offline cross-vendor collaboration - Additional burden on model storage within in 3GPP network RAN1 #117 Agreement From RAN1 perspective, for model delivery/transfer Case z4, further study the following alternatives (including the necessity/feasibility/benefits): - Alt. A - Step A-1: UE reports the supported known model structure(s) to network - Step A-2: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step A-1 - FFS: whether some additional step(s), and/or whether other information is needed - Alt. B - Step B-0: UE reports to NW its support of model transfer/delivery case z4 - Note: Step B-0 may be before or after Step B-1, or not necessary - Step B-1: NW indicates to UE the candidate known model structure(s) - Step B-2: UE reports to NW which model structure(s) out of the candidate known model structure(s) indicated in Step B-1 is supported - Step B-3: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step B-2 - FFS: whether some additional step(s), and/or whether other information is needed - Note: Other alternative(s) is not precluded - Note: Other method(s) of parameter exchange from NW to UE side is a separate discussion. RAN1 #118 Agreement From RAN1 perspective, the “known model structure(s)” of the model transfer/delivery Case z4 at least include known information on the following aspects - Model type/backbone (e.g., Transformer, CNN and so on) - In case model type is a neural network - Number of layers - Layer types/structure (e.g., full connected, activation layer and so on) - Layer size (e.g., the number of parameters of a layer) - Connection between different layers - model input/output related information

This contribution analyses the model transfer/delivery solutions from perspective of case z4.

2. Discussion

Case z4 is model transfer in open format of a known model structure at UE, i.e., an exact model structure as has been previously identified between NW and UE and for which the UE has explicitly indicated its support. According to RAN1 agreement, the known model structure related information can be indicated by the UE to the network. However, it does not mean that the model structure needs to be specified in 3GPP.

From RAN1 perspective, the “known model structure(s)” of the model transfer/delivery Case z4 at least include known information on the following aspects - Model type/backbone (e.g., Transformer, CNN and so on) - In case model type is a neural network - Number of layers - Layer types/structure (e.g., full connected, activation layer and so on) - Layer size (e.g., the number of parameters of a layer) - Connection between different layers - model input/output related information

If an AI/ML model structure is delivered through specified filed, the network entity responsible for AI/ML model transfer/delivery may need to understand the contents of the configuration. For example, if RRC is in charge of configuring a proper AI/ML model, it is required that the gNB have a complete understanding of the AI/ML model to use. Also, it requires the massive update of existing gNBs to support ML functionalities. As AI/ML technology develops, model scalability is also a very important issue. As new features are added to the AI/ML model, new model-specific settings may be added. Accordingly, RAN2 spec impact is significant, and RRC cannot catch up with the latest AI/ML model pool applicable in the field/ML community. Consequently, RAN2 could adopt the approach of using known model structures pre-defined between the UE and the network in an offline manner, which can be indicated by a structure ID.

Proposal 1. As a baseline, to assume the approach of using known model structures pre-defined between the UE and the network in an offline manner, which can be indicated by a structure ID. FFS whether a single structure ID can cover following information:

- Model type/backbone (e.g., Transformer, CNN and so on)

- In case model type is a neural network

  - Number of layers

  - Layer types/structure (e.g., full connected, activation layer and so on)

  - Layer size (e.g., the number of parameters of a layer)

  - Connection between different layers

- model input/output related information

According to RAN1 agreement, the network transfers parameters for supported known model structure(s).  Since not all parameters can be specified in RRC for the same reason as in proposal 1, model parameters can be delivered via container method (OCTET STRING) for CP solution, i.e., solution 1a, 2a, 3a, and 4b. To clarify this, RAN2 can add a note in the TR as an annex.

Agreement on RAN1 #117 From RAN1 perspective, for model delivery/transfer Case z4, further study the following alternatives (including the necessity/feasibility/benefits): - Alt. A - Step A-1: UE reports the supported known model structure(s) to network - Step A-2: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step A-1 - FFS: whether some additional step(s), and/or whether other information is needed - Alt. B - Step B-0: UE reports to NW its support of model transfer/delivery case z4 - Note: Step B-0 may be before or after Step B-1, or not necessary - Step B-1: NW indicates to UE the candidate known model structure(s) - Step B-2: UE reports to NW which model structure(s) out of the candidate known model structure(s) indicated in Step B-1 is supported - Step B-3: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step B-2 - FFS: whether some additional step(s), and/or whether other information is needed - Note: Other alternative(s) is not precluded - Note: Other method(s) of parameter exchange from NW to UE side is a separate discussion.

Proposal 2. As a baseline, to assume the approach of using container method (OCTET STRING) for CP solution, i.e., solution 1a, 2a, 3a, and 4b, i.e., no specify model parameters

3. Conclusion

Proposal 1. As a baseline, to assume the approach of using known model structures pre-defined between the UE and the network in an offline manner, which can be indicated by a structure ID. FFS whether a single structure ID can cover following information:

- Model type/backbone (e.g., Transformer, CNN and so on)

- In case model type is a neural network

  - Number of layers

  - Layer types/structure (e.g., full connected, activation layer and so on)

  - Layer size (e.g., the number of parameters of a layer)

  - Connection between different layers

- model input/output related information

Proposal 2. As a baseline, to assume the approach of using container method (OCTET STRING) for CP solution, i.e., solution 1a, 2a, 3a, and 4b, i.e., no specify model parameters

4. Annex

Table 7.3.1.4-1 Analysis of current status and gaps, and potential 
RAN specification impact for Solution 1a

| Discussion Area | Current status and Gaps | Potential RAN specification impact |
| --- | --- | --- |
| A1. Large, no upper limit model/model parameter size | Model size >45kBytes is not supported based on existing number of RRC segments | Extension of the number of RRC segments is required to support models larger than 45kBytes |
| A2. Model transfer/delivery continuity (i.e., resume transmission of model (segments) across gNBs) | Transmission is restarted upon mobility | - Requires service continuity support for SRBs with segmentations. - Xn/NGAP enhancement(s) for model transfer/delivery continuity |
| A3. Network controllability on model transfer/delivery and management at gNB | Management and interaction between UE and gNB is not supported | Requires management and interaction between UE and gNB (e.g., model identification, model transfer completion indication, etc.) when model management at gNB |
| A4. Model transfer/delivery QoS (for DRB) (including latency, etc.) and priority (for SRB) | Procedure latency depends on model size and SRB priority | Impact on SRB in DL, e.g., a new SRB with configurable priority, etc. |

Note:	RAN2 assumes that model/model parameters are delivered through a container type (e.g.,OCTET STRING)

Table 7.3.1.4-2 Analysis of current status and gaps, and potential 
RAN specification impact for Solutions 2a and 3a

| Discussion Area | Current status and Gaps | Potential RAN specification impact |
| --- | --- | --- |
| A1. Large, no upper limit model/model parameter size | - Model size >45kBytes is not supported based on existing number of RRC segments - Core Network supports NAS signalling segmentation - LMF supports LPP signalling segmentation | If NAS/LMF does not do segmentation for model transfer/delivery, it may need RRC segmentation, and extension of the number of RRC segments is required to support models larger than 45kBytes |
| A2. Model transfer/delivery continuity (i.e., resume transmission of model (segments) across gNBs) | Supported with limitation: - For Solution 2a, support within AMF coverage area based on NAS signalling segmentation - For Solution 3a, support within LMF coverage area based on LPP signalling segmentation | Note: Supporting service continuity across AMF/LMF is out of RAN scope and needs coordination with Core Network groups |
| A3. Network controllability on model transfer/delivery and management at gNB | - For Solution 2a, gNB cannot perform management directly, considering model transfer is transparent to gNB - Management and interaction between UE and gNB is not supported | - Requires management and model transfer interaction between Core Network/LMF and gNB, e.g., via NAS signalling or NRPPa signalling when model management at gNB - Requires management and interaction between UE and gNB (e.g., model identification, model transfer completion indication, etc.) when model management at gNB |
| A4. Model transfer/delivery QoS (for DRB) (including latency, etc.) and priority (for SRB) | Procedure latency depends on model size and SRB priority; other latency includes forwarding NAS message latency from Core Network to gNB | Impact on SRB in DL, e.g., a new SRB with configurable priority, etc. |

Note: 	NAS and LMF upper limits and potential impacts to NAS and LPP specifications have not been studied and feasibility on filling gaps is unknown.

Note:	RAN2 assumes that model/model parameters are delivered through a container type (e.g.,OCTET STRING)

Table 7.3.1.4-3 Analysis of current status and gaps, and potential 
RAN specification impact for Solutions 1b

| Discussion Area | Current status and Gaps | Potential RAN specification impact |
| --- | --- | --- |
| A1. Large, no upper limit model/model parameter size | - No model size limitation - PDU session termination at gNB is not supported | Requires PDU session termination at gNB if needed |
| A2. Model transfer/delivery continuity (i.e., resume transmission of model (segments) across gNBs) | Model transfer continuity if PDU session terminated at gNB is not studied | - Identify a solution to support service continuity support between gNBs when PDU session is terminated at gNB if needed - Xn/NGAP enhancement(s) for model transfer/delivery continuity |
| A3. Network controllability on model transfer/delivery and management at gNB | Management and interaction between UE and gNB appear to be feasible but not supported | Requires management and interaction between UE and gNB (e.g., model identification, model transfer completion indication, etc.) when model management at gNB |
| A4. Model transfer/delivery QoS (for DRB) (including latency, etc.) and priority (for SRB) | - Procedure latency depends on model size, QoS requirement and DRB priority - QoS management at gNB if PDU session is terminated at gNB is not supported | Identify a solution to support QoS management at gNB for model transfer when PDU session is terminated at gNB if needed |

Table 7.3.1.4-4 Analysis of current status and gaps, and potential 
RAN specification impact for Solutions 2b and 3b

| Discussion Area | Current status and Gaps | Potential RAN specification impact |
| --- | --- | --- |
| A1. Large, no upper limit model/model parameter size | No model size limitation | - No RAN impact - Note: The detail procedure of model transfer from Core Network/LMF to UE is out of RAN scope |
| A2. Model transfer/delivery continuity (i.e., resume transmission of model (segments) across gNBs) | - For Solution 2b, supported - For Solution 3b, depends on Rel-18 CT1 solution LPP message over a user plane connection between UE and LMF | Note: supporting service continuity across LMF is out of RAN scope |
| A3. Network controllability on model transfer/delivery and management at gNB | gNB cannot perform model management directly | Requires management and model transfer interaction between Core Network/LMF and gNB when model management at gNB |
|  | Management and interaction between UE and gNB is not supported | Requires management and interaction between UE and gNB (e.g., model identification, model transfer completion, etc.) when model management at gNB |
| A4. Model transfer/delivery QoS (for DRB) (including latency, etc.) and priority (for SRB) | - Procedure latency depends on model size, QoS requirement and DRB priority - Other latency includes forwarding data from Core Network to gNB | Note: The detail QoS requirement on Core Network for model transfer/delivery is out of RAN scope |

Table 7.3.1.4-5 Analysis of current status and gaps, and potential 
RAN specification impact for Solutions 4a

| Discussion Area | Current status and Gaps | Potential RAN specification impact |
| --- | --- | --- |
| A1. Large, no upper limit model/model parameter size | No model size limitation | No RAN impact |
| A2. Model transfer/delivery continuity (i.e., resume transmission of model (segments) across gNBs) | - If model transfer/delivery from OTT server via Core Network, supported - If model transfer/delivery from OTT server via LMF, depends on Rel-18 CT1 solution LPP message over a User Plane connection between UE and LMF | Note: supporting service continuity across LMF is out of RAN scope |
| A3. Network controllability on model transfer/delivery and management at gNB | Model transfer/delivery is transparent to RAN | - Requires management and model transfer interaction between OTT server and gNB when model management at gNB - Note: it is unclear whether this is within RAN scope - Requires interaction between UE and gNB for the network controllability of the model transfer/delivery (e.g., model identification, model transfer completion, etc.) if management is in gNB |
| A4. Model transfer/delivery QoS (for DRB) (including latency, etc.) and priority (for SRB) | - Procedure latency depends on model size, QoS requirement and DRB priority - Other latency includes forwarding data from OTT server to gNB | Note: The detail QoS requirement for model transfer/delivery of solution 4a is out of RAN scope |

Table 7.3.1.4-6 Analysis of current status and gaps, and potential 
RAN specification impact for Solutions 4b

| Discussion Area | Current status and Gaps | Potential RAN specification impact (Note: whether and how to support model transfer/delivery from OAM to gNB and OAM to UE directly is out of RAN scope) |
| --- | --- | --- |
| A1. Large, no upper limit model/model parameter size | - Over Control Plane (CP) signalling: model size >45kBytes is not supported based on existing number of RRC segments if OAM does not do segmentation for model transfer/delivery - Over, e.g., IP: no model size limitation, but direct connection between OAM and UE is not supported | - Over Control Plane (CP) signalling: If OAM does not do segmentation for model transfer/delivery, it may need RRC segmentation, and extend RRC segment number if model size larger than 45kBytes - Over, e.g., IP: NOTE: whether and how to support direct connection between OAM and UE is out of RAN scope |
| A2. Model transfer/delivery continuity (i.e., resume transmission of model (segments) across gNBs) | Support within OAM coverage |  |
| A3. Network controllability on model transfer/delivery and management at gNB | gNB cannot perform model management directly | Note: support management and model transfer interaction between OAM and gNB is out of RAN scope |
| A4. Model transfer/delivery QoS (for DRB) (including latency, etc.) and priority (for SRB) | - Over Control Plane (CP) signalling: 1) Procedure latency depends on model size and SRB priority 2) other latency includes forwarding data from OAM to gNB - Over, e.g., IP: direct connection between OAM and UE is not supported | - Over Control Plane (CP) signalling: Note: The detail QoS requirement for model transfer/delivery of solution 4b is out of RAN scope - Over, e.g., IP: Note: whether and how to support latency, QoS requirement between OAM and UE is out of RAN scope |

Note: 	For Solution 4b, RAN2 discussed the following two solutions but did not study or analyse their feasibility:
- OAM can transfer/deliver AI/ML models to UE via OAM→RAN→UE, where Control Plane (CP) signalling is used for RAN→UE.
- OAM can transfer/deliver AI/ML models to UE via OAM→UE, e.g., via IP tunnel.

Note:	RAN2 assumes that model/model parameters are delivered through a container type (e.g.,OCTET STRING)