3GPP TSG-RAN WG2 Meeting #127bis		R2-2409041

Hefei, China, Oct 14 – 18, 2024	revision of R2-2407105

Agenda Item:	8.10.2

Source: 	LG Electronics

Title:  	MRO for MOB DC/CA

Document for:	Discussion

Introduction

In this contribution, we focus on the Rel-18 DCCA mobility MRO based on the last agreements.

Discussion

CHO+CPAC MRO

    - Near failure logging

We would like to first consider how the UE can report logging information related to CHO+CPAC to the network.

Firstly, let's take a look at the flow chart shown in section 5.1.

This flow chart shows logging cases that can occur after the UE is configured with CHO+CPC. According to the flow chart, we can see that in all cases where CHO+CPC logging is required, except for clear success case, the CHO+CPC information can be reported to the network via an RLF report or SHR (Successful Handover Report).

In other words, although PSCell change can be performed together, CPC is an additional condition based on CHO and the configuration is the same as CHO+SCG, so there is no need to consider SPR (Successful PSCell change or addition Report) for new logging information of the CHO+CPC.

Proposal 1: For CHO with associated CPA/CPC, the UE reports information for the near failure case to the network via SHR not SPR.

Additionally, the flow chart also shows SCG failure information doesn’t need to include new information for the CHO+CPC because of the same reason above (i.e. in all cases where CHO+CPC logging is required, except for clear success case, the CHO+CPC information can be reported to the network via an RLF report or SHR).

If the CHO is successfully completed but a failure occurs in the associated CPA/CPC procedure, the UE sends SCG failure information according to the current specification. Thus, in this case, sending the SCG failure information for the associated CPA/CPC is not a new procedure. We think that the UE should send the SCG failure information without considering additional information. This is because if the SCG is failed in a situation that is not the near failure scenario, it is similar to a legacy failure rather than new scenario requiring additional information for CHO with the associated CPC/CPA.

Proposal 2: For CHO with associated CPA/CPC, the UE uses the SCG failure information to inform the network if only the associated CPC/CPA fails. However, no further information needs to be included in the SCG failure information if the SHR can be adequately enhanced.

Additionally, to simplify the SHR logging condition, we think that the existing SHR conditions can be used as it is. The network may prepare the CHO-only configuration and other execution conditions for candidate cells, so if the CHO+CPC execution condition is not met, most other CHO execution conditions will likely be met. Thus, we think there is no need to consider additional new conditions for SHR logging due to associated CPC/CPA execution condition.

Proposal 3: For CHO with associated CPA/CPC, the UE uses the existing condition of SHR to log the near failure case to the network.

Then, let's consider what detailed information can be included in the SHR. Currently, the following information is included for the SHR as like in 5.2.

We consider additional information that could be included based on the existing SHR information.

- First satisfied execution condition

It should be possible to determine which execution condition was satisfied first between the execution conditions provided for CHO and associated SCG. This is necessary to identify which condition was not satisfied or was satisfied later.

- Satisfaction time of the first execution condition

Information about the exact time when the first satisfied execution condition was met could be helpful. We believe the UE can easily report the network of this information using the existing time stamp information.

- Satisfaction status of the remaining execution condition

It is necessary to provide information about whether the remaining execution condition was satisfied. Even if the UE successfully performs handover, the handover may not be initiated by CHO+CPC. Thus, it would be helpful to know if the remaining execution condition was also met.

- Time gap between two execution conditions

Information about how much time elapsed between the two execution conditions is required. This information inform the network to determine how much time passed after the first execution condition was satisfied till the both execution conditions were met.

Additionally, this information can be also used to inform the network of the time between the first satisfied execution condition and the handover if the rest if execution condition is not met and another handover is initiated.

- Source PSCell signal quality

Information about the signalling quality of the source PSCell could be helpful. Currently, A3 or A5 is not used for the execution conditions of CPC for CHO+CPC configuration. Thus, even if the source PSCell is still good when performing CHO+CPC, PSCell change can happen. If the target PCell can maintain the source PSCell, the signal quality of the source PSCell is information that helps the network achieve more robust CHO+CPC.

- Candidate PSCell signal quality

Information about the signalling quality of the other candidate PSCell could be also helpful. Through this information, the network can figure out which other candidate PSCell are still considered or are not longer considered as candidate SCG for other UEs.

Based on the above information, we propose the following additional information to be included in the SHR:

Proposal 4. For CHO with associated CPA/CPC, the UE newly includes the below information in SHR as the information of the near failure:

- First satisfied execution condition

- Satisfaction time of the first execution condition

- Satisfaction status of the remaining execution condition

- Time gap between two execution conditions

- Source PSCell measurement result

- Candidate PSCell measurement results

    - Failure logging

Since RAN2 agreed that the UE use the RLF-report in case of the failure logging, let's consider what detailed information can be included in the RLF-report. Currently, the following information is included for the RLF-report as like in 5.3.

We already described which detailed information can be logged for SHR. Since the detailed information can also be included in RLF report for SCH+CPC, we propose that the following information should be considered for MCG RLF-report.

Proposal 5. For CHO with associated CPA/CPC, the UE includes the below information in MCG RLF-report as the information of the failure:

- First satisfied execution condition

- Satisfaction time of the first execution condition

- Satisfaction status of the remaining execution condition

- Time gap between two execution conditions

- Source PSCell signal quality

- Candidate PSCell signal quality

SCPAC MRO

SCPAC are not very different from the existing CPAC. Thus, we think that in most cases, existing information can be simply reused for both near-failure and failure cases of SCPAC.

Especially, in a state where pre-configuration is shared between candidate cells for subsequent mobility, if a failure or near failure occurs for the UE, the network will most likely implicitly recognize that the issue has occurred during the subsequent mobility scenario. Therefore, we think that no additional information would be required for mobility optimisation so far.

However, the newly introduced multiple security keys need to be considered as new logging information.

The UE receives multiple security keys in advance for each cell group (i.e., per gNB) to perform SCPAC. When the UE performs SCPAC and the target cell belongs to a different cell group (i.e., during inter-gNB mobility), the UE selects the unused security key of the cell group that includes the target cell to perform a security update. In this case, there might be cases where the network and the UE have a mismatch in the information regarding the used security keys, or the network may have failed to appropriately update the multiple security keys, resulting in no unused security key being available for the security update.

Therefore, it will be helpful for the network if the network knows which security keys have been used and when the security keys have been selected to prevent further security failure in various conditional mobility scenarios.

Proposal 6. For, SCPAC MRO, the UE performs logging for multiple security key status to RLF-report or SHR to help resolve the security key mismatch issue that occurs during SCPAC.

Conclusion

It is proposed to discuss and decide on the following proposals:

Proposal 1: For CHO with associated CPA/CPC, the UE reports information for the near failure case to the network via SHR not SPR.

Proposal 2: For CHO with associated CPA/CPC, the UE uses the SCG failure information to inform the network if only the associated CPC/CPA fails. However, no further information needs to be included in the SCG failure information if the SHR can be adequately enhanced.

Proposal 3: For CHO with associated CPA/CPC, the UE uses the existing condition of SHR to log the near failure case to the network.

Proposal 4. For CHO with associated CPA/CPC, the UE newly includes the below information in SHR as the information of the near failure:

- First satisfied execution condition

- Satisfaction time of the first execution condition

- Satisfaction status of the remaining execution condition

- Time gap between two execution conditions

- Source PSCell measurement result

- Candidate PSCell measurement results

Proposal 5. For CHO with associated CPA/CPC, the UE includes the below information in MCG RLF-report as the information of the failure:

- First satisfied execution condition

- Satisfaction time of the first execution condition

- Satisfaction status of the remaining execution condition

- Time gap between two execution conditions

- Source PSCell signal quality

- Candidate PSCell signal quality

Proposal 6. For, SCPAC MRO, the UE performs logging for multiple security key status to RLF-report or SHR to help resolve the security key mismatch issue that occurs during SCPAC.

References

- 3GPP TS 38.331 V18.2.0 (2024-06)

Annexes

CHO+CPC Flow Chart

SHR information

SuccessHO-Report-r17 ::= SEQUENCE { sourceCellInfo-r17 SEQUENCE { sourcePCellId-r17 CGI-Info-Logging-r16, sourceCellMeas-r17 MeasResultSuccessHONR-r17 OPTIONAL, rlf-InSourceDAPS-r17 ENUMERATED {true} OPTIONAL}, targetCellInfo-r17 SEQUENCE { targetPCellId-r17 CGI-Info-Logging-r16, targetCellMeas-r17 MeasResultSuccessHONR-r17 OPTIONAL}, measResultNeighCells-r17 SEQUENCE { measResultListNR-r17 MeasResultList2NR-r16 OPTIONAL, measResultListEUTRA-r17 MeasResultList2EUTRA-r16 OPTIONAL } OPTIONAL, locationInfo-r17 LocationInfo-r16 OPTIONAL, timeSinceCHO-Reconfig-r17 TimeSinceCHO-Reconfig-r17 OPTIONAL, shr-Cause-r17 SHR-Cause-r17 OPTIONAL, ra-InformationCommon-r17 RA-InformationCommon-r16 OPTIONAL, upInterruptionTimeAtHO-r17 UPInterruptionTimeAtHO-r17 OPTIONAL, c-RNTI-r17 RNTI-Value OPTIONAL, ..., [[ eutraTargetCellInfo-r18 SEQUENCE { targetPCellId-r18 CHOICE { cellGlobalId-r18 CGI-Info-Logging-r16, pci-arfcn-r18 PCI-ARFCN-EUTRA-r16}, targetCellMeas-r18 MeasQuantityResultsEUTRA OPTIONAL } OPTIONAL, measResultServCell-RSSI-r18 RSSI-Range-r16 OPTIONAL, measResultNeighFreqList-RSSI-r18 MeasResultNeighFreqList-RSSI-r18 OPTIONAL, eutra-C-RNTI-r18 EUTRA-C-RNTI OPTIONAL, timeSinceSHR-r18 TimeSinceSHR-r18 OPTIONAL ]]}

RLF Report information

RLF-Report-r16 ::= CHOICE { nr-RLF-Report-r16 SEQUENCE { measResultLastServCell-r16 MeasResultRLFNR-r16, measResultNeighCells-r16 SEQUENCE { measResultListNR-r16 MeasResultList2NR-r16 OPTIONAL, measResultListEUTRA-r16 MeasResultList2EUTRA-r16 OPTIONAL } OPTIONAL, c-RNTI-r16 RNTI-Value, previousPCellId-r16 CHOICE { nrPreviousCell-r16 CGI-Info-Logging-r16, eutraPreviousCell-r16 CGI-InfoEUTRALogging } OPTIONAL, failedPCellId-r16 CHOICE { nrFailedPCellId-r16 CHOICE { cellGlobalId-r16 CGI-Info-Logging-r16, pci-arfcn-r16 PCI-ARFCN-NR-r1 }, eutraFailedPCellId-r16 CHOICE { cellGlobalId-r16 CGI-InfoEUTRALogging, pci-arfcn-r16 PCI-ARFCN-EUTRA-r16}}, reconnectCellId-r16 CHOICE { nrReconnectCellId-r16 CGI-Info-Logging-r16, eutraReconnectCellId-r16 CGI-InfoEUTRALogging } OPTIONAL, timeUntilReconnection-r16 TimeUntilReconnection-r16 OPTIONAL, reestablishmentCellId-r16 CGI-Info-Logging-r16 OPTIONAL, timeConnFailure-r16 INTEGER (0..1023) OPTIONAL, timeSinceFailure-r16 TimeSinceFailure-r16, connectionFailureType-r16 ENUMERATED {rlf, hof}, rlf-Cause-r16 ENUMERATED {t310-Expiry, randomAccessProblem, rlc-MaxNumRetx, beamFailureRecoveryFailure, lbtFailure-r16, bh-rlfRecoveryFailure, t312-expiry-r17, spare1}, locationInfo-r16 LocationInfo-r16 OPTIONAL, noSuitableCellFound-r16 ENUMERATED {true} OPTIONAL, ra-InformationCommon-r16 RA-InformationCommon-r16 OPTIONAL, ..., [[ csi-rsRLMConfigBitmap-v1650 BIT STRING (SIZE (96)) OPTIONAL ]], [[ lastHO-Type-r17 ENUMERATED {cho, daps, spare2, spare1} OPTIONAL, timeConnSourceDAPS-Failure-r17 TimeConnSourceDAPS-Failure-r17 OPTIONAL, timeSinceCHO-Reconfig-r17 TimeSinceCHO-Reconfig-r17 OPTIONAL, choCellId-r17 CHOICE { cellGlobalId-r17 CGI-Info-Logging-r16, pci-arfcn-r17 PCI-ARFCN-NR-r16 } OPTIONAL, choCandidateCellList-r17 ChoCandidateCellList-r17 OPTIONAL ]], [[ pSCellId-r18 CHOICE { cellGlobalId-r18 CGI-Info-Logging-r16, pci-arfcn-r18 PCI-ARFCN-NR-r16 } OPTIONAL, mcgRecoveryFailureCause-r18 ENUMERATED {t316-Expiry, scgDeactivated, spare2, spare1} OPTIONAL, scgFailureCause-r18 ENUMERATED {t310-Expiry, randomAccessProblem, rlc-MaxNumRetx, synchReconfigFailureSCG, scg-ReconfigFailure, srb3-IntegrityFailure, scg-lbtFailure-r16, beamFailureRecoveryFailure-r16, t312-Expiry-r16, bh-RLF-r16, beamFailure-r17, spare3, spare2, spare1 } OPTIONAL, elapsedTimeSCGFailure-r18 ElapsedTimeSCGFailure-r18 OPTIONAL, voiceFallbackHO-r18 ENUMERATED {true} OPTIONAL, measResultLastServCell-RSSI-r18 RSSI-Range-r16 OPTIONAL, measResultNeighFreqList-RSSI-r18 MeasResultNeighFreqList-RSSI-r18 OPTIONAL, bwp-Info-r18 AttemptedBWP-Info-r18 OPTIONAL, elapsedTimeT316-r18 ElapsedTimeT316-r18 OPTIONAL ]] }, eutra-RLF-Report-r16 SEQUENCE { failedPCellId-EUTRA CGI-InfoEUTRALogging, measResult-RLF-Report-EUTRA-r16 OCTET STRING, ..., [[ measResult-RLF-Report-EUTRA-v1690 OCTET STRING OPTIONAL ]] } }