3GPP TSG-RAN WG2 Meeting #127bis		R2-2409042

Hefei, China, Oct 14 – 18, 2024	revision of R2-2407105

Agenda Item:	8.10.2

Source: 	LG Electronics

Title:  	MRO for MOB LTM

Document for:	Discussion

Introduction

In this contribution, we focus on the Rel-18 LTM MRO based on the last agreements.

Discussion

LTM MRO

    - Near failure logging

The first point to consider is what information is newly needed compared to the existing reporting.

A significant different point of LTM cell switch compared to existing PCell handover is the early synchronization procedure for RACH-less cell switch. This early synchronization procedure involves detailed steps for DL sync and UL sync. Except for the UE-based UL synchronization step (i.e. TA acquisition), all other steps are things where the network can already know the information before the UE reporting.

The reason why we think only the UE-based UL synchronization information is needed is because, even though the early synchronization procedure includes DL synchronization and PDCCH order-based UL synchronization, the DL synchronization and PDCCH order-based UL synchronization information is initiated by the network to the UE, meaning the network already knows this information with start time and results.

Thus, we think the most critical aspect is to report the UE-based synchronization to the network to provide more chance of the RACH-less cell switch in LTM.

Additionally, even though the UE has acquired the UE-based synchronization information, if TA values are provided in the cell switch command, the UE performs the RACH-less cell switch using the provided TA information instead of the UE-based synchronization information. Thus, informing whether the UE had acquired the UE-based synchronization information even in cases of performing the RACH-less cell switch becomes more helpful for the network to consider in the subsequent cell switch.

Furthermore, informing which candidate cells the UE acquired the UE-based synchronization information when performing the cell switch also becomes helpful information for the network to consider in the subsequent cell switch.

Based on the above points, for successful cell switch scenarios for MCG LTM, we can propose that the UE-based synchronization information can be mainly considered as new information for the UE reporting.

Proposal 1. In successful scenarios for MCG LTM, the UE logs new information related to the UE based synchronization in SHR to inform the network of the RACH-less cell switch logging in MCG LTM.

Proposal 2. In successful scenarios for MCG LTM, the UE doesn’t need to log information related to DL synchronization and PDCCH order-based UL synchronization in SHR that the network already knows without reporting.

Next is how the UE will report the SHR to the network. Except the UE based UL synchronization, we discuss what other information can be considered in SHR.

First, we think that Random Access information performed for LTM should be included. According to the current specifications, the RACH report only includes the last successful or failed RACH information, which limits its usefulness as it cannot directly help the network support subsequent mobility in LTM.

Second, similar to the existing logging report, it would be beneficial to include the measurement results of the last serving cell, neighboring cells, and candidate cells. Since LTM is a mobility procedure based on L1 measurements, it would be helpful to include both L3 and L1 measurement results.

Based on the above information, we propose that the following information be included in the SHR.

Proposal 3. In successful scenarios for MCG LTM, the UE includes the below other information in SHR as the information of the near failure:

- Source PSCell L3 and/or L1 measurement results

- Candidate PSCell L3 and/or L1 measurement results

- Other neighbour cell L3 and/or L1 measurement results

- Random access trial results if RACH-less isn’t allowed

Lastly, we think there is no need to log time-related information regarding when the UE received the LTM configuration and when the UE performed the LTM cell switching. This is because the LTM isn’t executed by the UE decision but by the network decision, and unlike conditional mobility, the network can fully track this sort of time information.

Proposal 4. In successful scenarios for MCG LTM, the UE does not include time-related information, such as when the UE received the LTM configuration and when the UE performed the LTM.

    - Failure logging

Since RAN2 agreed below for LTM failure logging in RLF report:

Only the field description associated to the timeConnFailure IE needs to be updated accordingly. Otherwise, we don’t expect any further specification impact for timeConnFailure and reconnectCellId is foreseen, TBC. introduce a new field in RLF report to indicate the LTM recovery cell id. RAN2 include the specific access type in the RLF report, i.e. whether it is RA-based or RA-less cell switch. FFS details, e.g. if explicit or implicitly signalled. We do not include in the RLF report, the time since the last reception of PDCCH order towards the target cell till reception of cell switch command. Reuse the existing approach of using timeUntilReconnection in RLF-report also for LTM failure scenarios. We will not define and log timeSinceLTM-Reconfig (like timeSinceCHO-Reconfig) within RLF-report and SHR in LTM failure and near failure cases. We will not log interruption time in SON reports (e.g. SHR) for LTM. We aim to log some info to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not, TBD if explicit/implicit. Log L3 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure. RAN2 assumes this is already possible with existing spec.

Thus, except L1 and L3 measurement results for source PCell, candidate cells, and neighbour cells, we think that for failure logging, only the information that the network cannot already know should be newly considered, just as in the case of SHR. We propose the following:

Proposal 5. In failure scenarios for MCG LTM, the UE includes the below information in RLF-report as like SHR:

- Random access trial results if RACH-less isn’t allowed

- the UE based synchronization information

In addition, we think beam information should be newly considered in RLF-report

The network triggers the RACH-less LTM, the network needs to indicate which beam to be applied for the RACH-less LTM. For the beam to be applied for the mobility, the network can indicate SSB index or TCI state ID associated with the target cell. For the association, the network needs to be pre-configured the UE with the SSB configuration/TCI state configuration of the target cell. Then, the UE applies the received beam information, during the RACH-less mobility, for monitoring of PDCCH at the target cell for receiving dynamic grant for transmitting mobility completion message or the received beam information is applied for transmission of mobility completion message via configured grant. The network may indicate TA value to be applied for the mobility or may indicate the UE to apply the UE-calculated TA value for the mobility.

The problem is that the received beam information may not be suitable for the UE due to, e.g., the beam information is outdated, or the UE’s local movement makes the beam information unsuitable. If such unsuitable beam is used for the RACH-less LTM, the mobility fails, i.e. the received TA or the UE calculated TA value may not be suitable for the UE, and in that case, the mobility fails. In case LTM failure happens, the current RLF-report cannot provide any information of the beam information used by the UE for the mobility. Hence, it is difficult for network to analyse the failure reason.

Thus, we propose to include the beam information for LTM trial in RLF-report.

For the beam information, the UE may log below information for logging the failed RACH-less LTM:

- SSB and TCI state information

This is for indicating which SSB index and associated TCI state was used for RACH-less LTM

- Other qualified beam information

This is for indicating whether there is any other qualified beam information for RACH-less LTM

Proposal 6. In failure scenarios for MCG LTM, the UE newly include the beam information (i.e. SSB and TCI state with other qualified beam) what was used and can be used for LTM in RLF-report.

Conclusion

It is proposed to discuss and decide on the following proposals:

Proposal 1. In successful scenarios for MCG LTM, the UE logs new information related to the UE based synchronization in SHR to inform the network of the RACH-less cell switch logging in MCG LTM.

Proposal 2. In successful scenarios for MCG LTM, the UE doesn’t need to log information related to DL synchronization and PDCCH order-based UL synchronization in SHR that the network already knows without reporting.

Proposal 3. In successful scenarios for MCG LTM, the UE includes the below other information in SHR as the information of the near failure:

- Source PSCell L3 and/or L1 measurement results

- Candidate PSCell L3 and/or L1 measurement results

- Other neighbour cell L3 and/or L1 measurement results

- Random access trial results if RACH-less isn’t allowed

Proposal 4. In successful scenarios for MCG LTM, the UE does not include time-related information, such as when the UE received the LTM configuration and when the UE performed the LTM.

Proposal 5. In failure scenarios for MCG LTM, the UE includes the below information in RLF-report as like SHR:

- Random access trial results if RACH-less isn’t allowed

- the UE based synchronization information

Proposal 6. In failure scenarios for MCG LTM, the UE newly include the beam information (i.e. SSB and TCI state with other qualified beam) what was used and can be used for LTM in RLF-report.

References

- 3GPP TS 38.331 V18.2.0 (2024-06)