3GPP TSG-RAN WG2 Meeting #127bis	R2-2409050

Hefei, China, October 14th – 18th, 2024

Agenda item:		8.2.3

Source:		LG Uplus

Title:	Discussion on Ambient IoT paging

Document for:		Discussion

- Introduction

In the previous RAN2 meetings [1]- [3], A-IoT paging has been discussed and following agreements have been reached.

RAN2#125bis Meeting Agreements: - Legacy paging message for device will not be supported. - Legacy paging occasion and legacy DRX for the device is not supported. This doesn’t preclude solutions that address device monitoring (taking into account discussions from RAN1 as well). - RAN2 assumes that the device will not support tracking/RAN area update procedure. - For the case of reaching single or group of devices, an identifier may be required to identify the device/group of devices in the trigger message. FFS pending the details from SA2. RAN2#126 Meeting Agreements: - RAN2 will study the following cases for A-IoT paging message: • a message containing an ID of a single A-IoT device. • a message containing a group ID that maps to multiple A-IoT devices. • a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. • a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. - What device ID and group ID and scenarios is depending on SA2 discussion. - A-IoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. RAN2#127 Meeting Agreements: - Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices.

In this contribution, the A-IoT paging aspects and the required contents for A-IoT in the identified procedures will be further discussed.

- Discussion

Contents of A-IoT Paging Message

In previous RAN2 meetings, there was an agreement regarding the inclusion of multiple IDs of A-IoT devices within the paging message. When conducting a contention-free random access (CFRA) procedure for multiple devices, it becomes necessary to allocate distinct time and frequency resources for each device.

To achieve this, the paging message should provide the time and frequency resources to each device. By assigning resources in accordance with the order of the device IDs within the paging message, the CFRA procedure can be executed without increasing the complexity of resource allocation. This ensures that each device can perform the D2R transmission efficiently while maintaining a straightforward and predictable structure in the paging message.

Proposal 1: When multiple device IDs are transmitted in the paging message, the A-IoT paging should provide the time and frequency resources for each device for CFRA.

The type of RACH to be used plays a significant role in the random access procedure for A-IoT devices. In contention-free access, as discussed in RAN2, devices can transmit their data directly to the reader without undergoing contention resolution. On the other hand, contention-based access requires devices to participate in contention resolution procedures (e.g., Msg1/Msg2) before being able to send data.

There is a clear need for the A-IoT paging message to distinguish between these two types of access procedures. If a device is unaware of the type of random access procedure being triggered, it may initiate an inappropriate access process, leading to unnecessary contention, delays, and even failure in establishing communication.

It was agreed that the paging message should contain information from which the device can determine the resources to be used for the D2R message. Additionally, distinguishing the RACH type allows the device to determine the appropriate random access procedure, improving efficiency and reducing unnecessary attempts.

Proposal 2 : the A-IoT paging message should clearly indicate the type of random access channel (RACH).  i.e., whether the access is contention-based or contention-free.

Multiple/subsequent A-IoT Paging

During the RAN2 #127[3] meeting, it was agreed that the Reader can trigger multiple or subsequent A-IoT paging messages associated with the same service request from the CN. However, when multiple paging messages are transmitted for the same service request, devices may unnecessarily respond to each subsequent paging message, leading to wasted resources and increased network collisions.

To prevent such redundant responses, it is essential for the paging message to include an indicator that allows devices to distinguish whether the current paging message is a new request or a re-transmission of a previous one. By providing this differentiation, devices can avoid duplicate responses and conserve air resources. Therefore, an indicator in the multiple/subsequent A-IoT paging message that signals whether the message pertains to a new paging request or a re-transmission is necessary.

Proposal 3 : A-IoT paging message need to include an indicator that allows devices to distinguish whether the current paging message is a new request or a re-transmission of a previous one.

Additionally, there should be a discussion on how to trigger multiple/subsequent paging messages. One option is for the Reader to autonomously manage the paging process based on its own criteria, while another option is for the CN to determine the number of paging transmissions based on the service's priority or criticality. Both methods should be considered to balance system efficiency and service requirements.

Proposal 4 : There is also a need for further discussion on how to trigger multiple/subsequent paging messages.

- Conclusion

In this paper, we discussed the issues relates to A-IoT paging procedure, and have following proposals:

Proposal 1: When multiple device IDs are transmitted in the paging message, the A-IoT paging should provide the time and frequency resources for each device for CFRA.

Proposal 2 : the A-IoT paging message should clearly indicate the type of random access channel (RACH).  i.e., whether the access is contention-based or contention-free.

Proposal 3 : A-IoT paging message need to include an indicator that allows devices to distinguish whether the current paging message is a new request or a re-transmission of a previous one.

Proposal 4 : There is also a need for further discussion on how to trigger multiple/subsequent paging messages.

- References

3GPP RAN2#125bis meeting, Chair notes.

3GPP RAN2#126 meeting, Chair notes,

3GPP RAN2#127 meeting, Chair notes.

3GPP TR 38.848, Study on Ambient IoT (Internet of Things) in RAN; (Release 18).