3GPP TSG-RAN WG2 Meeting #127-bis	R2-2409051

Hefei, China, Oct 14th – 18th, 2024

Agenda Item: 	8.8.2

Source:	NTPU

Title:	Discussion on the impact of SSB extension for NR NTN

Document for: 	Discussion

- Introduction

In RAN2#127, the following agreement was mad for considering the impact of SSB periodicity extension for NR NTN.

Agreement 1. From RAN2 point of view, if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration (there might still be impacts on DTX aspects) 2. From RAN2 point of view, If the SSB periodicity is larger than 160ms, for example ssb-PeriodicityServingCell, measurement gap periodicity, SMTC configuration, ssb-Periodicity-r17 for NonCellDefiningSSB-r17 may need to be extended. And the field description of nAndPagingFrameOffset may need to be enhanced to consider the SSB periodicity higher than 160ms. 3. RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity 4. If there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g. because of extreme SSB periodicity) the existing NTN bar bit can be used. FFS about the behaviour for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set.

In this contribution, we will discuss the impact of SSB extension for NR NTN.

- Discussion

As the satellite footprint moves, the relative position of the UE will shift from the edge of the satellite footprint to the central and then to the other edge of the satellite footprint, as shown in Figure 1. As the UE is located at the centre or edge area of ​​the satellite footprint, the requirements of handover measurement and decision are different. For example, if the UE is located at the centre area of the satellite footprint, the SSB periodicity may require the longer SSB periodicity. On the contrary, if it is in the edge area of ​​the satellite footprint, the SSB periodicity may require the shorter SSB periodicity.

Figure 1 Different position of the UE within the centre or edge area of satellite footprint

Observation 1: If the UE is at the edge of the satellite footprint, it may need to enter another satellite footprint or is about to leave the current satellite footprint. An excessively long SSB periodicity may affect the performance of handover measurement and decision.

Proposal 1: Different periodicity for each SSB can supported with the consideration on uneven handover requirements among different position of the UE within the satellite footprint.

- Conclusion

Observation 1: If the UE is at the edge of the satellite footprint, it may need to enter another satellite footprint or is about to leave the current satellite footprint. An excessively long SSB periodicity may affect the performance of handover measurement and decision.

Proposal 1: Different periodicity for each SSB can supported with the consideration on uneven handover requirements among different position of the UE within the satellite footprint.

- Reference

[1] “R2-2407573, Report from Break-out session on NR-NTN and IoT-NTN”, Maastricht, Netherlands, August 19th – 23rd, 2024.