3GPP TSG RAN WG2 Meeting #127bis	 R2-2409062

Hefei, China, October 14–18, 2024

Agenda Item:	8.1.2.2

Source:	Futurewei

Title:	Discussion on LCM for UE-Side Model for Beam Management use case

Document for:	Discussion

# Introduction

In Release 19, this agenda item has been discussed in the following three meetings.

During RAN2 meeting #127 (Maastricht)[5], discussion was based on the post-meeting 126 email discussion and was focused on the following two topics.

- Terminology definitions (a follow-up work of meeting #126)

- Procedures for UE-side LCM

The discussions on terminology definitions resulted in the following agreements.

Agreements (on definitions)

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling).

- Applicable functionalities refer to functionalities that the UE is ready to apply for inference.

- Activated functionalities refer to functionalities already enabled for performing inference.

The following proposals from the post-meeting 126 email discussions are more difficult to reach consensus so they were left for future discussions.

- Proposal 2: RAN2 discuss further whether definition on configured functionalities is needed.

- Proposal 4: RAN2 discuss further relationship between configured functionalities and applicable functionalities as a part of discussion on the need of defining configured functionalities.

- Proposal 6: RAN2 discuss further on the need of defining available functionalities separately from applicable functionalities.

The discussions on procedures for UE-side LCM resulted in the following agreements.

Agreements (on procedures)

- Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

- Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side.

- “Step 3”: Following configurations are provided from NW to UE:

1) UE is allowed to do UAI reporting via OtherConfig.

2) Network may provide NW-side additional condition.  FFS on the RRC signalling and whether it is mandatory or optional.

3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration.

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

  - FFS whether other configuration can be considered by UE (e.g. inference configuration).

  - FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

- “Step 4”: UE reports applicable functionality in the following scenarios:

- Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

- As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration).

  - FFS via UAI or RRCReconfigurationComplete, etc

- Step 5:

1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5).

2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not.

- The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5.

  - FFS the initial activation state.

  - FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3.

  - FFS on additional L1/L2 signalling for activation/deactivation.

  - FFS if multiple applicable functionalities can be activated at the same time.

  - FFS what is the granularity of functionality

- We will write an LS to RAN1 to provide our agreements and ask specific questions that RAN2 needs to enable progress.

The above procedure was also captured in the following figure (note it is not part of the agreements).

During RAN2 meeting #126 (Fukuoka) [1]. The discussion was focused on terminology definitions and additional conditions of both NW and UE side.

- Terminology alignment

- Availability/applicability of Functionality

- Network-side additional conditions

- UE side additional/applicability conditions

The discussions resulted in the following agreements.

Agreements

- RAN2 will support functionality activation/deactivation after inference configuration.   FFS initial state of configuration and how activation/deactivation is achieved.

  - FFS what Deactivation refers to:  examples discussed: 1) fallback to legacy 2) switching, etc.

- We will work offline on the definitions for functionality types and define what is availability.

- The UE will indicate the gNB/LMF whether the AI/ML functionality is available/applicable. For a functionality to be applicable at least there should at least one model available within it.   FFS other details on what is applicability/non-applicability.

- For NW-side additional conditions, RAN2 assumes that RRC signalling from gNB to UE can be designed for consistency between inference and training.  RAN2 will wait for RAN1 input for further details.   FFS if the same applies to positioning.

- For BM use case, as a baseline the UE determines whether a functionality is applicable.  Existing UAI framework is used at least for proactive reporting of applicable functionality.  FFS reactive.

Earlier in meeting #125bis [4] (Changsha), RAN2 reached the following agreements for LCM for UE-side model.

Agreements

- Which AI/ML-enabled Features/FGs and functionalities are supported should be standardized. The details wait for RAN1’s progress.

  - “supported” means that the UE is capable of supporting the functionality and doesn’t mean necessarily that the UE has the model available.

  - FFS what functionality refers to.

- Supported AI/ML-enabled Features/FGs and supported functionalities are included in UE capability.

Agreements

- Support proactive reporting of UE-sided applicable functionality, e.g., the UE reports its applicable AI/ML functionalities via UAI message/LPP message.

- Support reactive reporting of UE-sided applicable functionality.

  - The NW configures AI/ML functionalities via RRC/LPP message.

  - FFS what the configuration contains.

  - FFS how to report applicable functionality and what is applicable functionality.

- FFS how the two approaches will be specified and whether we can combine them into one procedure.

  - FFS how to report applicable functionality, what is applicable functionality, how the UE determines which function is applicable or not (if it is needed)

Agreements

- For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline.  The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach).

- “UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19.

In this contribution, we will further discuss LCM for UE-side models from the following aspects.

- The leftover of definitions of terminologies

- The details, clarifications and revisions of the LCM procedures for UE-side models

# Definitions of Terminologies

## Deactivation

In RAN2 meeting #126[1], the discussion of functionality activation/deactivation resulted in the following agreement.

RAN2 will support functionality activation/deactivation after inference configuration. FFS initial state of configuration and how activation/deactivation is achieved.

  - FFS what Deactivation refers to:  examples discussed: 1) fallback to legacy 2) switching, etc.

Some companies at the meeting think deactivation means switching to other models, while others think deactivation means falling back to the legacy solution at the functionality level (for which the discussion is about). We are the proposer of the latter; we think that at functionality level we do not need to control/select individual models to be used, for each functionality. Therefore, there are only two solutions: the non-AI/ML based, i.e., the existing solution, and the AI/ML-based solution. Once we deactivate the AI/ML-based solution (i.e., the functionality), we will have to fall back to the legacy solution. Note that switching means the LCM is at the model level, which is outside the scope of functionality LCM. Switching from a functionality (say BM) to another functionality (say Positioning) does not make sense.

Observation 1: Model switching means the LCM is at the model level, which is outside the scope of functionality LCM.

Proposal 1: For functionality-based LCM, deactivation refers to falling back to the legacy solution.

## Configured/Available Functionality

The following proposals from the post-meeting 126 email discussions are more difficult to reach consensus during meeting 127 so they were left for future discussions[5].

- Proposal 2: RAN2 discuss further whether definition on configured functionalities is needed.

- Proposal 4: RAN2 discuss further relationship between configured functionalities and applicable functionalities as a part of discussion on the need of defining configured functionalities.

- Proposal 6: RAN2 discuss further on the need of defining available functionalities separately from applicable functionalities.

The relationship of configured functionality and applicable functionality

In our opinion, once a functionality is configured by gNB or LMF, it becomes a configured functionality. An applicable functionality (functionality that is ready for inference) is a superset of configured functionality, because not every functionality needs to be configured by the gNB/LMF.

Some companies think configuration of functionalities may be different for different purposes of LCM (i.e. inference, training, performance monitoring). However, we think it should be used only for model inference. This is because we normally will not say configured functionality at training stage as we will not train a functionality, but a model. Similarly, we will not say configured functionality at performance monitoring stage; instead, we may say configuration for performance monitoring.

Proposal 2: Configured functionalities refer to configurable functionalities that have been configured by gNB/LMF.

Proposal 3: Configured functionality is a subset of applicable functionality.

Applicable functionality

In RAN2 meeting #127[5], the group has reached the following agreement on the definition of applicable functionality.

- Applicable functionalities refer to functionalities that the UE is ready to apply for inference.

It is our understanding that the applicability of a functionality, besides ready for model inference, refers to the fact that the functionality is suitable to be applied for inference in the current environment, including the additional conditions of the NW side (e.g., antenna setting etc.) and the capability/capacity of the UE side (computational power, memory/storage capacity etc.). This is because the underlying model supporting the functionality may not be suitable for all inference environments. We propose to modify the definition of applicable functionality as below.

Proposal 4: modify the definition of applicable functionality as below.

Applicable functionalities refer to functionalities that the UE is ready to apply for inference in the current environment.

Available functionality

One topic discussed during the post-meeting 126 email discussion and meeting 127 was whether we should separate available functionalities from applicable functionalities which has been agreed by most companies.

It is our understanding that the meaning of “available” could be expressed by either “supported” or “applicable” which have been agreed upon; it brings neither clarity nor additional meanings that cannot be covered by the other two terms. In addition, it could create more confusion between “available” and “applicable”, as well as “available” and “configured”.

Proposal 5: RAN2 does not define and use the term “available functionalities”.

# Procedures for UE-side LCM

In meeting #127, the procedures for UE-side LCM were discussed, based on the result of post-meeting 126 email discussion. As stated in the Introduction section, the following agreement was achieved[5].

- Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

- Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side.

- “Step 3”: Following configurations are provided from NW to UE:

1) UE is allowed to do UAI reporting via OtherConfig.

2) Network may provide NW-side additional condition.

- FFS on the RRC signalling and whether it is mandatory or optional.

3) FFS on configuration (e.g. inference configuration) of supported functionalities.

- FFS on the content of configuration.

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

  - FFS whether other configuration can be considered by UE (e.g. inference configuration).

  - FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

- “Step 4”: UE reports applicable functionality in the following scenarios:

- Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

- As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration).

  - FFS via UAI or RRCReconfigurationComplete, etc

- Step 5:

1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5).

2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not.

- The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5.

  - FFS the initial activation state.

  - FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3.

  - FFS on additional L1/L2 signalling for activation/deactivation.

  - FFS if multiple applicable functionalities can be activated at the same time.

  - FFS what is the granularity of functionality

- We will write an LS to RAN1 to provide our agreements and ask specific questions that RAN2 needs to enable progress.

## NW-side additional conditions

Sub-bullet 2) of Step 3 is on NW-side additional conditions.

In Step 1 and Step 2, NW has configured the UE to report supported functionalities and UE has reported them. In Step 3, the NW should configure the UE with its additional conditions and this step should be mandatory, unless the model was trained specifically for this NW. As we mentioned in the Applicability discussion of Section 2.2, this step will make sure the supported model is applicable for the current environment. This step also turns a supported functionality into a configured functionality and makes it applicable (i.e., ready) for inference.

Sub-bullet 3) of Step 3 says the configuration of supported functionalities is FFS. It is our understanding that the configuration of each functionality depends on its use case. Therefore, we can leave this to the discussion of each use case.

Proposal 6: Revised “Step 3” as below.

- “Step 3”: Following configurations are provided from NW to UE:

1) UE is allowed to do UAI reporting via OtherConfig.

2) Network shall may provide NW-side additional condition if applicable.  FFS on the RRC signalling. and whether it is mandatory or optional.

3) FFS on The configuration (e.g. inference configuration) of supported functionalities is up to individual use case.

## The granularity of functionality

Although there are no detailed statements in the TR or agreements in R19 discussions on the granularity of a functionality, from the following quotes from the TR, we can see that a functionality is roughly at the level of or lower than (i.e., smaller than) an AI/ML-enabled Feature or FG.

- Where AI/ML functionality resides depends on the specific use cases and sub use cases.

- For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature, whereby AI/ML-enabled Feature refers to a Feature where AI/ML may be used. Note: UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality.

- For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models, functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG.

Observation 2: The granularity of functionality has been defined in the TR; no further discussion is needed.

## Reporting of UE internal capability/capacity

Besides AI/ML-based UE capability reporting (which associates with AI/ML functionality), another important aspect the UE needs to report to the NW is the UE’s internal capability or capacity for model inference, for example, its computational power and memory. Note this is different from AI/ML-based UE capability reporting; it is used for the NW to decide whether the UE will be able to handle the task of certain model inference.

This is also toughed in the above agreements, between Step 3 and Step 4, as listed below.

  - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

    - FFS whether other configuration can be considered by UE (e.g. inference configuration).

    - FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

The “UE-side additional conditions (internally known by UE)” in this proposal can include the UE’s capability or capacity for model inference. we believe this information should also be made known to the NW (not only to the UE itself) for various reasons. This information may include UE’s computational power, memory, storage, model inference time, battery level etc. that are applicable for AI/ML functionality operations.

With this information, the NW can have a better understanding of the resources available at the UE side and activate/deactivate AI/ML-based functionality at the right time/situation. For example, if the UE’s current computational power or memory is not able to support the inference of a certain functionality, the NW will not instruct the UE to activate the functionality.

To capture this aspect, we proposed a revised agreement for the above text and Step 4.

Proposal 7: Revise the procedures for UE-side LCM as below.

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE, including UE’s capabilities and capacities that are related to AI/ML functionality operations) and model availability in device.

  - FFS whether other configuration can be considered by UE (e.g. inference configuration).

  - FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

- “Step 4”: UE reports applicable functionality and its internal capabilities/capacities in the following scenarios:

  - Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

  - As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration).

    - FFS via UAI or RRCReconfigurationComplete, etc

# Conclusions

In this contribution, we discussed our views on LCM for UE-side models for the beam management use cases. Based on the discussions in the previous sections, our proposals are as follows.

Observation 1: Model switching means the LCM is at the model level, which is outside the scope of functionality LCM.

Proposal 1: For functionality-based LCM, deactivation refers to falling back to the legacy solution.

Proposal 2: Configured functionalities refer to configurable functionalities that have been configured by gNB/LMF.

Proposal 3: Configured functionality is a subset of applicable functionality.

Proposal 4: modify the definition of applicable functionality as below.

Applicable functionalities refer to functionalities that the UE is ready to apply for inference in the current environment.

Proposal 5: RAN2 does not define and use the term “available functionalities”.

Proposal 6: Revised “Step 3” as below.

- “Step 3”: Following configurations are provided from NW to UE:

1) UE is allowed to do UAI reporting via OtherConfig.

2) Network shall may provide NW-side additional condition if applicable.  FFS on the RRC signalling. and whether it is mandatory or optional.

3) FFS on The configuration (e.g. inference configuration) of supported functionalities is up to individual use case.

Observation 2: The granularity of functionality has been defined in the TR; no further discussion is needed.

Proposal 7: Revise the procedures for UE-side LCM as below.

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE, including UE’s capabilities and capacities that are related to AI/ML functionality operations) and model availability in device.

  - FFS whether other configuration can be considered by UE (e.g. inference configuration).

  - FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

- “Step 4”: UE reports applicable functionality and its internal capabilities/capacities in the following scenarios:

  - Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

  - As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration).

    - FFS via UAI or RRCReconfigurationComplete, etc

# References

Chair’s notes, R2_126_ChairNotes_24-05-24_final, RAN2 meeting #126, Fukuoka, May 2024

RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN meeting #102, Qualcomm (moderator)

RP-233946, 3GPP TR 38.843, Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18) v2.0.1 (2023-12)

Chair’s notes, R2_125bis_ChairNotes_24-04-19_final, RAN2 meeting #125bis, Changsha, April 2004

Chair’s notes, R2_127_ChairNotes_ 24-08-23_17-00_eom, RAN2 meeting #127, Maastricht, Aug 2024