3GPP TSG RAN WG2 Meeting #127bis	 R2-2409063

Hefei, China, October 14–18, 2024

Agenda Item:	8.1.3

Source:	Futurewei

Title:	Discussion on Data Collection for NW-side Model Training

Document for:	Discussion

# Introduction

In the RAN19 WID [1], the following items are listed as RAN2 or RAN2-led WI research areas for the NR_AIML_Air project.

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]:

  - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback

    - Identification related signalling is part of the above objective

  - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models

  - Signalling mechanism of applicable functionalities/models

In the first R19 RAN2 meeting (#125bis, in Changsha) for this topic, we reached the following agreements for Data Collection for NW-side Model Training[3].

Agreements

- For the NW-side data collection related to beam management use cases, RAN2 to consider gNB-centric and OAM-centric approaches.

- We aim that the same measurement framework is applied to both gNB-centric data collection and OAM-centric data collection for NW-side data collection.

- RAN2 supports enhancements to MDT for data collection framework for training.

  - FSS Whether to enhance logged or immediate MDT.

In RAN2 meeting #126 (Fukuoka), we advanced the discussions further and reached the following agreements for Data Collection for NW-side Model Training[4].

Agreements

- Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

- Enhance the immediate MDT framework to support periodical reporting.

  - FFS whether and what event-based reporting is supported, and

  - FFS on network request reporting.

- For gNB centric and OAM centric (for RRC signalling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature.

  - FFS how to handle case when single RRC message is not sufficient.

  - FFS if there will be any further enhancement needed pending RAN1 agreement.

In summary, we made further decisions on the enhancement of MDT. We decided that immediate MDT will be the baseline, and it will be enhanced to support periodical reporting. However, both event-based reporting and on-demand based reporting are FFS. In addition, we decided that reporting multiple instances of logged L1 measurement results is an optional feature.

In RAN2 meeting #127 (Maastricht), we further discussed and agreed with the following[5].

- The NW configures the UE for data collection, including immediate start of data collection.

- UE-side internal conditions, such as memory size and power state are important for data collection; both memory size and power state can stop the UE from data collection.

- On-demand request of data collection from the network is supported. However, event-based data collection/logging is still undecided (after a fair amount of time of discussion).

- The UE needs to carefully decide the size of measured information to be transmitted so that it will not exceed the max PDCP SDU size; RRC segmentation will not be supported for data collection.

- Measured data will be transmitted over SRB1 (FFS which SRB is used). Note the discussion point here was whether high-priority or low-priority SRBs should be used. While Nokia thinks that it should be low priority SRB, Oppo thinks that if we use low priority SRB, periodic reporting is useless.

Detailed agreements were listed below[5].

Agreements (on the procedures and types of data collection, e.g., event-triggered, on-demand etc.)

- As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.

  - The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.

  - FFS if multiple configurations can be provided to the UE.

  - FFS if dynamic activation/deactivation is support.

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. This is across all use cases.

  - FFS on the memory size.

  - When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

- Measurements for data collection purposes and logging based can be controlled based on power state of the UE.

  - It is up to UE implementation how the UE determines power state.

  - FFS whether the UE stops autonomously or if it reports to the network.

- FFS whether AS buffer event-based reporting is supported.

- FFS if we send availability indication or full report if it is supported.

- FFS on event-based data collection/logging.

- On-demand request from the network is supported.   FFS details on signalling.

Agreements (on signaling)

- The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded.

    - No standardized RRC segmentation procedure is needed (as for the logged MDT measurements)

- Data collection report will not be transmitted over SRB1.

    - FFS which SRB is used.

In this contribution, our discussion will focus on the mechanisms and principles identified for data collection for network side model training during R18, as indicated by the Chair.

# Background

In the TR [2], Table 7.3.1.2-1 lists existing data collection mechanisms available in current RAN specifications for the UE to report measurements to another entity acting as termination point for this data. As indicated by the TR, the analysis/selection of the data collection frameworks should focus on the RRC CONNECTED state for both data generation and reporting.

Table 7.3.1.2-1. Existing data collection methods identified.

| Involved network entity (termination point) | RRC state to generate data | Max payload size per reporting* | Contents to be collected | - End-to-End report latency** | Report type | Security and Privacy |
| --- | --- | --- | --- | --- | --- | --- |
| Method: Logged MDT |  |  |  |  |  |  |
| TCE/OAM (Data can be utilized by gNB) | IDLE / INACTIVE | <9kbyte | - L3 cell/beam measurements - location information - sensor information - timing information | - Procedure latency***: - Latency to enter CONNECTED state - Latency to receive gNB request signalling (~20ms) - Air interface signalling latency****: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and TCE | Upon gNB request after entering RRC_CONNECTED | AS security via RRC message Privacy via user consent |
| Method: Immediate MDT |  |  |  |  |  |  |
| TCE/OAM (Data can be utilized by gNB) | CONNECTED | <9kbyte | - L3 cell/beam measurements - location information - sensor information | - Procedure latency: - Report interval: - 120ms~30min for periodic report - TTT for event triggered report - Air interface signalling latency: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and TCE | - Event triggered - Periodic reporting | AS security via RRC message Privacy via user consent |
| Method: L3 measurements |  |  |  |  |  |  |
| gNB | CONNECTED | <9kbyte | L3 cell/beam measurements | - Procedure latency: - Report interval: - l20ms~30min for periodic report - TTT for event triggered report - Air interface signalling latency: - 20ms (RRC) | - Event triggered report - Periodic reporting | AS security via RRC message |
| Method: L1 measurement (CSI reporting) |  |  |  |  |  |  |
| gNB | CONNECTED | <1706bit in PUCCH <3840bit in PUSCH | L1 CSI measurement | - Procedure latency: - Report interval: - 4-320 slot for periodic and semi-persistent report - 0-32 slot after reception of DCI for aperiodic report - Air interface signalling latency: - 1 TTI (PUCCH) | - Aperiodic report - Semi-persistent report - Periodic report | No AS security |
| Method: UE Assistance Information (UAI) |  |  |  |  |  |  |
| gNB | CONNECTED | <9kbyte | Assistance information to show UE preference | - Procedure latency: - Upon generation of UE's preference - Air interface signalling latency: - ~20ms (RRC) | Up to UE implementation when to report | AS security via RRC message |
| Method: Early measurements |  |  |  |  |  |  |
| gNB | IDLE / INACTIVE | <9kbyte | L3 cell/beam measurements | - Procedure latency: - Latency to enter CONNECTED state - Latency to receive gNB request signalling (~20ms) - Air interface signalling latency: - ~20ms (RRC) | Upon gNB request after entering RRC_CONNECTED | AS security via RRC message |
| Method: LPP |  |  |  |  |  |  |
| LMF | CONNECTED | <9kbyte | Location information | - Procedure latency: - Latency to get upper layer trigger (for UE triggered) - Or latency to receive network request message (~20ms) - Air interface signalling latency: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and LMF | - UE-triggered - Network-triggered | AS security via RRC message |

* The payload size doesn't consider signalling overhead.
** The End-to-End report latency is the latency from availability of the measurement report at the UE side to the availability of the measurement report at the terminated network entity. The time to generate data or perform measurements depends on RAN1/RAN4 specification.
*** Procedure latency is the latency caused by procedures, including procedure to ready for reporting (e.g., entering CONNECTED state, report interval).
**** Air interface signalling latency is the latency to transmit one report, e.g., RRC signalling latency or PUCCH signalling latency.

The TR [2] also captured the following considerations for network-side data collection (Section 7.2.1.3.1).

In addition, A set of general data collection principles is expected to be considered for network-side model training. These include:

- UE to support data logging,

- UE to report the collected data periodically, event-based, and on-demand,

- The UE memory, processing power, energy consumption, signalling overhead should be considered.

Furthermore, and regarding the use cases in this study, the following is considered.

For CSI and beam management use cases, the training of network-side models can consider both gNB and OAM-centric data collection mechanisms.

- The gNB-centric data collection implies that the gNB can configure the UE to initiate/terminate the data collection procedure. The potential impact of L3 signalling for the reporting of collected data should be assessed.

- On the other hand, OAM-centric data collection implies that the OAM provides the configuration (via the gNB) needed for the UE to initiate/terminate the data collection procedure. MDT framework can be considered to achieve this. The potential impact on MDT for RRC_CONNECTED state should be assessed.

For positioning use cases, when considering LMF-side inference, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF. While for LMF-side performance monitoring, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF.

# Discussion

## Event-triggered reporting

In RAN2 meeting #126 [4], one of the agreements we achieved was the following, on reporting mechanisms.

Enhance the immediate MDT framework to support periodical reporting.

- FFS whether and what event-based reporting is supported, and

- FFS on network request reporting.

In RAN2 meeting #127[5], we further achieved the following agreement, on the support of on-demand reporting.

- On-demand request from the network is supported.   FFS details on signaling.

Therefore, so far, we have agreed to support both enhanced periodical and on-demand (network request) reporting mechanisms. Meanwhile, the support of event-based reporting is still FFS, as agreed below.

- FFS whether AS buffer event-based reporting is supported.

However, the TR [2]  clearly says both event-based and on-demand based reporting approaches are part of the “general data collections principles”, as shown below.

A set of general data collection principles is expected to be considered for network-side model training. These include:

- UE to support data logging,

- UE to report the collected data periodically, event-based, and on-demand,

- The UE memory, processing power, energy consumption, signalling overhead should be considered.

In addition, if we looked at the data collection methods identified in R18 (Table 7.3.1.2-1 in Section 2 of this TDoc), we can see event-based reporting mechanisms is identified as one of the reporting types. For example, event-triggered reporting is identified for

- Immediate MDT: collected by TEC/OAM

- L3 measurement: collected by gNB

- LPP: collected by LMF

In fact, for some use cases, such as positioning, event-triggered reporting can be more efficient and up-to-date, because measurements only needs to be done when the UE starts to move, which saves the traffic generated by periodical reporting. In addition, allowing event-based reporting provides flexibility of control and reporting.

Based on this consideration, we propose to support event-based reporting mechanisms.

Proposal 1: Support event/trigger-based reporting mechanisms in addition to periodical and on-demand request reporting for NW-side data collection.

## Data logging, UE capability/capacity for data processing and storage

In the TR [2], the “general data collections principles” also include principles regarding UE’s capability and capacity for data collection for NW-side models.

A set of general data collection principles is expected to be considered for network-side model training. These include:

- UE to support data logging,

- UE to report the collected data periodically, event-based, and on-demand,

- The UE memory, processing power, energy consumption, signalling overhead should be considered.

In RAN2 meeting #125bis [3] (Changsha), we reached the following agreements for Data Collection for NW-side Model Training.

- For the NW-side data collection related to beam management use cases, RAN2 to consider gNB-centric and OAM-centric approaches.

- We aim that the same measurement framework is applied to both gNB-centric data collection and OAM-centric data collection for NW-side data collection.

- RAN2 supports enhancements to MDT for data collection framework for training.

  - FSS Whether to enhance logged or immediate MDT.

In RAN2 meeting #126 [4], the group has agreed to take immediate MDT as the baseline for OAM-centric data collection (see the agreement below).

- Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

In RAN2 meeting #127 [5], the group has agreed the following (see the agreement below).

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE.

- FFS on event-based data collection/logging.

These two agreements are quite contradictive to each other. The first one says the UE can store the logged training data, and the second one says data logging is FFS.

There are two issues we would like to discuss here, the support of data logging and UE’s capability/ capacity for logged data processing/storage.

Data logging

As we have shown, whether logged MDT will be used and enhanced has not been determined. Based on the TR text we quoted, it is expected that data logging will be supported by the UE.

- UE to support data logging

We understand there is concern regarding the use of UE memory by logged data (which we will address a bit later), we think overall if we can log collected data and send multiple of them in one RRC transmission, we can save the number of signaling exchanges and it should be encouraged (as long as delay requirement is met). Therefore, data logging at the UE should be supported.

Proposal 2: Support data logging and enhancement to logged MDT for data collection for NW-side models.

UE’s capability/ capacity for logged data processing/storage

We mentioned that there is concern regarding the use of UE memory for logging collected data for NW-side models, which was documented in the TR.

- The UE memory, processing power, energy consumption, signalling overhead should be considered.

We consider this a practical concern because the resources at the UE side are limited, for example, the computational power and memory (the collected data will be in the memory for the BM and Positioning use cases, unlike some of the application-level use cases). We believe it is necessary to provide a mechanism to report a UE’s capability/capacity for storing measured data temporarily for NW-side model training. Logged MDT may only be used when the UE has enough available resources to support it. Otherwise, immediate MDT should be used for data collection.

Proposal 3: Design a mechanism for a UE to report its capability and/or capacity to the NW, such as computational power, available memory and storage space. NW decides on whether immediate MDT or logged MDT to be used based on the UE’s capability and capacity.

# Conclusions

In this contribution, we continued to present our observations and views on topics related to NW-side data collection. Based on the discussions in the previous sections, our proposals are as follows.

Proposal 1: Support event/trigger-based reporting mechanisms in addition to periodical and on-demand request reporting for NW-side data collection.

Proposal 2: Support data logging and enhancement to logged MDT for data collection for NW-side models.

Proposal 3: Design a mechanism for a UE to report its capability and/or capacity to the NW, such as computational power, available memory and storage space. NW decides on whether immediate MDT or logged MDT to be used based on the UE’s capability and capacity.

# References

RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN meeting #102, Qualcomm (moderator)

RP-233946, 3GPP TR 38.843, Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18) v2.0.1 (2023-12)

Chair’s notes, R2_125bis_ChairNotes_24-04-19_final, RAN2 meeting #125bis, Changsha, April 2004

Chair’s notes, R2_126_ChairNotes_24-05-24_final, RAN2 meeting #126, Fukuoka, May 2024

Chair’s notes, R2_127_ChairNotes_ 24-08-23_17-00_eom, RAN2 meeting #127, Maastricht, Aug 2024