3GPP TSG RAN WG2 Meeting #127bis	 R2-2409067

Hefei, China, October 14–18, 2024

Agenda Item:	8.1.5

Source:	Futurewei

Title:	Discussion on model transfer/delivery

Document for:	Discussion

# Introduction

In RAN2, the topic of model transfer/delivery has not been actively discussed since the beginning of R19. In R18, RAN2 has done initial investigations, and the agreements and conclusions were captured in Section 7.2.1.4 of the TR [1]. For easy reference, we copied the most important agreements/conclusions here.

Whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) is unclear from the outcome of the present study. Nonetheless, to support AI/ML model transfer/delivery, the following solutions are considered:

- Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling.

- Solution 2a: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling.

- Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling.

- Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data.

- Solution 2b: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via User Plane (UP) data.

- Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data.

- Solution 4a: OTT server can transfer/deliver AI/ML model(s) to UE (e.g., transparent to 3GPP).

- Solution 4b: OAM can transfer/deliver AI/ML model(s) to UE.

Note: The relationships between model transfer/delivery solutions and use cases can be derived from what is captured in clauses 7.3.2, 7.3.3, and 7.3.4.

The following areas are considered to evaluate the different model transfer/delivery solutions:

- A1: Large, no upper limit model/model parameter size,

- A2: Model transfer/delivery continuity (i.e., resume transmission of model (segments) across gNBs),

- A3: Network controllability on model transfer/delivery (e.g., management decision at gNB),

- A4: Model transfer/delivery QoS (for DRB) (including latency, etc.) and priority (for SRB).

For every model transfer/delivery solution, each of the above areas is analysed, focusing on the current status and gaps, and the potential impacts on RAN specification. The analysis is shown in the Tables below.

The TR also provides analysis of current status, gaps, and potential RAN specification impact for each of the solutions, in the discussion areas A1 to A4.

For the mechanisms of model transfer/delivery, the TR says the following.

A reactive and a proactive approach for initiating a model transfer/delivery can be considered in a normative phase. For the reactive approach, an AI/ML model is transferred/delivered (i.e., downloaded) to the UE when needed. This could typically happen due to changes in scenarios, configurations, sites, etc. While for the proactive model transfer/delivery approach, an AI/ML model is pre-download to the UE, and a model switch can typically be performed due to changes in scenarios, configurations, sites, etc.

In RAN1, model transfer/delivery was discussed under the topic of inter-vendor collaboration levels, and the results of R18 discussion was captured in Section 4.3 (Collaboration levels) of the TR. Different options/cases of model transfer/delivery were be summarized in the following table.

Table 4.3-1: Model delivery/transfer cases

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| y | model delivery (if needed) over-the-top. | Outside 3gpp Network | UE-side / NW-side / neutral site |
| z1 | model transfer in proprietary format. | 3GPP Network | UE-side / neutral site |
| z2 | model transfer in proprietary format. | 3GPP Network | NW-side |
| z3 | model transfer in open format. | 3GPP Network | UE-side / neutral site |
| z4 | model transfer in open format of a known model structure at UE, i.e., an exact model structure as has been previously identified between NW and UE and for which the UE has explicitly indicated its support. | 3GPP Network | NW-side |
| z5 | model transfer in open format of an unknown model structure at UE, i.e., any other model structure not covered in z4, including any model structure that is only partially known. | 3GPP Network | NW-side |

These options/cases were further discussed in R19, resulting the following agreements.

- During RAN1 meeting #116[2], Case z5 has been deprioritized due to lack of interest.

- During RAN1 meeting #116bis[3], Case z2 and z3 have been further deprioritized, for R19.

As a summary, the options left on the table are Case y, z1 and z4. As Case y is achieved by over-the-top approach, what we really need to care about are Case z1 and z4.

With Case z1, model is trained at the UE side/neutral site and stored at the 3GPP network then delivered/transferred via proprietary format. The scenario for this use case could be that the UE vendor leases an OTT server from the operator within 3GPP NW. However, we are not clear how this is different or beneficial from Case y (in neither case the NW has the control of the procedure or the visibility of the data). In addition, model transferred in proprietary format means there would be large burden of offline cross-vendor collaboration, which is not desirable.

With Case z4, model is trained and stored at the network side then delivered/transferred via open format. It may bring more spec impact on 3GPP air interface compared to Case z1 in addition to collaboration between NW and UE for the “known model structure” and the transfer of model parameters.

Overall, in RAN1 discussions, Case z4 gets the most support and it is likely to become the only model transfer option for R19 study (and R20 normative work), as Case z1 may be further deprioritized.

In this contribution, we analyze model transfer/delivery solutions captured in the TR against the use cases of model transfer/delivery. We try to narrow down to the solutions that support inter-vendor Case y and Case z4.

# Discussion

## Supporting collaboration Case y

From the list of model transfer/delivery solutions, only Solution 4a was proposed for OTT server and the source/transmitter.

- Solution 4a: OTT server can transfer/deliver AI/ML model(s) to UE (e.g., transparent to 3GPP).

It is, therefore, obvious that Solution 4a is needed for collaboration Case y, which is 3GPP-transparent.

## Supporting collaboration Case z1 and 4z

As we mentioned in the introduction section, both Case z1 and z4 have their trained models stored at the network then transfer/deliver them to the UE.

- with Case z1, model is trained at the UE side/neutral site and stored at the 3GPP network then delivered/transferred via proprietary format.

- with Case 4z, model is trained and stored at the network side then delivered/transferred via open format.

However, the TR does not specify where in the NW the models are stored, whether it is gNB, CN/LMF, or OAM (in RAN2 discussions, all these are valid entities). RAN2 did compare all the solutions against discussion areas A1 to A4 and list major pros and cons of each solution.

We compare and summarize the pros and cons for different categories of solutions in the following table.

| Source of the transfer/delivery | CP vs UP (Solution) | A1. Model size support | A2. Transfer/ Delivery continuity | A3. Network controllability at gNB | A4. transfer/ delivery QoS and priority |
| --- | --- | --- | --- | --- | --- |
| gNB | CP (1a) | No support of model > 45kBytes | Must restart transmission after mobility | Not supported. | Shortest latency |
|  | UP (1b) | No size limitation | Same continuity issue as 1a, unless PDU session can terminate at gNb. | Feasible but currently not support. | Longer latency than 1a. |
| CN or LMF | CP (2a, 3a) | No support of model > 45kBytes; Support of segmentation. | Support with limitation: within AMF or LMF coverage areas | Not supported | Introduces extra latency bet. CN/LMF and gNB. |
|  | UP (2b, 3b) | No size limitation | 2b: supported 3b: depends on CT1 R18 solution for LPP msg over UP | Not supported | Introduces extra latency bet. CN/LMF and gNB. |
| OAM* | via CP (4b-CP) | No support of model > 45kBytes | Support within OAM coverage | Not supported | Introduces extra latency bet. OAM and gNB. |
|  | via IP tunnel (4b-IP) | No model size limitation. But direct connection between OAM and UE is not supported | Support within OAM coverage | Not supported | Direct connection OAM and UE is not supported. But expect longer latency than gNB. Could be similar to CN/LMF |

*Note: For Solution 4b, RAN2 discussed the following two solutions but did not study or analyze their feasibility:

- OAM can transfer/deliver AI/ML models to UE via OAM→RAN→UE, where Control Plane (CP) signalling is used for RAN→UE.

- OAM can transfer/deliver AI/ML models to UE via OAM→UE, e.g., via IP tunnel.

From the table above, we observed the following rough rankings (from the best to the worst). Note that this ranking does not consider the amount of potential standard’s work involved.

- For model size support (A1): UP (1b, 2b, 3b) > CP (1a, 2a, 3a, 4b-CP)

- For model transfer/delivery continuity (A2): CN/LMF/OAM > gNB

- For network controllability at gNB (A3): Same, as none of them currently supports.

- For transfer/delivery QoS and priority (A4): gNB (CP) > gNB (UP) > CN/LMF > OAM

With these observations, we think it may be fair to say that

- the CP solution’s biggest disadvantages are model size limitation and mobility/continuity support. While the UP solution’s biggest disadvantages may be QoS such as longer latency.

- the gNB solution’s biggest disadvantage is mobility/continuity support, comparing to CN/LMF/OAM.

As a summary, both CP and UP mechanisms, combining with both gNB and CN/LMF/OAM solutions, have their applicable scenarios. Further, to minimize spec impact and avoid having too many options, we think Solution 4a (the OTT approach) can be used to represent UP solutions, because they (UP Solution 1b, 2b, 3b and 4b-IP) do not provide much advantage over Solution 4a. By eliminating these UP solutions, we don’t need to address issues such as terminating PDU sessions at gNB. In addition, Solution 4a can support inter-vendor collaboration Case y perfectly.

When considering CP solutions and choosing between gNB and CN/LMF/OAM, the key factor to consider is the support for mobility/continuity. If the UE is more static than mobile, then choose gNB. Otherwise, choose CN/LMF/OAM. Another consideration is that gNB is more suitable for cell-specific models while CN/LMF/OAM may be more suitable for generalized models that can be transferred to multiple cells, which can be in different types (e.g., macro vs micro, indoor vs outdoor).

When choosing between OTT solutions (4a) and CP solutions, we think the key factor to look at can be model size. Based on company reports, we see model size can be either greater than 45KB or smaller than 45KB. Therefore, we support the use of both OTT and CP solutions (1a, 2a, 3a, 4b-CP).

- When the model size is greater than 45KB, use Solution 4a.

- When the model size is smaller than 45KB, use CP solutions (1a, 2a, 3a, 4b-IP).

Based on the above analysis, we have the following proposals.

Proposal 1: Deprioritized UP-based solutions 1b, 2b, 3b and 4b-IP tunnel as they are more complicated than Solution 4a (the OTT approach) but provide similar level of service.

Proposal 2: Prioritize Solution 4a as the baseline for model transfer/delivery. When Solution 4a is not suitable or satisfactory, use CP-based solutions as the model transfer mechanisms.

Proposal 3: Prioritize gNB/CP-based solutions for low-mobility UEs, and for cell-specific and/or small-size (<45KB) models.

Proposal 4: Prioritize CN/LMF/OAM and CP-based solutions for high-mobility UEs, and for big-size and/or generalized models.

# Conclusions

In this contribution, we presented our observations and views on topics related to model transfer/delivery. Based on the discussions in the previous sections, our proposals are as follows.

Proposal 1: Deprioritized UP-based solutions 1b, 2b, 3b and 4b-IP tunnel as they are more complicated than Solution 4a (the OTT approach) but provide similar level of service.

Proposal 2: Prioritize Solution 4a as the baseline for model transfer/delivery. When Solution 4a is not suitable or satisfactory, use CP-based solutions as the model transfer mechanisms.

Proposal 3: Prioritize gNB/CP-based solutions for low-mobility UEs, and for cell-specific and/or small-size (<45KB) models.

Proposal 4: Prioritize CN/LMF/OAM and CP-based solutions for high-mobility UEs, and for big-size and/or generalized models.

# References

RP-233946, 3GPP TR 38.843, Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18) v2.0.1 (2023-12)

Chair’s notes, RAN1 Meeting #116 eom, Feb/Mar 2024

Chair’s notes, RAN1 Meeting #116bis eom, Apr 2024