3GPP TSG RAN WG2 Meeting #127bis	R2- 2409071

Hefei, China, 14th - 18th Oct. 2024

Agenda Item:	8.8.5

Source:	ETRI

Title:		Discussion on support of regenerative payload

Document for:	Discussion and Decision

1	Introduction

In the previous RAN2 meeting #127, the following agreements were made for support of regenerative payload in NR NTN [1].

Agreements:

1.	RAN2 confirms the understanding that legacy UEs should be supported in a regenerative payload scenario.

2.	Send an LS to RAN1 and RAN4 asking whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for ta-Common or whether we should e.g. introduce negative values for ta-Common. Also indicate that in any case legacy UEs would have to rely on existing signalling and then on a minimum value equal to 0.

3.	RAN2 understanding is that for regenerative payload existing RACH-less HO could be applied directly, no further Uu enhancement is needed.

4.	We do not optimize UE power saving for SDT procedure in regenerative payload architecture

In this paper, we provide our views on the support of regenerative payload.

2	Discussion

2.1	Common TA

In the last meeting, introduction of negative ta-Common-r17 was proposed [2] and RAN2 made following agreements about the proposal:

1.	RAN2 confirms the understanding that legacy UEs should be supported in a regenerative payload scenario.

2.	Send an LS to RAN1 and RAN4 asking whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for ta-Common or whether we should e.g. introduce negative values for ta-Common. Also indicate that in any case legacy UEs would have to rely on existing signalling and then on a minimum value equal to 0.

From our perspective, the first agreement for support of legacy UEs contradicts introduction of negative ta-Common-r17. As specified in TS 38.331, ta-Common-r17 has a non-negative integer value (0..66485757) with a granularity 4.072∙10-3 μs. Our understanding is that the introduction of the negative ta-Common-r17 cannot avoid changing the definition of ta-Common-r17. That is, the way a UE interprets the value of ta-Common-r17 must be changed if the negative ta-Common-r17 is introduced. The change would be possible for Rel-19 UEs. However, for legacy UEs, the change will result in invalid common TA at the UE, which is not intended by NW.

Support of legacy UEs contradicts introduction of negative ta-Common-r17.

Based on the above observation, it is better to find another way other than the introduction of the negative ta-Common-r17, if non-negative ta-Common-r17 is finally turned out to be a problem to be solved. The motivation for the proposal of the negative ta-Common-r17 was TA error that may occur in TA pre-compensation by a UE based on NTN assistance information and UE’s GNSS. The error may occur due to various reasons: outdated GNSS information, inaccuracy in ephemeris information, and approximation error inherent in the calculation formula of NTA,adjcommon [3, Sec. 4.2] using ta-Common-r17, ta-CommonDrift-r17, and ta-CommonDriftVarfiant-r17 (the formula is basically Taylor approximation). Therefore, instead of concentrating on common TA (i.e., ta-Common-r17), it is strongly motivated to introduce a new parameter that can accommodate all these error factors.

Rather than introducing negative ta-Common, introduce a new parameter that can cope with overestimation or underestimation of TA by a UE

2.2	Impacts of the regenerative payload on Rel-18 time-based measurement initiation for feeder link switch

In Rel-18, to address feeder-link switch in NTN Earth moving system as well as quasi-Earth fixed system, time-based measurement initiation is used with the reused t-Service. According to Rel-18 spec., a UE in either an Earth moving cell or a quasi-Earth fixed cell shall perform intra-frequency, inter-frequency, or inter-RAT measurements before t-Service which is the time information on when the cell is going to stop serving the UE due to feeder link switch.

Figure 1. Example feeder link switch for a satellite with a regenerative payload in an Earth-moving cell

With a transparent payload, the serving gNB is always changed after feeder link switch for a satellite with a transparent payload. On the other hand, with a regenerative payload in Rel-19, the serving gNB is not changed after the feeder link switch as an example shown in Figure 1. This is because the regenerative payload plays role of a full-function gNB(s).

With the regenerative payload in Rel-19, the serving gNB is not changed after the feeder link switch.

This observation clearly indicates that the Rel-19 regenerative payload has impact on the existing Rel-18 time-based measurement initiation for feeder link switch. The motivation for the Rel-18 time-based measurement is for a UE to have enough time in finding the best cell to camp on before feeder link switch. This is because the serving cell abruptly stops serving the area covered by it after the switch. However, with Rel-19 regenerative payload, the serving cell keep serving the area after feeder link switch. Consequently, the Rel-18 time-based measurement initiation for feeder link switch is not needed for the case of Rel-19 regenerative payload.

With the regenerative payload in Rel-19, time-based measurement initiation for feeder link switch is not necessary.

Based on above observations, we propose the following:

Time-based measurement initiation for feeder link switch shall not be configured for regenerative payload.

3	Conclusion

In this paper, the following observations are made:

Observation 1	Support of legacy UEs contradicts introduction of negative ta-Common-r17.

Observation 2	With the regenerative payload in Rel-19, the serving gNB is not changed after the feeder link switch.

Observation 3	With the regenerative payload in Rel-19, time-based measurement initiation for feeder link switch is not necessary.

In this paper, we propose the following:

Proposal 1	Rather than introducing negative ta-Common, introduce a new parameter that can cope with overestimation or underestimation of TA by a UE

Proposal 2	Time-based measurement initiation for feeder link switch shall not be configured for regenerative payload.

References

R2-2407573, Report from session on NR NTN and IoT NTN, Session chair (ZTE), RAN2 #127, Aug. 2024.

R2-2406972, Considerations on regenerative payload, CMCC, RAN2 #127, Aug. 2024.

TS 38.213, NR; Physical layer procedures for control