3GPP TSG-RAN WG2 Meeting #127bis		                               R2-2409084

Hefei, China, 14-18 October, 2024

Agenda Item:		8.7.6

Source:			MediaTek Inc.

Title:				Uplink congestion control signalling

Document:			Discussion and decision.

### 1	Introduction

This document discusses the new objective: uplink congestion control in R19 XR WID [1].

-	Specify uplink congestion signalling [RAN2]:

-	Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

### 2	Discussion

In RAN plenary #105 meeting [2], companies bring a joint contribution to elaborate that RAN-Aware XR Rate Control is important to XR user experience. Compare to packet drop due to congestion problem which leads to broken image, reducing the data rate (lower resolution but not broken image) has lower negative impact to user experience. Therefore, to know whether there is congestion along the path to destination or not is important for XR services.

In MAC spec, we already have Recommended bit rate MAC CE to indicate the data rate, and its 6 bits bit rate range and bit rate multiplier (X) is enough to cover the maximum XR data rate defined in TR 38.838 [3].

Proposal 1: RAN2 agree to use Recommended bit rate MAC CE as a baseline for uplink congestion signalling.

However, the granularity of current Recommended bit rate MAC CE is per logical channel, which is not finer enough to support XR services, which might have multiple QoS mapped into the same DRB/logical. Therefore, it is better to use QoS flow ID rather than logical channel ID.

Proposal 2: To achieve finer granularity congestion control, RAN2 to agree to the congestion control signalling is per QoS flow.

Proposal 3: RAN2 to define a new MAC CE for congestion signalling.

- Its format is similar to Recommended bit rate MAC CE.

- The new MAC CE includes QoS Flow ID not LCID.

### 3	Conclusion

This document promulgated the following proposals:

Proposal 1: RAN2 agree to use Recommended bit rate MAC CE as a baseline for uplink congestion signalling.

Proposal 2: To achieve finer granularity congestion control, RAN2 to agree to the congestion control signalling is per QoS flow.

Proposal 3: RAN2 to define a new MAC CE for congestion signalling.

- Its format is similar to Recommended bit rate MAC CE.

- The new MAC CE includes QoS Flow ID not LCID.

### 4	Reference

[1]	RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur).

[2]       RP-242224, On RAN Awareness and UL Rate Control for XR, Meta, CATT, CMCC, Google, vivo, ZTE Corporation, Sanechips, Huawei, HiSilicon, Spreadtrum, Samsung, Apple, MediaTek Inc., Futurewei

[3]       TR 38.838, Study on XR (Extended Reality) evaluations for NR.