3GPP TSG-RAN WG2 Meeting #127bis	R2-2409086

Hefei, China, 14 – 18 October 2024

Agenda item:	8.5.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On-demand SSB SCell operation

WID/SID:	Netw_Energy_NR - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we will discuss the aspects related to the on-demand SSB SCell operation which is one of the objectives of the R19-network energy savings work item

# 2	On-demand SSB SCell Operation

## 2.1	Scenarios and signalling

RAN1 has made following agreements regarding on-demand SSB:

Agreement For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable Agreement For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE FFS: Whether the RRC is newly introduced or existing RRC is reused

Based on the agreements in RAN1#117, RRC and MAC-CE based signaling have been agreed to indicate on-demand SSB transmission. Our current understanding is these signaling are applicable to Scenario#2, Scenario#2A as depicted in the figure 1 below, where:

- For Scenario#2: the on-demand SSB transmission can be indicated by gNB at the time instance T1 (when UE receives SCell configuration via RRC message) or during the time period between T1 and T2. From RAN1 discussion it is not clear whether this would include scenario where SCell is directly activated when configured.

- For Scenario#2A: the on-demand SSB transmission can be indicated by gNB at the time instance T2 (when UE receives SCell activation via MAC CE command). From RAN1 discussion it is not clear whether this would include scenario where SCell is directly activated when configured.

For the RAN2 discussion we prefer to consider Scenario#2 and Scenario#2A for both Case#1(without always-on SSB) and Case#2(with always-on SSB).

Figure 1: Scenarios and indication for on-demand SSB transmission.

RAN2 could possibly start the discussion on how the on-demand SSB configurations that can be provided to the UE to support these scenarios.

Observation 1: During the RAN1#117 meeting, both RRC and MAC-CE based signaling have been agreed to indicate on-demand SSB transmission at least for Scenario#2 and Scenario#2A for both Case#1 and Case#2.

Agreement For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE - FFS: Whether the RRC is newly introduced or existing RRC is reused

During the RAN1#117, it has also been agreed to provide the on-demand SSB configuration via RRC signaling and may include at least the parameters below

- Frequency of the on-demand SSB

- SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst

- Periodicity of the on-demand SSB

Also, there are more than one SSB transmission patterns supported in RAN1 as listed below

- Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A.

- Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB

- Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B.

- Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted.

- Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.

Depending on the SSB pattern to be transmitted, we believe the configurations may have to include additional parameters for example,

- The periodicity parameter is required for all the on-demand transmission options.

- Number of SSB bursts is required for the Option 3

- time duration for which on-demand SSB are transmitted for Option 2 and Option 4

- alternate or a second periodicity for Option 4.

- and may also require a corresponding SMTC configuration.

Some of the patterns for on-demand SSB may be as indicated in the Figure 2.

Figure 2: On-demand SSB transmission patterns.

In general, on-demand SSB configuration may be provided via the Serving Cell Configuration. As per our understanding, the set of parameters required for the on-demand SSB operation may be kept minimal. As indicated in Figure 3 two options may be considered.

- the minimal set consisting of mandatory parameters for OD-SSB operation that can work on top of the legacy SSB configuration.

- the full set of parameters for OD-SSB operation that would replace the legacy SSB configuration.

Figure 3: On-demand SSB configuration structure

Proposal 1 : RAN2 to discuss the on-demand SSB configuration and consider the possibility of optimizing the configuration. Discuss the possibility of defining a minimal set for OD-SSB operation that may work on top of the legacy configuration or a full set of parameter for OD-SSB operation that replace the legacy configuration.

It may be beneficial to configure one or more patterns for on-demand SSB.

SSB-Pattern-Config :: SEQUENCE {

pattern-id INTEGER (1..M)

ssb-Periodicity-r19 ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1},

activeDuration ENUMERATED {sf5, sf10, sf20, sf40, sf80, sf160, sf320, sf640} OPTIONAL,

numberOfBurst INTEGER (1..N)

ssb-Frequency ARFCN-ValueNR,

subcarrierSpacing SubcarrierSpacing,

ssb-PositionsInBurst CHOICE {

shortBitmap BIT STRING (SIZE (4)),

medium Bitmap BIT STRING (SIZE (8)),

longBitmap BIT STRING (SIZE (64))

} OPTIONAL,

ss-PBCH-BlockPower INTEGER (-60..50) OPTIONAL,

}

From previous RAN2 discussions, we have the following open issues to be addressed:

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

On the open issue of whether to limit the activation/deactivation state indication in RRC only for initial configuration, we do not see any motivation to disallow changing the state with reconfiguration since the bit is anyway there.

Proposal 2: No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration.

For MAC CE design, RAN1 has agreed to support OD-SSB activation before SCell activation as well as upon SCell activation and also the possibility of configuring multiple candidate values at least for periodicity of the OD-SSB, but whether also for any other parameters is still FFS.

From RAN2 signalling point of view, the simplest way would be to indicate in the MAC CE the SSB configuration IDs configured in RRC and the RRC configuration can accommodate any parameter that RAN1 concludes to support multiple candidate values.

Proposal 3: The different periodicities are configured as different OD-SSB configurations in RRC and MAC indicates which OD-SSB configuration ID to be activated, which is forward compatible if other parameters also introduced in RAN1.

Proposal 4: The new MAC-CE for OD-SSB includes a bitmap for the SCell(s) with OD-SSB to be activated and the indication of the activated SSB configuration ID for each SCell.

Wrt. whether to optimize the case of OD-SSB activation also upon SCell activation, it depends on whether it is always the case as long as OD-SSB is configured for the SCell, since otherwise separate bitmap might need to be defined for OD-SSB activation and SCell activation which would be quite close to having separate MAC CEs.

Proposal 5: Joint MAC CE for OD-SSB activation and SCell activation can be considered on top only if OD-SSB is always considered as activated (if configured for the SCell) upon SCell activation without separate bitmap.

On L3 measurements based on on-demand SSB, RAN1 thinks that RAN2 should lead the discussions. Our understanding is that on-demand SSB based L3 measurements and reporting may depend on the on-demand SSB pattern.

It is required to align the SMTC window with the on-demand SSB transmission. The signaling used for on-demand SSB indication may provide the reference for the STMC. E.g time T slots after which SSB transmission may start would have impact to the SMTC window. The SMTC configuration may be explicitly configured or derived based on the on-demand SSB configuration.

Figure 4: on-demand SSB and SMTC

Proposal 6: RAN2 to discuss the configurations for SMTC along with the on-demand SSB configuration.

During the RAN1#118 meeting it has been agreed that L1 measurement based on on-demand SSB may be supported. The L1 measurements are configured using the CSI-reporting framework where the resources to be measured are included. The UE may be configured with CSI-ResourceConfig that contains,

- A list of NZP CSI-RS resource set and/or SSB resource set provided by nzp-CSI-RS-ResourceSetList and/or csi-SSB-ResourceSetList

- Or a list of CSI-IM resource set provided by csi-IM-ResourceSetList

The SSB resources are provided as the SSB-Index that maps to a SSB configuration.

CSI-ResourceConfig ::=      SEQUENCE {

csi-ResourceConfigId        CSI-ResourceConfigId,

csi-RS-ResourceSetList      CHOICE {

nzp-CSI-RS-SSB              SEQUENCE {

nzp-CSI-RS-ResourceSetList  SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL, -- Need R

csi-SSB-ResourceSetList     SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSetsPerConfig)) OF CSI-SSB-ResourceSetId  OPTIONAL  -- Need R

},

csi-IM-ResourceSetList      SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId

},

bwp-Id                      BWP-Id,

resourceType                ENUMERATED { aperiodic, semiPersistent, periodic },

...,

[[

csi-SSB-ResourceSetListExt-r17      CSI-SSB-ResourceSetId                                                  OPTIONAL  -- Need R

]]

}

CSI-SSB-ResourceSet ::=             SEQUENCE {

csi-SSB-ResourceSetId               CSI-SSB-ResourceSetId,

csi-SSB-ResourceList                SEQUENCE (SIZE(1..maxNrofCSI-SSB-ResourcePerSet)) OF SSB-Index,

...,

[[

servingAdditionalPCIList-r17        SEQUENCE (SIZE(1..maxNrofCSI-SSB-ResourcePerSet)) OF ServingAdditionalPCIIndex-r17  OPTIONAL  -- Need R

]]

}

ServingAdditionalPCIIndex-r17  ::=  INTEGER(0..maxNrofAdditionalPCI-r17)

It is uncertain whether UE would measure single SSB(e.g always-on) or multiple SSBs(on-demand SSB and always-on SSB) when on-demand SSB based L1 measurements enabled. This could affect the current CSI reporting framework and needs to be discussed in RAN1 before RAN2 can consider signaling enhancements.

Proposal-7: RAN2 to wait for RAN1 decision in the signaling enhancement needed for the CSI-Reporting framework.

# 3	Conclusion

Observation 1: During the RAN1#117 meeting, both RRC and MAC-CE based signaling have been agreed to indicate on-demand SSB transmission at least for Scenario#2 and Scenario#2A for both Case#1 and Case#2.

Proposal 1 : RAN2 to discuss the on-demand SSB configuration and consider the possibility of optimizing the configuration. Discuss the possibility of defining a minimal set for OD-SSB operation that may work on top of the legacy configuration or a full set of parameter for OD-SSB operation that replace the legacy configuration.

Proposal 2: No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration.

Proposal 3: The different periodicities are configured as different OD-SSB configurations in RRC and MAC indicates which OD-SSB configuration ID to be activated, which is forward compatible if other parameters also introduced in RAN1.

Proposal 4: The new MAC-CE for OD-SSB includes a bitmap for the SCell(s) with OD-SSB to be activated and the indication of the activated SSB configuration ID for each SCell.

Proposal 5: Joint MAC CE for OD-SSB activation and SCell activation can be considered on top only if OD-SSB is always considered as activated (if configured for the SCell) upon SCell activation without separate bitmap.

Proposal 6: RAN2 to discuss the configurations for SMTC along with the on-demand SSB configuration.

Proposal-7: RAN2 to wait for RAN1 decision in the signaling enhancement needed for the CSI-Reporting framework.