3GPP TSG-RAN WG2 Meeting #127bis	R2-2409088

Hefei, China, 14 – 18 October 2024

Agenda item:	8.5.4

Source:	Nokia, Nokia Shanghai Bell

Title:	On Adaptation of common signal/channel transmissions for NES

WID/SID:	Netw_Energy_NR_enh

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we discuss aspects related to the potential adaptations of the common signal/channel transmissions.

# 2	Adaptation of SSB in time domain

During the previous RAN2 meeting, there were no agreements on this topic. Pending agreements from the RAN1#118 meeting are

Agreement​ For adaptation mechanism(s) of SSB in time-domain,​ - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported ​ - FFS: Adaptation for SSB that is not CD-SSB is supported (A2)​ - FFS: Adaptation for SSB not on sync raster is supported (A3)​ - For Rel-19 NES-capable UE’s SCell​ - Adaptation of SSB configured for the SCell is supported for the following cases​ - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell​ - Adaptation for SSB that is not CD-SSB on sync raster (B2’)​ - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)​ Agreement​ - For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select from following options​ - Option 1: One SSB burst periodicity is configured for the UE and UEs assumes SSB transmissions are not present during Cell DTX non-active period ​ - Option 2: UE assumes SSB transmission with different periodicities during Cell DTX non-active period and during Cell DTX active period ​ - Option 3: Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior) – no spec impact​

In our view, RAN1 should clarify first for which scenarios adaptation of SSB burst periodicity can be considered as useful to achieve network energy saving (NES) for a cell with Rel-19 NES-capable UE in connected mode. So far, it is still unclear what are the targeted scenarios. RAN2 can then provide input.

Proposal 1: RAN2 to wait for RAN1 to clarify scenarios for SSB adaptation in the time domain network energy saving can be achieved before providing input.

# 3		PRACH Adaptation in time domain

As per the latest RAN1#117 meeting outcome, it was agreed to support at least the PRACH adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any). It was also noted that NES-capable UEs can use both additional PRACH resources and PRACH resources configured for legacy UEs.

In our view, the agreed PRACH adaptation based on additional PRACH resources should be applied for both 4-step RACH and 2-step RACH (if configured in the cell). Particularly for 2-step RACH, further clarifications and considerations would be needed. For instance, it is not clear whether additional PUSCH resources for NES-capable shall be considered for MsgA payload transmission or the same PUSCH as configured for legacy UEs shall be considered.

Proposal 2: Clarify whether further considerations are needed for the 2-step PRACH adaptation e.g., the resource used by the UE to transmit MsgA.

There are two types of PRACH that can be configured:

- PRACH associated with CBRA configured by cell-specific signaling.

- PRACH associated with CFRA configured by UE-dedicated signaling.

The agreed PRACH adaptation shall at least target CBRA procedures. PRACH adaptation associated with CFRA procedures shall be studied as well, if time allows.

Proposal 3: Study at least PRACH adaptation solution(s) that are applied to CBRA. PRACH adaptation applied to CFRA shall also be investigated, if time allows, however no CFRA-specific PRACH adaptation solutions should be considered.

The likelihood that cells are designed to only serve R19 UEs is low; thus, the PRACH adaptation should be applicable to cells that support both legacy and Rel-19 (NES-capable) UEs. Moreover, in order to avoid any impact on legacy UEs as noted in the WID, and to enable the initial access for all UEs before being able to identify whether they are legacy or NES-capable UEs, the cell should support both legacy and NES-capable UEs.

Proposal 4: PRACH adaptation should be applicable to cells that support both legacy and Rel-19 UEs.

The configuration of a PRACH configuration index for the additional PRACH resources different from the PRACH configuration index for the legacy resource offers more flexibility to the NW and can help accommodating scheduler needs. However, the possible overlaps between sub-sets of ROs would need to be further studied in this case, which is under discussion in RAN1.

The configuration of the PRACH configuration index for the additional PRACH resources as the legacy resource allows the network energy consumption for PRACH reception to occur mainly (or only) during system frames where legacy ROs are located and, at the same time, ensures that PRACH resources are not overlapping in time and/or frequency, but we should wait for RAN1 progress on this before discussing RAN2 implications.

Proposal 5: RAN2 to wait for RAN1 to conclude on PRACH adaptation mechanisms before discussing RAN2 implications.

One potential implication on RAN2 could be on the Conditional handover (CHO) procedure as the NW does not know the time instant when UE initiates CHO and the UE can’t be aware in advance about the potential activation/deactivation of additional PRACH resources at one or more potential target cell(s). Hence, further studies shall be conducted in order to ensure that the UE and the NW are perfectly aligned on the usage of additional PRACH resources, even during the CHO procedure.

Proposal 6: Study whether further enhancements are needed for the handover procedure when the additional PRACH resources are activated at one or more candidate target cells.

Another implication on RAN2 will be the beam failure recovery configuration as for beam failure recovery RAN2 needs to discuss further how additional PRACH resources can be used for beam failure recovery.

Proposal 7: RAN2 to consider how to enable usage of additional PRACH resources for beam failure recovery procedure.

Additionally RAN2 should continue studying more details regarding configuration e.g. do we need different signaling for RRC_IDLE/INACTIVE and RRC_CONNECTED UEs.. It is not yet clear whether parameter sets are exactly same for all the states so it is difficult yet to continue detailed discussions but unless otherwise indicated by RAN1 we assume same signaling structure can be used in all the states (e.g. same IE signalled in SIB and dedicated signaling.)

Proposal 8: RAN2 needs to wait for more RAN1 inputs before we can progress detailed signaling of PRACH adaptation

# Adaptation of paging occasions

After RAN2#127 we have had email discussion comparing different options for realizing paging bundling – it was generally understood that both options can achieve similar performance and option B would be simpler in 38.304 as we don’t need to touch paging frame calculation formulas. But what has not been identified in the email discussion is possible overhead of different options. Thus we propose before making final decision we should also see whether there are significant differences in the overhead. If option B required excessively more signaling then we would still consider option A as good way forward even though it is more complex to realize in the idle mode specification.

Proposal 9: Before making final decision on the paging bundling option RAN2 should consider also potential ASN.1 overhead

## PRACH Adaptation in time domain in conjunction with adaptation of paging occasions

The PRACH load level will inherently increase if the paging occasions are confined in time domain. This is because the UEs will have to be paged during such confined time period, thereby causing the UEs to respond to the paging also in a confined time interval. So it should be possible to temporarily increase the amount of PRACH resources in order to accommodate more clustered paging responses from UEs. The legacy approach of configuring a high amount/frequently occurring PRACH resources will impact the network’s resource availability and RO monitoring energy consumption.

Proposal 10: PRACH adaptation in time domain is supported to temporarily increase the PRACH capacity to be aligned with clustered paging and spread the PRACH load when paging occasion adaptation is used.

# Conclusion

Proposal 1: RAN2 to wait for RAN1 to clarify scenarios for SSB adaptation in the time domain network energy saving can be achieved before providing input.

Proposal 2: Clarify whether further considerations are needed for the 2-step PRACH adaptation e.g., the resource used by the UE to transmit MsgA.

Proposal 3: Study at least PRACH adaptation solution(s) that are applied to CBRA. PRACH adaptation applied to CFRA shall also be investigated, if time allows, however no CFRA-specific PRACH adaptation solutions should be considered.

Proposal 4: PRACH adaptation should be applicable to cells that support both legacy and Rel-19 UEs.

Proposal 5: RAN2 to wait for RAN1 to conclude on PRACH adaptation mechanisms before discussing RAN2

implications.

Proposal 6: Study whether further enhancements are needed for the handover procedure when the additional PRACH resources are activated at one or more candidate target cells.

Proposal 7: RAN2 to consider how to enable usage of additional PRACH resources for beam failure recovery procedure.

Proposal 8: RAN2 needs to wait for more RAN1 inputs before we can progress detailed signaling of PRACH adaptation

Proposal 9: Before making final decision on the paging bundling option RAN2 should consider also potential ASN.1 overhead

Proposal 10: PRACH adaptation in time domain is supported to temporarily increase the PRACH capacity to be aligned with clustered paging and spread the PRACH load when paging occasion adaptation is used.