3GPP TSG-RAN WG2 Meeting #127bis	R2-2409089

Hefei, China, 14 – 18 October 2024

Agenda item:	8.11.3

Source:	Nokia

Title:	Other Aspects of SBFD

WID/SID:	NR_duplex_evo-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN#102 meeting, a Work Item (WI) on Evolution of NR duplex operation: Subband full duplex (SBFD) was approved [1]. The work item description has been updated in RAN#104 meeting and contain the following objectives [RP-241614]:

For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of time location of SBFD subbands in SIB is not precluded - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of frequency domain location of SBFD subbands in SIB is not precluded - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2] - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2] - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon - UL transmissions within UL subband only - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols - Enhancement on resource allocation in frequency domain in SBFD symbols, including - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol - Followings are assumed based on TR 38.858 - SBFD at the gNB side - Half duplex operation at the UE side - FR1 and FR2-1 - SBFD operation Option 4, i.e., both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs - Coexistence between non-SBFD aware UEs (including legacy UEs) and SBFD aware UEs in the cell operating SBFD at gNB side - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies - One UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol/slot) within a TDD carrier - Mechanisms for SBFD operation shall also consider the adjacent channel coexistence between two operators - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4] - Note: RAN3 will not specify enhancements to network signalling to support inter-operator coordination for CLI handling

In this contribution we provide our view on relevant RAN2-aspects according to the above objectives regarding need for reselection control for SBFD supporting UEs We start with a general overview of SBFD in Section 2, taking into account the agreements reached during the RAN1 Rel-18 study item phase and Rel-19 RAN1 normative work so far, followed by more detailed discussion

# 2	SBFD signalling aspects

The following was agreed in RAN2#127:

Cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signalling to covey cell specific configuration). FFS on UE specific dedicated RRC configuration if needed, pending on RAN1 progress.

Meanwhile, RAN1 agreed the following regarding the signaling of time- and frequency resource configuration:

RAN1#117: Agreement For cell-specific indication of SBFD subband frequency location, adopt the following option. - Option 1: Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s). RAN1#118: Agreement For configuration of SBFD symbols within a TDD-UL-DL pattern period, the following parameters are supported - A starting slot index - A starting symbol index within the starting slot - An ending slot index - An ending symbol index within the ending slot Working Assumption For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. - One or two SBFD DL subbands can be configured

So based on the decision in RAN2 and RAN1 we should add signaling of SBFD symbols within TDD-UL-DL pattern configuration in SIB1 – and this will naturally mean that also the corresponding information elements are included in the “common” signaling part of dedicated signaling to allow RRC_CONNECTED mode UEs to be signalled SBFD parameters without requirement of reading SIB1 before utilizing RACH.

Observation 1: RAN2 needs to add signaling of SBFD parameters (up to RAN1) within SIB1 which implicitly also means they should be added to ServingCellConfigCommon

Whether one needs to signal these parameters in UE specific manner is up to RAN1 to decide and RAN2 does not need to react on this aspect until we get parameter list(s) from RAN1

Proposal 1: Regarding UE specific SBFD parameter signaling RAN2 should wait RAN1 (and possibly RAN4 need to give input as well) to confirm whether they are needed or not

# 3	Cross-link interference handling

There were various papers in RAN2#127 regarding CLI measurements/reporting and SBFD. This topic is in our view very much RAN1 issue and RAN2 should just wait for RAN1 to conclude on this regarding what new (if any) parameters are required.

Proposal 2: RAN2 can wait before discussing further details until we get more exact parameter lists from RAN1/RAN4 regarding detailed CLI and SBFD parameters.

# 4	Mobility and cell-selection aspects

## 4.1	Cell selection prioritization

In scenarios with a mix of SBFD- and non-SBFD cells, while there is no need to fully prevent legacy UEs to connect to SBFD cells, it is desirable that SBFD-aware UEs prioritize SBFD cells during cell (re-)selection over cells not supporting SBFD. Similarly, legacy UEs may get better performance if they are served by non-SBFD cells. For this reason, we think at least the knowledge on whether the cell supports or not SBFD operation should be available to the SBFD-aware UEs for cell (re-)selection purposes.

Observation 2: RAN1 has concluded that non-SBFD aware UEs, including legacy UEs, and SBFD aware UEs can coexist in cells with SBFD operation at gNB side. Nevertheless, only SBFD-aware UEs may get the coverage and latency benefit of SBFD, while legacy UEs may not experience any performance improvement or may even be subject to a performance loss instead.

Proposal 3: Support SBFD-aware UEs to prioritize SBFD cells over non-SBFD cells.

## 4.2	Mobility Measurements

Mobility measurements, e.g. RSRP and RSSQ, can be based on SBFD or CSI-RS signals. CSI-RS are impacted by SBFD operation as noted in the following agreements made by RAN1:

RAN1#117: Working Assumption For frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, support one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL usable PRBs. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured) - RAN1 to further study the impact on CSI processing timeline in SBFD symbols to process the CSI-RS across the two DL subbands Agreement For a contiguous CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS frequency resources within DL usable PRBs are valid for SBFD-aware. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured)

Essentially, the above agreements mean that CSI-RS configuration for the UE follows the legacy way (start and end RB), but only the CSI-RS resources in the DL subband are measured by the UE (and used to derive an eventual measurement report). The CSI-RS resources in the UL subband and guardbands are assumed not to be present, i.e. “punctured” by the gNB. This is illustrated in Figure 1.

While the UE knows the position of the SBFD subbands for the serving cell, this information is not available for the neighbouring cells. If the neighbour cell operates with SBFD and its CSI-RS transmission takes place in a SBFD symbol, the CSI-RS would be punctured and this needs to be known by the UE to ensure accurate e.g. RSRP or RSRQ measurement. RAN1 has not make any distinction between serving-cell or inter -cell CSI-RS based measurements, so it is assumed that RAN1 agreements apply to both cases.

Observation 3: According to RAN1 agreements, contiguous CSI-RS resources are configured to the UE, but only the CSI-RS resources in the DL subband are measured by the UE. The CSI-RS resources in the UL subband and guardbands are assumed not to be present, i.e. not transmitted by the gNB. RAN1 has not make any distinction between serving-cell or inter -cell CSI-RS based measurements.

Observation 4: For mobility measurements, if the neighbour cell operates with SBFD and its CSI-RS transmission takes place in a SBFD symbol, the CSI-RS would be punctured and this needs to be known by the UE to ensure accurate e.g. RSRP or RSRQ measurement.

Proposal 4: For inter-cell CSI-RS measurements, UE is provided with information of the SBFD configuration of neighbouring cells.

Figure 1: CSI-RS resources in SBFD symbols

# 5	Conclusion

Observation 1: RAN2 needs to add signaling of SBFD parameters (up to RAN1) within SIB1 which implicitly also means they should be added to ServingCellConfigCommon

Proposal 1: Regarding UE specific SBFD parameter signaling RAN2 should wait RAN1 (and possibly RAN4 need to give input as well) to confirm whether they are needed or not

Proposal 2: RAN2 can wait before discussing further details until we get more exact parameter lists from RAN1/RAN4 regarding detailed CLI and SBFD parameters.

Observation 2: RAN1 has concluded that non-SBFD aware UEs, including legacy UEs, and SBFD aware UEs can coexist in cells with SBFD operation at gNB side. Nevertheless, only SBFD-aware UEs may get the coverage and latency benefit of SBFD, while legacy UEs may not experience any performance improvement or may even be subject to a performance loss instead.

Proposal 3: Support SBFD-aware UEs to prioritize SBFD cells over non-SBFD cells.

Observation 3: According to RAN1 agreements, contiguous CSI-RS resources are configured to the UE, but only the CSI-RS resources in the DL subband are measured by the UE. The CSI-RS resources in the UL subband and guardbands are assumed not to be present, i.e. not transmitted by the gNB. RAN1 has not make any distinction between serving-cell or inter-cell CSI-RS based measurements.

Observation 4: For mobility measurements, if the neighbour cell operates with SBFD and its CSI-RS transmission takes place in a SBFD symbol, the CSI-RS would be punctured and this needs to be known by the UE to ensure accurate e.g. RSRP or RSRQ measurement.

Proposal 4: For inter-cell CSI-RS measurements, UE is provided with information of the SBFD configuration of neighbouring cells.