3GPP TSG-RAN2#127bis 	R2-2409092

Hefei, China, 14th – 18th Oct, 2024

Agenda item:		7.20.2 (NR_MIMO_evo_DL_UL-Core)

Source:	LG Electronics Inc.

Title: 	Remaining issue on STx2P PHR

Document for:	Discussion and Decision

1.	Introduction

In the last meeting, RAN2 discussed how to obtain Pcmax for STx2P PHR, i.e., whether to obtain two Pcmax values or one Pcmax value, but it was postponed.

In this contribution, we discuss how to obtain Pcmax values and how to determine PHR MAC CE format for STx2P.

2.	Discussion

In Rel-18, PHR MAC CE for STx2P is introduced. When PHR MAC CE for STx2P is generated, two Pcmax values can be included for a Serving Cell, otherwise one Pcmax value can be obtained.

Figure 6.1.3.82-1: Enhanced Multiple Entry PHR for multiple TRP STx2P MAC CE with the highest ServCellIndex of Serving Cell with configured uplink is less than 8

Meanwhile, how to obtain Pcmax value and how to determine PHR MAC CE format are specified as follows.

1> if the Power Headroom reporting procedure determines that at least one PHR has been triggered and not cancelled; and 1> if the allocated UL resources can accommodate the MAC CE for PHR which the MAC entity is configured to transmit, plus its subheader, as a result of LCP as defined in clause 5.4.3.1: 2> if multiplePHR with value true is configured: 3> for each activated Serving Cell with configured uplink associated with any MAC entity of which the active DL BWP is not dormant BWP; and 3> for each activated Serving Cell with configured uplink associated with E-UTRA MAC entity: … unnecessary part… 4> else (i.e. if this MAC entity is not configured with phr-AssumedPUSCH-Reporting): 5> if this MAC entity is configured with twoPHRMode and if this Serving Cell is configured with multipanelSchemeSDM or multipanelSchemeSFN: 6> obtain two values for the corresponding PCMAX,f,c,k fields from the physical layer. 6> if mpe-Reporting-FR2 is configured and this Serving Cell operates on FR2 and this Serving Cell is associated to this MAC entity: 7> obtain two values for the corresponding MPEk fields from the physical layer. 5> else: 6> if this MAC entity has UL resources allocated for transmission on this Serving Cell; or 6> if the other MAC entity, if configured, has UL resources allocated for transmission on this Serving Cell and phr-ModeOtherCG is set to real by upper layers: 7> obtain the value for the corresponding PCMAX,f,c field from the physical layer. 7> if mpe-Reporting-FR2 is configured and this Serving Cell operates on FR2 and this Serving Cell is associated to this MAC entity: 8> obtain the value for the corresponding MPE field from the physical layer. … unnecessary part… 3> else if this MAC entity is configured with twoPHRMode and any associated Serving Cell is configured with multipanelSchemeSDM or multipanelSchemeSFN: 4> instruct the Multiplexing and Assembly procedure to generate and transmit the Enhanced Multiple Entry PHR for multiple TRP STx2P MAC CE as defined in clause 6.1.3.82 based on the values reported by the physical layer. 3> else if this MAC entity is configured with twoPHRMode and any associated Serving Cell is configured with multiple TRP PUSCH repetition: 4> instruct the Multiplexing and Assembly procedure to generate and transmit the Enhanced Multiple Entry PHR for multiple TRP MAC CE as defined in clause 6.1.3.51 based on the values reported by the physical layer.

In [1], if this MAC entity transmitting PHR MAC CE is configured with twoPHRMode and if this Serving Cell associated with any MAC entity is configured with multipanelScheme, two Pcmax values are obtained for this Serving Cell.

In addition, if this MAC entity is configured with twoPHRMode and any associated Serving Cell is configured with multipanelScheme, PHR MAC CE for STx2P is generated.

This means, regardless of whether this MAC entity is configured with Rel-18 feature or not, as long as this MAC entity is configured with twoPHRMode and at least one Serving Cell in both MAC entities is configured with multipanelScheme, this MAC entity generates PHR MAC CE for STx2P and obtain two Pcmax values for the at least one Serving Cell.

However, there may be a case where the MAC entity transmitting PHR MAC CE is configured with twoPHRMode but is not configured for Rel-18 feature, i.e., the MAC entity is connected to a Rel-17 gNB, and a Serving Cell configured with multipanelScheme belongs to the other MAC entity configured with Rel-18 feature, i.e., the other MAC entity the Serving Cell belongs to is connected to a Rel-18 gNB.

Figure 1. Example of PHR MAC CE on Rel-17 MAC entity

One example is depicted in above figure. In DC case, if one gNB supports Rel-17 and the other gNB supports Rel-18, one MAC entity is configured with Rel-17 feature and the other MAC entity is configured with Rel-18 feature.

According to the current specification, when the MAC entity configured with Rel-17 feature generates PHR MAC CE, the MAC entity is allowed to generate Rel-18 PHR MAC CE for STx2P and to obtain two Pcmax values for Cell#3 and Cell#4 even if Cell#3 and Cell#4 belongs to the other MAC entity.

However, it does not make sense that the MAC entity configured with Rel-17 feature generates Rel-18 PHR MAC CE for STx2P because the Rel-17 gNB cannot decode Rel-18 PHR MAC CE.

Observation 1. According to the current specification, even if the MAC entity is connected to a Rel-17 gNB, the MAC entity generates Rel-18 PHR MAC CE (i.e. with two Pcmax values for STx2P).

Therefore, the MAC entity configured with Rel-17 feature should not be allowed to generate Rel-18 PHR MAC CE and to obtain two Pcmax values, and only the MAC entity configured with Rel-18 multipanelScheme should be allowed to generate Rel-18 PHR MAC CE for STx2P and to obtain two Pcmax values.

In other words, the MAC entity should be allowed to generate Rel-18 PHR MAC CE for STx2P only when the MAC entity is connected to a Rel-18 gNB supporting multipanelScheme feature.

Proposal 1. The MAC entity should be allowed to generate Rel-18 PHR MAC CE (i.e. with two Pcmax values for STx2P) only when the MAC entity is connected to a Rel-18 gNB supporting multipanelScheme feature.

Based on P1, the UE behaviour can be presented as follows:

- If the MAC entity transmitting PHR MAC CE is configured with twoPHRMode and if the MAC entity is connected to a Rel-18 gNB supporting multipanelScheme feature:

  - If two Pcmaxs value are provided for this Serving Cell from the physical layer:

    - Obtain two Pcmax value.

  - Else (i.e. one Pcmax value is provided for this Serving Cell from the physical layer):

    - Obtain one Pcmax value.

  - Instruct to generate PHR MAC CE for STx2P

- Else (i.e. the MAC entity is not configured with twoPHRMode or the MAC entity is not connected to Rel-18 gNB supporting multipanelScheme feature):

  - Obtain one Pcmax value.

  - Instruct to generate PHR MAC CE depending on other features.

However, currently, it has not been defined whether the MAC entity is connected to Rel-18 gNB supporting multipanelScheme feature or not because multipanelScheme feature is configured per Serving Cell, not per MAC entity.

Thus, the definition should be defined first, and we think that whether the MAC entity is connected to Rel-18 gNB supporting multipanelScheme feature or not can be easily defined as whether at least one Serving Cell belonging to the MAC entity is configured with multipanelScheme.

Proposal 2. The MAC entity is considered to be connected to Rel-18 gNB supporting multipanelScheme feature if at least one Serving Cell belonging to the MAC entity is configured with multipanelScheme.

If P2 is agreeable, the UE behaviour for Pcmax and PHR MAC CE format is specified as follows:

- If the MAC entity transmitting PHR MAC CE is configured with twoPHRMode and if at least one Serving Cell belonging to the MAC entity is configured with multipanelScheme:

  - If two Pcmaxs value are provided for this Serving Cell from the physical layer:

    - Obtain two Pcmax value.

  - Else (i.e. one Pcmax value is provided for this Serving Cell from the physical layer):

    - Obtain one Pcmax value.

  - Instruct to generate PHR MAC CE for STx2P

- Else (i.e. the MAC entity is not configured with twoPHRMode or if there is no Serving Cell configured with multipanelScheme belonging to the MAC entity):

  - Obtain one Pcmax value.

  - Instruct to generate PHR MAC CE depending on other features.

Proposal 3. The UE behaviour for Pcmax and PHR MAC CE format is specified as follows.

- If the MAC entity transmitting PHR MAC CE is configured with twoPHRMode and if at least one Serving Cell belonging to the MAC entity is configured with multipanelScheme:

  - If two Pcmaxs value are provided for this Serving Cell from the physical layer:

    - Obtain two Pcmax value.

  - Else (i.e. one Pcmax value is provided for this Serving Cell from the physical layer):

    - Obtain one Pcmax value.

  - Instruct to generate PHR MAC CE for STx2P

- Else (i.e. the MAC entity is not configured with twoPHRMode or if there is no Serving Cell configured with multipanelScheme belonging to the MAC entity):

  - Obtain one Pcmax value.

  - Instruct to generate PHR MAC CE depending on other features.

Based on P1-P3, we provide TP in Annex.

Proposal 4. Adopt TP in Annex.

3.	Conclusion

In this document, we discuss how to obtain Pcmax values and how to determine PHR MAC CE format for STx2P, and make an observation and proposals as follows.

Observation 1. According to the current specification, even if the MAC entity is connected to a Rel-17 gNB, the MAC entity generates Rel-18 PHR MAC CE (i.e. with two Pcmax values for STx2P).

Proposal 1. The MAC entity should be allowed to generate Rel-18 PHR MAC CE (i.e. with two Pcmax values for STx2P) only when the MAC entity is connected to a Rel-18 gNB supporting multipanelScheme feature.

Proposal 2. The MAC entity is considered to be connected to Rel-18 gNB supporting multipanelScheme feature if at least one Serving Cell belonging to the MAC entity is configured with multipanelScheme.

Proposal 3. The UE behaviour for Pcmax and PHR MAC CE format is specified as follows.

- If the MAC entity transmitting PHR MAC CE is configured with twoPHRMode and if at least one Serving Cell belonging to the MAC entity is configured with multipanelScheme:

  - If two Pcmaxs value are provided for this Serving Cell from the physical layer:

    - Obtain two Pcmax value.

  - Else (i.e. one Pcmax value is provided for this Serving Cell from the physical layer):

    - Obtain one Pcmax value.

  - Instruct to generate PHR MAC CE for STx2P

- Else (i.e. the MAC entity is not configured with twoPHRMode or if there is no Serving Cell configured with multipanelScheme belonging to the MAC entity):

  - Obtain one Pcmax value.

  - Instruct to generate PHR MAC CE depending on other features.

Proposal 4. Adopt TP in Annex.

4.	Reference

- 3GPP TS 38.321	Medium Access Control (MAC) protocol specification	V18.3.0

5.	Annex

### 5.4.6	Power Headroom Reporting

…unnecessary part…

4>	else (i.e. if this MAC entity is not configured with phr-AssumedPUSCH-Reporting):

5>	if this MAC entity is configured with twoPHRMode and if at least one Serving Cell belonging to this MAC entity is configured with multipanelSchemeSDM or multipanelSchemeSFN:

6>	if this Serving Cell is configured with multipanelSchemeSDM or multipanelSchemeSFN and the MAC entity this Serving Cell belongs to is configured with twoPHRMode:

7>	obtain two values for the corresponding PCMAX,f,c,k fields from the physical layer.

7>	if mpe-Reporting-FR2 is configured and this Serving Cell operates on FR2 and this Serving Cell is associated to this MAC entity:

8>	obtain two values for the corresponding MPEk fields from the physical layer.

6>	else:

7>	obtain the value for the corresponding PCMAX,f,c field from the physical layer.

7>	if mpe-Reporting-FR2 is configured and this Serving Cell operates on FR2 and this Serving Cell is associated to this MAC entity:

8>	obtain the value for the corresponding MPE field from the physical layer.

5>	else:

6>	if this MAC entity has UL resources allocated for transmission on this Serving Cell; or

6>	if the other MAC entity, if configured, has UL resources allocated for transmission on this Serving Cell and phr-ModeOtherCG is set to real by upper layers:

7>	obtain the value for the corresponding PCMAX,f,c field from the physical layer.

7>	if mpe-Reporting-FR2 is configured and this Serving Cell operates on FR2 and this Serving Cell is associated to this MAC entity:

8>	obtain the value for the corresponding MPE field from the physical layer.

…unnecessary part…

3>	else if this MAC entity is configured with twoPHRMode and at least one Serving Cell belonging to this MAC entity is configured with multipanelSchemeSDM or multipanelSchemeSFN:

4>	instruct the Multiplexing and Assembly procedure to generate and transmit the Enhanced Multiple Entry PHR for multiple TRP STx2P MAC CE as defined in clause 6.1.3.82 based on the values reported by the physical layer.

3>	else if this MAC entity is configured with twoPHRMode and at least one Serving Cell belonging to this MAC entity is configured with multiple TRP PUSCH repetition:

4>	instruct the Multiplexing and Assembly procedure to generate and transmit the Enhanced Multiple Entry PHR for multiple TRP MAC CE as defined in clause 6.1.3.51 based on the values reported by the physical layer.

…unnecessary part…